3GPP TSG-RAN4 Meeting #93 	R4-1914738

# Reno, USA, November 18th – 22nd, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6726 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Cat-1bis clarification to RSRQ measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sequans Communications, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_UE_cat_1RX-Perf |  |  |  |  |  | Date: |  |  | 2019-11-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Table A.3.22.2-1 of 36.133 provides a RSRP TDD requirement referencing A.9.1.2. But in the test requirement sub-clause A.9.1.2.3 it does not provide a requirement section for Cat1bis. - Table A.3.22.2-1 references sections A.8.1.17 and A.8.1.18 which are currently "Void" - Title for A.9.7.2 is incomplete and is too general. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add section requirement for cat-1bis in A.9.1.2.3 - Correct Table A.3.22.2-1 refrencing - Add specifics for title in A.9.7.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - It is unclear the specification requirements for category cat-1bis. - Event triggered reporting is unclear. - It is not in the format the title of A.9.7.2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.9.1.2, A.3.22, A.9.7.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

### A.9.1.2	TDD Intra frequency case

#### A.9.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the RSRP measurement accuracy is within the specified limits. This test will verify the requirements in Sections 9.1.2.1 and 9.1.2.2 for TDD intra frequency measurements.

#### A.9.1.2.2	Test parameters

In this set of test cases all cells are on the same carrier frequency. Both absolute and relative accuracy of RSRP intra frequency measurements are tested by using the parameters in Table A.9.1.2.2-1. In all test cases, Cell 1 is the PCell and Cell 2 the target cell.

Table A.9.1.2.2-1: RSRP TDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Special subframe configurationNote1 |  |  | 6 |  | 6 |  | 6 |  |
| Uplink/downlink configurationNote1 |  |  | 1 |  | 1 |  | 1 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.3.1.1.2 |  |  | R.0 TDD | - | R.0 TDD | - | R.0 TDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 |  |  | R.6 TDD |  | R.6 TDD |  | R.6 TDD |  |
| OCNG Patterns defined in A.3.2.2.1 (OP.1 TDD) and A.3.2.2.2 (OP.2 TDD) |  |  | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD | OP.1 TDD | OP.2 TDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote2 |  |  |  |  |  |  |  |  |
| OCNG_RBNote2 |  |  |  |  |  |  |  |  |
| Note3 | Bands TDD_A | dBm/15 kHz | -106 | -106 | -88 | -88 | -116 |  |
|  | Bands TDD_C |  |  |  |  |  | -115 |  |
|  | Bands TDD_E |  |  |  |  |  | -114 |  |
|  |  | dB | 2.5 | -6 | 2.5 | -6 | 0.5 | -5.76 |
| RSRPNote4 | Bands TDD_A | dBm/15 kHz | -100 | -105 | -82 | -87 | -113 | -117 |
|  | Bands TDD_C |  |  |  |  |  | -112 | -116 |
|  | Bands TDD_E |  |  |  |  |  | -111 | -115 |
| IoNote4 | Bands TDD_A | dBm/9 MHz | -70.27 | -70.27 | -52.27 | -52.27 | -82.43 |  |
|  | Bands TDD_C |  |  |  |  |  | -81.43 |  |
|  | Bands TDD_E |  |  |  |  |  | -80.43 |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 5: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

#### A.9.1.2.3	Test Requirements

The RSRP measurement accuracy shall fulfil the requirements in sections 9.1.2.1 and 9.1.2.2. The RSRP measurement accuracy for UE Category 1bis shall fulfil the requirements in sections 9.1.2.7 and 9.1.2.8.

< END OF CHANGE #1 >

< START OF CHANGE #2 >

### A.3.22.2	Principle of Category 1bis UE Testing

In Annex A, tests in table A.3.22.2-1 defined for Category ≥1 UE with 2 Rx antenna are applicable to Category 1bis UE with 1 Rx antenna. Unless otherwise specified, same test configurations are used except for propagation channel change to 1x1 or 2x1 according to number of Tx antennas. For RSRP and RSRQ measurement accuracy test, corresponding measurement accuracy requirement for Category 1bis UE is specified in the table. For band dependent RRM tests defined in section A.9, only subset of bands that are defined for Cat.1bis UE are applicable.

Table A.3.22.2-1: Test cases applicable to category 1bis UE

| Test category | Test case | Test case name |
| --- | --- | --- |
| Cell re-selection tests | A.4.2.20 | E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis |
|  | A.4.2.21 | E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis |
|  | A.4.2.31 | E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis |
|  | A.4.2.32 | E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis |
|  | A.4.2.33 | E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis |
|  | A.4.2.34 | E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis |
| Handover tests | A.5.1.19 | E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis |
|  | A.5.1.20 | E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis |
|  | A.5.1.3 | E-UTRAN FDD – FDD Inter frequency handover |
|  | A.5.1.4 | E-UTRAN TDD – TDD Inter frequency handover |
|  | A.5.1.5 | E-UTRAN FDD – FDD Inter frequency handover: unknown target cell |
|  | A.5.1.6 | E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell |
|  | A.5.1.7 | E-UTRAN FDD – TDD Inter frequency handover |
|  | A.5.1.8 | E-UTRAN TDD – FDD Inter frequency handover |
| RRC re-establishment tests | A.6.1.1 | E-UTRAN FDD Intra-frequency RRC Re-establishment |
|  | A.6.1.2 | E-UTRAN FDD Inter-frequency RRC Re-establishment |
|  | A.6.1.3 | E-UTRAN TDD Intra-frequency RRC Re-establishment |
|  | A.6.1.4 | E-UTRAN TDD Inter-frequency RRC Re-establishment |
| Random access tests | A.6.2.1 | E-UTRAN FDD – Contention Based Random Access Test |
|  | A.6.2.2 | E-UTRAN FDD – Non-Contention Based Random Access Test |
|  | A.6.2.3 | E-UTRAN TDD – Contention Based Random Access Test |
|  | A.6.2.4 | E-UTRAN TDD – Non-Contention Based Random Access Test |
| RRC connection release with redirection tests | A.6.3.1 | Redirection from E-UTRAN FDD to UTRAN FDD |
|  | A.6.3.2 | Redirection from E-UTRAN TDD to UTRAN FDD |
|  | A.6.3.3 | Redirection from E-UTRAN FDD to GERAN when System Information is provided |
|  | A.6.3.4 | Redirection from E-UTRAN TDD to GERAN when System Information is provided |
|  | A.6.3.5 | E-UTRA TDD RRC connection release redirection to UTRA TDD |
|  | A.6.3.6 | E-UTRA FDD RRC connection release redirection to UTRA TDD |
|  | A.6.3.7 | E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided |
|  | A.6.3.8 | E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided |
|  | A.6.3.9 | Redirection from E-UTRAN FDD to UTRAN FDD without System Information |
|  | A.6.3.10 | Redirection from E-UTRAN FDD to GERAN when System Information is not provided |
|  | A.6.3.11 | Redirection from E-UTRAN TDD to GERAN when System Information is not provided |
|  | A.6.3.12 | E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided |
| UE transmit timing tests | A.7.1.1 | E-UTRAN FDD – UE Transmit Timing Accuracy Tests |
|  | A.7.1.2 | E-UTRAN TDD - UE Transmit Timing Accuracy Tests |
| UE timing advance tests | A.7.2.1 | E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test |
|  | A.7.2.2 | E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test |
| Radio link monitoring tests | A.7.3.26 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0 |
|  | A.7.3.27 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0 |
|  | A.7.3.28 | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |
|  | A.7.3.29 | E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0 |
|  | A.7.3.34 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0 |
|  | A.7.3.35 | E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0 |
|  | A.7.3.36 | E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0 |
|  | A.7.3.37 | E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0 |
| Event-triggered reporting | A.8.1.11 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0 |
|  | A.8.1.12 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0 |
|  | A.8.1.13 | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0 |
|  | A.8.2.12 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells |
|  | A.8.2.13 | E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX |
|  | A.8.3.1 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells |
|  | A.8.3.2 | E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells |
|  | A.8.3.3 | E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used |
|  | A.8.4.1 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells |
|  | A.8.4.2 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells |
|  | A.8.4.3 | E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used |
|  | A.8.4.6 | E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0 |
| CGI reading tests | A.8.1.19 | E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0 |
|  | A.8.1.20 | E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0 |
|  | A.8.2.7 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps |
|  | A.8.2.8 | E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX |
| RSTD measurement reporting delay test | A.8.12.1 | E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case |
|  | A.8.12.2 | E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case |
|  | A.8.13.1 | E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency |
|  | A.8.13.2 | E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency |
| RSRP measurement accuracy tests | A.9.1.1 | FDD Intra frequency case Measurement accuracy requirement in 9.1.2.7 and 9.1.2.8 |
|  | A.9.1.2 | TDD Intra frequency case Measurement accuracy requirement in 9.1.2.7 and 9.1.2.8 |
|  | A.9.1.3 | FDD—FDD Inter frequency case Measurement accuracy requirement in 9.1.3.3 and 9.1.3.4 |
|  | A.9.1.4 | TDD—TDD Inter frequency case Measurement accuracy requirement in 9.1.3.3 and 9.1.3.4 |
|  | A.9.1.5 | FDD—TDD Inter frequency case Measurement accuracy requirement in 9.1.3.3 and 9.1.3.4 |
| RSRQ measurement accuracy tests | A.9.2.1 | FDD Intra frequency case Measurement accuracy requirement in 9.1.5.5 |
|  | A.9.2.2 | TDD Intra frequency case Measurement accuracy requirement in 9.1.5.5 |
|  | A.9.2.3 | FDD—FDD Inter frequency case Measurement accuracy requirement in 9.1.6.5 and 9.1.6.6 |
|  | A.9.2.4 | TDD—TDD Inter frequency case Measurement accuracy requirement in 9.1.6.5 and 9.1.6.6 |
|  | A.9.2.4A | FDD—TDD Inter frequency case Measurement accuracy requirement in 9.1.6.5 and 9.1.6.6 |
| UE Rx-Tx time difference measurement accuracy tests | A.9.7.1 | E-UTRAN FDD UE Rx – Tx time difference case |
|  | A.9.7.2 | E-UTRA TDD |
| RSTD measurement accuracy tests | A.9.8.1 | E-UTRAN FDD RSTD intra frequency case Measurement accuracy requirement in 9.1.10.5 |
|  | A.9.8.2 | E-UTRAN TDD RSTD intra frequency case Measurement accuracy requirement in 9.1.10.5 |
|  | A.9.8.3 | E-UTRAN FDD-FDD RSTD inter frequency case Measurement accuracy requirement in 9.1.10.6 |
|  | A.9.8.4 | E-UTRAN TDD-TDD RSTD inter frequency case Measurement accuracy requirement in 9.1.10.5 |

< END OF CHANGE #2 >

< START OF CHANGE #3 >

### A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case

#### A.9.7.2.1	Test Purpose and Environment

The purpose of this test is to verify that the E-UTRAN TDD UE Rx-Tx time difference measurement accuracy is within the specified limits in clause 9.1.9.

There is only one cell in the test. The tested UE is connected with the PCell, configured to transmit SRS signals periodcally, and signaled to report UE Rx – Tx time difference measurement. The test equipment measures the transmit timing of the UE using the transmitted SRS, and measures the receive timing using the downlink CRS. The test equipment then compares the difference of these two timings to the UE Rx – Tx measurement reported by the UE.

#### A.9.7.2.2	Test parameters

The parameters for this test case are defined in Table A.9.7.2.2-1, and the SRS configuration used is defined in Table A.9.7.2.2-2.

Table A.9.7.2.2-1: Cell specific test parameters for UE Rx-Tx time difference measurement

| Parameter | Unit | Tests 1 | Tests 2 |
| --- | --- | --- | --- |
| E-UTRAN RF Channel Number | - | 1 | 1 |
| BWchannel | MHz | 1.4 | 10 |
| Uplink-downlink configuration of cell Note1 |  | 1 | 1 |
| Special subframe configuration of cell Note1 |  | 6 | 6 |
| PDSCH Reference measurement channel defined in A.3.1.1.2 | - | R.2 TDD | R.0 TDD |
| PDSCH allocation |  | 2-3 | 13-36 |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.3.1.2.2 | - | R.8 TDD | R.6 TDD |
| OCNG Patterns defined in A.3.2.2.3 (OP.3 TDD) and A.3.2.2.1 (OP.1 TDD) | - | OP.3 TDD | OP.1 TDD |
| PBCH_RA | dB | 0 | 0 |
| PBCH_RB | dB |  |  |
| PSS_RA | dB |  |  |
| SSS_RA | dB |  |  |
| PCFICH_RB | dB |  |  |
| PHICH_RA | dB |  |  |
| PHICH_RB | dB |  |  |
| PDCCH_RA | dB |  |  |
| PDCCH_RB | dB |  |  |
| PDSCH_RA | dB |  |  |
| PDSCH_RB | dB |  |  |
| OCNG_RANote2 | dB |  |  |
| OCNG_RBNote2 | dB |  |  |
| Note 3 | dBm/15 kHz | -98 | -98 |
| RSRP Note 4 | dBm/15 kHz | -101 | -101 |
|  | dB | -3 | -3 |
| Io Note 4 | dBm/1.08 MHz | -77.66 | N/A |
|  | dBm/9 MHz | N/A | -68.45 |
|  | dB | -3 | -3 |
| Propagation Condition |  | AWGN |  |
| Note 1: For special subframe and uplink-downlink configurations see Tables 4.2-1 and 4.2-2 in TS 36.211. Note 2: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 3: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modeled as AWGN of appropriate power for to be fulfilled. Note 4: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |

Table A.9.7.2.2-2: Sounding Reference Symbol Configuration to be used in TDD UE Rx – Tx time difference test

| Field | Test 1 | Test 2 | Comment |
| --- | --- | --- | --- |
|  | Value |  |  |
| srsBandwidthConfiguration | bw7 | bw5 |  |
| srsSubframeConfiguration | sc1 |  |  |
| ackNackSrsSimultaneousTransmission | FALSE |  |  |
| srsMaxUpPTS | TRUE |  |  |
| srsBandwidth | 0 |  | No hopping |
| srsHoppingBandwidth | hbw0 |  |  |
| frequencyDomainPosition | 0 |  |  |
| Duration | TRUE |  | Indefinite duration |
| Srs-ConfigurationIndex | 10 |  | SRS periodicity of 10ms for all Tests. |
| transmissionComb | 0 |  |  |
| cyclicShift | cs0 |  | No cyclic shift |
| SRS-AntennaPort | an1 |  | Number of antenna ports used for SRS transmission |
| Note: For further information see clause 6.3.2 in TS 36.331. |  |  |  |

#### A.9.7.2.3	Test Requirements

The UE Rx – Tx time difference measurement accuracy shall fulfill the requirements in clause 9.1.9.1.

< END OF CHANGE #3 >