3GPP TSG-RAN4 Meeting #93	R4-1914895

# Reno, US, 18th – 22nd November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6750 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding interruption length requirements for direct Scell activation R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-Core |  |  |  |  |  | Date: |  |  | 2019-09-23 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Endorsed in R4-1912799 in RAN4#92bis. New change is that section number is changed from 7.8.2.18 to 7.8.2.21 due to conflicting with other CRs. For direct SCell activation or hibernation, the interruption window is specified in section 7.7.18 and 7.7.19, but the requirements on length of interruption are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new section for interruption length requirements for direct SCell activation. The interruption length is 1ms for inter-band CA and 5ms for intra-band CA. In RAN4#92, some companies mentioned that the interruption length should consider UE doing both SCell addition and activation/hibernation, so we propose the interruption length is 2ms for inter-band case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is no interruption requirements for direct SCell activation or hibernation, so it is unclear whether interruption is allowed or what interruption length is allowed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | new section 7.8.2.18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

7.8.2.21	Interruptions at direct SCell activation and hibernation

When any number of SCells between one and M is directly activated or hibernated using the same RRC message as defined in [2], for each of SCell(s) to be directly activated or hibernated, the UE is allowed an interruption on PCell and on any activated SCell during the direct SCell activation and hibernation procedure [2] as follows:

-	an interruption on PCell:

-	of up to 2 subframe, if the PCell is not in the same band as any of the SCells being directly activated or hibernated, or

-	of up to 5 subframes, if the PCell is in the same band as any of the SCells being directly activated or hibernated;

-	an interruption on any activated SCell:

-	of up to 2 subframe, if the activated SCell is not in the same band as any of the SCells being directly activated or hibernated, or

-	of up to 5 subframes, if the activated SCell is in the same band as any of the SCells being directly activated or hibernated.

where M is the number of SCells included in the RRC reconfiguration message and shall not exceed the maximum number of SCells supported by the UE.

The interruption shall be within the direct SCell activation and hibernation delay as defined in 7.7.18 and 7.7.19.

<End of Change 1>