3GPP TSG-RAN Meeting #86                           RP-193075

Sitges, Spain, 9-12 December 2019

Source:	Ericsson

Title:	Band combination proposals for NR-U WI in Rel-16

Agenda Item:	9.4.3

Document for:	Approval

# 1	Introduction

RAN4 has agreed to a WF in RAN4#93 on how to handle band combinations related to NR-U WI in Rel-16 [2].

The following band combination types are agreed in this WF [2]:

| Scenario A1 | - PCell in NR + up to 5 DL SCells in NR-U - PCell in NR + up to 5 DL SCells in NR-U + 1 UL SCell in NR-U |
| --- | --- |
| Scenario B | - PCell in LTE + PSCell in NR-U + up to 4 DL SCells in NR-U - PCell in LTE + PSCell in NR + up to 5 DL SCells in NR-U |
| Scenario C | PCell in NR-U + up to 4 DL Scells in NR-U |
| Note 1: PCell and PSCell have both DL and UL Note 2: Both contiguous and non-contiguous DL NR-U SCells are allowed |  |

Besides, we have also agreed on the following [2]:

- Maximum 400MHz aggregated BW is considered in DL for NR-U band

- Maximum 100MHz UL CBW is considered for NR-U band

- Ericsson to collect band combination proposals prior to RAN plenary

  - Interested companies are requested to send band combination proposals on RAN4 reflector

  - Deadline for providing band combination proposals by Friday, 29 November 2019 23.59 UTC

    - RAN plenary tdoc deadline: Friday 29 November 2019

    - RAN plenary submission deadline: Monday 2 December 2019

- The band combinations proposed for Rel-16 NR-U WI will be included in NR-U WID and to be approved in December RAN plenary

Based on agreements in this WF, we list the band combination proposals from different companies in this contribution.

# 2	Scenario A (NR CA)

The band combinations in this section relates to scenario A as defined in the WID [1] and the WF in [2].

| NR CA configuration | Uplink CA configuration | contact name, company | contact email | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- |
| CA_n1A-n46A | - | S. Truelove, BT plc | stephen.truelove@bt.com | na |
| CA_n1A-n46A | CA_n1A-n46A | S. Truelove, BT plc | stephen.truelove@bt.com | DL_n1A-n46A_UL_n1A, DL_n1A-n46A_UL_n46A |
| CA_n38A-n46A | - | S. Truelove, BT plc | stephen.truelove@bt.com | na |
| CA_n38A-n46A | CA_n38A-n46A | S. Truelove, BT plc | stephen.truelove@bt.com | DL_n38A-n46A_UL_n38A, DL_n38A-n46A_UL_n46A |
| CA_n46A-n78A | - | S. Truelove, BT plc | stephen.truelove@bt.com | na |
| CA_n46A-n78A | CA_n46A-n78A | S. Truelove, BT plc | stephen.truelove@bt.com | DL_n46A-n78A_UL_n46A, DL_n46A-n78A_UL_n78A |
| CA_n46A-n78(2A) | - | S. Truelove, BT plc | stephen.truelove@bt.com | na |
| CA_n46A-n78(2A) | CA_n46A-n78A | S. Truelove, BT plc | stephen.truelove@bt.com | DL_n46A-n78(2A)_UL_n46A, DL_n46A-n78A_UL_n46A-n78A, DL_n78(2A)_UL_n78A |
| CA_n25A-n46A | - | Nelson Ueng, T-Mobile USA | Neng-tsann.ueng@t-mobile.com | none |
| CA_n46A-n66A | - | Nelson Ueng, T-Mobile USA | Neng-tsann.ueng@t-mobile.com | none |
| CA_n48A-n46A | CA_n48A-n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_n48A_UL_n48A |
| CA_n48A-n46B | CA_n48A-n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_n46A_UL_n48A |
| CA_n48A-n46C | CA_n48A-n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_n46A_UL_n48A |
| CA_n48A-n46D | CA_n48A-n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_n46C_UL_n48A |
| CA_n48A-n46E | CA_n48A-n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_n46D_UL_n48A |

# 3	Scenario B (EN-DC)

The band combinations in this section relates to scenario B as defined in the WID [1] and the WF in [2].

| EN-DC configuration | Uplink EN-DC configurations | contact name, company | Contact Email | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- |
| DC_2A_n46A | DC_2A_n46A | Nelson Ueng, T-Mobile USA | Neng-tsann.ueng@t-mobile.com | none |
| DC_48A_n46A | DC_48A_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48A_n46A_UL_48A DL_48A_UL_48B |
| DC_48B_n46A | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48B_n46A_UL_48A (new) DL_48A_n46A_UL_48B DL_48B _UL48B |
| DC_48C_n46A | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48C_n46A_UL_48A (new) DL_48B_n46A_UL_48B DL_48C _UL_48B |
| DC_48D_n46A | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48D_n46A_UL_48A (new) DL_48C_n46A_UL_48B DL_48D_UL_48B |
| DC_48E_n46A | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48E_n46A_UL_48A (new) DL_48D_n46A_UL_48B DL_48E_UL48_B |
| DC_48A_n46B | DC_48A_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48A_n46B_UL_48A |
| DC_48B_n46B | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48B_n46B_UL_48A (new) DL_48B_n46A_UL_48B |
| DC_48C_n46B | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48C_n46B_UL_48A (new) DL_48B_n46B_UL_48B (new) DL_48C_n46A_UL_48B |
| DC_48D_n46B | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48D_n46B_UL_48A (new) DL_48C_n46B_UL_48B (new) DL_48D_n46A_UL_48B |
| DC_48E_n46B | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48E_n46B_UL_48A (new) DL_48C_n46B_UL_48B (new) DL_48E_n46A_UL_48B |
| DC_48A_n46C | DC_48A_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48A_n46C_UL_48A (new) DL_48A_n46B_UL_48B |
| DC_48B_n46C | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48B_n46C_UL_48A (new) DL_48B_n46B_UL_48B |
| DC_48C_n46C | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48C_n46C_UL_48A (new) DL_48B_n46C_UL_48B (new) DL_48C_n46B_UL_48B |
| DC_48D_n46C | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48D_n46C_UL_48A (new) DL_48C_n46C_UL_48B (new) DL_48D_n46B_UL_48B |
| DC_48E_n46C | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48E_n46C_UL_48A (new) DL_48C_n46C_UL_48B (new) DL_48E_ 46B_UL_48B |
| DC_48A_n46D | DC_48A_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48A_n46D_UL_48A (new) DL_48A_n46C_UL_48B |
| DC_48B_n46D | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48B_n46D_UL_48A (new) DL_48B_n46C_UL_48B |
| DC_48C_n46D | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48C_n46D_UL_48A (new) DL_48B_n46D_UL_48B (new) DL_48C_n46C_UL_48B |
| DC_48D_n46D | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48D_n46D_UL_48A (new) DL_48C_n46D_UL_48B (new) DL_48D_n46C_UL_48B |
| DC_48E_n46D | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48E_n46D_UL_48A (new) DL_48C_n46D_UL_48B (new) DL_48E_n46C_UL_48B |
| DC_48A_n46E | DC_48A_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48A_n46E_UL_48A (new) DL_48A_n46D_UL_48B |
| DC_48B_n46E | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48B_n46E_UL_48A (new) DL_48B_n46D_UL_48B |
| DC_48C_n46E | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48C_n46E_UL_48A (new) DL_48B_n46E_UL_48B (new) DL_48C_n46D_UL_48B |
| DC_48D_n46E | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48D_n46E_UL_48A (new) DL_48C_n46E_UL_48B (new) DL_48D_n46D_UL_48B |
| DC_48E_n46E | DC_48B_n46A | Frank Azcuy, Charter Communications | C-Frank.Azcuy@charter.com | (new) DL_48E_n46E_UL_48A (new) DL_48D_n46E_UL_48B (new) DL_48E_n46D_UL_48B |
| DC_66A_n46A | DC_66A_n46A | Nelson Ueng, T-Mobile USA | Neng-tsann.ueng@t-mobile.com | none |
| Note: Only one EN-DC configuration can be added per row in the first column while multiple UL configurations can be listed in the second column separated by a newline. |  |  |  |  |

# 4	Scenario C (NR-U standalone)

The band combinations in this section relates to scenario C as defined in the WID [1] and the WF in [2].

| NR-U standalone configuration | Uplink NR-U standalone configuration | contact name, company | contact email | supported next level fallback modes (in DL and UL) |
| --- | --- | --- | --- | --- |
| CA_n46E | CA_n46A | Imadur Rahman, Ericsson | imadur.rahman@ericsson.com | (new) DL_n46D_UL_n46A (new) DL_n46C_UL_n46A (new) DL_n46A_UL_n46A |
| CA_n46I | CA_n46A | Imadur Rahman, Ericsson | imadur.rahman@ericsson.com | (new) DL_n46H_UL_n46A (new) DL_n46G_UL_n46A (new) DL_n46B_UL_n46A (new) DL_n46A_UL_n46A |

# 5 	Proposal

We propose to approve the above band combinations for Rel-16 NR-U WI.

# References

- RP-182878, NR-based Access to Unlicensed Spectrum, Qualcomm Incorporated.

- R4-1915977, WF on Number of bands and CCs to be considered in Rel-16 NR-U WI, Ericsson