3GPP TSG RAN meeting #86								     RP-193102

Sitges, Spain, December 9-12, 2019

Agenda Item:	9.1.1

Source: 	Qualcomm

Title:	Response to RP-192980

Document for:	Discussion

# Introduction

This document is a “response document” to RP-192980 “Rel-17 V2X Automotive Perspective Challenges” from Volkswagen, General Motors, Toyota.

In that document, a number of points worth of discussions are raised related to the Rel-16 work on NR V2X. This contribution addresses those points one by one and provides some perspectives accordingly.

# Discussion

The text in colored square is a quote from RP-192980. The rest is response text

Observation 1: Vehicle high mobility introduces Doppler spread that may significantly impact link performance. Observation 2: The current standard implies that broadcast safety use cases can only be supported if all UEs use the same carrier spacing (numerology) regardless scenario and region. Observation 3: Appling the same numerology for all broadcast safety messages, regardless the scenario, may result in unfavourable performance. Proposal 1: A thorough research needs to be conducted to find the optimal numerology per scenario. Even if the optimal numerology wouldn’t be chosen for all the scenarios, it is essential to be aware of the performance degradation due to such a choice. As this work has to do with many of the system aspects, it all must be done within 3GPP purview. Proposal 2: The coexistence between UEs using different carrier spacing and the way UEs shift between various carrier spacing regimes should be considered within Rel-17 work. Proposal 4: The multiple options per topic in Rel-16 standard should be reduced so that a proper evaluation of the remaining options can be completed.

Response: It is our understanding that these observations and proposals are made by assuming that since NR-V2X supports different numerologies so numerology change must be used.

It is indeed true that NR-V2X provides support for different numerology, but, we assume that as part of “golden configuration” regional standards a single preferred numerology will be selected on a regional basis.

From our point of view (see R1-1911104, R1-1912944), a  30KHz subcarrier spacing is the optimal choice from performance point of view as it can handle very high relative speed of (250 Kmph + 250 Kmph) as well as the size of the cyclic prefix in case of 30KHz provides adequate range. Based on that, we would see no need to change the subcarrier spacing on the fly.

Observation 4: There are system level challenges with regards to synchronization and resource allocation for both Mode 1 and Mode 2. The option to rely on other UEs for synchronization hasn't been explored thoroughly enough from a system level perspective to be incorporated as a recommended solution. Proposal 3: The implementation challenges of Mode 1 and 2 should be addressed and resolved during Rel-17.

Response: We have submitted extensive link level and system level simulation results in RAN1 for synchronisation and resource allocation (eg see R1-1912947)

The sidelink synchronisation mechanism agreed so far works on the principle of searching for higher priority signal continuously. High priority sources include GNSS (Highest priority P0), UE directly synchronised to GNSS (Priority P1), UE indirectly synchronised to GNSS (Priority P2), UE synchronised to gNB/eNB (Priority P3), UE directly synchronised to gNB/eNB (Priority P4), UE indirectly synchronised to gNB/eNB (Priority P5) and the remaining UEs with lowest priority (Priority P6).

Our observation is that the synchronisation mechanism works quite well for (Priority levels P0 to P5) and will become quite robust once the last pending aspect of “cluster merge” for lowest priority synch sources (i.e. P6 and P6’) based on SLSS ID is agreed (R1-1912947).

Observation 5: NR V2X Rel-16 offers multiple options for numerous aspects of the standard. It is unclear how the different settings of each option interact with each other and which combinations are not advisable to use.

Response: It is our understanding that, similar to LTE-V2X, regional standards will specify the optimal value for all radio parameters for preferred deployment choice. From 3GPP point of view the objective is to provide a robust platform which is capable of handling different deployments e.g. Out of Coverage in ITS or in-coverage licensed network.

Observation 6: The system level simulation profiling does not use some of the scenarios and models that were agreed upon in the evaluation methodology stage. Simulations with idealized scenarios make it much harder to figure out the system performance in a real-world deployment. Proposal 5: - Rel-17 V2X should consider Highway Option B as an additional simulation scenario during the WI. - Rel-17 V2X simulation profiles should be complemented by realistic antenna models during the WI.

Response: 3GPP decided several profiles which were good mix of highway and Urban scenarios. Companies in RAN1 selected different profiles for simulation which they considered as representative models. We even provided simulation results for higher traffic density deployment which were beyond the agreed 3GPP scenario set to show the performance in congested setups (R1- 1913293).

Of course there can be many more possible scenarios to simulate and characterize. However, those simulations can also occour outside of 3GPP e.g in a characterization exercise in 5GAA.

Observation 7: The system level simulation profiling assumes the availability of 20MHz BW per a data traffic model (periodic/ aperiodic) in the sub 6GHz band, when some regional regulations allocate only 10MHz. Proposal 6: Rel-17 V2X should consider 10MHz BW for sub 6GHz frequencies as a baseline assumption in the simulation profiling of the standard. Proposal 7: There should be an effort to simulate system level performance for broadcast, data traffic periodic Model 1 with 2Rx antennas to allow the automotive industry to get a grasp of the NR V2X potential in the context of road safety.

Response: At the time of performing the simulation (even now) there is no guaranteed clear spectrum allocation for NR-V2X. So a “reasonable” bandwidth size was taken as candidate for simulation to accommodate the desired amount of traffic.

Observation 8: With the current simulation profiling defined in [3,5] the performed simulations don’t provide a clear understanding of the technology performance of a vehicle UE for safety use cases.

Response: As clearly indicated in the Work Item description (see below), the primary focus of NR-V2X were advanced use cases which have much higher reliability requirements compared to basic safety messages.

RP-191723: Revised WID on 5G V2X with NR sidelink

TSG RAN has already agreed in TR 38.913 that it is not intended for NR V2X to replace the services offered by LTE V2X. Instead, the NR V2X shall complement LTE V2X for advanced V2X services and support interworking with LTE V2X. At least from 3GPP RAN technology development standpoint, the focus and scope of NR V2X is to target advanced V2X use cases. However, this does not imply that NR V2X cannot support basic safety use cases. It is clearly up to the regional regulators and the stakeholders involved (i.e. Car OEMs and automotive ecosystem in general) to decide on the technology of choice for any service and any use case.

Observation 9: NR-V2X reuses TX resource (re-)selection procedure as in LTE MAC specification. Proposal 8: Rel-17 V2X should enhance the TX resource (re-) selection procedures to avoid the packet collisions, especially in congested scenarios.

Response: NR-V2X supports advanced resource allocation mechanism to handle aperiodic traffic. The overall design does handle both periodic and aperiodic traffic efficiently.

# Conclusion

This paper has provided a point by point set of comments to RP-192980.