3GPP TSG RAN Meeting #86	RP-193124

# Sitges, Spain, December 9th – 12th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0079 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of V2X in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL |  |  |  |  |  | Date: |  |  | 2019-12-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of V2X. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for synchronization procedures, power control, transmission prioritization, reporting of control information, and reception of control information on sidelink, and for reporting of control information on uplnik and reception of control information in downlink |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for V2X. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1, 16 (new clause), 16.1 (new), 16.2 (new), 16.2.1 (new), 16.2.2 (new), 16.2.3 (new), 16.2.4 (new), 16.3 (new), 16.4 (new), 16.5 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, 38.212, 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1-1913680 |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1, TR 21.905].

BPRE	Bits per resource element

BWP	Bandwidth part

CB	Code block

CBG	Code block group

CBR	Channel busy ratio

CCE	Control channel element

CORESET	Control resource set

CP	Cyclic prefix

CRC	Cyclic redundancy check

CSI	Channel state information

CSS	Common search space

DAI	Downlink assignment index

DC	Dual connectivity

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

EPRE	Energy per resource element

EN-DC	E-UTRA NR dual connectivity with MCG using E-UTRA and SCG using NR

FR1	Frequency range 1

FR2	Frequency range 2

GSCN	Global synchronization channel number

HARQ-ACK	Hybrid automatic repeat request acknowledgement

MCG	Master cell group

MCS	Modulation and coding scheme

NE-DC	E-UTRA NR dual connectivity with MCG using NR and SCG using E-UTRA

NR-DC	NR NR dual connectivity

PBCH	Physical broadcast channel

PCell	Primary cell

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PRACH	Physical random access channel

PRB	Physical resource block

PRG	Physical resource block group

PSCell	Primary secondary cell

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSSCH	Physical sidelink shared channel

PSS	Primary synchronization signal

PUCCH	Physical uplink control channel

PUCCH-SCell	PUCCH SCell

PUSCH	Physical uplink shared channel

QCL	Quasi co-location

RB	Resource block

RE	Resource element

RLM	Radio link monitoring

RRM	Radio resource management

RS	Reference signal

RSRP	Reference signal received power

SCG	Secondary cell group

SCS	Subcarrier spacing

SCI	Sidelink control information

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SLIV	Start and length indicator value

SPS	Semi-persistent scheduling

SR	Scheduling request

SRI	SRS resource indicator

SRS	Sounding reference signal

SSS	Secondary synchronization signal

TA	Timing advance

TAG	Timing advance group

TCI	Transmission Configuration Indicator

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

USS	UE-specific search space

## 10.1	UE procedure for determining physical downlink control channel assignment

A set of PDCCH candidates for a UE to monitor is defined in terms of PDCCH search space sets. A search space set can be a CSS set or a USS set. A UE monitors PDCCH candidates in one or more of the following search spaces sets

-	a Type0-PDCCH CSS set configured by pdcch-ConfigSIB1 in MIB or by searchSpaceSIB1 in PDCCH-ConfigCommon or by searchSpaceZero in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type0A-PDCCH CSS set configured by searchSpaceOtherSystemInformation in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a SI-RNTI on the primary cell of the MCG

-	a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI or a TC-RNTI on the primary cell

-	a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG

-	a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, or TPC-SRS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and

-	a USS set configured by SearchSpace in PDCCH-Config with searchSpaceType = ue-Specific for DCI formats with CRC scrambled by C-RNTI, MCS-C-RNTI, SP-CSI-RNTI, CS-RNTI, SL-RNTI, SL-CS-RNTI, or SL-L-CS-RNTI.

For a DL BWP, if a UE is not provided searchSpaceSIB1 for Type0-PDCCH CSS set by PDCCH-ConfigCommon, the UE does not monitor PDCCH candidates for a Type0-PDCCH CSS set on the DL BWP. The Type0-PDCCH CSS set is defined by the CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level given in Table 10.1-1. If the active DL BWP and the initial DL BWP have same SCS and same CP length and the active DL BWP includes all RBs of the CORESET with index 0, or the active DL BWP is the initial DL BWP, the CORESET configured for Type0-PDCCH CSS set has CORESET index 0 and the Type0-PDCCH CSS set has search space set index 0.

For a DL BWP, if the UE is not provided searchSpaceOtherSystemInformation for Type0A-PDCCH CSS set, the UE does not monitor PDCCH for Type0A-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type0A-PDCCH CSS set are given in Table 10.1-1.

For a DL BWP and a Type1-PDCCH CSS set, a UE is provided a configuration for a search space by ra-SearchSpace. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set.

If a UE is not provided pagingSearchSpace for Type2-PDCCH CSS set, the UE does not monitor PDCCH for Type2-PDCCH CSS set on the DL BWP. The CCE aggregation levels and the number of PDCCH candidates per CCE aggregation level for Type2-PDCCH CSS set are given in Table 10.1-1.

If a UE is provided a zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set as described in Subclause 13, and the UE is provided a C-RNTI, the UE monitors PDCCH candidates only at monitoring occasions associated with a SS/PBCH block, where the SS/PBCH block is determined by the most recent of

-	a MAC CE activation command indicating a TCI state of the active BWP that includes a CORESET with index 0, as described in [6, TS 38.214], where the TCI-state includes a CSI-RS which is quasi-co-located with the SS/PBCH block, or

-	a random access procedure that is not initiated by a PDCCH order that triggers a contention-free random access procedure

If a UE monitors PDCCH candidates for DCI formats with CRC scrambled by a C-RNTI and the UE is provided a non-zero value for searchSpaceID in PDCCH-ConfigCommon for a Type0/0A/2-PDCCH CSS set, the UE determines monitoring occasions for PDCCH candidates of the Type0/0A/2-PDCCH CSS set based on the search space set associated with the value of searchSpaceID.

The UE may assume that the DM-RS antenna port associated with PDCCH receptions in the CORESET configured by pdcch-ConfigSIB1 in MIB and for corresponding PDSCH receptions, and the corresponding SS/PBCH block are quasi co-located with respect to average gain, QCL-TypeA, and QCL-TypeD properties, when applicable [6, TS 38.214], and if the UE is not provided a TCI state indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in the CORESET. The value for the DM-RS scrambling sequence initialization is the cell ID. A SCS is provided by subCarrierSpacingCommon in MIB.

For single cell operation or for operation with carrier aggregation in a same frequency band, a UE does not expect to monitor a PDCCH in a Type0/0A/2/3-PDCCH CSS set or in a USS set if a DM-RS for monitoring a PDCCH in a Type1-PDCCH CSS set does not have same QCL-TypeD properties [6, TS 38.214] with a DM-RS for monitoring the PDCCH in the Type0/0A/2/3-PDCCH CSS set or in the USS set, and if the PDCCH or an associated PDSCH overlaps in at least one symbol with a PDCCH the UE monitors in a Type1-PDCCH CSS set or with an associated PDSCH.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, and

-	a C-RNTI, an MCS-C-RNTI, a CS-RNTI, a SL-RNTI, a SL-CS-RNTI, or a SL-L-CS-RNTI

the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI, the MCS-C-RNTI, the CS-RNTI in the one or more search space sets in a slot where the UE monitors PDCCH candidates for at least a DCI format 0_0 or a DCI format 1_0 with CRC scrambled by SI-RNTI, RA-RNTI or P-RNTI.

If a UE is provided

-	one or more search space sets by corresponding one or more of searchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation, pagingSearchSpace, ra-SearchSpace, or a CSS set by PDCCH-Config, and

-	a SI-RNTI, a P-RNTI, a RA-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI

then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot.

Table 10.1-1: CCE aggregation levels and maximum number of PDCCH candidates per CCE aggregation level for CSS sets configured by searchSpaceSIB1

| CCE Aggregation Level | Number of Candidates |
| --- | --- |
| 4 | 4 |
| 8 | 2 |
| 16 | 1 |

For each DL BWP configured to a UE in a serving cell, a UE can be provided by higher layer signalling with  CORESETs. For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index , , by controlResourceSetId;

-	a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID;

-	a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity;

-	a number of consecutive symbols provided by duration;

-	a set of resource blocks provided by frequencyDomainResources;

-	CCE-to-REG mapping parameters provided by cce-REG-MappingType;

-	an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET;

-	an indication for a presence or absence of a transmission configuration indication (TCI) field for DCI format 1_1 transmitted by a PDCCH in CORESET , by tci-PresentInDCI.

When precoderGranularity = allContiguousRBs, a UE does not expect

-	to be configured a set of resource blocks of a CORESET that includes more than four sub-sets of resource blocks that are not contiguous in frequency

-	any RE of a CORESET to overlap with any RE determined from lte-CRS-ToMatchAround or with any RE of a SS/PBCH block.

For each CORESET in a DL BWP of a serving cell, a respective frequencyDomainResources provides a bitmap. The bits of the bitmap have a one-to-one mapping with non-overlapping groups of 6 consecutive PRBs, in ascending order of the PRB index in the DL BWP bandwidth of  PRBs with starting common RB position  where the first common RB of the first group of 6 PRBs has common RB index .

For a CORESET other than a CORESET with index 0,

-	if a UE has not been provided a configuration of TCI state(s) by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET, or has been provided initial configuration of more than one TCI states for the CORESET by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block the UE identified during the initial access procedure;

-	if a UE has been provided a configuration of more than one TCI states by tci-StatesPDCCH-ToAddList and tci-StatesPDCCH-ToReleaseList for the CORESET as part of Reconfiguration with sync procedure as described in [12, TS 38.331] but has not received a MAC CE activation command for one of the TCI states as described in [11, TS 38.321], the UE assumes that the DM-RS antenna port associated with PDCCH receptions is quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331].

For a CORESET with index 0, the UE assumes that a DM-RS antenna port for PDCCH receptions in the CORESET is quasi co-located with

-	the one or more DL RS configured by a TCI state, where the TCI state is indicated by a MAC CE activation command for the CORESET, if any, or

-	a SS/PBCH block the UE identified during a most recent random access procedure not initiated by a PDCCH order that triggers a contention-free random access procedure, if no MAC CE activation command indicating a TCI state for the CORESET is received after the most recent random access procedure.

For a CORESET other than a CORESET with index 0, if a UE is provided a single TCI state for a CORESET, or if the UE receives a MAC CE activation command for one of the provided TCI states for a CORESET, the UE assumes that the DM-RS antenna port associated with PDCCH receptions in the CORESET is quasi co-located with the one or more DL RS configured by the TCI state. For a CORESET with index 0, the UE expects that QCL-TypeD of a CSI-RS in a TCI state indicated by a MAC CE activation command for the CORESET is provided by a SS/PBCH block

-	if the UE receives a MAC CE activation command for one of the TCI states, the UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH. The active BWP is defined as the active BWP in the slot when the activation command is applied.

For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with  search space sets where, for each search space set from the  search space sets, the UE is provided the following by SearchSpace:

-	a search space set index , , by searchSpaceId

-	an association between the search space set  and a CORESET  by controlResourceSetId

-	a PDCCH monitoring periodicity of  slots and a PDCCH monitoring offset of  slots, by monitoringSlotPeriodicityAndOffset

-	a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot

-	a duration of  slots indicating a number of slots that the search space set  exists by duration

-	a number of PDCCH candidates  per CCE aggregation level  by aggregationLevel1, aggregationLevel2, aggregationLevel4, aggregationLevel8, and aggregationLevel16, for CCE aggregation level 1, CCE aggregation level 2, CCE aggregation level 4, CCE aggregation level 8, and CCE aggregation level 16, respectively

-	an indication that search space set  is either a CSS set or a USS set by searchSpaceType

-	if search space set  is a CSS set

-	an indication by dci-Format0-0-AndFormat1-0 to monitor PDCCH candidates for DCI format 0_0 and DCI format 1_0

-	an indication by dci-Format2-0 to monitor one or two PDCCH candidates for DCI format 2_0 and a corresponding CCE aggregation level

-	an indication by dci-Format2-1 to monitor PDCCH candidates for DCI format 2_1

-	an indication by dci-Format2-2 to monitor PDCCH candidates for DCI format 2_2

-	an indication by dci-Format2-3 to monitor PDCCH candidates for DCI format 2_3

-	if search space set  is a USS set, an indication by dci-Formats to monitor PDCCH candidates either for DCI format 0_0 and DCI format 1_0, or for DCI format 0_1 and DCI format 1_1, or for DCI format 3_0, or for DCI format 3_1, or for DCI format 3_0 and DCI format 3_1

If the monitoringSymbolsWithinSlot indicates to a UE to monitor PDCCH in a subset of up to three consecutive symbols that are same in every slot where the UE monitors PDCCH for all search space sets, the UE does not expect to be configured with a PDCCH SCS other than 15 kHz if the subset includes at least one symbol after the third symbol.

A UE does not expect to be provided a first symbol and a number of consecutive symbols for a CORESET that results to a PDCCH candidate mapping to symbols of different slots.

A UE does not expect any two PDCCH monitoring occasions on an active DL BWP, for a same search space set or for different search space sets, in a same CORESET to be separated by a non-zero number of symbols that is smaller than the CORESET duration.

A UE determines a PDCCH monitoring occasion on an active DL BWP from the PDCCH monitoring periodicity, the PDCCH monitoring offset, and the PDCCH monitoring pattern within a slot. For search space set , the UE determines that a PDCCH monitoring occasion(s) exists in a slot with number  [4, TS 38.211] in a frame with number  if nfNslotframe,μ+ns,fμ-op,smod kp,s=0. The UE monitors PDCCH candidates for search space set  for  consecutive slots, starting from slot , and does not monitor PDCCH candidates for search space set  for the next  consecutive slots.

A USS at CCE aggregation level  is defined by a set of PDCCH candidates for CCE aggregation level .

If a UE is configured with CrossCarrierSchedulingConfig for a serving cell the carrier indicator field value corresponds to the value indicated by CrossCarrierSchedulingConfig.

For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if the UE is not configured with a carrier indicator field, the UE monitors the PDCCH candidates without carrier indicator field. For an active DL BWP of a serving cell on which a UE monitors PDCCH candidates in a USS, if a UE is configured with a carrier indicator field, the UE monitors the PDCCH candidates with carrier indicator field.

A UE does not expect to monitor PDCCH candidates on an active DL BWP of a secondary cell if the UE is configured to monitor PDCCH candidates with carrier indicator field corresponding to that secondary cell in another serving cell. For the active DL BWP of a serving cell on which the UE monitors PDCCH candidates, the UE monitors PDCCH candidates at least for the same serving cell.

For a search space set  associated with CORESET , the CCE indexes for aggregation level  corresponding to PDCCH candidate  of the search space set in slot  for an active DL BWP of a serving cell corresponding to carrier indicator field value  are given by

where

for any CSS, ;

for a USS, , ,  for ,  for ,  for , and ;

;

is the number of CCEs, numbered from 0 to , in CORESET ;

is the carrier indicator field value if the UE is configured with a carrier indicator field by CrossCarrierSchedulingConfig for the serving cell on which PDCCH is monitored; otherwise, including for any CSS, ;

, where  is the number of PDCCH candidates the UE is configured to monitor for aggregation level  of a search space set  for a serving cell corresponding to ;

for any CSS, ;

for a USS,  is the maximum of  over all configured  values for a CCE aggregation level  of search space set  ;

the RNTI value used for  is the C-RNTI.

A UE that

-	is configured for operation with carrier aggregation, and

-	indicates support of search space sharing through searchSpaceSharingCA-UL or through searchSpaceSharingCA-DL, and

-	has a PDCCH candidate with CCE aggregation level  in CORESET  for a DCI format 0_1 or a DCI format 1_1 having a first size and associated with serving cell ,

can receive a corresponding PDCCH through a PDCCH candidate with CCE aggregation level  in CORESET  for a DCI format 0_1 or a DCI format 1_1, respectively, having a second size and associated with serving cell  if the first size and the second size are same.

A UE expects to monitor PDCCH candidates for up to 4 sizes of DCI formats that include up to 3 sizes of DCI formats with CRC scrambled by C-RNTI per serving cell. The UE counts a number of sizes for DCI formats per serving cell based on a number of configured PDCCH candidates in respective search space sets for the corresponding active DL BWP.

A UE does not expect to detect, in a same PDCCH monitoring occasion, a DCI format with CRC scrambled by a SI-RNTI, RA-RNTI, TC-RNTI, P-RNTI, C-RNTI, CS-RNTI, or MCS-RNTI and a DCI format with CRC scrambled by a SL-RNTI or a SL-CS-RNTI for scheduling respective PDSCH and PSSCH receptions on a same serving cell.

A PDCCH candidate with index  for a search space set  using a set of CCEs in a CORESET  on the active DL BWP for serving cell  is not counted for monitoring if there is a PDCCH candidate with index  for a search space set , or if there is a PDCCH candidate with index  and , in the CORESET  on the active DL BWP for serving cell  using a same set of CCEs, the PDCCH candidates have identical scrambling, and the corresponding DCI formats for the PDCCH candidates have a same size; otherwise, the PDCCH candidate with index  is counted for monitoring.

Table 10.1-2 provides the maximum number of monitored PDCCH candidates, , for a DL BWP with SCS configuration  for a UE per slot for operation with a single serving cell.

Table 10.1-2: Maximum number  of monitored PDCCH candidates per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of monitored PDCCH candidates per slot and per serving cell |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 10.1-3 provides the maximum number of non-overlapped CCEs, , for a DL BWP with SCS configuration  that a UE is expected to monitor corresponding PDCCH candidates per slot for operation with a single serving cell.

CCEs for PDCCH candidates are non-overlapped if they correspond to

-	different CORESET indexes, or

-	different first symbols for the reception of the respective PDCCH candidates.

Table 10.1-3: Maximum number  of non-overlapped CCEs per slot for a DL BWP with SCS configuration  for a single serving cell

|  | Maximum number of non-overlapped CCEs per slot and per serving cell |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

If a UE is configured with  downlink cells with DL BWPs having SCS configuration  where , the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than  PDCCH candidates or more than  non-overlapped CCEs per slot for each scheduled cell.

If a UE is configured with  downlink cells with DL BWPs having SCS configuration , where , a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells.

For each scheduled cell, the UE is not required to monitor on the active DL BWP with SCS configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

A UE does not expect to be configured CSS sets that result to corresponding total, or per scheduled cell, numbers of monitored PDCCH candidates and non-overlapped CCEs per slot that exceed the corresponding maximum numbers per slot.

For same cell scheduling or for cross-carrier scheduling where a scheduling cell and scheduled cell(s) have DL BWPs with same SCS configuration , a UE does not expect a number of PDCCH candidates, and a number of corresponding non-overlapped CCEs per slot on a secondary cell to be larger than the corresponding numbers that the UE is capable of monitoring on the secondary cell per slot.

For cross-carrier scheduling, the number of PDCCH candidates for monitoring and the number of non-overlapped CCEs per slot are separately counted for each scheduled cell.

For all search space sets within a slot , denote by  a set of CSS sets with cardinality of  and by  a set of USS sets with cardinality of . The location of USS sets , , in  is according to an ascending order of the search space set index.

Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set .

For the CSS sets, a UE monitors  PDCCH candidates requiring a total of  non-overlapping CCEs in a slot.

The UE allocates PDCCH candidates for monitoring to USS sets for the primary cell having an active DL BWP with SCS configuration  in slot  according to the following pseudocode. A UE does not expect to monitor PDCCH in a USS set without allocated PDCCH candidates for monitoring.

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , .

Set

Set

Set

while  AND

allocate  PDCCH candidates for monitoring to USS set

;

;

;

end while

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells

the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells

-	the CORESET corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index

-	the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions

-	for the purpose of determining the CORESET, a SS/PBCH block is considered to have different QCL-TypeD properties than a CSI-RS

-	for the purpose of determining the CORESET, a first CSI-RS associated with a SS/PBCH block in a first cell and a second CSI-RS in a second cell that is also associated with the SS/PBCH block are assumed to have same QCL-TypeD properties

-	the allocation of non-overlapping CCEs and of PDCCH candidates for PDCCH monitoring is according to all search space sets associated with the multiple CORESETs on the active DL BWP(s) of the one or more cells

-	the number of active TCI states is determined from the multiple CORESETs

If a UE

-	is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and

-	monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs where none of the CORESETs has TCI-states with 'QCL-TypeD',

the UE is required to monitor PDCCH candidates in overlapping PDCCH monitoring occasions for search space sets associated with different CORESETs.

For a scheduled cell and at any time, a UE expects to have received at most 16 PDCCHs for DCI formats 1_0 or 1_1 with CRC scrambled by C-RNTI, CS-RNTI, or MCS-RNTI scheduling 16 PDSCH receptions for which the UE has not received any corresponding PDSCH symbol and at most 16 PDCCHs for DCI formats 0_0 or 0_1 with CRC scrambled by C-RNTI, CS-RNTI, or MCS-RNTI scheduling 16 PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol.

If a UE

-	is not configured for NR-DC operation and indicates through pdcch-BlindDetectionCA a capability to monitor PDCCH candidates for  downlink cells and the UE is configured with  downlink cells or  uplink cells, or

-	is configured with NR-DC operation and for a cell group with  downlink cells or  uplink cells

the UE expects to have respectively received at most  PDCCHs for

-	DCI formats 1_0 or 1_1 with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-RNTI scheduling  PDSCH receptions for which the UE has not received any corresponding PDSCH symbol over all  downlink cells

-	DCI formats 0_0 or 0_1 with CRC scrambled by a C-RNTI, or a CS-RNTI, or a MCS-RNTI scheduling  PUSCH transmissions for which the UE has not transmitted any corresponding PUSCH symbol over all  uplink cells

If a UE

-	is configured to monitor a first PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a CSS set and a second PDCCH candidate for a DCI format 0_0 and a DCI format 1_0 from a USS set in a CORESET with index zero on an active DL BWP, and

-	the DCI formats 0_0/1_0 associated with the first PDCCH candidate and the DCI formats 0_0/1_0 associated with the second PDCCH candidate have same size, and

-	the UE receives the first PDCCH candidate and the second PDCCH candidate over a same set of CCEs, and

-	the first PDCCH candidate and the second PDCCH candidate have identical scrambling, and

-	the DCI formats 0_0/1_0 for the first PDCCH candidate and the DCI formats 0_0/1_0 for the second PDCCH candidate have CRC scrambled by either C-RNTI, or MCS-C-RNTI, or CS-RNTI

the UE decodes only the DCI formats 0_0/1_0 associated with the first PDCCH candidate.

If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format.

A UE configured with a bandwidth part indicator in DCI formats 0_1 or 1_1 determines, in case of an active DL BWP or of an active UL BWP change, the DCI information applicable to the new active DL BWP or UL BWP, respectively, as described in Subclause 12.

For unpaired spectrum operation, if a UE is not configured for PUSCH/PUCCH transmission on serving cell , the UE does not expect to monitor PDCCH on serving cell  if the PDCCH overlaps in time with SRS transmission (including any interruption due to uplink or downlink RF retuning time [10, TS 38.133]) on serving cell  and if the UE is not capable of simultaneous reception and transmission on serving cell and serving cell .

If a UE is provided resourceBlocks and symbolsInResourceBlock in RateMatchPattern, or if the UE is additionally provided periodicityAndPattern in RateMatchPattern, the UE can determine a set of RBs in symbols of a slot that are not available for PDSCH reception as described in [6, TS 38.214]. If a PDCCH candidate in a slot is mapped to one or more REs that overlap with REs of any RB in the set of RBs in symbols of the slot, the UE does not expect to monitor the PDCCH candidate.

*** Unchanged text is omitted ***

# 16	UE procedures for sidelink

A UE is provided by locationAndBandwidth-SL a BWP for SL transmissions (SL BWP) with numerology and resource grid determined as described in [4, TS38.211]. For a resource pool within the SL BWP, the UE is provided by numSubchannel a number of sub-channels where each sub-channel includes a number of contiguous RBs provided by subchannelsize. The first RB of the first sub-channel in the SL BWP is indicated by startRB-Subchannel. Available slots for a resource pool are provided by timeresourcepool and occur with a periodicity provided by ‘periodResourcePool’. For an available slot without S-SS/PBCH blocks, SL transmissions can start from a first symbol indicated by startSLsymbols and be within a number of consecutive symbols indicated by lengthSLsymbols. For an available slot with S-SS/PSBCH blocks, the first symbol and the number of consecutive symbols is predetermined.

The UE expects to use a same numerology in the SL BWP and in an active UL BWP in a same carrier of a same cell. If the active UL BWP numerology is different than the SL BWP numerology, the SL BWP is deactivated.

A UE transmitting using a Mode-1 grant uses the corresponding fields in SCI to reserve the next resource(s) allocated by the same grant.

A priority of a PSSCH according to NR radio access or according to E-UTRA radio access is indicated by a priority field in a respective scheduling SCI format. A priority of a PSSS/SSSS/PSBCH according to E-UTRA radio access is provided by LTESidelinkSSBPriority. A priority of an S-SS/PBCH block is provided by NRSidelinkSSBPriority. A priority of a PSFCH is same as the priority of a corresponding PSSCH.

## 16.1	Synchronization procedures

A UE receives the following SL synchronization signals in order to perform synchronization procedures based on S-SS/PSBCH blocks: SL primary synchronization signals (S-PSS) and SL secondary synchronization signals (S-SSS) [4, TS 38.211].

A UE assumes that reception occasions of a physical sidelink broadcast channel (PSBCH), S-PSS, and S-SSS are in consecutive symbols [4, TS 38.211] and form a S-SS/PSBCH block.

For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by absoluteFrequencySSB-SL. The UE assumes that S-PSS, S-SSS, PSBCH DM-RS, and PSBCH data have same EPRE. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PBCH block is aligned with a subcarrier with index 0 in the SL BWP.

A UE is provided, by numSSBwithinPeriod-SL, a number NperiodS-SSB of S-SS/PSBCH blocks in a period of 16 frames. The UE assumes that a transmission of the S-SS/PSBCH blocks in the period is with a periodicity of 16 frames. The UE determines indexes of slots that include S-SS/PSBCH block as NoffsetS-SSB+NintervalS-SSB⋅iS-SSB, where

-	index 0 corresponds to a first slot in a frame with SFN satisfying (SFN mod 16)=0

-	iS-SSB is a S-SS/PSBCH block index within the number of S-SS/PSBCH blocks in the period, with 0≤iS-SSB≤NperiodS-SSB-1

-	NoffsetS-SSB is a slot offset from a start of the period to the first slot including S-SS/PSBCH block, provided by timeOffsetSSB-SL

-	NintervalS-SSB is a slot interval between S-SS/PSBCH blocks, provided by timeIntervalSSB-SL

If a UE would transmit or receive an S-SS/PSBCH block or, for E-UTRA radio access, sidelink synchronization signals, and the transmission or reception would overlap in time with other transmissions and/or receptions on the sidelink, the UE transmits or receives the signal/channel with the higher priority.

## 16.2	Power control

A UE transmits a PSSCH, or a PSCCH, or a PSFCH with a same non-zero power in all corresponding symbols.

### 16.2.1	PSSCH

A UE determines a power PPSSCH,b,c(i) for a PSSCH transmission on a resource pool in PSSCH transmission occasion i as

PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PMAX,CBR is determined by a value of maximumtransmitPower-SL based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if maximumtransmitPower-SL is not provided, PMAX,CBR=0

-	if p0-DL-PSCCHPSSCH is provided

-	PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm]

-	elseif p0-SL-PSCCHPSSCH is provided

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR,PPSSCH,SL(i) [dBm]

-	else

-	PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm]

where

-	PO,D is a value of p0-DL-PSCCHPSSCH if provided

-	αD is a value of alpha-DL-PSCCHPSSCH, if provided; else, αD=1

-	PLD=PLb,f,c(qd) as described in Subclause 7.1.1

-	MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration

-	if p0-SL-PSCCHPSSCH and alpha-SL-PSCCHPSSCH are provided

-	PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm]

-	else

-	PPSSCH(i)=minPCMAX,PPSSCH,D(i) [dBm]

where

-	PO,SL is a value of p0-SL-PSCCHPSSCH, if provided

-	αSL is a value of alpha-SL-PSCCHPSSCH, if provided; else, αSL=1

-	PLSL= referenceSignalPower – higher layer filtered RSRP, where

-	referenceSignalPower is …, and

-	higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by filterCoefficient-SL

-	MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration

### 16.2.2	PSCCH

TBD

### 16.2.3	PSFCH

A UE determines a power PPSFCH(i) for a PSFCH transmission on a resource pool in PSFCH transmission occasion i as

PPSFCH(i)=minPCMAX,PO,PSFCH+10log102μ+αPFSCH⋅PL [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PO,PSFCH is a value of p0-DL-PSFCH if provided; else, PPSFCH(i)=PCMAX

-	αPFSCH is a value of alpha-DL-PSFCH, if provided; else, αPFSCH=1

-	PL=PLb,f,c(qd) as described in Subclause 7.1.1

### 16.2.4	Prioritization of transmissions/receptions

#### 16.2.4.1	Simultaneous NR and E-UTRA transmission/reception

## If a UE

-	would transmit a first channel/signal using E-UTRA radio access and a second channel/signal using NR radio access, and

-	a transmission of the first channel/signal would overlap in time with a transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the earlier of the two transmissions

the UE transmits only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

## If a UE

-	would respectively transmit or receive a first channel/signal using E-UTRA radio access and receive or transmit a second channel/signal using NR radio access, and

-	a transmission or reception of the first channel/signal would respectively overlap in time with a reception or transmission of the second channel/signal, and

-	the priorities of the two channels/signals are known to the UE T msec prior to the start of the later transmission or reception

the UE transmits or receives only the channel/signal with the higher priority as determined by the SCI formats scheduling the transmissions or, in case of a S-SS/PSBCH block or a sidelink synchronization signal using E-UTRA radio access, as indicated by higher layers.

#### 16.2.4.2	Simultaneous PSFCH transmission/reception

## If a UE

-	would transmit a first PSFCH and receive a second PSFCH, and

-	a transmission of the first PSFCH would overlap in time with a reception of the second PSFCH

the UE transmits or receives only the PSFCH with the higher priority as determined by a first SCI format 0_1 and a second SCI format 0_1 [5, TS 38.212] that are respectively associated with the first PSFCH and the second PSFCH.

## If a UE

-	would transmit a first PSFCH to a first UE and a second PSFCH to a second UE, and

-	a transmission of the first PSFCH would overlap in time with a transmission of the second PSFCH

the transmits only the PSFCH with the higher priority as determined by a first SCI format 0_1 and a second SCI format 0_1 [5, TS 38.212] that are respectively associated the first PSFCH and the second PSFCH.

## If a UE

-	would transmit a first PSFCH and a second PSFCH to another UE, and

-	a transmission of the first PSFCH would overlap in time with a transmission of the second PSFCH

the UE transmits only the PSFCH with the higher priority as determined by a first SCI format 0_1 and a second SCI format 0_1 that are respectively associated the first PSFCH and the second PSFCH.

#### 16.2.4.3	Simultaneous SL and UL transmissions

## If a UE

-	would simultaneously transmit on the UL and on the SL of a serving cell, and

-	the UE is not capable of simultaneous transmissions on the UL and on the SL of the serving cell

the UE transmits only on the link, UL or SL, with the higher priority.

## If a UE

-	would transmit on the UL and on the SL of two respective carriers of a serving cell, or of two respective serving cells,

-	the transmission on the UL would overlap with the transmission on the SL over a time period, and

-	the total UE transmission power over the time period would exceed PCMAX

the UE

-	reduces the power for the UL transmission prior to the start of the UL transmission, if the SL transmission has higher priority than the UL transmission, so that the total UE transmission power would not exceed PCMAX

-	reduces the power for the SL transmission prior to the start of the SL transmission, if the UL transmission has higher priority than the SL transmission, so that the total UE transmission power would not exceed PCMAX

## 16.3	UE procedure for reporting HARQ-ACK on sidelink

A UE can be indicated by an SCI format scheduling a PSSCH, in one or more sub-channels from a number of Nsubch PSSCH sub-channels, to transmit a PSFCH with HARQ-ACK information in response to the PSSCH reception. The UE provides HARQ-ACK information that includes ACK or NACK, or only NACK.

## A UE can be provided, by periodPSFCHresource, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions in the resource pool are disabled.

A UE may be indicated by higher layers to not transmit a PSFCH in response to a PSSCH reception [11, TS 38.321].

If a UE receives a PSSCH in a resource pool and a ZYX field in a SCI format 02 scheduling the PSSCH reception indicates to the UE to report HARQ-ACK information for the PSSCH reception [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception.

A UE is provided, by rbSetPSFCH, a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission in a PRB of the resource pool. For a number of Nsubch sub-channels for the resource pool, provided by numSubchannel, and a number of NPSSCHPSFCH PSSCH slots associated with a PSFCH slot, provided by periodPSFCHresource, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i  and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j.

A UE determines a number of PSFCH resources available for multiplexing HARQ-ACK information in a PSFCH transmission as RPRB, CSPSFCH=Ntype PSFCH⋅Msubch, slotPSFCH⋅NCSPSFCH where NCSPSFCH is a number of cyclic shift pairs for the resource pool and, based on an indication by higher layers,

-	Ntype PSFCH=1 and the Msubch, slotPSFCH PRBs are in one sub-channel

-	Ntype PSFCH=Nsubch PSSCH and the Nsubch PSSCH⋅Msubch, slotPSFCH are located in one or more sub-channels from the Nsubch PSSCH sub-channels

The UE applies one cyclic shift from a cyclic shift pair to a sequence used for the PSFCH transmission [4, TS 38.211]. The PSFCH resources are first indexed according to an ascending order of the PRB index, from the Ntype PSFCH⋅Msubch, slotPSFCH PRBs, and then according to an ascending order of the cyclic shift pair index from the NCSPSFCH cyclic shift pairs.

A UE determines an index of a PSFCH resource for a PSFCH transmission in response to a PSSCH reception, using a sequence associated with the resource pool [4, TS 38.211], as PID+MIDmodRPRB, CSPSFCH where PID is a physical layer source ID provided by SCI format 0-2 [5, TS 38.212] scheduling the PSSCH reception, MID is zero or MID is the identity of the UE receiving the PSSCH as indicated by higher layers.

A UE determines a mcs value, for computing a value of cyclic shift α [4, TS 38.211], from a cyclic shift pair of a PSFCH resource as in Table 16.6-1.

Table 16.6-1: Mapping of HARQ-ACK information bit values to a cyclic shift, from a cyclic shift pair, of a sequence for a PSFCH transmission

| HARQ-ACK Value | 0 (NACK) | 1 (ACK) |
| --- | --- | --- |
| Sequence cyclic shift | First cyclic shift | Second cyclic shift |

## 16.4	UE procedure for transmitting PSCCH

A UE can be provided a number of symbols in a resource pool, by timeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by frequencyResourcePSCCH, for a PSCCH transmission with a SCI format 0_1.

## 16.5	UE procedure for reporting HARQ-ACK on uplink

A UE can be provided PUCCH resources [12, TS 38.331] to report HARQ-ACK information to a serving gNB that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions.

For SL configured grant Type 1 or Type 2 PSSCH receptions by a UE within a time period provided by periodSlCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources.

For each PSFCH reception occasion, from a number of PSFCH reception occasions that the UE generates HARQ-ACK information to report to the gNB in a PUCCH or PUSCH transmission, the UE can be indicated by higher layers to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable.

-	generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK

-	generate ACK when the UE determines ACK from each PSFCH reception for the number of PSFCH reception occasions; otherwise, generate NACK if the UE determines absence of PSFCH reception or determines a NACK value from a PSFCH reception at a corresponding PSFCH reception occasion

-	generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK

With reference to slots for PUCCH transmissions and for a number of PSFCH reception occasions ending in slot n, the UE provides the generated HARQ-ACK information in a PUCCH transmission within slot n+k, subject to the overlapping conditions in Subclause 9.2.5, where k is a number of slots indicated by a PSFCH-to-HARQ_feedback timing indicator field, if present, in a DCI format indicating a slot for PUCCH transmission to report the HARQ-ACK information, or k is provided by sl-ACKToUL-ACK. k=0 corresponds to a last slot for a PUCCH transmission that would overlap with the last PSFCH reception occasion.

For a PSSCH reception by a UE that is scheduled by a DCI format, or for a SL configured grant Type 2 PSCCH reception activated by a DCI format, the DCI format can indicate to the UE that a PUCCH resource is not provided. For a SL configured grant Type 1 PSSCH reception, a PUCCH resource can be provided PUCCH-SL-Config. If a PUCCH resource is not provided, the UE does not transmit a PUCCH with generated HARQ-ACK information from PSFCH reception occasions.