3GPP TSG RAN Meeting #86	RP-193129

# Sitges, Spain, December 9th – 12th, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0080 | rev | 1 | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of positioning support in NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Samsung |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2019-12-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of positioning support. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description for power determination of SRS transmission related to positioning support and capture that a UE does not receive positioning RS if corresponding symbols are indicated as UL by TDD configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete support for positioning. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7, 7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, 38.214, 38.215 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1-1913681 |  |  |  |  |  |  |  |  |

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Subclauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config as described in Subclause 7.3.1.

A PUSCH/PUCCH/SRS/PRACH transmission occasion  is defined by a slot index  within a frame with system frame number , a first symbol  within the slot, and a number of consecutive symbols .

## 7.3	Sounding reference signals

For SRS, a UE splits a linear value  of the transmit power  on active UL BWP  of carrier  of serving cell  equally across the configured antenna ports for SRS.

### 7.3.1	UE behaviour

If a UE transmits SRS based on a configuration by IE SRS-Config on active UL BWP  of carrier  of serving cell  using SRS power control adjustment state with index , the UE determines the SRS transmission power  in SRS transmission occasion  as

[dBm]

where,

-	 is the UE configured maximum output power defined in [8, TS 38.101-1], [8-2, TS38.101-2] and [TS 38.101-3] for carrier  of serving cell  in SRS transmission occasion

-	 is provided by p0 for active UL BWP  of carrier  of serving cell  and SRS resource set  provided by SRS-ResourceSet and SRS-ResourceSetId

-	 is a SRS bandwidth expressed in number of resource blocks for SRS transmission occasion  on active UL BWP  of carrier  of serving cell  and  is a SCS configuration defined in [4, TS 38.211]

-	 is provided by alpha for active UL BWP  of carrier  of serving cell  and SRS resource set

-	 is a downlink pathloss estimate in dB calculated by the UE using RS resource index  as described in Subclause 7.1.1 for the active DL BWP of serving cell  and SRS resource set  [6, TS 38.214]. The RS resource index  is provided by pathlossReferenceRS associated with the SRS resource set  and is either a ssb-Index providing a SS/PBCH block index or a csi-RS-Index providing a CSI-RS resource index

-	If the UE is not provided pathlossReferenceRS or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB

-	If the UE is provided pathlossReferenceLinking, the RS resource is on a serving cell indicated by a value of pathlossReferenceLinking

-	For the SRS power control adjustment state for active UL BWP  of carrier  of serving cell  and SRS transmission occasion

-	, where  is the current PUSCH power control adjustment state as described in Subclause 7.1.1, if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions; or

-	 if the UE is not configured for PUSCH transmissions on active UL BWP  of carrier  of serving cell , or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and if tpc-Accumulation is not provided, where

-	The  values are given in Table 7.1.1-1

-	 is jointly coded with other TPC commands in a PDCCH with DCI format 2_3, as described in Subclause 11.4

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before SRS transmission occasion  and  symbols before SRS transmission occasion  on active UL BWP  of carrier  of serving cell  for SRS power control adjustment state, where  is the smallest integer for which  symbols before SRS transmission occasion  is earlier than  symbols before SRS transmission occasion

-	if the SRS transmission is aperiodic,  is a number of symbols for active UL BWP  of carrier  of serving cell  after a last symbol of a corresponding PDCCH triggering the SRS transmission and before a first symbol of the SRS transmission

-	if the SRS transmission is semi-persistent or periodic,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of serving cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of serving cell  at SRS transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of serving cell  at SRS transmission occasion  and , then

-	If a configuration for a  value or for a  value for a corresponding SRS power control adjustment state  for active UL BWP  of carrier  of serving cell  is provided by higher layers

-

-	Else

-

where

is the TPC command value indicated in the random access response grant corresponding to the random access preamble that the UE transmitted on active UL BWP  of carrier  of the serving cell , and

;

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of serving cell .

-	 if the UE is not configured for PUSCH transmissions on active UL BWP  of carrier  of serving cell , or if srs-PowerControlAdjustmentStates indicates separate power control adjustment states between SRS transmissions and PUSCH transmissions, and tpc-Accumulation is provided, and the UE detects a DCI format 2_3  symbols before a first symbol of SRS transmission occasion , where absolute values of  are provided in Table 7.1.1-1

-	if srs-PowerControlAdjustmentStates indicates a same power control adjustment state for SRS transmissions and PUSCH transmissions, the update of the power control adjustment state for SRS transmission occasion  occurs at the beginning of each SRS resource in the SRS resource set ; otherwise, the update of the power control adjustment state SRS transmission occasion  occurs at the beginning of the first transmitted SRS resource in the SRS resource set .

If a UE transmits SRS based on a configuration by IE SRS-Positioning-Config on active UL BWP b of carrier f of serving cell c, the UE determines the SRS transmission power PSRS,b,f,ci,qs in SRS transmission occasion i as

[dBm]

where,

-	PO,SRS,b,f,cqs and αO,SRS,b,f,cqs are provided by p0 and alpha respectively, for active UL BWP b of carrier f of serving cell c, and SRS resource set qs is indicated by SRS-ResourceSetId from SRS-ResourceSet, and

-	PLb,f,cqd is a downlink pathloss estimate in dB calculated by the UE, as described in Subclause 7.1.1 in case of an active DL BWP of a serving cell c, using RS resource indexed qd in a serving or non-serving cell for SRS resource set qs [6, TS 38.214]. A configuration for RS resource index qd associated with SRS resource set qs is provided by pathlossReferenceRS

-	if a ssb-Index is provided, referenceSignalPower is provided by ss-PBCH-BlockPower

-	if a dl-PRS-ResourceId is provided, referenceSignalPower is provided by dl-PRS-ResourcePower

If the UE determines that the UE is not able to accurately measure PLb,f,cqd, the UE calculates PLb,f,cqd using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB

The UE indicates a capability for a number of pathloss estimates that the UE can simultaneously maintain.

## *** Unchanged text is omitted ***