3GPP TSG RAN meeting #86										     RP-193189

Sitges, Spain, December 9-12, 2019						(revision of RP-192762)

Status Report to TSG

Agenda item:			9.4.2

| WI / SI Name | Cross Link Interference (CLI) handling and Remote Interference Management (RIM) for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_CLI_RIM |  |  |  |  |
| Unique ID | 800082 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-191997 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2020 | Performance part: 06/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 90% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Hyunsoo Ko |
|  | Company | LG Electronics |
|  | Email | hyunsoo.ko@lge.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | YES |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#99

Following 3 CRs for CLI/RIM were agreed.

R1-1912986	Introduction of cross layer interference measurements (38.215, CR0008, Rev.1)	Intel Corporation (UK) Ltd

R1-1912988	Introduction of remote interference management (38.211, CR0020, Rev. 1)	Ericsson

R1-1912989	Introduction of UE behaviour for SRS measurements for CLI (38.214, CR0043, Rev. 1)	Nokia

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#107bis

18 contributions were submitted to RAN2#107bis.

An LS to RAN3 was agreed:

- R2-1914021	LS to RAN3 For Exchange of Information related to UE CLI measurement configuration and reporting.

Detail agreements are as follows:

- Multiple CLI measurement objects can be configured for an UE

- SRS-RSRP results and CLI-RSSI results cannot be included in the same measurement report. (I.e. report quantity cannot be set to ‘SRS-RSRP and CLI-RSSI’.)

- s-measure is not applicable to CLI measurements.

- Define a UE radio capability to indicate whether serving cell DL signal/channel (e.g. PDSCH/PDCCH) and SRS-RSRP FDMed reception is supported. FFS - UE behavior when DL signal/channel and SRS RSRP are FDMed for UEs indicating the FDMed reception is not supported.

- Define a UE radio capability to indicate whether serving cell DL signal/channel (e.g. PDSCH/PDCCH) and CLI-RSSI FDMed reception is supported. FFS - UE behavior when DL signal/channel and CLI RSSI are FDMed for UEs indicating the FDMed reception is not supported.

RAN2#108

22 contributions were submitted to RAN2#108.

An LS to RAN1 was agreed:

- R2-1916348	LS on CLI measurements UE capabilities

Detail agreements are as follows:

- FR1/FR2 diff should be "Yes" in general for all CLI capabilities.

- UE shall prioritize the DL transmission in case DL signal/channel and SRS RSRP resources are FDMed for UEs indicating the FDMed reception is not supported.

- UE shall prioritize the DL transmission in case DL signal/channel and CLI RSSI resources are FDMed for UEs indicating the FDMed reception is not supported.

- We reconfirm that CLI measurements can be configured in (NG)EN-DC and NE-DC

- CLI measurements can be configured in NR-DC (and NR CA). No additional RAN3 work is expected for this.

- In NR-DC, both MN and SN can configure CLI measurement at the same time. Network ensures the CLI measurements configured by MN and SN do not go beyond UE's capability.

- For CLI measurement coordination, MN informs SN in CG-ConfigInfo:

o	Maximum number of SRS RSRP resources that can be configured by SN;

o	Maximum number of CLI-RSSI resources that can be configured by SN.

Following CRs were endorsed as running CR

- R2-1916347	Introduction of cross link interfernce management	38.300

- R2-1916340	Introduction of cross link interference	37.340

- R2-1915716	Introduction of Cross Link Interference (CLI) handling and Remote Interference Management (RIM)	38.306

2.2.2	Remaining Open issues

- Whether additional information needs to be configured for SRS RSRP resource to indicate the position of the SRS RSRP resource in frequency-domain.

2.3	RAN3

2.3.1	Agreements

RAN3#106

CLI:

- The following CRs are agreed.

  - CR to TS 38.401 in R3-197513

  - CR to TS 38.423 in R3-196441

  - CR to TS 38.470 in R3-196443

  - CR to TS 38.473 in R3-196446

Following request from RAN2 in R3-196406, RAN3 analysed the information exchange required between gNBs and CU-DU for SRS-RSRP measurements, with no consensus.

RIM:

- The following CRs and the following LS are agreed.

  - CR to TS 38.401 in R3-197542

  - CR to TS 38.410 in R3-196447

  - CR to TS 38.413 in R3-196453

  - CR to TS 38.470 in R3-196598

  - CR to TS 38.473 in R3-197541

  - LS to SA5 in R3-197540

- The following CR is endorsed.

  - CR to TS 38.300 in R3-196444

Reply LS to SA5 on OAM requirements was agreed in R3-197540.

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4#92bis

23 contributions were submitted to RAN4#92bis.

In RRM session, the WF on CLI RRM was agreed, reply LS to RAN2 was agreed, and one draft CR for CLI abbreviation was endorsed.

Detail agreements are as follows:

- Definition for DL timing error

- Residual timing error for measurement accuracy

- The required number of symbol for scheduling restriction during CLI measurement

The reporting range for SRS-RSRP and CLI-RSSI were agreed and send LS to RAN2

RAN4#93

11 contributions were submitted to RAN4#93.

In RRM session, 3 CRs for CLI core part was agreed, and remaining issues for RRM core part were cleared.

Detail agreements are as follows:

- 1dB SNR side condition and 3 samples averaging for SRS-RSRP measurement accuracy

  - Define separate SRS-RSRP measurement accuracy per frequency range

  - Define separate SRS-RSRP measurement accuracy depending on SCS

Based on these agreements, following CRs were agreed

- R4-1915516	CR for Abbreviations for cross link interference (section 3)

- R4-1915517	CR for cross link interference measurements (section 9)

- R4-1915780	CR to introduce CLI measurement accuracy requirements

2.4.2	Remaining Open issues

RRM performance aspect

- Define measurement accuracy for CLI measurement

- Define test cases for CLI measurement

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#98bis

- R1-1909958	Reply LS on measurement gap for SRS-RSRP or CLI-RSSI measurements	RAN4, LG Electronics

- R1-1910891	Introduction of remote interference management	Ericsson

- R1-1910896	Introduction of UE behaviour for SRS measurements for CLI	Nokia

- R1-1911239	Introduction of cross layer interference measurements	Intel Corporation (UK) Ltd

RAN1#99

- R1-1911816	LS For Exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI	RAN2, Nokia

- R1-1912986	Introduction of cross layer interference measurements (38.215, CR0008, Rev.1)	Intel Corporation (UK) Ltd

- R1-1912988	Introduction of remote interference management (38.211, CR0020, Rev. 1)	Ericsson

- R1-1912989	Introduction of UE behaviour for SRS measurements for CLI (38.214, CR0043, Rev. 1)	Nokia

RAN2 #107bis

- R2-1912039	Reply LS on measurement gap for SRS-RSRP or CLI-RSSI measurements LGE

- R2-1914168	LS on CLI measurement reporting range	LGE

- R2-1913714	Introduction of cross link interfernce management	Huawei, HiSilicon

- R2-1913535	Draft CR on CLI for TS 38.306	Ericsson

- R2-1913748	draft CR on CLI for TS38 331_v3_Alt1	LGE

- R2-1913749	draft CR on CLI for TS38 331_v3_Alt2	LG Electronics Inc

- R2-1914012	draft CR on CLI for TS38 331_v3_AltX	LG Electronics Inc

- R2-1913747	Remaining issues on CLI	LG Electronics Inc

- R2-1912775	Remaining issues on CLI measurement	ZTE

- R2-1913393	Too strong interference during CLI-RSSI/SRS-RSRP measurement	Qualcomm

- R2-1913536	Remaining issues on CLI	Ericsson

- R2-1913537	On s measure for CLI	Ericsson

- R2-1913715	Leftover issues on CLI	Huawei, HiSilicon

- R2-1912408	UE CLI Measurement Configuration, Reporting and NW Signalling	Nokia

- R2-1914013	Draft LS to RAN3 For Exchange of Information related to UE CLI measurement configuration and reporting	Nokia

- R2-1914021	LS to RAN3 For Exchange of Information related to UE CLI measurement configuration and reporting	Nokia

- R2-1912409	LS to RAN3 For Exchange of Information related to UE CLI measurement configuration and reporting	Nokia

- R2-1913392	CLI measurements UE capabilities	Qualcomm

RAN2 #108

- R2-1914333	LS on CLI measurement reporting range (R4-1911416; contact: LGE

- R2-1915882	Introduction of cross link interfernce management	Huawei, HiSilicon

- R2-1916331	Introduction of cross link interfernce management	Huawei, HiSilicon

- R2-1916347	Introduction of cross link interfernce management	Huawei, HiSilicon

- R2-1916340	Introduction of cross link interfernce management	Huawei, HiSilicon, ZTE

- R2-1915715	Report of [107bis#58]CLI measurements UE capabilities	Qualcomm

- R2-1915716	Introduction of Cross Link Interference (CLI) handling and Remote Interference Management (RIM)	Qualcomm

- R2-1916258	[DRAFT] LS on CLI measurements UE capabilities	Qualcomm

- R2-1916332	[DRAFT] LS on CLI measurements UE capabilities	Qualcomm

- R2-1916348	LS on CLI measurements UE capabilities	Qualcomm

- R2-1916211	Introduction of Cross Link Interference (CLI) handling and Remote Interference Management (RIM)	LGE

- R2-1916558	Report of offline discussion #202	LG Electronics Inc

- R2-1916559	[DRAFT] LS on remaining CLI issues LG Electronics Inc

- R2-1916338	Introduction of Cross Link Interference (CLI) handling and Remote Interference Management (RIM)	LG Electronics Inc

- R2-1916128	Remaining issues on CLI measurement	ZTE

- R2-1914590	UE CLI Measurements for Multiconnectivity	Nokia

- R2-1915881	Leftover issues on CLI	Huawei, HiSilicon

- R2-1916246	Remaining issues on CLI	Ericsson

- R2-1915714	Too strong interference during CLI-RSSI measurement	Qualcomm

- R2-1914554	Open issues for CLI measurment	OPPO

- R2-1914568	Draft LS on configuration exchange for CLI measurement	OPPO

- R2-1914591	Remaning Issues for UE-CLI	Nokia

RAN3#105bis

- OAM Requirements for RIM

- R3-195229, Discussion on LS reply from 3GPP SA5 to RAN1 and RAN3 on OAM requirements for RIM, Huawei

- R3-195230, [Draft] Reply LS on LS reply from 3GPP SA5 to RAN1 and RAN3 on OAM requirements for RIM, Huawei

- R3-195713, OAM requirements for RIM, ZTE

- R3-195833, Discussion on reply LS from SA5 on OAM RIM requirements, CMCC

- R3-195834, [Draft] Reply LS to SA5 LS on clarification of OAM requirements for RIM, CMCC

- R3-195987, Answers to SA5 on OAM requirements for RIM, Nokia, Nokia Shanghai Bell

- R3-195988, [DRAFT] Reply LS on clarification of OAM requirements for RIM, Nokia, Nokia Shanghai Bell

- R3-196105, [DRAFT] Reply LS on clarification of OAM requirements for RIM, Nokia, Nokia Shanghai Bell

- R3-196249, Summary of discussion at RAN3#105bis on RIM, Nokia, Nokia Shanghai Bell

RAN3#106

- Rel-16 CRs Technically Endorsed from Previous Meetings

- R3-196441, BL CR to 38.423: CLI support on XnAP, LG Electronics, ZTE, Ericsson

- R3-196442, CR TS 38.300 Cross-link Interference Management, Ericsson

- R3-196443, Intended DL&UL configuration for TS38.470, ZTE, LGE, Ericsson

- R3-196444, CR TS 38.300 Remote Interference Management, Ericsson

- R3-196445, Introduction of Remote Interference Management, Huawei

- R3-196446, BL CR to 38.473: Support for CLI, Ericsson, ZTE

- R3-196447, Remote Interference Management Message Transfer Support, ZTE, Ericsson

- R3-196448, CR to F1-AP for RIM new message, Samsung

- R3-196453, CR to 38.413 for signalling design for RIM, Huawei, Nokia, Nokia Shanghai Bell

- R3-196598, Remote Interference Management Message Transfer Support, Ericsson

- R3-197491, Response to R3-196442: Introduction of Cross-link Interference Management, Huawei

- New Incoming LSs

- R3-196406, LS For Exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI, 3GPP RAN2, Nokia

- R3-196785, SRS configuration exchange for CLI measurement configuration, Qualcomm Incorporated

- R3-196850, Network signalling related to SRS-RSRP measurements, Nokia, Nokia Shanghai Bell

- R3-196851, [DRAFT] Reply LS on exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI, Nokia, Nokia Shanghai Bell

- R3-196852, [TP for TS 38.423] Introduction of SRS-RSRP configuration, Nokia, Nokia Shanghai Bell

- R3-196853, [TP for TS 38.473] Introduction of SRS-RSRP configuration, Nokia, Nokia Shanghai Bell

- R3-197194, [Draft] Reply LS on Exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI, Ericsson

- R3-197492, Response to R3-196850, R3-196851, R3-196852, R3-196853 and R3-196785, Ericsson

- R3-197513, Introduction of Cross-link Interference Management, Huawei

- R3-197524, Summary of discussion on SRS-RSRP configuration, Nokia, Nokia Shanghai Bell

- R3-197744, Summary of discussion on SRS-RSRP configuration, Nokia, Nokia Shanghai Bell

- OAM Requirements for RIM

- R3-196848, Summary of discussion prior to RAN3#106 on RIM, Nokia, Nokia Shanghai Bell

- R3-196849, [DRAFT] Reply LS on clarification of OAM requirements for RIM, Nokia, Nokia Shanghai Bell

- R3-197122, CR to F1-AP for RIM new message, Samsung

- R3-197209, Correction to RIM BL CR for TS 38.401, Huawei

- R3-197540, Reply LS on clarification of OAM requirements for RIM, Nokia, Nokia Shanghai Bell

- R3-197541, CR to F1-AP for RIM new message, Samsung, Ericsson, CMCC, Nokia, ZTE

- R3-197542, Introduction of RIM in TS 38.401, Huawei, CMCC, Ericsson, Nokia, Samsung, ZTE, LG

RAN4#92bis

- R4-1911349	Remaining issues for CLI measurements	LG Electronics Inc.

- R4-1911412	LS on SRS-RSRP measurement objective for CLI	LG Electronics Inc.

- R4-1911414	Discussion on required measurement period for CLI	LG Electronics Inc.

- R4-1911415	Discussion on reporting range for CLI measurement	LG Electronics Inc.

- R4-1911416	LS on CLI measurement reporting range	LG Electronics Inc.

- R4-1911417	Discussion on scheduling restriction for CLI measurement	LG Electronics Inc.

- R4-1911420	Draft CR for cross link interference measurements performance requirements (scetion 10)	LG Electronics Inc.

- R4-1911442	Draft CR for Abbreviations for cross link interference (scetion 3)	LG Electronics Inc.

- R4-1911628	Scheduling restriction for CLI measurement	NEC

- R4-1911632	Discussion on TCLI side condition and measurement accuracy requirements	NEC

- R4-1911718	Timing Error as side condition for CLI SRS-RSRP measurement	Nokia, Nokia Shanghai Bell

- R4-1911719	Simulation results for timing error impact on CLI measurement	Nokia, Nokia Shanghai Bell

- R4-1911720	RRM Core requirements for CLI SRS-RSRP measurements	Nokia, Nokia Shanghai Bell

- R4-1911721	CR to 38.133 introduction of Cross Link Interference measurement	Nokia, Nokia Shanghai Bell

- R4-1911968	Discussion on timing error in CLI measurement	Huawei, HiSilicon

- R4-1911969	Discussion on remaining issues in CLI measurement	Huawei, HiSilicon

- R4-1911970	CR to introduce CLI measurement requirements	Huawei, HiSilicon

- R4-1912379	On CLI measurements	Ericsson

- R4-1912536	Discussion of remaining issues on CLI	Qualcomm Incorporated

- R4-1912667	Way forward on CLI remaining RRM open issues	LGE

- R4-1912794	CR to introduce CLI measurement accuracy requirements	Huawei, HiSilicon

- R4-1912795	Draft CR for cross link interference measurements (scetion 9)	LG Electronics Inc.

- R4-1912796	Draft CR for conditions for cross link interference measurements (scetion B)	LG Electronics Inc.

RAN4#93

- R4-1913936	Discussion on SRS-RSRP measurement requirements for CLI	LG Electronics Inc.

- R4-1913937	Simulation results for SRS-RSRP measurement accuracy	LG Electronics Inc.

- R4-1913940	CR for conditions for cross link interference measurements (section B)	LG Electronics Inc.

- R4-1914001	Discussion on number of samples for CLI measurement accuracy	NEC

- R4-1914457	Open issues for CLI SRS-RSRP measurement	Nokia, Nokia Shanghai Bell

- R4-1914458	Simulation results for timing error impact on CLI measurement	Nokia, Nokia Shanghai Bell

- R4-1914942	Discussion on remaining issues in CLI measurement	Huawei, HiSilicon

- R4-1915059	On remaining CLI RRM open issues	Ericsson

- R4-1915516	CR for Abbreviations for cross link interference (section 3)	LG Electronics Inc.

- R4-1915517	CR for cross link interference measurements (section 9)	LG Electronics Inc.

- R4-1915780	CR to introduce CLI measurement accuracy requirements	Huawei, HiSilicon

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template