3GPP TSG-RAN WG1 Meeting #101-e	R1-2005145

# e-Meeting, May 25 – June 5, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0109 | rev | 1 | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on MIMO enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2020-06-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The following updates and corrections are required: - Replace Ncells,0DL+Ncells,1DL with Ncells,0DL,μ+Ncells,1DL,μ in Clause 10.1 to avoid incorrect determination for number of PDCCH candidates and overlapping CCEs per cell ([100b-e-NR-eMIMO-multiTRP-01]) - Remove text related to a UE not reporting R for a PDCCH monitoring capability associated with first and second scheduling cells corresponding to first and second CORESETPoolIndex values in Clause 10 because UE always reports R ([100b-e-NR-eMIMO-multiTRP-01]) - Clarify and simplify text for search space set indexing in Clause 9.1.3.1 and achieve conistent behavior for PDCCH monitoring with intended on when a UE is also provided CORESETPoolIndex ([100b-e-NR-eMIMO-multiTRP-02]) - Capture counter/total DAI accumulation when a UE monitors PDCCH on scheduling cells corresponding to first and second CORESETPoolIndex values and the UE is configured for joint HARQ-ACK feedback in Clause 9.1.3.1 ([100b-e-NR-eMIMO-multiTRP-02]) - Include RepNumR16, in the parameters used for a UE to determine PDCCH monitoring occasions for Type-2 HARQ-ACK codebook in Clause 9.1.3.1 ([100b-e-NR-eMIMO-multiTRP-02]) - Capture application time for pathloss when a corresponding RS resource is updated by MAC-CE ([100b-e-NR-eMIMO-MB1-01]) - Capture the RS resources a UE is required to use for pathloss measurements when the UE is configured a number of RS resources that is larger than 4 in Clause 7 ([100b-e-NR-eMIMO-MB1-02]) - Clarify that the “lowest ID CORESET” in clauses 7.1.1 and 7.3.1 refers to “the lowest ID CORESET of the active DL BWP of serving cell c” ([100b-e-NR-eMIMO-MB1-03]) - Align RRC parameters names for multi-cell TCI/spatial relation update and clarify that a number of lists of cells for simultaneous TCI state activation is up to two in Clause 10.1 ([100b-e-NR-eMIMO-MB1-03]) - Clarify that default spatial relation applies “at least for the single TRP case” in Clauses 7.2.1, 7.3.1 and 9.2.2 and avoid mixing “primary cell” and “serving cell” (use consistently “primary cell”) in statements related to determination of PUCCH transmission power ([100b-e-NR-eMIMO-MB1-03]) - Align with the description for the SCell MAC CE in TS 38.321 for SCell link recovery and add description for a candidate beam availability indication field to indicate whether or not qnew is available for a corresponding SCell in Clause 6 ([100b-e-NR-eMIMO-MB2-03]) - Alignment with RRC parameter names in Clause 7.1 ([100b-e-NR-eMIMO-ULFPTx-01]) - Clarify power scaling behaviour in Mode2 when SRI indicates SRS resource with single port in Clause 7.1 ([100b-e-NR-eMIMO-ULFPTx-03]) - Delete the condition of UE reporting beamCorrespondenceWithoutUL-BeamSweeping as an enabling condition for default PL RS/spatial relation for PUCCH to align with ones for SRS in Clause 9.2.2 ([101-e-NR-eMIMO-MB1-01]) - Capture that if UE is provided enableDefaultBeamPlForSRS and is not provided PUSCH-PathlossReferenceRS or PUSCH-PathlossReferenceRS-r16, and a PUSCH is scheduled by DCI format 0_1, the UE uses same RS resource index qd for PUSCH as for SRS resource set with associated SRS resource in Clause 7.1.1 ([101-e-NR-eMIMO-MB1-03]) - Capture that for multi-slot PUCCH, a same spatial relation/PL RS applies in all PUCCH slots in Clauses 7.2.1 and 9.2.2 ([101-e-NR-eMIMO-MB1-03]) - Align parameter names with TS 38.331 in Clause 6 ([101-e-NR-eMIMO-MB2-01]) - Capture that a UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in set in Clause 6 ([101-e-NR-eMIMO-MB2-02]) - Add description of SRI indication for Type1 configured grant for full power UL transmission in Clause 7.1 ([101-e-NR-eMIMO-ULFPTx-02]) - Clarify that power scaling for PUSCH does not apply if the PUSCH is scheduled by a DCI format 0_0 in Clause 7.1 ([101-e-NR-eMIMO-ULFPTx-02]) - Remove the conditions that “codebook subset in PUSCH-Config set to nonCoherent or partialAndNonCoherent” in order to not limit applicability of power scaling to other cases in Clause 7.1 ([101-e-NR-eMIMO-ULFPTx-03]) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the above updates and corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete/incorrect support for MIMO enhancements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6, 7, 7.1, 7.1.1, 7.2.1, 7.3.1, 9.1.3.1, 9.2.2, 10, 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.211, TS 38.212, TS 38.214 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In revision of R1-2003181 |  |  |  |  |  |  |  |  |

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1, TR 21.905].

BPRE	Bits per resource element

BWP	Bandwidth part

CB	Code block

CBG	Code block group

CBR	Channel busy ratio

CCE	Control channel element

CORESET	Control resource set

CP	Cyclic prefix

CRC	Cyclic redundancy check

CSI	Channel state information

CSS	Common search space

DAI	Downlink assignment index

DC	Dual connectivity

DCI	Downlink control information

DL	Downlink

DL-SCH	Downlink shared channel

EPRE	Energy per resource element

EN-DC	E-UTRA NR dual connectivity with MCG using E-UTRA and SCG using NR

FR1	Frequency range 1

FR2	Frequency range 2

GSCN	Global synchronization channel number

HARQ-ACK	Hybrid automatic repeat request acknowledgement

MCG	Master cell group

MCS	Modulation and coding scheme

NDI	New Data Indicator

NE-DC	E-UTRA NR dual connectivity with MCG using NR and SCG using E-UTRA

NR-DC	NR NR dual connectivity

PBCH	Physical broadcast channel

PCell	Primary cell

PDCCH	Physical downlink control channel

PDSCH	Physical downlink shared channel

PRACH	Physical random access channel

PRB	Physical resource block

PRG	Physical resource block group

PSCell	Primary secondary cell

PSBCH	Physical sidelink broadcast channel

PSCCH	Physical sidelink control channel

PSFCH	Physical sidelink feedback channel

PSS	Primary synchronization signal

PSSCH	Physical sidelink shared channel

PUCCH	Physical uplink control channel

PUCCH-SCell	PUCCH SCell

PUSCH	Physical uplink shared channel

QCL	Quasi co-location

RB	Resource block

RE	Resource element

RLM	Radio link monitoring

RRM	Radio resource management

RS	Reference signal

RSRP	Reference signal received power

SCG	Secondary cell group

SCI	Sidelink control information

SCS	Subcarrier spacing

SFCI	Sidelink feedback control information

SFN	System frame number

SL	Sidelink

SLIV	Start and length indicator value

SPS	Semi-persistent scheduling

SR	Scheduling request

SRI	SRS resource indicator

SRS	Sounding reference signal

SSS	Secondary synchronization signal

TA	Timing advance

TAG	Timing advance group

TCI	Transmission Configuration Indicator

UCI	Uplink control information

UE	User equipment

UL	Uplink

UL-SCH	Uplink shared channel

USS	UE-specific search space

# 6	Link recovery procedures

A UE can be provided, for each BWP of a serving cell, a set  of periodic CSI-RS resource configuration indexes by failureDetectionResources and a set  of periodic CSI-RS resource configuration indexes and/or SS/PBCH block indexes by candidateBeamRSList or candidateBeamRSListExt-r16 or candidateBeamRSSCellList-r16 for radio link quality measurements on the BWP of the serving cell. If the UE is not provided  by failureDetectionResources or beamFailureDetectionResourceList for a BWP of the serving cell, the UE determines the set  to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set  includes RS indexes with QCL-TypeD configuration for the corresponding TCI states. The UE expects the set  to include up to two RS indexes. The UE expects single port RS in the set . The UE expects single-port or two-port CSI-RS with frequency density equal to 1 or 3 REs per RB in the set .

The thresholds Qout,LR and Qin,LR correspond to the default value of rlmInSyncOutOfSyncThreshold, as described in [10, TS 38.133] for Qout, and to the value provided by rsrp-ThresholdSSB or rsrp-ThresholdBFR-r16, respectively.

*** Unchanged text is omitted ***

For the PCell or the PSCell, upon request from higher layers, the UE provides to higher layers the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set  and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold.

For the SCell, upon request from higher layers, the UE indicates to higher layers whether there is at least one periodic CSI-RS configuration index and/or SS/PBCH block index from the set  with corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, and provides the periodic CSI-RS configuration indexes and/or SS/PBCH block indexes from the set  and the corresponding L1-RSRP measurements that are larger than or equal to the Qin,LR threshold, if any.

*** Unchanged text is omitted ***

A UE can be provided, by schedulingRequestID-BFR-SCell-r16, a configuration for PUCCH transmission with a link recovery request (LRR) as described in Clause 9.2.4. The UE can transmit in a first PUSCH one MAC CE providing index(es) for at least corresponding SCell(s) with radio link quality worse than Qout,LR, indication(s) of presence of qnew for corresponding SCell(s), and index(es) qnew for a periodic CSI-RS configuration or for a SS/PBCH block provided by higher layers, as described in [11, TS 38.321], if any, for corresponding SCell(s). After 28 symbols from a last symbol of a PDCCH reception with a DCI format scheduling a PUSCH transmission with a same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value, the UE

-	monitors PDCCH in all CORESETs on the SCell(s) indicated by the MAC CE using the same antenna port quasi co-location parameters as the ones associated with the corresponding index(es) qnew, if any

-	transmits PUCCH on a PUCCH-SCell using a same spatial domain filter as the one corresponding to qnew for periodic CSI-RS or SS/PBCH block reception, as described in Clause 9.2.2, and using a power determined as described in Clause 7.2.1 with qu=0, qd=qnew, and l=0, if

-	the UE is provided PUCCH-SpatialRelationInfo for the PUCCH,

-	a PUCCH with the LRR was either not transmitted or was transmitted on the PCell or the PSCell, and

-	the PUCCH-SCell is included in the SCell(s) indicated by the MAC-CE

where the SCS configuration for the 28 symbols is the smallest of the SCS configurations of the active DL BWP for the PDCCH reception and of the active DL BWP(s) of the at least one SCell.

# 7	Uplink Power control

Uplink power control determines a power for PUSCH, PUCCH, SRS, and PRACH transmissions.

A UE does not expect to simultaneously maintain more than four pathloss estimates per serving cell for all PUSCH/PUCCH/SRS transmissions as described in Clauses 7.1.1, 7.2.1, and 7.3.1, except for SRS transmissions configured by IE SRS-Positioning-Config as described in Clause 7.3.1. If the UE is provided a number of RS resources for pathloss estimation for PUSCH/PUCCH/SRS transmissions that is larger than 4, the UE maintains for pathloss estimation RS resources corresponding to RS resource indexes qd as described in Clauses 7.1.1, 7.2.1, and 7.3.1. If an RS resource updated by MAC CE, as described in Clauses 7.1.1, 7.2.1 and 7.3.1, is one from the RS resources the UE maintains for pathloss estimation for PUSCH/PUCCH/SRS transmissions, the UE applies the pathloss estimation based on the RS resources starting from the first slot that is after slot k+3∙Nslotsubframe,  μ where k is the slot where the UE would transmit a PUCCH or PUSCH with HARQ-ACK information for the PDSCH providing the MAC CE and μ  is the SCS configuration for the PUCCH or PUSCH, respectively.

A PUSCH/PUCCH/SRS/PRACH transmission occasion  is defined by a slot index  within a frame with system frame number , a first symbol  within the slot, and a number of consecutive symbols .

*** Unchanged text is omitted ***

## 7.1	Physical uplink shared channel

For a PUSCH transmission on active UL BWP , as described in Clause 12, of carrier  of serving cell , a UE first calculates a linear value  of the transmit power , with parameters as defined in Clause 7.1.1. For a PUSCH transmission scheduled by a DCI format other than DCI format 0_0, or configured by ConfiguredGrantConfig or semiPersistentOnPUSCH, if txConfig in PUSCH-Config is set to 'codebook',

-	if ul-FullPowerTransmission in PUSCH-Config is provided, the UE scales PPUSCH,b,f,c(i,j,qd,l) by s where:

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode1, and each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over the maximum number of SRS ports supported by the UE in one SRS resource

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpowerMode2

-	s=1 for full power TPMIs reported by the UE [16, TS 38.306], and s is the ratio of a number of antenna ports with non-zero PUSCH transmission power over a number of SRS ports for remaining TPMIs, where the number of SRS ports is associated with a SRS resource indicated by a SRI field in a DCI format scheduling the PUSCH transmission if more than one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or the number of SRS ports is associated with the SRS resource if only one SRS resource is configured in the SRS-ResourceSet with usage set to 'codebook',

-	s=1, if a SRS resource with a single port is indicated by a SRI field in a DCI format scheduling the PUSCH transmission when more than one SRS resource is provided in the SRS-ResourceSet with usage set to 'codebook', or indicated by Type 1 configured grant, or if only one SRS resource with a single port is provided in the SRS-ResourceSet with usage set to 'codebook', and

-	if ul-FullPowerTransmission in PUSCH-Config is set to fullpower, s=1

-	else, if each SRS resource in the SRS-ResourceSet with usage set to 'codebook' has more than one SRS port, the UE scales the linear value by the ratio of the number of antenna ports with a non-zero PUSCH transmission power to the maximum number of SRS ports supported by the UE in one SRS resource.

The UE splits the power equally across the antenna ports on which the UE transmits the PUSCH with non-zero power.

### 7.1.1	UE behaviour

*** Unchanged text is omitted ***

-	is a downlink pathloss estimate in dB calculated by the UE using reference signal (RS) index qd for the active DL BWP, as described in Clause 12, of carrier f of serving cell c

-	If the UE is not provided PUSCH-PathlossReferenceRS and enableDefaultBeamPlForSRS, or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource from the SS/PBCH block that the UE uses to obtain MIB

-	If the UE is configured with a number of RS resource indexes, up to the value of maxNrofPUSCH-PathlossReferenceRSs, and a respective set of RS configurations for the number of RS resource indexes by PUSCH-PathlossReferenceRS, the set of RS resource indexes can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pusch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource index qd in the set of RS resource indexes to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pusch-PathlossReferenceRS-Id in PUSCH-PathlossReferenceRS

-	If the PUSCH transmission is scheduled by a RAR UL grant as described in Clause 8.3, or for a PUSCH transmission for Type-2 random access procedure as described in Clause 8.1A, the UE uses the same RS resource index qd as for a corresponding PRACH transmission

-	If the UE is provided SRI-PUSCH-PowerControl and more than one values of PUSCH-PathlossReferenceRS-Id, the UE obtains a mapping from sri-PUSCH-PowerControlId in SRI-PUSCH-PowerControl between a set of values for the SRI field in a DCI format scheduling the PUSCH transmission and a set of PUSCH-PathlossReferenceRS-Id values and determines the RS resource index qd from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the SRI field value where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the PUSCH transmission is scheduled by a DCI format 0_0, and if the UE is provided a spatial setting by PUCCH-SpatialRelationInfo for a PUCCH resource with a lowest index for active UL BWP b of each carrier f and serving cell c, as described in Clause 9.2.2, the UE uses the same RS resource index qd as for a PUCCH transmission in the PUCCH resource with the lowest index

-     If the PUSCH transmission is scheduled by a DCI format 0_1, and if the UE is provided enableDefaultBeamPlForSRS and is not provided PUSCH-PathlossReferenceRS and PUSCH-PathlossReferenceRS-r16, the UE uses the same RS resource index qd as for a SRS resource set with an SRS resource associated with the PUSCH transmission

-	If

-	the PUSCH transmission is scheduled by a DCI format 0_0 and the UE is not provided a spatial setting for a PUCCH transmission, or

-	the PUSCH transmission is scheduled by a DCI format 0_1 that does not include an SRI field, or

-	SRI-PUSCH-PowerControl is not provided to the UE,

the UE determines a RS resource index qd with a respective PUSCH-PathlossReferenceRS-Id value being equal to zero where the RS resource is either on serving cell c or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If

-	the PUSCH transmission is scheduled by a DCI format 0_0 on serving cell c,

-	the UE is not provided PUCCH resources for the active UL BWP of serving cell c, and

-	the UE is provided enableDefaultBeamPlForPUSCH0_0

the UE determines a RS resource index qd providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c

-	If

-	the PUSCH transmission is scheduled by a DCI format 0_0 on serving cell c,

-	the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and

-	the UE is provided enableDefaultBeamPlForPUSCH0_0

the UE determines a RS resource index qd providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of serving cell c

*** Unchanged text is omitted ***

### 7.2.1	UE behaviour

If a UE transmits a PUCCH on active UL BWP  of carrier  in the primary cell  using PUCCH power control adjustment state with index , the UE determines the PUCCH transmission power  in PUCCH transmission occasion  as

[dBm]

where

-	 is the UE configured maximum output power defined in [8-1, TS 38.101-1], [8-2, TS38.101-2] and [8-3, TS38.101-3] for carrier  of primary cell  in PUCCH transmission occasion

-	 is a parameter composed of the sum of a component , provided by p0-nominal, or  dBm if p0-nominal is not provided, for carrier  of primary cell  and, if provided, a component  provided by p0-PUCCH-Value in P0-PUCCH for active UL BWP  of carrier  of primary cell , where .  is a size for a set of  values provided by maxNrofPUCCH-P0-PerSet. The set of  values is provided by p0-Set. If p0-Set is not provided to the UE, ,

-	If the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of p0-PUCCH-Value values. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the p0-PUCCH-Value value through the link to a corresponding p0-PUCCH-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If the UE is not provided PUCCH-SpatialRelationInfo, the UE obtains the p0-PUCCH-Value value from the P0-PUCCH with p0-PUCCH-Id value equal to the minimum p0-PUCCH-Id value in p0-Set

-	 is a bandwidth of the PUCCH resource assignment expressed in number of resource blocks for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell  and  is a SCS configuration defined in [4, TS 38.211]

-	is a downlink pathloss estimate in dB calculated by the UE using RS resource index  as described in Clause 7.1.1 for the active DL BWP  of carrier  of the primary cell  as described in Clause 12

-	If the UE is not provided pathlossReferenceRSs or before the UE is provided dedicated higher layer parameters, the UE calculates  using a RS resource obtained from the SS/PBCH block that the UE uses to obtain MIB

-	If the UE is provided a number of RS resource indexes, the UE calculates  using RS resource with index , where .  is a size for a set of RS resources provided by maxNrofPUCCH-PathlossReferenceRSs. The set of RS resources is provided by pathlossReferenceRSs. The set of RS resources can include one or both of a set of SS/PBCH block indexes, each provided by ssb-Index in PUCCH-PathlossReferenceRS when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a SS/PBCH block index, and a set of CSI-RS resource indexes, each provided by csi-RS-Index when a value of a corresponding pucch-PathlossReferenceRS-Id maps to a CSI-RS resource index. The UE identifies a RS resource in the set of RS resources to correspond either to a SS/PBCH block index or to a CSI-RS resource index as provided by pucch-PathlossReferenceRS-Id in PUCCH-PathlossReferenceRS

-	If the UE is provided pathlossReferenceRSs and PUCCH-SpatialRelationInfo, the UE obtains a mapping, by indexes provided by corresponding values of pucch-PathlossReferenceRS-Id, between a set of pucch-SpatialRelationInfoId values and a set of referenceSignal values provided by PUCCH-PathlossReferenceRS. If the UE is provided more than one values for pucch-SpatialRelationInfoId and the UE receives an activation command [11, TS 38.321] indicating a value of pucch-SpatialRelationInfoId, the UE determines the referenceSignal value in PUCCH-PathlossReferenceRS through the link to a corresponding pucch-PathlossReferenceRS-Id index. The UE applies the activation command in the first slot that is after slot  where  is the slot where the UE would transmit a PUCCH with HARQ-ACK information for the PDSCH providing the activation command and  is the SCS configuration for the PUCCH

-	If PUCCH-SpatialRelationInfo includes servingCellId indicating a serving cell, the UE receives the RS for resource index  on the active DL BWP of the serving cell

-	If the UE is provided pathlossReferenceRSs and is not provided PUCCH-SpatialRelationInfo, the UE obtains the referenceSignal value in PUCCH-PathlossReferenceRS from the pucch-PathlossReferenceRS-Id with index 0 in PUCCH-PathlossReferenceRS where the RS resource is either on the primary cell or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

-	If the UE

-	is not provided pathlossReferenceRSs, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is provided enableDefaultBeamPlForPUCCH, and

-	is not provided CORESETPoolIndex value of 1 for any CORESET, or is provided CORESETPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

the UE determines a RS resource index qd providing a periodic RS resource with 'QCL-TypeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots.

-	The parameter  is a value of deltaF-PUCCH-f0 for PUCCH format 0, deltaF-PUCCH-f1 for PUCCH format 1, deltaF-PUCCH-f2 for PUCCH format 2, deltaF-PUCCH-f3 for PUCCH format 3, and deltaF-PUCCH-f4 for PUCCH format 4, if provided; otherwise ΔF_PUCCHF=0.

-	 is a PUCCH transmission power adjustment component on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 0 or PUCCH format 1,  where

-	 is a number of PUCCH format 0 symbols or PUCCH format 1 symbols for the PUCCH transmission as described in Clause 9.2.

-	 for PUCCH format 0

-	 for PUCCH format 1

-	 for PUCCH format 0

-	 for PUCCH format 1, where  is a number of UCI bits in PUCCH transmission occasion

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits smaller than or equal to 11, , where

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided with pdsch-HARQ-ACK-Codebook,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Clause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

-	 is a number of resource elements determined as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Clause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell

-	For a PUCCH transmission using PUCCH format 2 or PUCCH format 3 or PUCCH format 4 and for a number of UCI bits larger than 11, , where

-

-

-	 is a number of HARQ-ACK information bits that the UE determines as described in Clause 9.1.2.1 for Type-1 HARQ-ACK codebook and as described in Clause 9.1.3.1 for Type-2 HARQ-ACK codebook. If the UE is not provided pdsch-HARQ-ACK-Codebook,  if the UE includes a HARQ-ACK information bit in the PUCCH transmission; otherwise,

-	 is a number of SR information bits that the UE determines as described in Clause 9.2.5.1

-	 is a number of CSI information bits that the UE determines as described in Clause 9.2.5.2

-	 is a number of CRC bits that the UE determines as described in Clause 9.2

-	 is a number of resource elements that the UE determines as , where  is a number of subcarriers per resource block excluding subcarriers used for DM-RS transmission, and  is a number of symbols excluding symbols used for DM-RS transmission, as defined in Clause 9.2.5.2, for PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell .

-	For the PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion

-	 is a TPC command value and is included in a DCI format 1_0 or DCI format 1_1 for active UL BWP  of carrier  of the primary cell  that the UE detects for PUCCH transmission occasion  or is jointly coded with other TPC commands in a DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI [5, TS 36.212], as described in Clause 11.3

-	 if the UE is provided twoPUCCH-PC-AdjustmentStates and PUCCH-SpatialRelationInfo and  if the UE is not provided twoPUCCH-PC-AdjustmentStates or PUCCH-SpatialRelationInfo

-	If the UE obtains a TPC command value from a DCI format 1_0 or a DCI format 1_1 and if the UE is provided PUCCH-SpatialRelationInfo, the UE obtains a mapping, by an index provided by p0-PUCCH-Id, between a set of pucch-SpatialRelationInfoId values and a set of values for closedLoopIndex that provide the  value(s). If the UE receives an activation command indicating a value of pucch-SpatialRelationInfoId, the UE determines the value closedLoopIndex that provides the value of  through the link to a corresponding p0-PUCCH-Id index

-	If the UE obtains one TPC command from a DCI format 2_2 with CRC scrambled by a TPC-PUCCH-RNTI, the  value is provided by the closed loop indicator field in DCI format 2_2

-	 is the current PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  and PUCCH transmission occasion , where

-	The  values are given in Table 7.1.2-1

-	 is a sum of TPC command values in a set  of TPC command values with cardinality  that the UE receives between  symbols before PUCCH transmission occasion  and  symbols before PUCCH transmission occasion  on active UL BWP  of carrier  of primary cell  for PUCCH power control adjustment state, where  is the smallest integer for which  symbols before PUCCH transmission occasion  is earlier than  symbols before PUCCH transmission occasion

-	If the PUCCH transmission is in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of symbols for active UL BWP  of carrier  of primary cell  after a last symbol of a corresponding PDCCH reception and before a first symbol of the PUCCH transmission

-	If the PUCCH transmission is not in response to a detection by the UE of a DCI format 1_0 or DCI format 1_1,  is a number of  symbols equal to the product of a number of symbols per slot, , and the minimum of the values provided by k2 in PUSCH-ConfigCommon for active UL BWP  of carrier  of primary cell

-	If the UE has reached maximum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If UE has reached minimum power for active UL BWP  of carrier  of primary cell  at PUCCH transmission occasion  and , then

-	If a configuration of a  value for a corresponding PUCCH power control adjustment state  for active UL BWP  of carrier  of primary cell  is provided by higher layers,

-

If the UE is provided PUCCH-SpatialRelationInfo, the UE determines the value of  from the value of  based on a pucch-SpatialRelationInfoId value associated with the p0-PUCCH-Id value corresponding to  and with the closedLoopIndex value corresponding to ; otherwise,

-	Else,

-	, where , and  is

-	the TPC command value indicated in a random access response grant corresponding to a PRACH transmission according to Type-1 random access procedure, or in a random access response grant corresponding to MsgA transmissions according to Type-2 random access procedure with RAR message(s) for fallbackRAR, or

-	the TPC command value indicated in a successRAR corresponding to MsgA transmissions for Type-2 random access procedure, or

-	the TPC command value in a DCI format with CRC scrambled by C-RNTI or MCS-C-RNTI that the UE detects in a first PDCCH reception in a search space set provided by recoverySearchSpaceId if the PUCCH transmission is a first PUCCH transmission after 28 symbols from a last symbol of the first PDCCH reception,

and, if the UE transmits PUCCH on active UL BWP  of carrier  of primary cell ,

;

otherwise,

where  is provided by higher layers and corresponds to the total power ramp-up requested by higher layers from the first to the last preamble for active UL BWP  of carrier  of primary cell , and  corresponds to PUCCH format 0 or PUCCH format 1

Table 7.2.1-1: Mapping of TPC Command Field in DCI format 1_0 or DCI format 1_1 or DCI format 2_2 with CRC scrambled by TPC-PUCCH-RNTI to accumulated  values

| TPC Command Field | Accumulated [dB] |
| --- | --- |
| 0 | -1 |
| 1 | 0 |
| 2 | 1 |
| 3 | 3 |

### 7.3.1	UE behaviour

*** Unchanged text is omitted ***

-	If the UE

-	is not provided pathlossReferenceRS or SRS-PathlossReferenceRS,

-	is not provided spatialRelationInfo, and

-	is provided enableDefaultBeamPlForSRS, and

-	is not provided CORESETPoolIndex value of 1 for any CORESET, or is provided CORESETPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

-	the UE determines a RS resource index  providing a periodic RS resource with 'QCL-TypeD' in

-	the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP, if CORESETs are provided in the active DL BWP of serving cell c

-	the active PDSCH TCI state with lowest ID [6, TS 38.214] in the active DL BWP, if CORESETs are not provided in the active DL BWP of serving cell c

*** Unchanged text is omitted ***

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format scheduling PDSCH receptions or SPS PDSCH release on an active DL BWP of a serving cell c, as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot n based on

-	PDSCH-to-HARQ_feedback timing indicator field values for PUCCH transmission with HARQ-ACK information in slot n in response to PDSCH receptions or SPS PDSCH release

-	slot offsets K0 [6, TS 38.214] provided by time domain resource assignment field in a DCI format scheduling PDSCH receptions or SPS PDSCH release and by pdsch-AggregationFactor or RepNumR16, when provided.

The set of PDCCH monitoring occasions for a DCI format scheduling PDSCH receptions or SPS PDSCH release is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells. PDCCH monitoring occasions are indexed in an ascending order of start time of the search space sets associated with a PDCCH monitoring occasion. The cardinality of the set of PDCCH monitoring occasions defines a total number M of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI formats denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with the DCI formats is present up to the current serving cell and current PDCCH monitoring occasion, first in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index m, where m 0≤m<M. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the value of the counter DAI is in the order of the first CORESETs and then the second CORESETs for a same serving cell index and a same PDCCH monitoring occasion index.

The value of the total DAI, when present [5, TS 38.212], in a DCI format denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI formats is present, up to the current PDCCH monitoring occasion m and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion. If, for an active DL BWP of a serving cell, the UE is not provided CORESETPoolIndex or is provided CORESETPoolIndex with value 0 for one or more first CORESETs and is provided CORESETPoolIndex with value 1 for one or more second CORESETs, and is provided ACKNACKFeedbackMode = JointFeedback, the total DAI value counts the {serving cell, PDCCH monitoring occasion}-pair(s) for both the first CORESETs and the second CORESETs.

*** Unchanged text is omitted ***

### 9.2.2	PUCCH Formats for UCI transmission

*** Unchanged text is omitted ***

If a UE

-	is not provided pathlossReferenceRSs in PUCCH-PowerControl,

-	is provided enableDefaultBeamPlForPUCCH, and

-	is not provided PUCCH-SpatialRelationInfo, and

-	is not provided CORESETPoolIndex value of 1 for any CORESET, or is provided CORESETPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212]

a spatial setting for a PUCCH transmission from the UE is same as a spatial setting for PDCCH receptions by the UE in the CORESET with the lowest ID on the active DL BWP of the PCell. For a PUCCH transmission over multiple slots, a same spatial setting applies to the PUCCH transmission in each of the multiple slots.

A number of DMRS symbols for a PUCCH transmission using PUCCH format 3 or 4 is provided by additionalDMRS.

Use of /2-PBSK, instead of QPSK, for a PUCCH transmission using PUCCH format 3 or 4 is indicated by pi2BPSK.

*** Unchanged text is omitted ***

# 10	UE procedure for receiving control information

*** Unchanged text is omitted ***

If a UE can support

-	a first set of Ncells,0DL serving cells where the UE is either not provided CORESETPoolIndex or is provided CORESETPoolIndex with a single value for all CORESETs on all DL BWPs of each serving cell from the first set of serving cells, and

-	a second set of Ncells,1DL serving cells where the UE is provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of each serving cell from the second set of serving cells

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is a value reported by the UER=TBD.

*** Unchanged text is omitted ***

## 10.1	UE procedure for determining physical downlink control channel assignment

*** Unchanged text is omitted ***

For each CORESET, the UE is provided the following by ControlResourceSet:

-	a CORESET index , by controlResourceSetId, where

-	 if CORESETPoolIndex is not provided, or if a value of CORESETPoolIndex is same for all CORESETs if CORESETPoolIndex is provided;

-	0<p<16 if CORESETPoolIndex is not provided for a first CORESET, or is provided and has a value 0 for a first CORESET, and is provided and has a value 1 for a second CORESET;

-	a DM-RS scrambling sequence initialization value by pdcch-DMRS-ScramblingID;

-	a precoder granularity for a number of REGs in the frequency domain where the UE can assume use of a same DM-RS precoder by precoderGranularity;

-	a number of consecutive symbols provided by duration;

-	a set of resource blocks provided by frequencyDomainResources;

-	CCE-to-REG mapping parameters provided by cce-REG-MappingType;

-	an antenna port quasi co-location, from a set of antenna port quasi co-locations provided by TCI-State, indicating quasi co-location information of the DM-RS antenna port for PDCCH reception in a respective CORESET;

-	if the UE is provided by simultaneousTCI-UpdateList-r16 or simultaneousTCI-UpdateListSecond-r16 up to two lists of cells for simultaneous TCI state activation, the UE applies the antenna port quasi co-location provided by TCI-States with same activated tci-StateID value to CORESETs with index p in all configured DL BWPs of all configured cells in a list determined from a serving cell index provided by a MAC CE command

-	an indication for a presence or absence of a transmission configuration indication (TCI) field for a DCI format, other than DCI format 1_0, that schedules PDSCH receptions or indicates SPS PDSCH release and is transmitted by a PDCCH in CORESET , by tci-PresentInDCI or tci-PresentInDCI-ForDCIFormat1_2

*** Unchanged text is omitted ***

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells using Rel-15 PDCCH monitoring capability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cell(s) using SCS configuration μ , where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μNcells,0DL+Ncells,1DL downlink cells. If a UE is configured with downlink cells using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability,  Ncellscap is replaced by Ncells,r15cap-r16.

*** Unchanged text is omitted ***