3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002957

# E-Meeting, April 20 - April 30, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1328 | rev | - | Current version: | 14.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on NPDCCH monitoring for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2020-04-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | UE should not monitor DCI format N2 in a NPDCCH UE-specific search space. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove “DCI N1” from UE-spceific search space and two HARQ processes case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the first change, specification may cause confusion on whether DCI format N2 is intended to be valid or not. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged parts are omitted >

## 16.6	Narrowband physical downlink control channel related procedures

#### < Unchanged parts are omitted >

For a NPDCCH UE-specific search space, if a NB-IoT UE is configured with higher layer parameter twoHARQ-ProcessesConfig

-	and if the NB-IoT UE detects NPDCCH with DCI Format N1 ending in subframe n, and if a NPDSCH transmission starts from n+k, the UE is not required to monitor an NPDCCH candidate in any subframe starting from subframe n+k-2 to subframe n+k-1;

otherwise

-	if the NB-IoT UE detects NPDCCH with DCI Format N1 or N2 ending in subframe n, and if the corresponding NPDSCH transmission starts from n+k, the UE is not required to monitor NPDCCH in any subframe starting from subframe n+1 to subframe n+k-1.

#### < Unchanged parts are omitted >