3GPP TSG RAN meeting #88e										    RP-200726

Electronic Meeting, June 29 - July 3, 2020						（Revision of RP-200458）

Status Report to TSG

Agenda item:			9.3.18

| WI / SI Name | Add support of NR DL 256QAM for FR2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_DL256QAM_FR2 |  |  |  |  |
| Unique ID | 830088 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-190760 RP-190925 RP-191891 RP-200124 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 06/2020 | Performance part: 12/2020 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 100% | Performance Part: 35% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Liu, Bo |
|  | Company | China Telecom |
|  | Email | liubo1@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The time budget was changed to request additional 1Q meetings for both core part and performance part due to UE requirements were not finished.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #91

R4-1906108	work plan on the WI: add support of NR DL 256QAM for FR2

R4-1907711	Simulation assumption of feasibility study for FR2 DL 256QAM

RAN4 #92

R4-1910478	TR skeleton for TR 38.8xx: Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2)

R4-1910591	TP for TR 38.8xx: Simulation assumptions for the feasibility study of FR2 DL 256QAM

R4-1910480	TP for TR 38.8xx: Link level simulation results for the feasibility study of FR2 DL 256QAM

R4-1910479	TP for TR 38.8xx: System level simulation results for the feasibility study of FR2 DL 256QAM

  - The agreed simulation assumptions for link level simulation in RAN4 #91 meeting were captured in the TR.

  - Initial link level simulation results and SNR ranges in which 256QAM has benefit were captured in the TR.

  - System level simulation results were captured in the TR and initial conclusion was made for scenarios in which 256QAM could apply.

RAN4 #92bis

R4-1911086	Draft TR 38.883 for FR2 DL 256QAM v0.1.0

R4-1910847	Additional simulation results on DL 256QAM

R4-1912892	TP to TR 38.883: FR2 DL 256QAM link level simulation results

R4-1912893	TP to TR 38.883: Section 5.2.1 Link level simulation

R4-1911088	TP for TR 38.883: Updated link level simulation for FR2 DL 256QAM

R4-1913051	TP to TR 38.883 NR FR2 256QAM link level and system level simulation results

R4-1912894	TP to TR 38.883: Section 5 System level simulation

R4-1912895	TP to TR: BS Implementation aspects

R4-1912896	TP for TR 38.883: Conclusion for the feasibility study of FR2 DL 256QAM

  - Updated link level simulation results and conclusion were captured in the TR.

  - Updated system level simulation results and conclusion were captured in the TR.

  - BS implementation study was finished to concluded that support of FR2 DL 256QAM is feasible from BS implementation perspective

  - Final conclusion was made that FR2 DL 256QAM is feasible and beneficial.

RAN4 #93

R4-1913902	Draft TR 38.883 for FR2 DL 256QAM v0.2.0

R4-1913493	TP to TR 38.883 Updated link level simulation results for FR2 DL 256QAM

R4-1913733	Updated simulation results for 256QAM for FR2

R4-1913734	TP for TR 38.883: Updated link level simulation for 256QAM for FR2

R4-1914081	TP to TR 38.883 Updated FR2 DL 256QAM link level simulation results

R4-1914568	TP to TR 38.883: Updated Section 5 System level simulation

  - Further supplementary and editorial updated link level simulation results were captured in the TR.

  - BS TX EVM core requirement for DL 256QAM FR2 is agreed as 3.5%

RAN4 #94_e

R4-2000909	Draft TR 38.883 for FR2 DL 256QAM v1.1.0

R4-2000910	TP for TR 38.883 BS RF requirements for FR2 DL 256QAM

R4-2001189	NR BS TX EVM for FR2 DL 256QAM

R4-2002802	CR to TS 38.104 Introduction of FR2 DL 256QAM

R4-2001427	CR to TS 38.141-2: Introduction of FR2 DL 256QAM

R4-2002803	WF on the requirements for FR2 DL 256QAM

  - Agreements for core requirements:

    - Discuss on how to define the UE maximum input level for 256QAM in the next meeting

    - Agree the BS and UE CRs in a package in Rel-16 WI

    - Technically endorse the CR to 38.104 (R4-2002802) since no technical concern

  - Agreement for BS conformance requirement:

    - Technically endorse the CR to 38.141-2 (R4-2001427) since no technical concern

      - Modification may be provided in the next meeting for the CR from Spec clear point

RAN4 #94bis_e

For core part:

R4-2003656	Draft TR 38.883 for FR2 DL 256QAM v1.2.0

R4-2003658	TP for TR 38.883 UE RF requirements for FR2 DL 256QAM

R4-2003772	Draft CR to TS 38.101-2: Introduction of FR2 DL 256QAM

R4-2003773	Draft CR to TS 38.104: Introduction of FR2 DL 256QAM

R4-2005655	WF on the requirements for FR2 DL 256QAM and 64QAM

- Define UE maximum input level for 256QAM in Rel-16

    - Technically endorse the CR R4-2003772 for UE maximum input level for 256QAM

    - Technically endorse the CR R4-2003773 for BS core requirement for 256QAM.

- Further discussion on UE maximum input level for 64QAM in Rel-16

    - Options on handling the requirement for 64QAM

      - Will be introduced jointly with that for 256QAM in Rel-16 from the consistence point

      - Will not be introduced considering the requirement has been removed before due to the redundancy

For perf. part:

R4-2004904	Work plan for FR2 DL 256QAM demodulation and CSI reporting requirements

R4-2005531	WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM

- Agreed to define FR2 PDSCH demodulation requirements for 256QAM.

- For PDSCH demodulation, following parameters were agreed:

    - SCS

    - PT-RS configuration

    - Overhead for TBS determination

    - PDSCH mapping type

    - TDD Configuration

    - PDCCH duration

    - PDSCH configuration:

    - SSB and TRS configuration

    - HARQ RV sequence

    - Receiver assumption

    - Test metric

RAN4 #95_e

For core part:

R4-2006927	Draft TR 38.883 for FR2 DL 256QAM v1.3.0

R4-2008968	Draft TR 38.883 for FR2 DL 256QAM v1.4.0

R4-2006928	TP for TR 38.883 Editoral corrections

R4-2007088	Maximum Input Level for 256 QAM FR2

R4-2007128	CR to TS 38.104: Introduction of FR2 DL 256QAM

R4-2007129	CR to TS 38.141-2: Introduction of FR2 DL 256QAM

R4-2007127	CR to TS 38.101-2: Introduction of FR2 DL 256QAM

  - Agreements for core requirements and BS conformance requirement:

    - CRs R4-2007128, R4-2007129, R4-2007127 were agreed

  - The core part for FR2 DL 256QAM is completed

For perf. part:

R4-2008817	WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM

- For PDSCH demodulation

    - Following main parameters were agreed:

      - Tx EVM

      - Rx impairment modelling and band agnostic requirements

      - MIMO configuration

    - Following other parameters were agreed:

      - DM-RS configuration

      - PRB bundling size and Precoding model

      - HARQ process number

2.4.2	Remaining Open issues

- For PDSCH demodulation, decide main parameters which are remained open, including rank, propagation condition and MCS.

- Whether to define SDR requirements for FR2 256QAM, if so, need to decide test parameters including MCS and rank.

- Whether to define FR2 CQI reporting requirements for CQI table 2, if so, need to decide test parameters including propagation condition, SNR testing point and others.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #91:

- R4-1907710, TR skeleton for TR 38.8xx: Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2), China Telecom

- R4-1906685, Summary of 256QAM for FR2 analysis, Ericsson

- R4-1905395, Simulation assumption for 256QAM feasibility study, CATT

- R4-1905776, Views on NR FR2 256QAM requirements, Intel corporation

- R4-1906014, System level simulation for 256QAM DL in FR2, Nokia, Nokia Shanghai Bell

- R4-1906883, Impact of Tx and Rx impairments, Nokia, Nokia Shanghai Bell

- R4-1906108, Work plan on the WI: add support of NR DL 256QAM for FR2, China Telecom

- R4-1907711, Simulation assumption of feasibility study for FR2 DL 256QAM, China Telecom

RAN4 #92:

- R4-1908012, Views on the testability of DL 256QAM in FR2, Apple Inc.

- R4-1908144, Views on feasibility of 256QAM for FR2, Qualcomm Incorporated

- R4-1908211, Discussion on feasibility of DL 256QAM in FR2 scenarios, Intel Corporation

- R4-1908391, Discussion on 256QAM for FR2, CATT

- R4-1908660, Feasibility evaluation for NR FR2 DL256QAM, NTT DOCOMO, INC.

- R4-1909002, Discussion on 256QAM for FR2 system level simulation assumptions, Nokia, Nokia Shanghai Bell

- R4-1909091, Link level simulation results for the feasibility study of FR2 DL 256QAM, China Telecom

- R4-1909185, Phase noise on 256QAM support for FR2, Huawei, HiSilicon

- R4-1909186, Simulation results on DL 256QAM, Huawei, HiSilicon

- R4-1909269, Initial FR2 DL 256QAM link level simulation results, Nokia, Nokia Shanghai Bell

- R4-1909401, Initial Link Simulation Results on DL 256 QAM FR2, Ericsson

- R4-1909402, Simulation Assumptions for 256 QAM FR2 Analysis, Ericsson

- R4-1909403, Feasibility of UE Demodulation Testing	Ericsson

- R4-1909982, On testability aspects of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell

- R4-1907710, TR skeleton for TR 38.8xx: Study on support of NR downlink 256 Quadrature Amplitude Modulation, China Telecom

- R4-1910591, TP for TR 38.8xx: Simulation assumptions for the feasibility study of FR2 DL 256QAM, China Telecom

- R4-1910480, TP for TR 38.8xx: Link level simulation results for the feasibility study of FR2 DL 256QAM, China Telecom

- R4-1910479, TP for TR 38.8xx: System level simulation results for the feasibility study of FR2 DL 256QAM, China Telecom

RAN4 #92bis:

- R4-1910846, BS TX impairment for 256 QAM, Huawei, HiSilicon

- R4-1910847, Additional simulation results on DL 256QAM,	Huawei, HiSilicon

- R4-1911002, Discussion on feasibility of DL 256QAM in FR2 scenarios, Intel Corporation

- R4-1911086, Draft TR 38.883 for FR2 DL 256QAM v0.1.0, China Telecom

- R4-1911087, Updated link level simulation results for the feasibility study of FR2 DL 256QAM, China Telecom

- R4-1911088, TP for TR 38.883: Updated link level simulation for FR2 DL 256QAM, China Telecom

- R4-1911227, Views on feasibility of 256QAM for FR2, Qualcomm Incorporated

- R4-1911684, Views on testability of DL 256QAM for FR2, Qualcomm Incorporated

- R4-1911800, TP to TR 38.883: BS implementation aspects in subclause 5.3, Ericsson

- R4-1912439, On FR2 DL 256QAM BS implementation feasibility, Nokia, Nokia Shanghai Bell

- R4-1912892, TP to TR 38.883: FR2 DL 256QAM link level simulation results, Nokia, Nokia Shanghai Bell

- R4-1912893, TP to TR 38.883: Section 5.2.1 Link level simulation, Ericsson

- R4-1912894, TP to TR 38.883: Section 5 System level simulation, Ericsson

- R4-1912895, TP to TR 38.883 on BS implementation aspects, China Telecom

- R4-1912896, TP for TR 38.883: Conclusion for the feasibility study of FR2 DL 256QAM, China Telecom

- R4-1913050, Feasibility evaluation for NR FR2 DL256QAM, NTT DOCOMO

- R4-1913051, TP to TR 38.883 NR FR2 DL256QAM link level and system level simulation results, NTT DoComo

RAN4 #93:

- R4-1913493, TP to TR 38.883 Updated link level simulation results for FR2 DL 256QAM, Qualcomm Incorporated

- R4-1913733, Updated simulation results for 256QAM for FR2, CATT

- R4-1913734, TP for TR 38.883: Updated link level simulation for 256QAM for FR2 CATT

- R4-1913902, Draft TR 38.883 for FR2 DL 256QAM v0.2.0, China Telecom

- R4-1913903, BS RF requirements for FR2 DL 256QAM, China Telecom

- R4-1913904, UE RF requirements for FR2 DL 256QAM, China Telecom

- R4-1914080, NR FR2 BS Tx EVM for DL 256QAM, NTT DOCOMO, INC.

- R4-1914081, TP to TR 38.883 Updated FR2 DL 256QAM link level simulation results, NTT DOCOMO, INC.

- R4-1914265, FR2 DL 256QAM requirements, Nokia, Nokia Shanghai Bell

- R4-1914568, TP to TR 38.883: Updated Section 5 System level simulation, Ericsson

- R4-1914569, TP to TR 38.883: Section 7 Demod test challenges, Ericsson

- R4-1914575, EVM requirements for DL 256QAM, Huawei, HiSilicon

- R4-1915572, CR to TS 38.141-2: EVM level added for 256 QAM, Ericsson

- R4-1915573, CR to TS 38.104: EVM level added for 256 QAM, Ericsson

- R4-1916008, TP for TR 38.883: Specification impact for FR2 DL 256QAM, China Telecom

RAN4 #94e:

- R4-2002886, Email discussion summary for RAN4#94e_#17_NR_DL256QAM_FR2, Moderator (China Telecom)

- R4-2002803, WF on the requirements for FR2 DL 256QAM, China Telecom

- R4-2000823, UE maximum input level  for DL 256QAM, Huawei, HiSilicon

- R4-2000909, Draft TR 38.883 for FR2 DL 256QAM v1.1.0, China Telecom

- R4-2000910, TP for TR 38.883 BS RF requirements for FR2 DL 256QAM, China Telecom

- R4-2000911, TP for TR 38.883 UE RF requirements for FR2 DL 256QAM, China Telecom

- R4-2000954, Discussion on FR2 Maximum Input Level requirements for 256QAM, Intel Corporation

- R4-2001189, NR BS TX EVM for FR2 DL 256QAM, NTT DOCOMO, INC.

- R4-2001190, NR UE maximum input level for FR2 DL 256QAM, NTT DOCOMO, INC.

- R4-2001425, CR to TS 38.101-2: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile

- R4-2002802, CR to TS 38.104 Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile, Ericsson

- R4-2001427, CR to TS 38.141-2: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile

- R4-2001729, CR to TS 38.104: Addition of EVM for 256 QAM, Ericsson

- R4-2002103, CR to TS 38.141-2: Addition of EVM for 256 QAM, Ericsson Inc.

RAN4 #94bis_e:

- R4-2003184, Discussion on UE performance requirements for FR2 DL 256QAM, Intel Corporation

- R4-2003625, NR UE maximum input level for FR2 DL 256QAM, NTT DOCOMO, INC.

- R4-2003656, Draft TR 38.883 for FR2 DL 256QAM v1.2.0, China Telecom

- R4-2003657, Maximum input level for FR2 DL 64QAM and 256QAM, China Telecom

- R4-2003658, TP for TR 38.883 UE RF requirements for FR2 DL 256QAM, China Telecom

- R4-2003707, Discussion on the performance requirements for NR DL 256QAM FR2, Huawei, HiSilicon

- R4-2003746, Discussion on the performance requirements for NR DL 256QAM FR2, Huawei, HiSilicon

- R4-2003771, UE RF core requirements for FR2 DL 256QAM, Nokia, Nokia Shanghai Bell

- R4-2003772, Draft CR to TS 38.101-2: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile

- R4-2003773, Draft CR to TS 38.104: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile, Ericsson

- R4-2003908, UE maximum input level for FR2 DL 256QAM	, OPPO

- R4-2004007, Views on DL 256QAM requirements for FR2, NTT DOCOMO, INC.

- R4-2004485, UE maximum input level for DL 256QAM, Huawei, HiSilicon

- R4-2004904, Work plan for FR2 DL 256QAM demodulation and CSI reporting requirements, China Telecom

- R4-2004905, UE demodulation and CSI reporting requirements for FR2 DL 256QAM, China Telecom

- R4-2005531, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, China Telecomm

- R4-2005592, Email discussion summary for [94e Bis][221] NR_DL256QAM_FR2_Demod, Moderator (China Telecomm)

- R4-2005655, WF on the requirements for FR2 DL 256QAM and 64QAM, China Telecom

- R4-2005696, Email discussion summary for [94e Bis][13] NR_DL256QAM_FR2, Moderator (China Telecom)

RAN4 #95_e:

- R4-2008947, Email discussion summary for [95e][117] NR_DL256QAM_FR2, Moderator (China Telecom)

- R4-2006927, Draft TR 38.883 for FR2 DL 256QAM v1.3.0, China Telecom

- R4-2008968, Draft TR 38.883 for FR2 DL 256QAM v1.4.0, China Telecom

- R4-2006928, TP for TR 38.883 Editoral corrections, China Telecom

- R4-2007128, CR to TS 38.104: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile, Ericsson

- R4-2007129, CR to TS 38.141-2: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile

- R4-2007088, Maximum Input Level for 256 QAM FR2, OPPO

- R4-2007127, CR to TS 38.101-2: Introduction of FR2 DL 256QAM, Nokia, Nokia Shanghai Bell, China Telecom, Verizon, NTT Docomo, T-Mobile

- R4-2009040, Email discussion summary for [95e][320] NR_DL256QAM_FR2_Demod, Moderator (China Telecomm)

- R4-2008817, WF on UE demodulation and CSI reporting requirements for FR2 DL 256QAM, China Telecomm

- R4-2006041, UE demodulation and CSI reporting requirements for FR2 DL 256QAM, China Telecom

- R4-2006529, Discussion on UE performance requirements for FR2 DL 256QAM, Intel Corporation

- R4-2007138, Views on DL 256QAM requirements for FR2, NTT DOCOMO, INC.

- R4-2007230, Discussion on the performance requirements for NR DL 256QAM for FR2, Huawei, HiSilicon

- R4-2008818, TP to TR 38.883: Section 7 Demod test challenges, Ericsson

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template