3GPP TSG-RAN Meeting #88                                                                             RP-200735

Electronic Meeting, June 29 - July 3, 2020

Agenda Item:	9.1

Source:	China Telecom, Nokia, Nokia Shanghai Bell

Title:	Motivation on enhancement of Private Network Support for NG-RAN

Document for:		Discussion and Decision

1. Introduction

The Private Network Support for NG-RAN is specified in Rel-16 in RP-191563[1]. In this document, the motivation of new work item towards Rel-17 is elaborated.

2. Discussion

2.1 Enhancement of Rel-16 topics

- The support of reporting the serving CAG ID usage

In Rel-16, the function of signalling the serving SNPN ID to RAN from AMF has been supported, but the functions of signalling the serving CAG ID to RAN in case of the access and handover procedures do not be supported yet.

However, RAN knowing the serving CAG ID of UEs could be helpful for operator’s network optimization and showing the effectiveness of PNI-NPN to the vertical customers who are willing to pay for the private network. Currently, RAN cannot know the access rate of each CAG cells if the cell is shared by multiple CAG IDs because RAN cannot collect the serving CAG ID information. For example, there are 5 CAG users (3 for CAG#A and 2 for CAG#B) accessing the Cell#X, currently, operators only know that 5 CAG users are in Cell#X, but cannot distinguish how many users of CAG#A and CAG#B are in the Cell#X.

Currently from the perspective of cells, there are three types of cells, i.e. CAG cell, SNPN cell, PLMN cell, the CAG cell is one of three types of cells. The usage conditions of other two types of cells (SNPN cell and PLMN cell) have been supported to report, only the usage of CAG cells has not been supported to report. However, the usage of CAG cells can help the operator to analysis whether the CAG cell has been utilized efficiently and whether the coverage, direction and location of CAG cells need to be adjusted. Operators also need to use the usage report of CAG cells to show their vertical or enterprise customers the effectiveness of PNI-NPNs.

Hence, from the point of PNI-NPN optimization and business promotion view, the functions of reporting the serving CAG ID usage in case of the access and handover procedures should be supported.

It should be further considered for the support of reporting the serving CAG ID usage in case of the access and handover procedures.

Figure 1. the serving cell for users of PNI-NPN

- The support of SNPN and PNI-NPN access for eMTC/NB-IoT

In Rel-16, the NB-IoT/eMTC connected to 5GC has been supported. However, the connection of NB-IoT/eMTC to a 5G NPN has not been defined (e.g. the broadcast of CAG-ID or NID by NB-IoT/eMTC RAN nodes not supported). As many operators have deployed the NB-IoT networks, they want these networks to be sufficiently utilized in 5G. NB-IoT/eMTC with NPN can provide operators additional scenarios for NB-IoT, so the connecting NB-IoT/eMTC devices to NPN can be part of the further work of eNPN to have a more comprehensive support of IoT use cases.

It should be further considered for the support of SNPN and PNI-NPN access for eMTC/NB-IoT.

2.2 Impact of SA2 work in Rel-17 on RAN

In SA2#133 meeting, the eNPN SI [3] was approved. In SA#87-e, the objectives of eNPN SI have been narrowed down to four key issues [4]:

- KI#1: ‘Study enhancements to enable support for SNPN along with subscription / credentials owned by an entity separate from the SNPN.’

This key issue aims at addressing the following points for SNPN along with subscription owned by an entity separate from the SNPN: - How to identify the separate entity providing the subscription; - Network selection enhancements, including UEs with multiple subscriptions; - E.g. how does the UE discover and select an SNPN which provides authentication in an external entity; - Architecture enhancements needed to support multiple separate entities, e.g.: - What are the interfaces exposed and/or used by SNPN and the separate entity; - What is the architecture and solution for a UE accessing a separate entity via SNPN access network; - How to exchange authentication signalling between the SNPN and the separate entity, including: - Authentication by the PLMN, based on PLMN identities and credentials, for access to the SNPN; - Authentication via SNPN to separate entity based on non-3GPP identities (e.g. non-IMSI) and credentials; - Mobility scenarios, including service continuity, for: - UE moving from SNPN#1 with separate entity#1 to SNPN#2 with separate entity#1 available; and - UE moving between SNPN#1 (where separate entity=PLMN) and PLMN. NOTE: Security aspects should be defined by SA WG3.

- KI#2: Study how to support UE onboarding and provisioning for non-public networks

The Key Issue is to study the architecture and solutions to support UE onboarding and provisioning for the NPN, including: - Architecture including which NFs are involved, and which scenario(s) the solution is addressing, including: - Which network entity performs UE's subscription provisioning and where is the network entity located. - If the network entity performing UE subscription provisioning is external to the SNPN, what is the service-based interface exposed by the SNPN towards that network entity for UE onboarding and provisioning. - Means for a UE, that is verifiably secure and uniquely identifiable to 5GS, for remote provisioning, including: - How does the UE discover and select the NPN before UE subscription is provisioned. - How does the network authenticate the UE before the UE's subscription is provisioned. - How does the 5G system provides and updates the subscription of an authorized UE in order to allow the UE to request connectivity to a desired NPN. - Means to remotely provision the required new or updated information to the UE for enabling the UE to access the NPN using 5GS, including e.g.: - Triggers and procedures used to initiate the provisioning procedure. - How to establish a secure connectivity between the UE and the network entity for provisioning. - How the network entity provisions the NPN subscription to the UE. - Support of exposure via APIs to support onboarding and remote provisioning, if required. The associated solutions need to consider the following UE characteristics: - Before the onboarding process there should be information in the UE for it to be "uniquely identifiable and verifiably secure". NOTE 1: This does not mean the UE is required to support the frequency bands the PLMN deploys for public network. - A TE might not have an interface that can be used to provision the MT. NOTE 2: This key issue covers devices with and without a UICC. NOTE 3: Security aspects should be discussed and confirmed by SA WG3.

- KI#3: Study enhancements to the 5GS for NPN to support NPN related service requirements for production of audio-visual content and services e.g. for service continuity and enabling reception of data services from two networks.

This key issue aims at addressing the following aspects: - Study whether there are support for service continuity (assuming PSA may reside in either PLMN or in the NPN) between PLMN and NPN (SNPN or PNI-NPN) with overlapping radio coverage areas; - Study means to enable a UE to receive data services from one network (e.g. NPN), and paging as well as data services from another network (e.g. PLMN) simultaneously. NOTE: The data service from NPN can be the low latency and high data rate service while serving massive number of UEs in a small area, e.g. the integrated audience multicast service in large live production events, such as music festivals (such as those listed in TS 22.263 [3] Table 6.3.1-1: Performance requirements for low latency deterministic periodic traffic with multicast service). It is assumed that the FS_IIoT will cover aspects to enable low latency data services, and that FS_5MBS will cover aspects to enable low latency multicast downlink services, while the scope of the FS_eNPN is to enable these services while the UE is using two networks e.g. NPN and PLMN.

- KI#4: Study support for IMS and emergency services for SNPN

3GPP Rel-16 includes IMS voice and emergency services support for Public network integrated Non-Public Networks, while for SNPNs the following was captured in TS 23.501 [4]:

"Emergency services are not supported in SNPN access mode.

NOTE 1:	Voice support with emergency services in SNPN access mode is not specified in this release".

This key issue aims at addressing the following points for SNPN:

-	Study the architectural impacts for support of IMS voice and emergency services offered by SNPN;

-	Study whether basic IMS functionality for SNPN via 3GPP access requires any specification changes to enable non-IMSI based IMPI usage over 3GPP access.

Based on the current progress of SA2 eNPN SI and RAN email discussion [5], the KI#1, KI#2 and KI#4 are most likely to have the RAN impact, the finial impact on RAN depends on SA2 final solutions of these key issues.

Check the RAN impact of SA2 study on enhanced support of NPN, and specify the necessary RAN functionality.

3. Conclusion

In this document, the motivation and the potential topics for NPN enhancement towards Rel-17 are described and the proposals were as follows:

- It should be further considered for the support of reporting the serving CAG ID usage in case of the access and handover procedures.

It should be further considered for the support of SNPN and PNI-NPN access for eMTC/NB-IoT.

Check the RAN impact of SA2 study on enhanced support of NPN, and specify the necessary RAN functionality.

4. Reference

- RP-191563, ‘Revised WID Private Network Support for NG-RAN’, China Telecom.

- TS23.501, ‘System architecture for the 5G System (5GS); Stage 2(Release 16)’, 3GPP.

- RP‑192422, ‘NB-IoT/eMTC access to Non-Public Networks (NPN)’, QUALCOMM Europe Inc.

- S2-1906726, ‘New SID: Study on enhanced support of Non-Public Networks’, Ericsson.

- SP-200094, ‘Revised SID: Study on enhanced support of Non-Public Networks’, Ericsson.

- RP-200734, ‘Email discussion on eNPN’, China Telecom.