3GPP TSG RAN Meeting #88e	 RP-200818

Electronic Meeting, June 29- July 3, 2020

Source:		ZTE, Sanechips

Title:		Discussion on the scope of NR MBS

Agenda item:		9.10.8

Document for:	  	Discussion

# 1 Introduction

The evolution of Multicast/Broadcast in 3GPP especially in 5G has gone through waves of email discussion as early as in 2018. Companies had recognized that there were two typical scenarios [1]:

Terrestrial Broadcast: notion of a downlink-only, ‘large area coverage up to nation-wide’ broadcast on dedicated spectrum, e.g. “TV-like” distribution of content

Mixed mode multicasting: notion of downlink multicast/broadcast with the potential to leverage downlink unicast and/or uplink unicast, with configurable/dynamic coverage ranging between a single cell to a large area, and multiplexed and possibly seamlessly switched with unicast traffic

Thanks to the efforts being made in 3GPP Rel-16, the 5G Terrestrial Broadcast whose requirement referred in TR 38.913 [2] has been well addressed to meet the requirements of expanded 3GPP ecosystem including the traditional broadcasters like ABS, BBC, and EBU [3].

It was then endorsed that TSG RAN will not target any study/work for NR on broadcast with MPMT/HPHT in Rel-17 if the requirement for 5G Terrestrial Broadcast can be addressed in Rel-16 [4].

MBS for 5G in Rel-17 plans to focus on small area mixed mode multicast (also termed as Objective A) [5-7]. Besides the basic broadcast functions, reliability and flexibility are pursued without compromise, which allows network operators to provide SLA for multicast services. This is of great importance for operators to broaden the range of NR MBS use cases, from traditional video content distribution to V2X, Public Safety and IIoT with higher requirements

In this contribution, discussion on the working scope of Rel-17 NR MBS in RAN is presented, including the support of Receive only Mode/free-to-air services, and considerations on potential WID prioritization, to help us focus on what's of importance and emergency for the upcoming RAN WG especially, under the circumstances of already delayed schedule and the foreseeable slow progress in online e-meeting.

# 2 Discussion

## 2.1 Support of Receive Only Mode and Free-to-air services

Receive Only Mode together with free-to-air services introduced in 3GPP Release-14 enable devices without a SIM card or 3GPP subscription to access a subset of the 3GPP eMBMS TV services, thus, to expand the reach of cellular broadcast into more diverse need.

The applicability of mobile broadcast to support public broadcasting requirements shall not be prohibited in NR MBS. However, in current WID of NR MBS, such technique which requires very minor RAN standard impacts had been excluded. The corresponding work in RAN can be done in a manner that associates with the progress of SA to fulfill the need of devices receiving without SIM card. Therefore, we propose:

Proposal 1: Include Receive Only Mode and Free-to-air services in Rel-17 NR MBS.

## 2.2 Core enhancements

NR MBS in Rel-17 will focus on small area mixed mode only for multicast/broadcast services/scenarios including at least V2X, Public Safety and multimedia delivery in hot-spot areas. Besides the basic functional blocks, Rel-17 NR MBS is an ambitious work item that includes a list of completely new features compared to LTE eMBMS to support the core use cases as listed above. The key words here are service continuity and reliability, while the measures to address the requirement is through mode switch, and feedback for multicast services:

Delivery mode switch with service continuity. Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

Mobility Support with service continuity. Specify support for basic mobility with service continuity [RAN2, RAN3]

Reliability enhancement. Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

Below is a non-exhaustive list for key issue analysis for further prioritization.

Delivery mode switch with service continuity

Recognize the requirements on mode switch, especially the so called "service continuity", where (levels of) KPIs like switch latency and data loss can be defined first.

Define a common QoS model for PTM and PTP transmission to enable the switch, rather than separated user plane and also control plane between unicast and broadcast service in eMBMS.

- Define the user plane behaviour/procedure for delivery mode switch and how to achieve lossless in such delivery mode switch if needed.

- Signalling in Uu/F1/E1, and potential NG-C interfaces to support such switch.

Mobility Support with service continuity

Recognize/clarify the requirements on mobility and the KPIs. In current WID the "basic mobility support" and "service continuity" can lead to ambiguity, like if lossless is pursued for multicast or broadcast services and considerations on supported UE RRC states are not clear, which shall be clarified before any further actions.

Impact on the current measurement config, and related NW/UE behaviour to reflect such.

Xn/NG-C signalling design which is dependent on the MBS session management.

Reliability enhancement

Reliability can be achieved through Layer 1 feedback (i.e., HARQ feedback or CSI report) or higher layer feedback (reception status report) with potential re-transmission. Mechanisms needed for such enhancement shall be clarified or evaluated before further actions.

For Layer 1 feedback with high RAN1 impacts, benefit shall be evaluated as in Rel-13 SC-PTM did.

Based on the evaluation, such enhancement will then be consolidated and standardized in the next phase.

Observation 1: There are still much clarification on the requirements and related KPIs work needed.

There are a few principles that can be followed to reduce the workload and have a more harmonized design that can cope with unicast and multicast:

Be aligned with Rel-15/16 NR unicast design as much as possible, from physical layer signal to RRC signaling.

LTE eMBMS, especially SC-PTM design as the baseline if possible, to avoid over design/starting from scratch.

Further define the requirement and KPIs.

According to the above discussions, we have the following proposal:

Proposal 2: In current WID for NR MBS, further prioritization is needed for the sake of workload and also productivity, it is suggested that:

- Prioritization based on the clarified requirements and recognized KPIs, to tackle the WID with clearer goals.

- Recognize the core part and any enhancement that can be left to future Releases due to time constraints.

# 3 Conclusions

In this contribution, we have presented some scope analysis on NR MBS. The following proposals are given:

Proposal 1: Include Receive Only Mode and Free-to-air services in Rel-17 NR MBS scope.

Proposal 2: In current WID for NR MBS, further prioritization is needed for the sake of workload and also productivity, it is suggested that:

- Prioritization based on the clarified requirements and recognized KPIs, to tackle the WID with clearer goals.

- Recognize the core part and any enhancement that can be left to future Releases due to time constraints.

# 4 References

RP-180668 Final report from email discussion on 5G Broadcast evolution, 3GPP RAN #80, June 2018

TR 38.913 Study on Scenarios and Requirements for Next Generation Access Technologies;

TR 36.976 Overall description of LTE-based 5G broadcast

RP-190717 Way forward on 5G Broadcast, 3GPP TSG RAN Meeting #83, March 2019

RP-191859 Summary of moderated email discussion on Rel-17 NR multicast broadcast

SP-200092 Revised SID: Architectural enhancements for 5G multicast-broadcast services

RP-193248 New Work Item on NR support of Multicast and Broadcast Services