3GPP TSG RAN meeting #88e									      RP-200927

Electronic Meeting, June 29 - July 3, 2020

## Status Report to TSG

Agenda item:			9.9.3

| WI / SI Name | NR Positioning Enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_Pos_Enh |  |  |  |  |
| Unique ID | 860034 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-193237 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2020 (updated from 09/2020) | Core part: NA | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: 15% | Core part: NA | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Ren DA |
|  | Company | CATT |
|  | Email | renda@catt.cn |
| Rapporteur | Name | Yi GUO |
|  | Company | Intel Corporation |
|  | Email | yi.guo@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

Due to COVID-19, the start of the SI was delayed from Feb. 2020 to May 2020. RAN1#101-e is the first RAN1 meeting of this SI with total 45minutes (<0.5TU) online session time. With the consideration of the late start of the SI and the change from F2F meetings to e-meetings, it is proposed to shift the completion dates of both RAN1 and RAN2 work for the SI to 12/2020.

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Agreement:

- InF-SH and InF-DH models in TR 38.901 are adopted as the baseline scenarios for defining the channel models, parameters and modelling techniques for performance evaluations in the Rel. 17 positioning enhancements at least for IIoT use cases

- Note: Modifications to parameters in the InF-DH models will be discussed separately.

- Note: Target performance and performance gap identification will be discussed separately.

- Note: Individual companies may consider additional InF models in TR 38.901 as complementary evaluation scenarios in their simulation investigation and the evaluation results can be considered to be captured in the TR 38.857.

- Note: Target positioning requirements may not necessarily be reached for all scenarios.

Agreement (Proposal 4.1-1, Revision #2, in Section 4.1 of R1-2004868):

- Adopt the parameters defined in Table below as the baseline parameters for all scenarios in the evaluation of the positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

- Note: Optional scenarios and assumptions will be discussed separately and can be included

Table: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: FFS |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Agreement:

Optional: The following UE antenna configuration can be considered

- 4 UE panels:

  - The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- FFS: Other details

Agreement:

Absolute-time-of arrival model defined in TR 38.901 without modification is considered in the evaluation of all scenarios.

Agreement:

Blockage model is not considered in the evaluation of all scenarios

Agreement: (Proposal 5.1-4, Revision 3, in Section 5.1 of R1-2004961)

- Adopt the parameters defined in the Table below as the baseline parameters for all InF scenarios in the evaluation of positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

Table: Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: See Proposal 5.1-7 |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

Agreement:

Optional: For evaluating vertical positioning performance, UE antenna height can be uniformly distributed within [0.5, X2]m, where X2 = 2m for InF-SH and X2= for InF-DH defined in TR 38.901.

Agreement:

Clutter parameters {density , height ,size } for high clutter density are set as follows:

- (Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height

- (Optional): {40%, 3m, 5m}

- (Optional): {60%, 6m, 2m}

Agreement:

It will be left to companies to define the configurations for DL PRS and UL SRS for the evaluation of positioning performance.

- Note: Configurations of DL PRS and UL SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Agreement:

CDFs of positioning errors are used as performance metrics in NR positioning evaluation with at least the following percentiles 50%, 67%, 80%, 90%.

- Note: In addition to overall positioning accuracy performance, companies are encouraged to report the estimation accuracy of UE/gNB measurements (e.g., RSTD) for performance comparison.

Agreement:

For TR 38.857, the template used in TR 38.855 for the inclusion of simulation results is reused. In addition, the following parameters should be provided for each scenario together with the simulation results.

| Parameter | [Source 1, scenario, FRx] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) |  |
| Reference signal (type of sequence, number of ports, …) |  |
| Number of sites |  |
| Number of symbols used per slot per positioning estimate |  |
| Number of slots per positioning estimate |  |
| Power-boosting level |  |
| Uplink power control (applied/not applied) |  |
| interference modelling (ideal muting, or other) |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) |  |
| Network synchronization assumptions |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) |  |
| Additional notes, if any |  |

Agreement:

CDF values for positioning accuracy for IIoT scenarios are derived based on:

- Case 1 (Required): The UEs inside the convex hull of the horizontal BS deployment area.

- Case 2 (Optional): All the UEs

Agreement:

Optional: For evaluating vertical positioning performance, gNB antenna height can also be set to two fixed heights, which is either {4, 8} m, or {max(4,), 8}.

Agreement:

Network efficiency and UE efficiency can be evaluated at least in an analytical manner.

- FFS: the definition of efficiency metric (e.g., the positioning performance (accuracy, latency) vs. PRS/SRS resource utilization etc.)

- Note: It will be up to each company on whether to use other methods (e.g., numerical simulation) for the evaluation.

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

Agreement:

Optional: UE mobility can be considered in evaluation with the consideration of the spatial consistency procedure defined in TR 38.901.

- FFS: the details of the mobility models

Agreement:

- UE power consumption for NR positioning can be optionally evaluated in the SI.

- Note: It is up to each company on how to evaluate the power consumption for positioning. The UE power consumption models developed in TR38.840 can be considered as the starting point for defining the UE power consumption model for the evaluation for NR positioning

Agreement:

The TR skeleton in R1-2004948 is endorsed.

Agreement:

Optional: The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1:

        T1:  [X] ns for gNB and [Y] ns for UE

- FFS: X, Y

        Note: RX and TX timing errors are generated per panel independently

        FFS: how the Rx and Tx timing errors are applied

Agreement:

- In Rel-17 SI, for the evaluation of positioning enhancements for commercial use cases, no baseline scenario is defined. UMi, UMa and IOO scenario(s) defined in TR 38.855 can be considered as optional scenarios without modifications to existing configuration parameters.

- FFS: absolute time of arrival model for UMi, UMa and IOO scenarios

Agreement:

Physical layer latency can be evaluated through analysis and, optionally, numerical evaluation.

Agreement:

Higher layer positioning latency can be evaluated in this SI.

- FFS: how to evaluate higher-layer positioning latency

- FFS: which higher-layers should be included in the evaluation

#### 2.1.2	Remaining Open issues

RAN1#101-e is the first RAN1 meeting of this SI. According to the meeting arrangement, RAN1 started working on the following objectives in SID:

Defining the additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

In RAN1#101-e, RAN1 has defined the additional scenarios and completed the baseline simulation assumptions and some optional simulation assumptions for the additional scenarios.

RAN1 has not started discussing of the following objective in SID, namely

Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2005095	Session notes for 8.2 (Study on NR positioning enhancements)	Ad-Hoc Chair (Ericsson)

- R1-2003638	Work plan for Study on NR Positioning Enhancements	CATT, Intel, Ericsson

- R1-2004649	TR skeleton for TR 38.857	Ericsson

- R1-2004948	TR skeleton for TR 38.857	Ericsson

- R1-2004701	FL Summary for NR Positioning Enhancements	Moderator (CATT)

- R1-2004868	FL Summary #2 for NR Positioning Enhancements	Moderator (CATT)

- R1-2004961	FL Summary #3 for NR Positioning Enhancements	Moderator (CATT)

- R1-2005049	FL Summary #4 for NR Positioning Enhancements	Moderator (CATT)

- R1-2005049	FL Summary #4 for NR Positioning Enhancements	Moderator (CATT)

- R1-2005102 Summary of Email Discussion [101-e-Post-NR-Pos-Enh] Moderator (CATT)

- R1-2005188 Summary #2 of Email Discussion [101-e-Post-NR-Pos-Enh] Moderator (CATT)

- R1-2003639	Summary of discussion on IIoT Scenarios for NR Positioning Enhancements	CATT

- R1-2003906	Additional scenarios for evaluation	Samsung

- R1-2003963	Discussions on IIoT scenarios for positioning	CMCC

- R1-2004141	Discussion on additional scenarios for evaluation	LG Electronics

- R1-2004490	Considerations on Additional Scenarios for Evaluation	Qualcomm Incorporated

- R1-2003284	IIOT Scenarios for Positioning	Futurewei

- R1-2003295	Discussion on scenarios and evaluation methodology for Rel-17 positioning	Huawei, HiSilicon

- R1-2003427	Discussion on additional scenarios for NR positioning evaluation	vivo

- R1-2003479	Additional scenarios for evaluation on positioning enhancements	ZTE

- R1-2003640	IIoT use cases and scenarios for evaluation of NR Positioning Enhancements	CATT

- R1-2003719	Additional scenarios for evaluation of NR positioning	Nokia, Nokia Shanghai Bell

- R1-2003767	I-IoT scenarios for NR positioning evaluations	Intel Corporation

- R1-2004063	Discussion on Scenarios for Evaluation	OPPO

- R1-2004190	Considerations on Scenarios for Evaluations of IIoT Positioning	Sony

- R1-2004199	View on scenarios and evaluation parameters for Rel 17 positioning enhancement	CEWiT

- R1-2004517	Additional scenarios and considerations for NR positioning	Fraunhofer IIS, Fraunhofer HHI

- R1-2004650	Additional scenarios for performance evaluations	Ericsson

- R1-2003296	Performance evaluation for Rel-17 positioning	Huawei, HiSilicon

- R1-2003428	Evaluation of achievable accuracy and latency for NR positioning enhancements	vivo

- R1-2003480	Evaluation results of additional scenarios for positioning	ZTE

- R1-2003668	Evaluation of DL-AoD technique under IIOT scenario	MediaTek Inc.

- R1-2003768	Initial analysis of NR positioning performance in I-IoT scenarios	Intel Corporation

- R1-2004064	Evaluation of NR positioning in IIOT scenario	OPPO

- R1-2004651	Evaluation of Achievable Positioning Accuracy and Latency	Ericsson

- R1-2003547	Evaluation of Rel-16 Positioning for IIOT	Futurewei

- R1-2003641	Discussion of evaluation of NR positioning performance	CATT

- R1-2003720	Views on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

- R1-2003907	Evaluation of achievable positioning accuracy and latency	Samsung

- R1-2003964	Discussions on evaluation methodology of latency	CMCC

- R1-2004191	Considerations on Evaluation of Positioning Accuracy and Latency	Sony

- R1-2004491	Initial Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

- R1-2004518	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

- R1-2003285	IIOT Positioning techniques Consideration	Futurewei

- R1-2003297	Discussion of positioning enhancement	Huawei, HiSilicon

- R1-2003429	Discussion on potential positioning enhancements	vivo

- R1-2003481	Potential NR positioning enhancements for Rel.17	ZTE

- R1-2003642	Discussion of NR positioning enhancements	CATT

- R1-2003669	Views on Positioning enhancement for Rel-17	MediaTek Inc.

- R1-2003701	Potential positioning enhancements	BUPT

- R1-2003721	Initial views on potential positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2003769	Potential NR positioning enhancements	Intel Corporation

- R1-2003908	Potential positioning enhancements	Samsung

- R1-2003965	Discussions on potential positioning enhancements	CMCC

- R1-2003977	Positioning enhancements for RRC IDLE and RRC INACTIVE state UE	Beijing Xiaomi Mobile Software

- R1-2003988	Discussion on potential positioning enhancements	Spreadtrum Communications

- R1-2004065	Discussions on NR Positioning Enhancements	OPPO

- R1-2004142	Discussion on potential positioning enhancements	LG Electronics

- R1-2004175	On Potential Rel-17 NR Positioning Enhancements	Lenovo, Motorola Mobility

- R1-2004192	Potential Techniques for Positioning Enhancements	Sony

- R1-2004420	Discussion on potential techniques for NR Positioning Enhancements	NTT DOCOMO, INC.

- R1-2004492	Initial thoughts on Potential Positioning Enhancements	Qualcomm Incorporated

- R1-2004652	Potential enhancements for NR positioning in release 17	Ericsson

- R1-2003430	Discussion on power consumption model for NR positioning enhancements	vivo

- R1-2003643	Discussion of  NLOS IIoT channel modelling for NR positioning enhancement	CATT

- R1-2003909	Uplink Transmission Based Relative Positioning	Samsung

- R1-2004066	Analysis of NR Positioning for IIOT Scenarios	OPPO

- R1-2004609	Discussion of sidelink positioning	Huawei, HiSilicon

- R1-2004653	PRS separation based on Cyclic shifts	Ericsson

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template