3GPP TSG-RAN4 Meeting #95-e	R4-2007335

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.307 | CR | 4442 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding UE category NB1 supporting LTE band 42/43 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_bands_R16_M1_NB1-Perf |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In Rel-16, TDD bands 42 and 43 are introduced for UE cat. NB2, but whether UE cat. NB1 supports LTE band 42/43 is not clarified. Considering that UE cat. NB1 shall be supported if UE supports cat. NB2, it is reasonable to make the statement that bands 42 and 43 also apply to UE cat. NB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Making the statement that LTE bands 42 and 43 apply to UE cat. NB1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Whether LTE bands 42 and 43 are supported by UE cat. NB1 remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

## 3A.3	Additional operating bands and/or CA configurations for specific features

For a specific feature introduced in an earlier release, it may be decided in a later release to apply this specific feature in a release independent way for additional operating bands and/or CA configurations. For a Rel-16 UE corresponding requirements are then introduced via this clause.

Table 3A.3-1: Operating bands for specific features

| Feature | Release independent from | Requirements to be fulfilled (see 36.307 of the REL when the feature was introduced) | Further information |
| --- | --- | --- | --- |
| Operating bands for UE category 0 | Rel-12 | Table B.2.9-1, Table B.3.5-1, Table B.4.10-1 | Rel-14 WI LC_MTC_LTE_cat0_B25_B26-Core introduced RF, RRM, demodulation and CSI requirements for bands 25 and 26, see Table B.2.9-1, Table B.3.5-1, Table B.4.10-1 |
| Operating bands for UE category M1 | Rel-13 | Table B.2.10-1, Table B.3.6-1, Table B.4.11-1 | Rel-14 WI LTE_MTCe2_L1_cat1_B25_B40-Core introduced RF, RRM, demodulation and CSI requirements for bands 25 and 40, see Table B.2.10-1, Table B.3.6-1, Table B.4.11-1. Rel-15 WI LTE_bands_R15_M1_NB1-Core introduced RF, RRM, demodulation and CSI requirements for bands 14 and 71, see Table B.2.10-1, Table B.3.6-1, Table B.4.11-1. |
| Operating bands for HD-FDD UE category M1, Power class 2 | Rel-13 | Table B.2.10-1, Table B.3.6-1, Table B.4.11-1 | Rel-16 WI LTE_PC2_B31_B72 introduced power class 2 feature for bands 31 and 72. |
| Operating bands for UE category M2 | Rel-14 | Table B.2.11-1, Table B.4.11-1 | Rel-15 WI LTE_bands_R15_M2_NB2-Core introduced RF and RRM requirements for bands 14 and 71, see Table B.2.11-1, Table B.4.11-1. |
| Operating bands for HD-FDD UE category M2, Power class 2 | Rel-14 | Table B.2.11-1, Table B.4.11-1 | Rel-16 WI LTE_PC2_B31_B72 introduced power class 2 feature for bands 31 and 72. |
| Operating bands for (FDD) UE category NB1 | Rel-13 | Table B.2.8-1, Table B.3.7-1, Table B.4.9-1 | Rel-14 WI NB_IOT_R14_bands introduced RF, RRM and demodulation requirements for bands 11, 21, 25, 31, 70, see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. Rel-15 WI LTE_bands_R15_M1_NB1-Core introduced RF, RRM and demodulation for bands 4, 14 and 71 see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. Rel-16 WI LTE_bands_R16_M1_NB1 introduced RF, RRM, demodulation for band 65, see Table B.2.8-1, Table B.3.7-1, Table B.4.9-1. |
| Operating bands (FDD) for UE category NB2 | Rel-14 | Table B.2.12-1, Table 3.7-1, Table B.4.9-1 | Rel-15 WI LTE_bands_R15_M2_NB2-Core introduced RF, RRM and demodulation requirements for bands 4, 14 and 71, see Table B.2.12-1, Table 3.7-1, Table B.4.9-1. Rel-16 WI LTE_bands_R16_M2_NB2 introduced RF, RRM, demodulation for band 65 , see Table B.2.12-1, Table B.3.7-1, Table B.4.9-1. |
| Operating bands (TDD) for UE category NB1 and NB2 | Rel-15 | Table B.2.12-1, Table 3.7-1, Table B.4.9-1 | Rel-16 WI LTE_bands_R16_M1_NB1 and Rel-16 WI LTE_bands_R16_M2_NB2 introduced RF, RRM, demodulation for band 42 and band 43, see Table B.2.12-1, Table B.3.7-1, Table B.4.9-1. |
| Operating bands for UE category 1bis | Rel-14 | Table B.2.15-1, Table B.3.8-1, Table B.4.14-1 | Rel-16 RF, RRM, demodulation and CSI requirements for Band 40, see Table B.2.15-1, Table B.3.8-1, Table B.4.14-1. |

<< End of changes >>