3GPP TSG-RAN WG4 Meeting #95-e	R4-2007178

Electronic Meeting, May 25th – June 5th 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.104 | CR | 4895 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 36.104 Finalization on PUSCH performance requirements for enhanced HST scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2-Perf |  |  |  |  |  | Date: |  |  | 2020-5-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resubmission of endorsed Draft CR R4-2003632. This CR is to finalize PUSCH requirements under high speed train conditions assuming a UE velocity of up to 500km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove squre brackets.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Square brackets are remained. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS36.141 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.2.3.1	Minimum requirements

The throughput shall be equal to or larger than the fraction of maximum throughput stated in table 8.2.3.1-1 at the given SNR.

Table 8.2.3.1-1 Minimum requirements of PUSCH for high speed train

| Channel Bandwidth [MHz] | Cyclic prefix | FRC (Annex A) | Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | Normal | A3-2 | 1 | 1 | HST Scenario 3 | 30% | -1.5 |
|  |  |  |  |  |  | 70% | 1.9 |
|  |  |  |  |  | HST Scenario 3-LTE500a (NOTE 1) | 30% | -1.5 |
|  |  |  |  |  |  | 70% | 1.9 |
|  |  |  |  |  | HST Scenario 3-LTE500b (NOTE 2) | 30% | -1.5 |
|  |  |  |  |  |  | 70% | 1.9 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -3.9 |
|  |  |  |  |  |  | 70% | -0.6 |
|  |  |  |  |  | HST Scenario 1-LTE500a Low (NOTE 1) | 30% | -3.9 |
|  |  |  |  |  |  | 70% | -0.6 |
|  |  |  |  |  | HST Scenario 1-LTE500b Low (NOTE 2) | 30% | -3.9 |
|  |  |  |  |  |  | 70% | -0.6 |
| 3 | Normal | A3-3 |  | 1 | HST Scenario 3 | 30% | -2.1 |
|  |  |  |  |  |  | 70% | 1.6 |
|  |  |  |  |  | HST Scenario 3-LTE500a (NOTE 1) | 30% | -2.1 |
|  |  |  |  |  |  | 70% | 1.8 |
|  |  |  |  |  | HST Scenario 3-LTE500b (NOTE 2) | 30% | -2.1 |
|  |  |  |  |  |  | 70% | 1.7 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -4.5 |
|  |  |  |  |  |  | 70% | -1.0 |
|  |  |  |  |  | HST Scenario 1-LTE500a Low (NOTE 1) | 30% | -4.5 |
|  |  |  |  |  |  | 70% | -1.0 |
|  |  |  |  |  | HST Scenario 1-LTE500b Low (NOTE 2) | 30% | -4.5 |
|  |  |  |  |  |  | 70% | -1.0 |
| 5 | Normal | A3-4 |  | 1 | HST Scenario 3 | 30% | -2.6 |
|  |  |  |  |  |  | 70% | 1.3 |
|  |  |  |  |  | HST Scenario 3-LTE500a (NOTE 1) | 30% | -2.6 |
|  |  |  |  |  |  | 70% | 1.3 |
|  |  |  |  |  | HST Scenario 3-LTE500b (NOTE 2) | 30% | -2.6 |
|  |  |  |  |  |  | 70% | 1.3 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.1 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500a Low (NOTE 1) | 30% | -5.1 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500b Low (NOTE 2) | 30% | -5.1 |
|  |  |  |  |  |  | 70% | -1.4 |
| 10 | Normal | A3-5 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500a (NOTE 1) | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.7 |
|  |  |  |  |  | HST Scenario 3-LTE500b (NOTE 2) | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.7 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.4 |
|  |  |  |  |  |  | 70% | -1.5 |
|  |  |  |  |  | HST Scenario 1-LTE500a Low (NOTE 1) | 30% | -5.4 |
|  |  |  |  |  |  | 70% | -1.5 |
|  |  |  |  |  | HST Scenario 1-LTE500b Low (NOTE 2) | 30% | -5.4 |
|  |  |  |  |  |  | 70% | -1.2 |
| 15 | Normal | A3-6 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500a (NOTE 1) | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500b (NOTE 2) | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.2 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500a Low (NOTE 1) | 30% | -5.2 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500b Low (NOTE 2) | 30% | -5.2 |
|  |  |  |  |  |  | 70% | -1.4 |
| 20 | Normal | A3-7 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500a (NOTE 1) | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.5 |
|  |  |  |  |  | HST Scenario 3-LTE500b (NOTE 2) | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.8 |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.3 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500a Low (NOTE 1) | 30% | -5.3 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500b Low (NOTE 2) | 30% | -5.3 |
|  |  |  |  |  |  | 70% | -1.4 |
| NOTE 1: Not applicable if the BS manufacturer declares supported maximum Doppler frequency is 1750 Hz NOTE 2: Not applicable if the BS manufacturer declares supported maximum Doppler frequency is 1944 Hz |  |  |  |  |  |  |  |

--------------End of text proposal-------------