# 3GPP TSG-RAN WG4 Meeting #95-e														R4-2008063

# Electronic meeting, 25 May – 5 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.141 | CR | 0940 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.141: Rel-16 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For transmitter tests, the “edges” in generation of TC18 are not clear since the GSM carriers are placed adjacent to each other starting from the high Base Station RF Bandwidth edge. During RAN4#93 (Reno) meeting, CR to 37.141 (R4-1914277) was agreed that updated sub-cluase 6.5.1.4.2 Procedure to modulation quality test. Changes in R4-1916083 updated references to single RAT test specification without updated steps. However, due to the clash with other CR agreed before (R4-1913770) for last bullet in step 2 of 6.5.1.4.2, references with steps 1 to 2 were not removed. This CR removes for NB-IoT carrier steps in references. Multi-band operation for CS18 and CS19 has been introduced in Clause 4.8.7 but is missing in Clause 5.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | “edges” is replaced by “edge” in clause 4.8.18.1 In sub-clause 6.5.1.4.2 Additional space added in “..in step 1” Removal of “step 1 to 2” in reference to TS 36.141. CS18 and CS19 are added to multi-band operation in Clause 5.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.8.18.1, 5.3, 6.5.1.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 4.8.18.1	TC18 generation

TC18 is constructed using the following method:

-	The Base Station RF Bandwidth shall be the declared maximum Base Station RF Bandwidth.

-	Place a GSM carrier at the lower Base Station RF Bandwidth edge. Place a 10 MHz E-UTRA carrier adjacent to the upper Base Station RF Bandwidth edge. Place the power boosted NB-IoT PRB at the outermost guard-band position eligible for NB-IoT PRB (according to sub-clause 4.5.3) at the upper Base Station RF Bandwidth edge and adjacent to the E-UTRA PRB edge as close as possible (i.e., away from the upper Base Station RF Bandwidth edge). The specified FOffset-RAT shall apply.

-	For transmitter tests, add GSM carriers at the edge using 600 kHz spacing until no more GSM carriers are supported or no more GSM carriers fit, then select as many 10 MHz E-UTRA carriers that the BS supports and that fit in the rest of the Base Station RF Bandwidth. Place the carriers adjacent to each other starting from the high Base Station RF Bandwidth edge. The nominal carrier spacing defined in subclause 4.5.1 shall apply.

-	If 10 MHz E-UTRA carriers are not supported by the BS the narrowest supported channel BW shall be selected instead.

--------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------------

## 5.3	Multi-band capable Base Stations

Table 5.3-1: Test configurations for Multi-Band capable BS (CS1-CS7, CS16 and CS18-CS19)

| BS test case | Test for Multi-Band capable BS |  | Test configuration for MBT |  |
| --- | --- | --- | --- | --- |
|  | Common antenna connector | Separate antenna connector | BC1/BC2 | BC3 |
| 6.2 Base Station output power | - | - | - | - |
| Base Station maximum output power | SBT, MBT | SBT, MBT | TC7a | TC7a |
| Additional regional requirement (only for band 34) | N/A | N/A | N/A | - |
| E-UTRA for DL RS power | SBT | SBT | - | - |
| UTRA FDD primary CPICH power | SBT | SBT | - | - |
| UTRA TDD primary CCPCH power | SBT | SBT | - | - |
| NB-IoT for DL RS power | SBT | SBT | - | - |
| 6.3 Output power dynamics | - | - |  |  |
| E-UTRA | SBT | SBT | - | - |
| UTRA FDD | SBT | SBT | - | - |
| UTRA TDD | SBT | SBT | - | - |
| GSM/EDGE | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| 6.4 Transmit ON/OFF power |  |  | - | - |
| Transmitter OFF power | MBT, SBT7 | MBT, SBT7 | N/A | TC7a |
| Transmitter transient period | MBT, SBT7 | MBT, SBT7 | N/A | TC7a |
| 6.5 Transmitted signal quality |  |  |  |  |
| 6.5.1 Modulation quality |  |  |  |  |
| E-UTRA | SBT, MBT | SBT, MBT | TC7a | TC7a |
| UTRA FDD | SBT, MBT | SBT, MBT | TC7a | N/A |
| UTRA TDD | SBT, MBT | SBT, MBT | N/A | TC7a |
| GSM/EDGE | SBT, MBT | SBT, MBT | TC7a | N/A |
| NB-IoT | N/A (Note 8) | N/A (Note 8) | - | - |
| NR | SBT, MBT | SBT, MBT | TC7a | TC7a |
| 6.5.2 Frequency error |  |  |  |  |
| E-UTRA | SBT, MBT | SBT, MBT | TC7a | TC7a |
| UTRA FDD | SBT, MBT | SBT, MBT | TC7a | N/A |
| UTRA TDD | SBT, MBT | SBT, MBT | N/A | TC7a |
| GSM/EDGE | SBT, MBT | SBT, MBT | TC7a | N/A |
| NB-IoT | N/A (Note 8) | N/A (Note 8) | - | - |
| NR | SBT, MBT | SBT, MBT | TC7a | TC7a |
| 6.5.3 Time alignment between transmitter branches |  |  |  |  |
| E-UTRA | SBT, MBT1 | SBT, MBT1 | TC7b | TC7b |
| UTRA FDD | SBT, MBT1 | SBT, MBT1 | TC7b | N/A |
| UTRA TDD | SBT | SBT | N/A | - |
| NB-IoT | N/A (Note 8) | N/A (Note 8) | - | - |
| NR | SBT, MBT1 | SBT, MBT1 | TC7b | TC7b |
| 6.6 Unwanted emissions |  |  |  |  |
| 6.6.1 Transmitter spurious emissions |  |  |  |  |
| (Category A) | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| (Category B) | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| Additional requirement for BC2 (Category B) | SBT, MBT3 | SBT2, MBT2, 3 | TC7b | N/A |
| Protection of the BS receiver of own or different BS | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| Additional spurious emissions requirements | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| Co-location with other Base Stations | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| 6.6.2 Operating band unwanted emissions |  |  |  |  |
| General requirement for Band Categories 1 and 3 | SBT, MBT | SBT2, MBT2 | TC7b | TC7b |
| General requirement for Band Category 2 | SBT, MBT | SBT2, MBT2 | TC7b | N/A |
| GSM/EDGE single-RAT requirement | SBT, MBT6 | SBT2, MBT2,6 | TC7c | N/A |
| Additional requirements | SBT, MBT | SBT2, MBT2 | - | - |
| 6.6.3 Occupied bandwidth |  |  |  |  |
| Minimum requirement | SBT | SBT | - | - |
| 6.6.4 Adjacent Channel Leakage Power Ratio (ACLR) |  |  | - | - |
| E- UTRA | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | TC7b |
| UTRA FDD | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | N/A |
| UTRA TDD | SBT, MBT4 | SBT2, MBT2, 4 | N/A | TC7b |
| Cumulative ACLR | SBT, MBT4 | SBT2 | TC7b | TC7b |
| NB-IoT | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | TC7b |
| NR | SBT, MBT4 | SBT2, MBT2, 4 | TC7b | TC7b |
| 6.7 Transmitter intermodulation |  |  |  |  |
| General requirement | SBT | SBT2 | - | - |
| Additional requirement (BC1 and BC2) | SBT | SBT2 | - | N/A |
| Additional requirement (BC3) | SBT | SBT2 | N/A | - |
| 7.2 Reference sensitivity level |  |  |  |  |
| E-UTRA | SBT | SBT | - | - |
| UTRA FDD | SBT | SBT | - | - |
| UTRA TDD | SBT | SBT | - | - |
| GSM/EDGE | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| 7.3 Dynamic range |  |  |  |  |
| E-UTRA | SBT | SBT | - | - |
| UTRA FDD | SBT | SBT | - | - |
| UTRA TDD | SBT | SBT | - | - |
| GSM/EDGE | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| 7.4 In- band selectivity and blocking |  |  |  |  |
| General blocking requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| General narrowband blocking requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| Additional narrowband blocking requirement for GSM/EDGE | SBT | SBT | - |  |
| GSM/EDGE requirements for AM suppression | SBT | SBT | - |  |
| Additional BC3 blocking requirement | MBT, SBT7 | SBT, MBT5 | N/A | TC7b |
| 7.5 Out-of-band blocking |  |  |  |  |
| General requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| Co-location requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| 7.6 Receiver spurious emissions |  |  |  |  |
| General requirement | SBT, MBT | SBT2 MBT2 | TC7b | TC7b |
| Additional requirement for BC2 (Category B) | SBT, MBT3 | SBT2, MBT2, 3 | TC7b | N/A |
| 7.7 Receiver intermodulation |  |  |  |  |
| General intermodulation requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| General narrowband intermodulation requirement | MBT, SBT7 | SBT, MBT5 | TC7b | TC7b |
| Additional narrowband intermodulation requirement for GSM/EDGE | SBT | SBT | - | N/A |
| 7.8 In-channel selectivity |  |  |  |  |
| E-UTRA requirement | SBT | SBT | - | - |
| NB-IoT | SBT | SBT | - | - |
| NR | SBT | SBT | - | - |
| NOTE 1: MBT is only applicable when DB-DC-HSDPA / inter-band CA is supported. NOTE 2: Single-band requirement apply to each antenna connector for both multi-band operation test and single-band operation test. For single-band operation test, other antenna connector(s) is (are) terminated. NOTE 3: For multi-band operation, this additional requirement for BC2 is applicable only when all supported operating bands belong to BC2 and GSM/EDGE is configured in all operating bands. NOTE 4: For ACLR, MBT shall be applied for the Inter RF Bandwidth gap only. In case of a BS capable of CS4, CS5, CS6, CS7 or CS18, the referenced test configuration shall be the corresponding one for CS1, CS2, CS3 or CS16 respectively, i.e. without GSM/EDGE carriers. NOTE 5: MBT is only applied for multi-band receiver. NOTE 6: MBT is only applicable for multi-band BS supporting CS4, CS5 or CS6 in at least one band. NOTE 7: SBT is only applicable if different Capability Sets are declared for single-band and multi-band operation. NOTE 8: There is no specific test with NB-IoT for those requirements, tests could be performed using E-UTRA signal only, without NB-IoT. |  |  |  |  |

--------------------------------------------------------- NEXT CHANGE ----------------------------------------------------------------

## 6.5	Transmitted signal quality

### 6.5.1	Modulation quality

#### 6.5.1.1	Definition and applicability

Modulation quality is defined by the difference between the measured carrier signal and a reference signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM), Peak Code Domain Error (PCDE) or Relative Code Domain Error (RCDE).

#### 6.5.1.2	Minimum Requirement

The minimum requirement is in TS 37.104 [2] subclause 6.5.1.

#### 6.5.1.3		Test purpose

The test purpose is to verify that modulation quality is within the limit specified by the minimum requirement.

#### 6.5.1.4	Method of test

##### 6.5.1.4.1	Initial conditions

Test environment: 	normal; see Annex B.2.

Base Station RF Bandwidth position to be tested: 	BRFBW, MRFBW and TRFBW single-band operation, see subclause 4.9.1 single-band operation.

1)	Connect measurement device to the base station antenna connector as shown in Annex D.1.1.

##### 6.5.1.4.2	Procedure

1)	Set the BS to transmit according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in subclause 4.9.2.

2)	Perform the modulation quality measurement on each carrier as outlined in respective measurement procedures in RAT-specific specifications, with the test configurations, test models or corresponding set of physical channels as defined in step 1:

-	For EVM measurement on a NR carrier, see TS 38.141-1 [26] subclause 6.5.3.4.2.

-	For EVM measurement on an E-UTRA carrier, see TS 36.141 [9] subclause 6.5.2.4.2.

-	For EVM measurement on a UTRA FDD carrier, see TS 25.141 [10] subclause 6.7.1.4.2.

-	For PCDE measurement on a UTRA FDD carrier, see TS 25.141 [10] subclause 6.7.2.4.2.

-	For RCDE measurement on a UTRA FDD carrier, see TS 25.141 [10] subclause 6.7.4.4.2.

-	For EVM measurement on a UTRA TDD carrier, see TS 25.142 [12] subclause 6.8.1.4.2.2.

-	For PCDE measurement on a UTRA TDD carrier, see TS 25.142 [12] subclause 6.8.2.4.1.2.

-	For RCDE measurement on a UTRA TDD carrier, see TS 25.142 [12] subclause 6.8.3.4.1.1.

-	For EVM measurement on a GSM/EDGE carrier, see TS 51.021 [11] subclause 6.2.2.

- 	For EVM measurement on an NB-IoT carrier, see TS 36.141 [9] subclause 6.5.2.4.2.

In addition, for a multi-band capable BS, the following step shall apply:

3)	For multi-band capable BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band. For multi-band capable BS with separate antenna connector, the antenna connector not being under test in case of SBT or MBT shall be terminated.