3GPP TSG-WG4 Meeting #95-e 	R4-2006960

# Online, 25th May – 05th June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0334 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to FR1 QPSK UL RMC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The TBS size for QPSK UL RMCs with 1 RB allocation are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct TBS size from 56 to 48 bits. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong information remains in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 38.521-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.2.2.2	DFT-s-OFDM QPSK

Table A.2.2.2-1: Reference Channels for DFT-s-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 5 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 10 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 15 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 15 | 15 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 15 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 20 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 20 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 20 | 15 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 25 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 15 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 25 | 15 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 30 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2-2: Reference Channels for DFT-s-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 5 | 30 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 15 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 20 | 30 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 30 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 25 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 30 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 30 | 30 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 40 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 40 | 30 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 50 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 50 | 30 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 80 | 30 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 90 | 30 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 90 | 30 | 243 | 11 | QPSK | 2 | 1/6 | 12040 | 24 | 2 | 4 | 64152 | 32076 |
|  | 100 | 30 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 100 | 30 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.2-3: Reference Channels for DFT-s-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 10 | 60 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 15 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 3960 | 1980 |
|  | 25 | 60 | 30 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 7920 | 3960 |
|  | 30 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 30 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 40 | 60 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 40 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 50 | 60 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 50 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 60 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 60 | 60 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 80 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 80 | 60 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 90 | 60 | 60 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 15840 | 7920 |
|  | 90 | 60 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 100 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.2.2.6	CP-OFDM QPSK

Table A.2.2.6-1: Reference Channels for CP-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 13 | 11 | QPSK | 2 | 1/6 | 672 | 16 | 2 | 1 | 3432 | 1716 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 10 | 15 | 52 | 11 | QPSK | 2 | 1/6 | 2600 | 16 | 2 | 1 | 13728 | 6864 |
|  | 15 | 15 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 15 | 15 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 20 | 15 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 20 | 15 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 25 | 15 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 25 | 15 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.6-2: Reference Channels for CP-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 5 | 30 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 15 | 30 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 20 | 30 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 20 | 30 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 25 | 30 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 25 | 30 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 30 | 30 | 39 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10296 | 5148 |
|  | 30 | 30 | 78 | 11 | QPSK | 2 | 1/6 | 3848 | 24 | 2 | 2 | 20592 | 10296 |
|  | 40 | 30 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 40 | 30 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 50 | 30 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 50 | 30 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 109 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28776 | 14388 |
|  | 80 | 30 | 217 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57288 | 28644 |
|  | 90 | 30 | 123 | 11 | QPSK | 2 | 1/6 | 6152 | 24 | 2 | 2 | 32472 | 16236 |
|  | 90 | 30 | 245 | 11 | QPSK | 2 | 1/6 | 12296 | 24 | 2 | 4 | 64680 | 32340 |
|  | 100 | 30 | 137 | 11 | QPSK | 2 | 1/6 | 6792 | 24 | 2 | 2 | 36168 | 18084 |
|  | 100 | 30 | 273 | 11 | QPSK | 2 | 1/6 | 13576 | 24 | 2 | 4 | 72072 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.2.6-3: Reference Channels for CP-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size | Transport block CRC | LDPC Base Graph | Number of code blocks per slot (Note 3) | Total number of bits per slot | Total modulated symbols per slot |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 10 | 60 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 25 | 60 | 31 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 8184 | 4092 |
|  | 30 | 60 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 30 | 60 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 40 | 60 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 40 | 60 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 50 | 60 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 50 | 60 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 60 | 60 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 60 | 60 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 80 | 60 | 54 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 14256 | 7128 |
|  | 80 | 60 | 107 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 28248 | 14124 |
|  | 90 | 60 | 61 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 16104 | 8052 |
|  | 90 | 60 | 121 | 11 | QPSK | 2 | 1/6 | 6024 | 24 | 2 | 2 | 31944 | 15972 |
|  | 100 | 60 | 68 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17952 | 8976 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.2.3.2	DFT-s-OFDM QPSK

Table A.2.3.2-1: Reference channels for DFT-s-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 5 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 10 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 15 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 15 | 15 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 15 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 15 | 15 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 20 | 15 | 20 | 11 | QPSK | 2 | 1/6 | 1032 | 16 | 2 | 1 | 5280 | 2640 |
|  | 20 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 20 | 15 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 25 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 15 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 25 | 15 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 30 | 15 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-2: Reference channels for DFT-s-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 5 | 30 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 15 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 20 | 30 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 20 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 25 | 30 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 25 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 30 | 30 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 30 | 30 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 40 | 30 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 40 | 30 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 50 | 30 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 50 | 30 | 128 | 11 | QPSK | 2 | 1/6 | 6408 | 24 | 2 | 2 | 33792 | 16896 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 80 | 30 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 90 | 30 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 90 | 30 | 243 | 11 | QPSK | 2 | 1/6 | 12040 | 24 | 2 | 4 | 64152 | 32076 |
|  | 100 | 30 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 100 | 30 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.2-3: Reference channels for DFT-s-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | DFT-s-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 5 | 11 | QPSK | 2 | 1/6 | 256 | 16 | 2 | 1 | 1320 | 660 |
|  | 10 | 60 | 10 | 11 | QPSK | 2 | 1/6 | 504 | 16 | 2 | 1 | 2640 | 1320 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 15 | 11 | QPSK | 2 | 1/6 | 768 | 16 | 2 | 1 | 3960 | 1980 |
|  | 25 | 60 | 30 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 7920 | 3960 |
|  | 30 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 30 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 40 | 60 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 40 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 50 | 60 | 32 | 11 | QPSK | 2 | 1/6 | 1608 | 16 | 2 | 1 | 8448 | 4224 |
|  | 50 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 60 | 60 | 36 | 11 | QPSK | 2 | 1/6 | 1800 | 16 | 2 | 1 | 9504 | 4752 |
|  | 60 | 60 | 75 | 11 | QPSK | 2 | 1/6 | 3752 | 16 | 2 | 1 | 19800 | 9900 |
|  | 80 | 60 | 50 | 11 | QPSK | 2 | 1/6 | 2472 | 16 | 2 | 1 | 13200 | 6600 |
|  | 80 | 60 | 100 | 11 | QPSK | 2 | 1/6 | 5000 | 24 | 2 | 2 | 26400 | 13200 |
|  | 90 | 60 | 60 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 15840 | 7920 |
|  | 90 | 60 | 120 | 11 | QPSK | 2 | 1/6 | 5896 | 24 | 2 | 2 | 31680 | 15840 |
|  | 100 | 60 | 64 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 16896 | 8448 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 6.1.4.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

### A.2.3.6	CP-OFDM QPSK

Table A.2.3.6-1: Reference channels for CP-OFDM QPSK for 15 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 4 and 9 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 4 and 9 (Note 3) | Total number of bits per slot for slots 4 and 9 | Total modulated symbols per slot for slots 4 and 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-50 | 15 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 15 | 13 | 11 | QPSK | 2 | 1/6 | 672 | 16 | 2 | 1 | 3432 | 1716 |
|  | 5 | 15 | 25 | 11 | QPSK | 2 | 1/6 | 1256 | 16 | 2 | 1 | 6600 | 3300 |
|  | 10 | 15 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 10 | 15 | 52 | 11 | QPSK | 2 | 1/6 | 2600 | 16 | 2 | 1 | 13728 | 6864 |
|  | 15 | 15 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 15 | 15 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 20 | 15 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 20 | 15 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 25 | 15 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 25 | 15 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 30 | 15 | 80 | 11 | QPSK | 2 | 1/6 | 3976 | 24 | 2 | 2 | 21120 | 10560 |
|  | 30 | 15 | 160 | 11 | QPSK | 2 | 1/6 | 7944 | 24 | 2 | 3 | 42240 | 21120 |
|  | 40 | 15 | 108 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28512 | 14256 |
|  | 40 | 15 | 216 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57024 | 28512 |
|  | 50 | 15 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
|  | 50 | 15 | 270 | 11 | QPSK | 2 | 1/6 | 13320 | 24 | 2 | 4 | 71280 | 35640 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-2: Reference channels for CP-OFDM QPSK for 30 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 8, 9, 18 and 19 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 8, 9, 18 and 19 (Note 3) | Total number of bits per slot for slots 8, 9, 18 and 19 | Total modulated symbols per slot for slots 8, 9, 18 and 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 5-100 | 30 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 5 | 30 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 5 | 30 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 10 | 30 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 10 | 30 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 15 | 30 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 15 | 30 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 20 | 30 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 20 | 30 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 25 | 30 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 25 | 30 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 30 | 30 | 39 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10296 | 5148 |
|  | 30 | 30 | 78 | 11 | QPSK | 2 | 1/6 | 3848 | 24 | 2 | 2 | 20592 | 10296 |
|  | 40 | 30 | 53 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 13992 | 6996 |
|  | 40 | 30 | 106 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 27984 | 13992 |
|  | 50 | 30 | 67 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17688 | 8844 |
|  | 50 | 30 | 133 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35112 | 17556 |
|  | 60 | 30 | 81 | 11 | QPSK | 2 | 1/6 | 4040 | 24 | 2 | 2 | 21384 | 10692 |
|  | 60 | 30 | 162 | 11 | QPSK | 2 | 1/6 | 8064 | 24 | 2 | 3 | 42768 | 21384 |
|  | 80 | 30 | 109 | 11 | QPSK | 2 | 1/6 | 5384 | 24 | 2 | 2 | 28776 | 14388 |
|  | 80 | 30 | 217 | 11 | QPSK | 2 | 1/6 | 10752 | 24 | 2 | 3 | 57288 | 28644 |
|  | 90 | 30 | 123 | 11 | QPSK | 2 | 1/6 | 6152 | 24 | 2 | 2 | 32472 | 16236 |
|  | 90 | 30 | 245 | 11 | QPSK | 2 | 1/6 | 12296 | 24 | 2 | 4 | 64680 | 32340 |
|  | 100 | 30 | 137 | 11 | QPSK | 2 | 1/6 | 6792 | 24 | 2 | 2 | 36168 | 18084 |
|  | 100 | 30 | 273 | 11 | QPSK | 2 | 1/6 | 13576 | 24 | 2 | 4 | 72072 | 36036 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.2.3.6-3: Reference channels for CP-OFDM QPSK for 60 kHz SCS

| Parameter | Channel bandwidth | Subcarrier Spacing | Allocated resource blocks | CP-OFDM Symbols per slot (Note 1) | Modulation | MCS Index (Note 2) | Target Coding Rate | Payload size for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Transport block CRC | LDPC Base Graph | Number of code blocks per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 (Note 3) | Total number of bits per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 | Total modulated symbols per slot for slots 16, 17, 18, 19, 36, 37, 38 and 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unit | MHz | KHz |  |  |  |  |  | Bits | Bits |  |  | Bits |  |
|  | 10-100 | 60 | 1 | 11 | QPSK | 2 | 1/6 | 48 | 16 | 2 | 1 | 264 | 132 |
|  | 10 | 60 | 6 | 11 | QPSK | 2 | 1/6 | 304 | 16 | 2 | 1 | 1584 | 792 |
|  | 10 | 60 | 11 | 11 | QPSK | 2 | 1/6 | 552 | 16 | 2 | 1 | 2904 | 1452 |
|  | 15 | 60 | 9 | 11 | QPSK | 2 | 1/6 | 456 | 16 | 2 | 1 | 2376 | 1188 |
|  | 15 | 60 | 18 | 11 | QPSK | 2 | 1/6 | 928 | 16 | 2 | 1 | 4752 | 2376 |
|  | 20 | 60 | 12 | 11 | QPSK | 2 | 1/6 | 608 | 16 | 2 | 1 | 3168 | 1584 |
|  | 20 | 60 | 24 | 11 | QPSK | 2 | 1/6 | 1192 | 16 | 2 | 1 | 6336 | 3168 |
|  | 25 | 60 | 16 | 11 | QPSK | 2 | 1/6 | 808 | 16 | 2 | 1 | 4224 | 2112 |
|  | 25 | 60 | 31 | 11 | QPSK | 2 | 1/6 | 1544 | 16 | 2 | 1 | 8184 | 4092 |
|  | 30 | 60 | 19 | 11 | QPSK | 2 | 1/6 | 984 | 16 | 2 | 1 | 5016 | 2508 |
|  | 30 | 60 | 38 | 11 | QPSK | 2 | 1/6 | 1928 | 16 | 2 | 1 | 10032 | 5016 |
|  | 40 | 60 | 26 | 11 | QPSK | 2 | 1/6 | 1288 | 16 | 2 | 1 | 6864 | 3432 |
|  | 40 | 60 | 51 | 11 | QPSK | 2 | 1/6 | 2536 | 16 | 2 | 1 | 13464 | 6732 |
|  | 50 | 60 | 33 | 11 | QPSK | 2 | 1/6 | 1672 | 16 | 2 | 1 | 8712 | 4356 |
|  | 50 | 60 | 65 | 11 | QPSK | 2 | 1/6 | 3240 | 16 | 2 | 1 | 17160 | 8580 |
|  | 60 | 60 | 40 | 11 | QPSK | 2 | 1/6 | 2024 | 16 | 2 | 1 | 10560 | 5280 |
|  | 60 | 60 | 79 | 11 | QPSK | 2 | 1/6 | 3912 | 24 | 2 | 2 | 20856 | 10428 |
|  | 80 | 60 | 54 | 11 | QPSK | 2 | 1/6 | 2664 | 16 | 2 | 1 | 14256 | 7128 |
|  | 80 | 60 | 107 | 11 | QPSK | 2 | 1/6 | 5256 | 24 | 2 | 2 | 28248 | 14124 |
|  | 90 | 60 | 61 | 11 | QPSK | 2 | 1/6 | 3104 | 16 | 2 | 1 | 16104 | 8052 |
|  | 90 | 60 | 121 | 11 | QPSK | 2 | 1/6 | 6024 | 24 | 2 | 2 | 31944 | 15972 |
|  | 100 | 60 | 68 | 11 | QPSK | 2 | 1/6 | 3368 | 16 | 2 | 1 | 17952 | 8976 |
|  | 100 | 60 | 135 | 11 | QPSK | 2 | 1/6 | 6664 | 24 | 2 | 2 | 35640 | 17820 |
| NOTE 1: PUSCH mapping Type-A and single-symbol DM-RS configuration Type-1 with 2 additional DM-RS symbols, such that the DM-RS positions are set to symbols 2, 7, 11. DMRS is [TDM'ed] with PUSCH data. NOTE 2: MCS Index is based on MCS table 5.1.3.1-1 defined in TS 38.214 [10]. NOTE 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit) |  |  |  |  |  |  |  |  |  |  |  |  |  |

< End of changes >