3GPP TSG-RAN WG4 Meeting #95-e	R4-2008982

Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0233 | rev | 1 | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-3: corrections on ACS for intra-band contiguous EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Resubmission of endorsed Draft CR R4-2005208. - For NR ACS case 2 for intra-band Contiguous EN-DC, the equation -42.7 +10log10(NRB,cSCSc/ BWagg) shall be changed to -42.7 +10log10(12*NRB,cSCSc/ BWagg) otherwise it would cause more stringent requirement when testing ACS case 2 compared to single carrier. Furthermore, to align with TS38.01-1, it is better to change to -42.7+10log10(NRB,c/NRRB_agg) - There is no clear definition for BWagg, and it should be replaced by ENBW |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TS38.211 as a reference 13 is added - The definition of NRB,c and NR_agg are added into Symbol section - Replace 10log10(NRB,cSCSc/ BWagg) by 10log10(NRB,c / NRagg) in table 7.5B.1-2. - BWagg is replaced by ENBW in the first column of table 7.5B.1-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It would cause stringent requirement when testing ACS case 2 for intra-band contiguous EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 7.5B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

--------------Start of change-------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone"

[3]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone"

[4]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception"

[5]	3GPP TS 38.521-3: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[7]	3GPP TS 36.211: "E-UTRA; Physical channels and modulation"

[8]	3GPP TS 36.331: " Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification"

[9]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification"

[10]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities

[12]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[13]	3GPP TS 38.211: "NR; Physical channels and modulation".

-------------End of change-------------

--------------Start of change-------------

3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for CA or DC operation, for serving cell c.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for CA or DC operation, for serving cell c

BWE-UTRA_Channel	Channel bandwidth of E-UTRA carrier

BWE-UTRA_Channel_CA	Channel bandwidth of E-UTRA sub-block which is composed of intra-band contiguous CA E-UTRA carriers

BWNR_Channel	Channel bandwidth of NR carrier

BWNR_Channel_CA	Channel bandwidth of NR sub-block which is composed of intra-band contiguous CA NR carriers

Ceil(x)	Rounding upwards; ceil(x) is the smallest integer such that ceil(x) ≥ x

EN-DCACLR	The ratio of the filtered mean power centred on the aggregated sub-block bandwidth ENBW to the filtered mean power centred on an adjacent bandwidth of the same size ENBW

E-UTRAACLR	E-UTRA ACLR

FC	RF reference frequency for the carrier center on the channel raster

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

FOOB	The boundary between the NR out of band emission and spurious emission domains

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resource blocks

Max()	The largest of given numbers

Min()	The smallest of given numbers

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated aggregated channel bandwidth

NRB_agg=1jNRB,cj*2μj  for carrier 1 to j, where μ is defined in TS 38.211 [13]

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,cj=NRBj*2μj  for carrier j, where μ is defined in TS 38.211 [13]

PCMAX	The configured maximum UE output power

RBstart	Indicates the lowest RB index of transmitted resource blocks

Wgap	The sub-block gap between the two sub-blocks

-------------End of change-------------

--------------Start of change-------------

7.5B	Adjacent channel selectivity for DC in FR1

7.5B.1	Intra-band contiguous EN-DC in FR1

Intra-band contiguous EN-DC ACS requirement and parameters are defined for test case 1 in Table 7.5B.1-1 and for test case 2 in Table 7.5B.1-2.

Table 7.5B.1-1: ACS test case 1

| EN-DC Aggregated Bandwidth, MHz | <=100 | >100, <=120 | >120, <=140 | >140, <=160 |
| --- | --- | --- | --- | --- |
| ACS, dB | X1 | 19.2 | 18.5 | 17.9 |
| Pinterferer, dBm | PI 2 | Aggregated power + 17.7 dB | Aggregated power + 17 dB | Aggregated power + 16.4dB |
| Pw in Transmission BW configuration, per CC, dBm | REFSENS +14dB |  |  |  |
| NOTE 1: X is ACS level at the specified EN-DC aggregated bandwidth from Table 7.5.1A-1 in TS 36.101 [4] NOTE 2: PI is from Table 7.5.1A-2 in TS 36.101 [4] NOTE 3: Jammer BW and offset is from Table 7.5.1A-2 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 4: For NR carrier, the transmitter shall be set to 4dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in subclause 6.2B.4. NOTE 5: For E-UTRA carrier, the transmitter shall be set to 4 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in subclause 6.2B.4 for single carrier. |  |  |  |  |

Table 7.5B.1-2: ACS test case 2

| EN-DC Aggregated Bandwidth, ENBW, MHz | ≤100 | >100, ≤120 | >120, ≤140 | >140, ≤160 |
| --- | --- | --- | --- | --- |
| Pw in Transmission Bandwidth Configuration, perCC, dBm | PW 1 | -42.7 +10log10(NRB,c/ NRB_agg) | -42 +10log10(NRB,c/ NRB_agg) | -41.4 +10log10(NRB,c/ NRB_agg) |
| Pinterferer, dBm | -25 |  |  |  |
| NOTE 1: PW is wanted signal power level at the specified EN-DC aggregated Bandwidth from Table 7.5.1A-3 in TS 36.101 [4] NOTE 2: Jammer BW and offset is from Table 7.5.1A-3 [4] and is applied from the lowest edge of the lowest carrier and the highest edge of the highest carrier NOTE 3: For NR carrier, the transmitter shall be set to 24 dB below PCMAX_L,f,c,NR at the minimum uplink configuration specified in Table 7.3.2-3 [2] with PCMAX_L,f,c,NR as defined in subclause 6.2B.4. NOTE 4: For E-UTRA carrier, the transmitter shall be set to 24 dB below PCMAX_L_E-UTRA,c at the minimum uplink configuration specified in Table 7.3.1-2 [4] with PCMAX_L_E-UTRA,c as defined in subclause 6.2B.4 for single carrier. |  |  |  |  |

7.5B.2	Intra-band non-contiguous EN-DC in FR1

-------------End of change-------------