3GPP TSG-RAN4 Meeting #95-e	R4-2006650

# Online, , 25th May 2020 - 5th Jun 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 0251 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mirror CR for 38.101-3: Corrections for Ppowerclass and referenced sections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-5 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Several instances of Ppowerclass are missing “,E-UTRA” or “,NR” and there are some incorrect references. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds missing “,E-UTRA” or “,NR” to Ppowerclass as needed and corrects the incorrect section and/or spec references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ppowerclass may be misinterpreted and reference errors will persist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2B.2.1.2, 6.2B.2.2.2, 6.2B.4.1.1, 6.2B.4.1.3, 6.2B.4.1.3a, 6.4B.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<First changed section >

##### 6.2B.2.1.2	MPR for power class 3 and power class 2

MPR in this clause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with EN-DC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in clause 6.2.4 of TS 36.101 [4] and 6.2.2 of TS 38.101-1 [2] apply. For a UE supporting dynamic power sharing for DC_(n)71AA for which dual simultaneous uplink transmissions are mandatory and A-MPR defined in clause 6.2B.3.1.1 is applied as MPR. The allowed maximum output power reduction applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA = 	15	;	 0 ≤ B < 0.5

10	;	 0.5 ≤ B < 1.0

8	; 	1.0 ≤ B < 2.0

6	; 	2.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

For UEs not supporting dynamic power sharing,

For E-UTRA

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000,000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000,000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

<Next changed section >

##### 6.2B.2.2.2	MPR for power class 3 and power class 2

MPR in this clause is applicable for power class 3 and power class 2 UEs indicating IE dualPA-Architecture supported with EN-DC power class being the same as the E-UTRA and NR power class, otherwise the UE can use as much MPR as needed to fulfil emissions requirements when scheduled with dual uplink transmission. For UEs scheduled with single uplink transmission, MPR in clause 6.2.4 of TS 36.101 [4] and 6.2.2 of TS 38.101-1 [2] apply.  The allowed maximum output power reduction for IM3 related emissions applied to transmission on the MCG and the SCG is defined as follows:

MPRENDC = MA

Where MA is defined as follows

MA = 	18	; 	0 ≤ B < 1.0

17	; 	1.0 ≤ B < 2.0

16	; 	2.0 ≤ B < 5.0

15	; 	5.0 ≤ B

Where:

For UEs supporting dynamic power sharing,

B = (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

For UEs not supporting dynamic power sharing,

For E-UTRA

B= (LCRB_alloc, E-UTRA * 12* SCSE-UTRA + 12 * SCSNR)/1,000.000

Where SCSNR = 15 kHz is assumed in calculation of B.

For NR

B = (12 * SCSE-UTRA + LCRB_alloc,NR * 12 * SCSNR)/1,000.000

Where SCSE-UTRA = 15 kHz is assumed in calculation of B.

and MA is reduced by 1 dB for B < 2.

<Next changed section>

##### 6.2B.4.1.1	Intra-band contiguous EN-DC

The following requirements apply for one component carrier per CG configured for synchronous DC.

For intra-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10].

The configured maximum output power PCMAX,f,c,NR (q) in physical channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified  as follows:

PCMAX_L,f,c,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PNR) - TC_NR, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ),  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MAX(MPRc,A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC, PNR, PPowerClass, EN-DC, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8] which is the same as PLTE in TS 38.213 [10];

-	∆tC_EUTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 of TS 36.101 [4] applies; ∆tC_EUTRA, c = 0 dB otherwise;

and whenever NS_01 is not indicated within CG 1:

-	for a UE indicating support of dynamicPowerSharing, the MPRc and the A-MPRc are determined in accordance with the DCI of serving cell c of the CG 1 and the specification in clause 6.2.4 of TS 36.101 [4];

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

and whenever NS_01 is indicated in CG 1:

-	for a UE indicating support of dynamicPowerSharing, the MPRc is determined in accordance with the DCI of serving cell c of the CG 1 and the specification in clause 6.2.4 of TS 36.101 [4];

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

The configured maximum output power PCMAX_ NR,c (q) in physical channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L_ NR,c and PCMAX H _ NR,c are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified  as follows:

PCMAX_L,f,c,,NR = MIN {MIN(PEMAX,c , PEMAX, EN-DC, PNR) - TC_NR, c, (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ),  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc,A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC, PNR, PPowerClass, EN-DC, PPowerClass,NR – ΔPPowerClass,NR }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	PLTE signalled by RRC as p-MaxEUTRA-r15 in TS 36.331 [8]

-	PNR is the value given by the field  p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in  [9] and signalled by RRC;

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in clause 6.2B.4.2.1 for EN-DC, the individual Power Class defined in table 6.2B.1.1 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3 for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	PPowerClass, EN-DC is defined in clause 6.2B.1.1 for intra-band contiguous EN-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔPPowerClass,EN-DC is 3 dB for a power class 2 capable EN-DC UE when  LTE UL/DL configuration is 0 or 6; or LTE UL/DL configuration is 1 and special subframe configuration is 0 or 5; ΔPPowerClass,EN-DC = 3 dB when the IE p-maxUE-FR1 as defined in TS 36.331 [4] is provided and set to the maximum output power of the default power class or lower; otherwise ΔPPowerClass,EN-DC = 0 dB;

and whenever an NS signalling other than NS_01 is indicated within CG 2:

-	for a UE indicating support of dynamicPowerSharing, A-MPRc = A-MPR'c with A-MPR'c determined in accordance with clause 6.2B.3.1 and MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, A-MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the A-MPRc is determined in accordance with clause 6.2B.3.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and MPRc = 0 dB;

and whenever NS_01 is indicated in CG 2.

-	for a UE indicating support of dynamicPowerSharing, MPRc = MPR'c with MPR'c determined in accordance with clause 6.2B.2.1 and A-MPRc = 0 dB if transmission(s) in subframe p on CG 1 overlap in time with physical channel q on CG 2;

-	for a UE indicating support of dynamicPowerSharing, MPRc is determined in accordance with TS 38.101-1 [2] if transmission(s) in subframe p on CG 1 does not overlap in time with physical channel q on CG 2;

-	for a UE not indicating support of dynamicPowerSharing, the MPRc is determined in accordance with clause 6.2B.2.1 with parameters applicable for UEs not indicating support of dynamicPowerSharing and A-MPRc = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

If the EN-DC UE is not supporting dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above.

If the UE does not support dynamic power sharing,

PTotalEN-DC = MIN { PEMAX, EN-DC , PPowerClass, EN-DC - ΔPPowerClass,EN-DC } + 0.3 dB

For UEs indicating support of dynamicPowerSharing in the UE-MRDC-Capability IE the UE can configure the total maximum transmission power PTotalEN-DC within the range

PEN-DC,tot_L ≤ PTotalEN-DC ≤  PEN-DC,tot_H

where

PEN-DC,tot_L (p,q) = MIN{ PPowerClass,EN-DC - ΔPPowerClass,EN-DC – MAX{MPRtot, A-MPRtot}, PEMAX,EN-DC}

PEN-DC,tot_H (p,q) = MIN{PPowerClass,EN-DC, PEMAX,EN-DC }

for sub-frame p on CG 1 overlapping with physical channel q on CG 2 and with MPRtot and A-MPRtot in accordance with 6.2B.2.1 and clause 6.2B.3.1, respectively.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,f,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

For UEs indicating support of dynamicPowerSharing, the measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX_L and PCMAX_L specified in Table 6.2B.4.1.1-2.

When an UL subframe transmission p from E-UTRA overlap with a physical channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.1-1 when same or different subframes and physical channel durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.1-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical channel overlapping with E-UTRA subframe p,

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PEN-DC,tot_L

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PEN-DC,tot_L

If a= FALSE and the configured transmission power spectral density between the MCG and SCG differs by less than [6] dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If (a=TRUE) AND (b=FALSE) and the configured transmission power spectral density between the MCG and SCG differs by less than [6] dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,,NR (q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC - ΔPPowerClass,EN-DC }

ELSE If b= TRUE or the transmission power after power scaling spectral density between the MCG and SCG differs by more than [6] dB

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC- ΔPPowerClass,EN-DC }

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.1 for intra-band EN-DC;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6] dB

-	pCMAX  E-UTRA,c (p) is the linear value of PCMAX  E-UTRA,c (p), the real configured max power for E-UTRA

-	pCMAX,f,c  NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.4.1.1-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |

If the UE supports dynamic power sharing, and when E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions and unless otherwise stated

10log(pCMAX L,f,c,NR(q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,NR(q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.1-2.

If the UE supports dynamic power sharing, the measured maximum output power in subframe p on CG 1, pUMAX,c,E-UTRA,  shall meet the requirements in clause 6.2.5 in TS 36.101 [4] with the limits PCMAX_L,c and PCMAX_H,c replaced by PCMAX_L_ E-UTRA,c and PCMAX_H_ E- UTRA,c as specified above, respectively.

If the configured transmission power spectral density between the MCG and SCG differs by more than [6] dB, then

PUMAX,f,c,NR (q) ≤ 10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

<Next changed section>

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

For inter-band dual connectivity with one uplink serving cell or more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG and one uplink serving cell on the NR CG, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation, PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10]. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, the PCMAX applies to the entire E-UTRA CG.

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell and E-UTRA Pcell is FDD, the UE is not expected to be configured with more than one serving cells in the uplink.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier(s) shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, EN-DC , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + tC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for the E-UTRA CG as specified in TS 36.101 [4] clause 6.2.5A modified by PLTE as follows:

PCMAX_L_ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass – ΔPPowerClass) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PLTE, Ppowerclass,ENDC }

PCMAX H _ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c , PPowerClass ,PLTE, Ppowerclass,ENDC}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PPowerClass refers to the maximum output power of the E-UTRA intra-band CA power class given in Table 6.2.2A-1 of TS 36.101 [4],

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in clause 6.2.3A and clause 6.2.4A of TS 36.101 [4] respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX_ E-UTRA,c is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers within the E-UTRA CG.

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in TS 38.331 [9];

-	Δtc_E-UTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0 dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0 dB;

-	ΔTIB,c specified in clause 6.2B.4.2.3 for EN-DC, the individual Power Class defined in table 6.2B.1.3 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3 for inter-band EN-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	∆PPowerClass,EN-DC = 3 dB for a power class 2 capable EN-DC UE when the IE p-maxUE-FR1, as defined in TS 38.331 [9], is provided and set to the maximum output power of the default power class or lower; otherwise ∆PPowerClass,EN-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

PTotalEN-DC = 10log10(PtotalEN-DC) with PTotalEN-DC the configured maximum transmission power for EN-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC – ΔPPowerClass, EN-DC }

If the UE does not support dynamic power sharing,

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC  – ΔPPowerClass, EN-DC } + 0.3 dB

If the EN-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalEN-DC applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalEN-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,EN-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe on all aggregated cells of E-UTRA | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p,

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalEN-DC

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PTotalEN-DC

If a= FALSE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If (a=TRUE) AND (b=FALSE)

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If b= TRUE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3-1 for inter-band EN-DC;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6]

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the configured max power for E-UTRA. If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PCMAX_ E-UTRA,c (p) will be replaced by PCMAX(p) which is the configured maximum power for the entire E-UTRA CG.

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the configured max power of NR

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,NR (q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,NR (q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

##### 6.2B.4.1.3a	Inter-band NE-DC within FR1

For inter-band dual connectivity with one uplink serving cell per CG on E-UTRA and NR respectively, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission  power for NE-DC operation, PTotalNE-DC= 10log10(PtotalNE-DC) with PtotalNE-DC as specified in clause 7.6.1A of TS 38.213 [10].

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

with exception that

-	if no symbol of slot  of the NR that is indicated as uplink or flexible by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated overlaps with subframe  of the E-UTRA; or

-	if NR slot(s) that is indicated as downlink by TDD-UL-DL-ConfigurationCommon or TDD-UL-DL-ConfigDedicated does not overlap with subframe  of the E-UTRA; then

PCMAX_L_ E-UTRA,c = MIN { PEMAX, NE-DC , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PEMAX,c  – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + TC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified by PNR as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MPRc + A-MPRc+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, NE-DC  , (PPowerClass, NE-DC – ΔPPowerClass,NE-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

-	PLTE signalled by RRC as p-MaxEUTRA in TS 36.331 [8]

-	PNR signalled by RRC as p-NR-FR1 defined in TS 38.331 [9]

-	ΔTc_E-UTRA, c = 1.5dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0dB;

-	ΔTIB,c specified in clause  6.2B.4.2.3 for NE-DC, the individual Power Class defined in table 6.2B.1.3a and any other additional power reductions parameters specified in clauses  6.2B.2.3a for NE-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	PPowerClass, NE-DC is defined in clause 6.2B.1.3a for inter-band NE-DC;

-	PPowerClass,NR is the nominal UE power of the power class that the UE supports for the NR band of the EN-DC combination as defined in clause 6.2.1 of 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class that the UE supports for the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of 36.101 [4];

-	ΔPPowerClass,NE-DC = 3 dB for a power class 2 capable NE-DC UE when the IE p-maxUE-FR1 as defined in TS 38.331 [9] is provided and set to the maximum output power of the default power class or lower; otherwise ΔPPowerClass,NE-DC = 0 dB;

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, NE-DC or PEMAX, NE-DC shall not be exceeded at any time by UE.

PTotalNE-DC = 10log10(PtotalNE-DC) with PTotalNE-DC the configured maximum transmission power for NE-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC – ΔPPowerClass, NE-DC }

If the UE does not support dynamic power sharing,

PTotalNE-DC= MIN { PEMAX, NE-DC ,PPowerClass, NE-DC  – ΔPPowerClass, NE-DC } + 0.3 dB

If the NE-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalNE-DC applies.

When a UE supporting dynamic  sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for NE-DC operation, PTotalNE-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_L) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3a-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3a-1 when same or different subframe and physical-channel durations are used in aggregated carriers. PPowerClass ,NE-DC shall not be exceeded by the UE during any evaluation period of time.

Table 6.2B.4.1.3a-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | LTE Subframe | Min(Tno_hopping, Physical Channel Length) |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ NE-DC _H (p,q) , PCMAX_ NE-DC _H (p,q+1), … , PCMAX_ NE-DC _H (p,q+n) }

where PCMAX_ NE-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ NE-DC _L (p,q) , PCMAX_ NE-DC _L (p,q+1), … , PCMAX_ NE-DC _L (p,q+n)}

where PCMAX_NE-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with LTE subframe p,

With

PCMAX_ NE-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

And:

a = 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalNE-DC

If a = TRUE

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, NE-DC ,PPowerClass, NE-DC}

Else

PCMAX_ NE-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, NE-DC ,PPowerClass, NE-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX H,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, NE-DC is defined in clause 6.2B.1.3a for inter-band NE-DC;

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the real configured max power for E-UTRA

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the real configured max power of NR

Table 6.2B.4.1.3a-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | [3.0] | [2.0] |
| 22 ≤ PCMAX < 23 | [5.0] | [2.0] |
| 21 ≤ PCMAX< 22 | [5.0] | [3.0] |
| 20 ≤ PCMAX < 21 | [6.0] | [4.0] |
| 16 ≤ PCMAX < 20 | [5.0] |  |
| 11 ≤ PCMAX < 16 | [6.0] |  |
| -40 ≤ PCMAX < 11 | [7.0] |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition a = TRUE, PUMAX,f,c,NR (q) for MCG, under nominal conditions, shall meet

PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,,NR c (q)) + THIGH (10log(pCMAX H, f,c,,NR c (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4a.1.3-2.

When LTE and NR transmissions overlap and the condition a = FALSE), then PUMAX, under nominal conditions, shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

where PCMAX_L, PCMAX_H, and PUMAX are specified above with the tolerances TLOW and THIGH specified in Table 6.2B.4a.1.3-2 for applicable values of PCMAX_L and PCMAX_H.

<Next changed section>

##### 6.4B.2.2.3	In-band emissions

For the MCG the in-band emission requirements in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 in TS 36.101 [4] apply within the transmission bandwidth configuration of the MCG with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the MCG at the edge of the transmission bandwidth configuration.

For the SCG the in-band emission requirements in Table 6.4.2.3-1 in TS 38.101-1 [2] apply within the transmission bandwidth configuration of the SCG with the carriers of both CGs active and one single contiguous PRB allocation of bandwidth LCRB within the SCG at the edge of the transmission bandwidth configuration.

<End of changes>