3GPP TSG-RAN WG4 Meeting #95-e	R4-2008102

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.104 | CR | 0903 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 37.104 on Removal of TBD for NB-IoT (Rel-16) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Core |  |  |  |  |  | Date: |  |  | 2020-06-05 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 3GPP specifications will be included as one part of the ITU-R submission of NR. For this reason, there can be TBDs remaining in the specifications. There are presently a few remaining TBDs in the applicability tables for NB-IoT. While they refer to performance requirements, this issue should be solved since the applicability table is central to the document. |  |  |  |  |  |  |  |  |
| Summary of change: |  | TBDs for NB-IoT performance are replaced with a reference to a new section 8.6, which in turn refers to the 36.104 specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TBDs would remain, making the specifications for ITU-R submission ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5, 8.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 5	Applicability of requirements

## 5.1	Band category 1

For all BS operating in bands belonging to Band Category 1, the RF requirements listed in Table 5.1-1 apply for each supported operating band. Requirements apply according to the RAT configuration of the Base Station, as listed in the heading of the table. Some requirements listed in the table may not be mandatory or they may apply only regionally. This is further specified in the clause of each requirement and in Table 4.4-1. For multiband operation, the applicability of the requirements for each operating band is determined based on the RAT configuration within only that operating band, unless otherwise stated.

Table 5.1-1: Applicability of requirements for MSR BS operation in Band Category 1

| RF requirement | BS configured for multi-RAT operation in the band | BS configured for single-RAT E-UTRA FDD operation in the band | BS configured for single-RAT UTRA FDD operation in the band | BS configured for single-RAT NB-IoT FDD standalone operation in the band | BS configured for single-RAT NR operation in the band |
| --- | --- | --- | --- | --- | --- |
| Base station output power | 6.2.1 6.2.3 6.2.4 6.2.4A 6.2.6 | 6.2.1 6.2.3 | 6.2.1 6.2.4 6.2.4A | 6.2.1 6.2.6 | 6.2.1 |
| Output power dynamics | 6.3.1 6.3.2 6.3.5 6.3.6 | 6.3.1 | 6.3.2 | 6.3.5 | 6.3.6 |
| Transmitted signal quality |  |  |  |  |  |
| Modulation quality | 6.5.1.1 6.5.1.2 6.5.1.5 6.5.1.6 | 6.5.1.1 | 6.5.1.2 | 6.5.1.5 | 6.5.1.6 |
| Frequency error | 6.5.2.1 6.5.2.2 6.5.2.5 6.5.2.6 | 6.5.2.1 | 6.5.2.2 | 6.5.2.5 | 6.5.2.6 |
| Time alignment error | 6.5.3.1 6.5.3.2 6.5.3.4 6.5.3.5 | 6.5.3.1 | 6.5.3.2 | 6.5.3.4 | 6.5.3.5 |
| Unwanted emissions |  |  |  |  |  |
| Transmitter spurious emissions | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (except for 6.6.1.1.3) |
| Operating band unwanted emissions | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 |
| Occupied bandwidth | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 |
| ACLR | 6.6.4.1 6.6.4.2 6.6.4.5 6.6.4.6 | 6.6.4.1 | 6.6.4.2 | 6.6.4.5 | 6.6.4.6 |
| Cumulative ACLR | 6.6.4.4 (NOTE 3) | 6.6.4.4 (NOTE 3) | 6.6.4.4 (NOTE 3) | 6.6.4.4 (NOTE 3) | 6.6.4.4 (NOTE 3) |
| Transmitter intermodulation | 6.7.1 6.7.2 (NOTE 2) | 6.7.1 6.7.2 (NOTE 2) | 6.7.1 6.7.2 (NOTE 2) | 6.7.1 6.7.2 (NOTE 2) | 6.7.1 6.7.2 (NOTE 2) |
| Reference sensitivity level | 7.2.1 7.2.2 7.2.5 7.2.6 | 7.2.1 | 7.2.2 | 7.2.5 | 7.2.6 |
| Dynamic range | 7.3.1 7.3.2 7.3.5 7.3.6 | 7.3.1 | 7.3.2 | 7.3.5 | 7.3.6 |
| In-band selectivity and blocking |  |  |  |  |  |
| Blocking | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 |
| Narrowband blocking | 7.4.2 | 7.4.2 | 7.4.2 | 7.4.2 | 7.4.2 |
| Out-of-band blocking | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |
| Receiver spurious emissions | 7.6.1 | 7.6.1 | 7.6.1 | 7.6.1 | 7.6.1 |
| Receiver intermodulation |  |  |  |  |  |
| Intermodulation | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 |
| Narrowband intermodulation | 7.7.2 | 7.7.2 | 7.7.2 | 7.7.2 | 7.7.2 |
| In-channel selectivity | 7.8 | 7.8 | - | 7.8 | 7.8.2 |
| Performance requirements | 8.1 8.2 8.5 | 8.1 | 8.2 | 8.6 | 8.5 |
| NOTE 1: For some requirements in BS configured for multi-RAT operation, there is no general MSR multi-RAT requirement. Instead, the requirement is defined by the respective single-RAT requirement for each RAT supported by the BS as referenced in the table. NOTE 2: The requirement in sub-clause 6.7.2 is only applied for BS operating in non-contiguous spectrum. NOTE 3: The requirement in sub-clause 6.6.4.4 is only applied for BS operating in non-contiguous spectrum. |  |  |  |  |  |

## 5.2	Band category 2

For all BS operating in bands belonging to Band Category 2, the RF requirements listed in Table 5.2-1 apply for each supported operating band. Requirements apply according to the RAT configuration of the Base Station, as listed in the heading of the table. Some requirements listed in the table may not be mandatory or they may apply only regionally. This is further specified in the clause of each requirement and in Table 4.4-1. For multiband operation, the applicability of the requirements for each operating band is determined based on the RAT configuration within only that operating band, unless otherwise stated.

Table 5.2-1: Applicability of requirements for MSR BS operation in Band Category 2

| RF requirement | BS configured for multi-RAT operation not including GSM/EDGE in the band | BS configured for multi-RAT operation including GSM/EDGE in the band | BS configured for single-RAT E-UTRA FDD operation in the band | BS configured for single-RAT UTRA FDD operation in the band | BS configured for single-RAT GSM/EDGE operation in the band | BS configured for single-RAT NB-IoT FDD standalone operation in the band | BS configured for single-RAT NR operation in the band |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Base station output power | 6.2.1 6.2.3 6.2.4 6.2.4A 6.2.6 | 6.2.1 6.2.3 6.2.4 6.2.4A 6.2.6 | 6.2.1 6.2.3 | 6.2.1 6.2.4 6.2.4A | 6.2.1 | 6.2.1 6.2.6 | 6.2.1 |
| Output power dynamics | 6.3.1 6.3.2 6.3.5 6.3.6 | 6.3.1 6.3.2 6.3.4 6.3.5 6.3.6 | 6.3.1 | 6.3.2 | 6.3.4 | 6.3.5 | 6.3.6 |
| Transmit ON/OFF power | - | - | - | - | - | - | - |
| Transmitted signal quality |  |  |  |  |  |  |  |
| Modulation quality | 6.5.1.1 6.5.1.2 6.5.1.5 6.5.1.6 | 6.5.1.1 6.5.1.2 6.5.1.4 6.5.1.5 6.5.1.6 | 6.5.1.1 | 6.5.1.2 | 6.5.1.4 | 6.5.1.5 | 6.5.1.6 |
| Frequency error | 6.5.2.1 6.5.2.2 6.5.2.5 6.5.2.6 | 6.5.2.1 6.5.2.2 6.5.2.4 6.5.2.5 6.5.2.6 | 6.5.2.1 | 6.5.2.2 | 6.5.2.4 | 6.5.2.5 | 6.5.2.6 |
| Time alignment error | 6.5.3.1 6.5.3.2 6.5.3.4 6.5.3.5 | 6.5.3.1 6.5.3.2 6.5.3.4 6.5.3.5 | 6.5.3.1 | 6.5.3.2 | - | 6.5.3.4 | 6.5.3.5 |
| Unwanted emissions |  |  |  |  |  |  |  |
| Transmitter spurious emissions | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (NOTE 3) | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (NOTE 3) | 6.6.1 (except for 6.6.1.1.3) | 6.6.1 (except for 6.6.1.1.3) |
| Operating band unwanted emissions | 6.6.2.2 6.6.2.4 | 6.6.2.2 6.6.2.4 | 6.6.2.2 6.6.2.4 | 6.6.2.2 6.6.2.4 | 6.6.2.3 6.6.2.4 | 6.6.2.2 6.6.2.4 | 6.6.2.2, 6.6.2.4 |
| Occupied bandwidth | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 | - | 6.6.3 | 6.6.3 |
| ACLR | 6.6.4.1 6.6.4.2 6.6.4.5 6.6.4.6 | 6.6.4.1 6.6.4.2 6.6.4.5 6.6.4.6 | 6.6.4.1 | 6.6.4.2 | - | 6.6.4.5 | 6.6.4.6 |
| Cumulative ACLR | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | - | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) |
| Transmitter intermodulation | 6.7.1 6.7.2 | 6.7.1 6.7.2 | 6.7.1 6.7.2 | 6.7.1 6.7.2 | 6.7.2 | 6.7.1 6.7.2 | 6.7.1 6.7.2 |
| Reference sensitivity level | 7.2.1 7.2.2 7.2.5 7.2.6 | 7.2.1 7.2.2 7.2.4 7.2.5 7.2.6 | 7.2.1 | 7.2.2 | 7.2.4 | 7.2.5 | 7.2.6 |
| Dynamic range | 7.3.1 7.3.2 7.3.5 7.3.6 | 7.3.1 7.3.2 7.3.4 7.3.5 7.3.6 | 7.3.1 | 7.3.2 | 7.3.4 | 7.3.5 | 7.3.6 |
| In-band selectivity and blocking |  |  |  |  |  |  |  |
| Blocking | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 | 7.4.1 |
| Narrowband blocking | 7.4.2 | 7.4.2 7.4.3 | 7.4.2 | 7.4.2 | 7.4.2 7.4.3 7.4.4 | 7.4.2 | 7.4.2 |
| Out-of-band blocking | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |
| Receiver spurious emissions | 7.6.1 | 7.6.1 7.6.2 (NOTE 3) | 7.6.1 | 7.6.1 | 7.6.1 7.6.2 (NOTE 3) | 7.6.1 | 7.6.1 |
| Receiver intermodulation |  |  |  |  |  |  |  |
| Intermodulation | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 |
| Narrowband intermodulation | 7.7.2 | 7.7.2 7.7.3 | 7.7.2 | 7.7.2 | 7.7.2 7.7.3 | 7.7.2 | 7.7.2 |
| In-channel selectivity | 7.8 | 7.8 | 7.8 | - | - | 7.8 | 7.8.2 |
| Performance requirements | 8.1 8.2 8.5 | 8.1 8.2 8.4 8.5 | 8.1 | 8.2 | 8.4 | 8.6 | 8.5 |
| NOTE 1: For some requirements in BS configured for multi-RAT operation, there is no general MSR multi-RAT requirement. Instead, the requirement is defined by the respective single-RAT requirement for each RAT supported by the BS as referenced in the table. NOTE 2: The requirement in sub-clause 6.6.4.4 is only applied for BS operating in non-contiguous spectrum. NOTE 3: For BS capable of multi-band operation, the limits in subclause 6.6.1.1.3 and 7.6.2 are only applicable when all supported operating bands belong to BC2 and GSM/EDGE is configured in all operating bands. |  |  |  |  |  |  |  |

## 5.3	Band category 3

For all BS operating in bands belonging to Band Category 3, the RF requirements listed in Table 5.3-1 apply for each supported operating band. Requirements apply according to the RAT configuration of the Base Station, as listed in the heading of the table. Some requirements listed in the table may not be mandatory or they may apply only regionally. This is further specified in the clause of each requirement and in Table 4.4-1.

Table 5.3-1: Applicability of requirements for MSR BS operation in Band Category 3

| RF requirement | BS configured for multi-RAT operation | BS configured for single-RAT E-UTRA TDD operation | BS configured for single-RAT UTRA TDD operation | BS configured for single-RAT NR TDD operation | BS configured for single-RAT NB-IoT standalone operation |
| --- | --- | --- | --- | --- | --- |
| Base station output power | 6.2.1 6.2.2 6.2.3 6.2.5 | 6.2.1 6.2.2 6.2.3 | 6.2.1 6.2.5 | 6.2.1 | 6.2.1 6.2.6 |
| Output power dynamics | 6.3.1 6.3.3 6.3.6 | 6.3.1 | 6.3.3 | 6.3.6 | 6.3.5 |
| Transmit ON/OFF power | 6.4 | 6.4 | 6.4 | 6.4 | 6.4 |
| Transmitted signal quality |  |  |  |  |  |
| Modulation quality | 6.5.1.1 6.5.1.3 6.5.1.6 | 6.5.1.1 | 6.5.1.3 | 6.5.1.6 | 6.5.1.5 |
| Frequency error | 6.5.2.1 6.5.2.3 6.5.2.6 | 6.5.2.1 | 6.5.2.3 | 6.5.2.6 | 6.5.2.5 |
| Time alignment error | 6.5.3.1 6.5.3.3 6.5.3.5 | 6.5.3.1 | 6.5.3.3 | 6.5.3.5 | 6.5.3.4 |
| Unwanted emissions |  |  |  |  |  |
| Transmitter spurious emissions | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3 and 6.6.1.2) | 6.6.1 (except for 6.6.1.1.3) |
| Operating band unwanted emissions | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 | 6.6.2.1 6.6.2.4 |
| Occupied bandwidth | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 | 6.6.3 |
| ACLR | 6.6.4.1 6.6.4.3 6.6.4.6 | 6.6.4.1 | 6.6.4.3 | 6.6.4.6 | 6.6.4.5 |
| Cumulative ACLR | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) | 6.6.4.4 (NOTE 2) |
| Transmitter intermodulation | 6.7.1 6.7.3 | 6.7.1 6.7.3 6.7.4 | 6.7.1 6.7.3 | 6.7.1 6.7.3 | 6.7.1 6.7.3 |
| Reference sensitivity level | 7.2.1 7.2.3 7.2.6 | 7.2.1 | 7.2.3 | 7.2.6 | 7.2.5 |
| Dynamic range | 7.3.1 7.3.3 7.3.6 | 7.3.1 | 7.3.3 | 7.3.6 | 7.3.5 |
| In-band selectivity and blocking |  |  |  |  |  |
| Blocking | 7.4.1 7.4.5 | 7.4.1 7.4.5 | 7.4.1 7.4.5 | 7.4.1 | 7.4.1 |
| Narrowband blocking | 7.4.2 | 7.4.2 | 7.4.2 | 7.4.2 | 7.4.2 |
| Out-of-band blocking | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |
| Receiver spurious emissions | 7.6.1 | 7.6.1 | 7.6.1 | 7.6.1 | 7.6.1 |
| Receiver intermodulation |  |  |  |  |  |
| Intermodulation | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 | 7.7.1 |
| Narrowband intermodulation | 7.7.2 | 7.7.2 | 7.7.2 | 7.7.2 | 7.7.2 |
| In-channel selectivity | 7.8 | 7.8 | - | 7.8.2 | 7.8 |
| Performance requirements | 8.1 8.3 8.5 | 8.1 | 8.3 | 8.5 | 8.6 |
| Note 1: For some requirements in BS configured for multi-RAT operation, there is no general MSR multi-RAT requirement. Instead, the requirement is defined by the respective single-RAT requirement for each RAT supported by the BS as referenced in the table. NOTE 2: The requirement in sub-clause 6.6.4.4 is only applied for BS operating in non-contiguous spectrum. |  |  |  |  |  |

*********************End of change*****************

*********************Next changed section*****************

# 8	Performance requirements

Performance requirements specify the ability of the BS to correctly demodulate signals in various conditions and configurations. For NR, UTRA and E-UTRA the requirements specify a minimum throughput or maximum BLER or BER that shall be achieved at a specific SNR. For GSM/EDGE the requirements specify a maximum FER, BLER or BER that shall be achieved at specific sensitivity levels (C) and specific carrier-to-interference ratios (C/I).

## 8.1	E-UTRA minimum requirement

For E-UTRA, the minimum requirements for performance are specified in TS 36.104 [4], clause 8.

## 8.2	UTRA FDD minimum requirement

For UTRA FDD, the minimum requirements for performance are specified in TS 25.104 [2], clause 8.

## 8.3	UTRA TDD minimum requirement

For UTRA TDD, the minimum requirements for performance are specified in TS 25.105 [3], clause 8.

## 8.4	GSM/EDGE minimum requirement

For GSM/EDGE, the minimum requirements for reference sensitivity level and reference interference level are specified in TS 45.005 [5], applicable parts of clauses 6.2, 6.3, 6.4, 6.5 and 6.6. The conditions specified in TS 45.005 [5], Annex P.1 are valid for GSM sensitivity and interference performance.

## 8.5	NR minimum requirement

For NR, the minimum requirements for performance (BS type 1-C) are specified in TS 38.104 [17], clause 8.

## 8.6	NB-IoT minimum requirement

For NB-IoT, the minimum requirements for performance are specified in TS 36.104 [4], clause 8.5.