3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2008862

# Electronic Meeting, 25 May – 5 June, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.843 | CR | 0040 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 37.843: internal TR references corrections and content redundancy removal (wrt. TR 37.941 for OTA BS testing), Rel-15 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | OTA_BS_testing |  |  |  |  |  | Date: |  |  | 2020-05-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In relation to the OTA BS testing WI and the new TR 37.941, multiple TR/TS were reviewed with the goal to capture the OTA BS testing content in a single external TR 37.941, as well as to remove any outstanding references to internal TRs. This CR provides corrections to the internal TR references in TR 37.843 and removes technical content already captured in the TR 37.941. Further discussion may be needed for the solution of Voiding multiple clauses and their concent vs. resulting TR readability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Scope clarifiaction added. - Removal of the references to TR 37.840, TR 37.842. - Multiple ”specific references” were removed, replaced by “non-specific references” for simplicity. - Multiple sections removed (to avoid redundant content), as already covered in TR 37.941. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference to the internal TR (not allowed by the drafting rules) and redundant content among RAN4 TRs would exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2 4.5, 5.1.1, 5.6.3.2, 6.2.2.3.1, 7.6, 7.7, 7.8, 8.1, 8.3, 8.4, 8.5, 8.6, 10, B, C, D, E, F, G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

# 1	Scope

The present document is the Technical Report for the Work Item on Base Station (BS) radiated Radio Frequency (RF) requirements for Active Antenna System (AAS) [2]. The present document captures the background and the decisions on the specification of radiated RF requirements that are applicable to AAS BS.

The AAS BS specification contains requirements for both single RAT UTRA FDD, UTRA TDD and E-UTRA as well as MSR (including single RAT MSR requirements for each of the RATS), it was agreed that it may not be necessary to support so many variants for the OTA AAS BS hence a specification complexity reduction investigation was carried out. It was found the following was not needed for the OTA AAS BS requirements:

1.	Radiated requirements applicable for eAAS not support UTRA TDD.

2.	Radiated requirements applicable for eAAS not include UTRA pilot requirements.

NOTE: 	In Rel-15, multiple clauses related to the OTA measurements of the BS were shifted to the OTA BS testing TR 37.941 [36], which includes such aspects as e.g., test tolerance and measurement uncertainty derivations, OTA test chambers descriptions, calibration and test procedure descriptions, etc.

------------------------------ Next modified section ------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	RP-170844: "Work Item Description for Enhancements of Base Station (BS) RF and EMC requirements for Active Antenna System (AAS)"

[3]	3GPP TS 37.105: "Active Antenna System (AAS) Base Station (BS) transmission and reception"

[4]	3GPP TR 37.842: "Frequency (RF) requirement background for Active Antenna System (AAS) Base Station (BS)"

[5]	3GPP TR 36.942: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) system scenarios"

[6]	3GPP TR 37.840: "Study of Radio Frequency (RF) and Electromagnetic Compatibility (EMC) requirements for Active Antenna Array System (AAS) base station"

[7]	3GPP TS 37.104: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) radio transmission and reception"

[8]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception"

[9]	Recommendation ITU-R SM.328-11: "Spectra and bandwidth of emissions", ITU/R

[10]	3GPP TR 25.951: "FDD Base Station (BS) classification"

[11]	3GPP TR 37.809: "Evolved Universal Terrestrial Radio Access (E-UTRA) medium range and Multi-Standard Radio (MSR) medium range / local area Base Station (BS) class requirements"

[12]	3GPP TR 36.931: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Frequency (RF) requirements for LTE Pico Node B"

[13]	3GPP TS 37.114: "Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)"

[14]	3GPP TS 37.113: "E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)"

[15]	3GPP TS 36.113: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[16]	3GPP TS 25.113: "Base Station (BS) and repeater ElectroMagnetic Compatibility (EMC)"

[17]	Conversion of power to field strength in the book "Automotive Antenna Design and Applications", by Victor Rabinovich, Nikolai Alexandrov, Basim Alkhateeb; 2010

[18]	Recommendation ITU-R SM.329-10: "Unwanted emissions in the spurious domain"

[19]	3GPP TS 25.104: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (FDD)"

[20]	3GPP TS 25.105: "Universal Terrestrial Radio Access (UTRA); Base Station (BS) radio transmission and reception (TDD)"

[21]	3GPP TS 25.141: "Base Station (BS) conformance testing (FDD)"

[22]	3GPP TS 36.141: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing"

[23]	3GPP TS 37.145-1: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 1: Conducted conformance testing"

[24]	3GPP TS 37.145-2: "Active Antenna System (AAS) Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[25]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE

[26]	TSGR4#8(99)631: "Antenna-to-Antenna Isolation Measurements", Allgon

[27]	Report ITU-R M.2244: "Isolation between antennas of IMT base stations in the land mobile service", ITU-R

[28]	IEC 61000-4-3: 2006+AMD1:2007+AMD2:2010: “Electromagnetic compatibility (EMC) - Part 4-3: Testing and measurement techniques - Radiated, radio-frequency, electromagnetic field immunity test”

[29]	Hald, J.; Hansen, J. E.; Jensen, F. & Holm Larsen, F. Hansen, J. (Ed.) Spherical Near-Field Antenna Measurements, Peter Peregrinus Ltd., 1998, vol 26.

[30]	"Sparse Sampling Analysis Tool" Matlab code: https://se.mathworks.com/matlabcentral/fileexchange/67143-sparse-sampling-analysis-tool?s_tid=srchtitle

[31]	J. Fridén, A. Razavi, and A. Stjernman, “Angular sampling, Test Signal, and Near-Field Aspects for Over-the-Air Total Radiated Power Assessment in Anechoic Chambers”, IEEE Access, 2018, https://ieeexplore.ieee.org/stamp/stamp.jsp?arnumber=8470084.

[32]	IEEE Std 145: “IEEE Standard Definitions of Terms for Antennas”, IEEE

[33]	IEC 61000-4-21. “Electromagnetic compatibility (EMC) - Part 4-21: Testing and measurement techniques - Reverberation chamber test methods” Edition 2.0 2011-01, The International Electrotechnical Commision (IEC), 2011

[34]	Hill, D.A., “Boundary fields in reverberation chambers,” IEEE Transactions on Electromagnetic Compatibility, vol. 47, no. 2, pp. 281-290, May 2005

[35]	Krauthäuser, H. G.; Winzerling, T.; J., N.; Eulig, N. & Enders, A. “Statistical interpretation of autocorrelation coefficients for fields in mode-stirred chambers” 2005 International Symposium on Electromagnetic Compatibility, EMC 2005, 2005, 2, 550-555

[36]		3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

------------------------------ Next modified section ------------------------------

## 4.5	Coordinate system

For the description of the coordinate system for the OTA BS measurements, refer to TR 37.941 [36].

------------------------------ Next modified section ------------------------------

### 5.1.1	Spatial requirements

OTA requirements for the AAS BS are belonging to one of the three groups:

-	Directional requirements

-	TRP requirements

-	Co-location requirements

For the requirements classification and descriptions of the above requirement groups, refer to TR 37.941 [36].

All OTA requirements are met in either the OTA coverage range or OTA peak direction set(s) as shown in the overview table of the radiated Tx requirements in table 5.1-1.

Table 5.1-1: Overview of radiated Tx requirements

| AAS BS requirement |  | OTA requirement type | Coverage range | Notes |
| --- | --- | --- | --- | --- |
| Base station output power | Output power accuracy for EIRP | Directional requirement | OTA peak directions set | Output power accuracy for EIRP requirement is already included as a core requirement in TS 37.105. |
|  | Output power accuracy for TRP | TRP | n/a |  |
| E-UTRA DL RS power |  | Directional Requirement | OTA peak directions set | Conformance testing is carried out in the reference direction |
| Output power dynamics |  | Directional requirement | OTA peak directions set | Conformance testing is carried out in the reference direction. |
| Transmit ON/OFF power |  | co-location requirement | n/a |  |
| Frequency Error |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Time Alignment Error |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Modulation Quality (EVM) |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction and the maximum directions of the OTA coverage range on each axis. |
| Unwanted emissions |  |  |  |  |
| Occupied Bandwidth |  | Directional requirement | OTA coverage range | Conformance testing is carried out in the reference direction. |
| Adjacent Channel Leakage Radio (ACLR) |  | TRP | n/a |  |
| Spectrum emission mask |  | TRP | n/a |  |
| Spurious emissions |  |  |  |  |
| Mandatory Requirements |  | TRP | n/a |  |
| Protection of the BS receiver of own or different BS |  | co-location requirement | n/a |  |
| Additional spurious emissions requirements |  | TRP | n/a | Includes co-existence in same geographical area |
| Co-location with other base stations |  | co-location requirement | n/a |  |
| Transmitter intermodulation |  | co-location requirement | n/a | The interferer is applied as a co-location requirements, the radiated emissions requirements are specified in the appropriated referenced sub-clause. Generally TRP |

The OTA peak directions set(s) are the same as the EIRP accuracy directions set(s) used in the REL13 specification, the name has been changed as they are now used for other requirements. Examples of OTA peak directions sets remain the same as related EIRP accuracy directions set(s) in TR 37.842 [4].

------------------------------ Next modified section ------------------------------

------------------------------ Next modified section ------------------------------

##### 6.2.2.3.1	OTA Sensitivity

Conformance testing for OTA sensitivity is performed for the five directions same as the Rel-13 OTA sensitivity requirements as described in TR 37.941 [36]. Same requirements for Rel-13 OTA sensitivity apply.

------------------------------ Next modified section ------------------------------

## 7.6	Conformance requirements

The OTA related parts of the conformance testing test procedures for the demodulation requirements are based upon the RX dynamic range requirement OTA procedure, as described in subclause 10.3.4. The setup and operation of the signal generator and measurement instruments are the same as for the corresponding conducted requirements.

Measurement Uncertainties for the OTA requirements are the same as for the conducted requirements, as described in subclause 10.11.

It is noted, that the possibility of OTA testing is considered also for those BS demodulation requirements, which were declared as Optional in TS 25.141 [21] and TS 36.141 [22].

For OTA test setup for the BS demodulation requirements and for description of the BS demodulation requirements feasible OTA, refer to TR 37.941 [36].

## 7.7	OTA test setup

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

Figure 7.7-1: Void

Figure 7.7-2:  Void

Figure 7.7-3:  Void

Figure 7.7-4:  Void

## 7.8	BS demodulation requirements feasible OTA

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

Table 7.8-1: Void

Table 7.8-2:  Void

------------------------------ Next modified section ------------------------------

# 8	EMC requirements

## 8.1	General

The Rel-13 AAS EMC specification [13] was drafted with a simple principle of applying the existing EMC requirements to the AAS BS architecture hence it was possible to refer to the existing UTRA , E-UTRA and MSR EMC specifications [16][15][14]. It was possible to use this approach since release 13 of AAS BS specification [3] only allows for AAS BS architectures with a conducted interface, which means that the antenna array can be disconnected and TAB connectors can be terminated. Hence the system can be treated in the same way as in the existing specifications and therefore the EMC requirements remained applicable in exactly the same way.

The OTA AAS BS architecture does not have a conducted interface and so the antennas cannot be disconnected. This means that the conducted methods where the EMC antenna ports are terminated are no longer be possible and hence new methods for EMC testing are needed.

Figure 8.1-1 shows the prime difference in architecture from an EMC view point when comparing the ports on a conventional non-AAS BS to the ports on an AAS BS.

Figure 8.1-1: Comparison between BS architectures for EMC testing

Since the antenna array for the OTA AAS BS is integrated within the EUT, the transmissions from the EUT within the test chamber have antenna gain included in them.

Figure 8.1-2 illustrates the status of EMC antenna ports during EMC testing where it is noted that the ports are connected to a terminating load and the hybrid AAS BS is then set to transmit at full power.

Figure 8.1-2: EMC Antenna port status during EMC testing

NOTE: 	In Rel-15 version of this TR, multiple clauses related to the OTA measurements of the EMC requirements were shifted to the OTA BS testing TR 37.941 [36].

## 8.2	Regulatory EMC requirements

The following table provides a summary of the existing EMC regulatory requirements in EU and US market. These requirements should serve as a reference when specifying the EMC requirements for the AAS BS.

Table 8.2-1: Overview of the regulatory EMC requirements for EU and US markets

|  | EU market | US market |
| --- | --- | --- |
| RF Radiated Spurious Emission (RSE) requirements | -36dBm/100kHz ERP below 1 GHz and -30dBm/1MHz ERP above 1 GHz Requirements specified in ETSI EN 301 908-1, and are tested with the antenna port connected in a terminating load. No TX exclusion bands defined for BS equipment today. | General requirement of -13 dBm/MHz ERP is applicable Requirements specified in FCC Part 2. FCC does not define any TX exclusion bands for BS equipment today. |
| EMC Radiated Emission (RE) requirements | -50dBm/MHz ERP general requirement. Not applicable for BS equipment Defined in ETSI EN 55032. | -50dBm/MHz above 1 GHz mandatory requirement. Specified in FCC Part 15, §15.35. |
| Radiated Immunity (RF electromagnetic field) requirements | Two type of requirements defined: 3 V/m, 80 MHz – 690 MHz, and 10 V/m 690 MHz – 6 GHz. Defined in ETSI EN 301 489-1 and ENSI EN 301 489-50 RX Exclusion bands defined 20 MHz above and below the operating band | No requirements today |

Note:	From the above regulatory requirements, the radiated spurious emission requirements (-36 dBm/100kHz and -30 dBm/1MHz) are reflected in the existing EMC specifications for AAS BS [13] and non-AAS BS [14]. Similarly, the 3 V/m immunity requirement is captured in the existing EMC specifications.

## 8.3	Field strength in EMC chamber

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

## 8.4	Protection of measurement equipment

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

## 8.5	EMC radiated emission requirement

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

Figure 8.5-1: Void

## 8.6	Radiated Immunity

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

------------------------------ Next modified section ------------------------------

# 10	Conformance testing aspects

NOTE:	In Rel-15, content of this clause was shifted to the OTA BS testing TR 37.941 [36].

10log1-ΓSGH2

LSGHcal

10log1-ΓSGH2

LSGHcal

PIsotropic Radiator=Preference antenna+Greference antenna

Lpath loss=Preference antenna+ Greference antenna- Ptest equipment

Ltest equipment-receiver

Preceiver Lpath loss

ΓSGH

LC↔SGH

10log1-ΓSGH2

LSGHcal

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

| MUwanted_conducted |  |  |  |  |
| --- | --- | --- | --- | --- |
| MUinterfer_conducted |  |  |  |  |
| MUmatching_conducted |  |  |  |  |
| MUwanted |  |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUinterfer |  |  |  | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA |  |  |  |  |
| MUTestEquipment_OTA |  |  |  |  |
| MUmatching_OTA |  |  |  |  |
| MUchamber_OTA |  |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| MUACS_OTA |  |  |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

MUmatching_conducted= MUmatching_OTA

|  | MUwanted_conducted |  |  |  |
| --- | --- | --- | --- | --- |
|  | MUinterfer_conducted |  |  |  |
|  | MUmatching_conducted |  |  |  |
|  | MUwanted |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUinterfer |  |  | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA |  |  |  |
|  | MUTestEquipiment_OTA |  |  |  |
|  | MUmatching_OTA |  |  |  |
|  | MUchamber_OTA |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  |  |  |  | MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect |
|  | MUACS_OTA |  |  |  |

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUinterfer=MUinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

| MUwanted_conducted |  |  |  |
| --- | --- | --- | --- |
| MUCWinterfer_conducted |  |  |  |
| MUMODinterfer_conducted |  |  |  |
| MUmatching_conducted |  |  |  |
| MUwanted |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
| MUCWinterfer |  |  | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
| MUMODinterfer |  |  | MUinterfer=MUinterfer_conducted2-MUmatching_conducted2 |
| MUEIS_OTA |  |  |  |
| MUTestEquipment_OTA |  |  |  |
| MUmatching_OTA |  |  |  |
| MUchamber_OTA |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment2-MUmatching_OTA2 |
| MURXIM_OTA |  |  | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

MUACS_OTA=MUwanted2+MUinterfer2+MUchamber_OTA2+MUmatching_OTA2+ACLReffect MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect

MUwanted=MUwanted_conducted2-MUmatching_conducted2

MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2

MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2

MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2

MUmatching_conducted= MUmatching_OTA

|  | MUwanted_conducted |  |  |  |
| --- | --- | --- | --- | --- |
|  | MUCWinterfer_conducted |  |  |  |
|  | MUMODinterfer_conducted |  |  |  |
|  | MUmatching_conducted |  |  |  |
|  | MUwanted |  |  | MUwanted=MUwanted_conducted2-MUmatching_conducted2 |
|  | MUCWinterfer |  |  | MUCWinterfer=MUCWinterfer_conducted2-MUmatching_conducted2 |
|  | MUMODinterfer |  |  | MUMODinterfer=MUMODinterfer_conducted2-MUmatching_conducted2 |
|  | MUEIS_OTA |  |  |  |
|  | MUTestEquipiment_OTA |  |  |  |
|  | MUmatching_OTA |  |  |  |
|  | MUchamber_OTA |  |  | MUchamber_OTA=MUEIS_OTA2-MUTestEquipment_OTA2-MUmatching_OTA2 |
|  | MURXIM_OTA |  |  | MURX IM_OTA=MUwanted2+(2×MUCWinterfer)2+MUMODinterfer2+MUchamberOTA2+MUmatching_OTA2+ACLReffect |

MUper-point2+SE2

f≤3GHz

[MUTRP=0.592+0.752

3GHz≤f≤4.2GHz

[MUTRP=0.702+0.752

MUTRP=MUper-point2+SE2

f≤3GHz

MUTRP=1.152+0.752

3GHz≤f≤4.2GHz

MUTRP=1.372+0.752

MUTRP=MUper-point2+SE2

f≤3GHz

[MUTRP=1.152+0.752

3GHz≤f≤4.2GHz

[MUTRP=1.372+0.752

klim≤0.37(1-7.22/N0.64)

klim klimr(k)≤klimrk≤rlim

f

Pn=S212

Pn  = Ur250 PtMtLs

Mt= 1-S112

Pn

Pn

1/Nref

Pr=S21r2

UambPamb=Uamb250

TRPamb=ηM1rPrPamb

PEUT=UEUT250

TRPEUTTRPamb

AAS BS

Test system Calibrated point

θ

ϕ

0

AAS declared coordinate reference point and orientation

Test system enclosure

Test antenna (out of band)

Signal Generator for the interfering signal

Signal Generator for the wanted signal

Test antenna (in band)

Test antenna polarisation can be adjusted

U(θ,ϕ)

Uθ,ϕ=EIRP(θ,ϕ)4π

TRPReference= 14π θ=0πϕ=02πEIRPθ,ϕ sinθdθdϕ

Etr,θ,ϕ,TRPReference

PDr,θ,ϕ=Etr,θ,ϕ2ηη=μ0ε0≈377 Ω

(θ,ϕ)

θϕθNϕMθϕ

Δθ= πN∆ϕ= 2πM

(N+1)×M

nmnθmϕθn, ϕmEIRPp1(θn,ϕm)EIRPp2(θn,ϕm)θϕ

TRPEstimate=π2NMn=1N-1m=0M-1EIRP(θn,ϕm)sinθn

sinθ0sinθN

NM∞

ΔϕrefΔθref,

15∘λ

EIRPθ,ϕ=sin2θcos2kcosθL2

sin2θcos2(kcosθ L/2)Δθref=λ/Lϕϕ

x,y,z=(0,0,±L/2)

ΔθλL=14,18,112,116,132

θϕθ

ϕ

Δθref=Δϕref=λ/L

Δϕref=λ/LΔθref=λ/L2+h2

Δθref=λL2+h2=λDΔϕref=λL=λDcyl

DDcyl

D=d2+w2+h2

Dcyl=d2+w2

Δθref=min⁡(180∘πarcsinλDZ, 15∘ )

Δϕref=min180∘ πarcsinλDY , 15∘

∆ϕref≅BeWϕ≅FNBWϕ2Δθref≅BeWθ≅FNBWθ2

FNBWϕFNBWθ

Δϕref=λλoBeWϕ=λλo FNBWϕ2Δθref=λλoBeWθ=λλo FNBWθ2

Dcyl=λoBeWϕ

Nθn,ϕn

TRPEstimate=1Nn=1NEIRP(θn,ϕn)

Nsin θθϕ

N

N=4π∆θref∆ϕref

ΔθrefΔϕref

I

TRPEstimate=1Ii=0I-1EIRP(θi,ϕi)

I-1

θi=cos-11-2i+1I

ϕi=2πiΨ

Ψ=1+52

sin θ

EIRPav,cut-n=1Pj=1PEIRP(j)

TRPEstimate=1Nn=1NEIRPav,cut-n

xz

EIRPcut1ϕθ=θH EIRPcut2θϕ=0

u=sinθsinϕv=cosθ

EIRPcut1u vH=cosθHEIRPcut2vu=0u2+v2≤1

EIRPu,v=EIRPcut1uEIRPcut2vEIRP0,vH

vH=0

TRP=14πfwdEIRPfwdu,vdudv1-u2-v2+bwdEIRPbwdu,vdudv1-u2-v2

u2+v2=1

u=sinθsinϕ, v=cos⁡(θ)

dudv=∂u,v∂θ,ϕdθdϕ=sin2θcosϕdθdϕ

TRP=14πfwdEIRPθ,ϕsinθcosϕdudv+14πbwdEIRPθ,ϕsinθcosϕdudv

TRP=ΔugridΔvgrid4πu2+v2<1cosϕ>0EIRPθn,ϕm,nsinθncosϕm,n +u2+v2<1cosϕ<0EIRPθn,ϕm,nsinθncosϕm,n

Δugrid=λDy

Δvgrid=λDz

um vnumvn

ΔθrefΔϕref

TRPEstimate=EIRPpeakDEUT

DEUT

DEUT,BSOutputpower= Prated,c,EIRPPrated,c,TRP

ACLR=TRPdTRPe

K

ΔϕΔθ

SF=maxΔϕΔϕref,ΔθΔθref

SF-1SFmax-1 dB,

SFmaxΔϕrefΔθref

4πd2

maximum EIRP-measured EIRP

| HPBW2 |  |
| --- | --- |
| HPBW3 |  |
| HPBW4 |  |

HPBW4HPBW3

HPBW2

----------------------------- End of modified section ------------------------------

Annex B:
OTA sensitivity measurement error contribution descriptions

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

Annex C:
Beam sweeping

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

0.55λ

D=5λ=>Δϕref=λ/DΔϕref

Annex D:
Sparse sampling for spurious emissions

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

ΔϕΔθΔϕrefΔθref

ρmax =0.5

Annex E:
Radiated transmit measurement error contribution descriptions

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

N σ/Nσ

Nσ/Nσ

S212

Annex F:
Test equipment uncertainty values

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

|  |  | - |  |  |  |
| --- | --- | --- | --- | --- | --- |

Annex G:
Power density measurements close to EUT

NOTE:	In Rel-15, content of this annex was shifted to the OTA BS testing TR 37.941 [36].

r0≥2DMA2/λ