3GPP TSG RAN meeting #88e										RP-201020

Electronic Meeting, June 29 - July 3, 2020

## Status Report to TSG

Agenda item:			9.3.25

| WI / SI Name | Over the air (OTA) base station (BS) testing TR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | OTA_BS_testing |  |  |  |  |
| Unique ID | 860254 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-200276 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: | Performance part: 09/2020 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: | Performance Part: 90% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN 4 |
| --- | --- | --- |
| Rapporteur | Name | Michal Szydelko |
|  | Company | Huawei |
|  | Email | michal.szydelko@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#94e (24 February – 6 March, 2020)

The work item work has been triggered during RAN4#94e meeting. Documents [1-28] were submitted to RAN4#94e meeting.

The summary of the email discussion on the RAN4#94e_#84_OTA_BS_testing topic during the electronic meeting was captured in [1].

The following topics were discussed during the electronic meeting:

- General issues

Work-plan for the creation of the OTA BS testing TR 37.941 was discussed in [2]. The first draft of the TR was shared before the electronic meeting for the companies to provide the feedback on the TR content. The content of the External TR 37.941 for the OTA BS testing has been generated based on the inputs from the following legacy technical reports:

-	AAS BS technical reports: TR 37.842, TR 37.843

- 	NR technical report: TR 38.817-02

Skeleton of the TR 37.941 was agreed in [3].

- Measurement uncertainty derivation

Multiple Excel spreadsheets were shared before the electronic meeting to allow companies familiarize with the approach taken for the MU values cross-check and alignment:

R4-2002431	OTA BS testing Tx FR1 MU calculation tables [4]

R4-2002432	OTA BS testing Tx FR2 MU calculation tables [5]

R4-2002433	OTA BS testing RX FR1 MU calculation tables [6]. This contribution was approved.

R4-2001702	OTA BS testing FR1 co-location MU calculation tables [7]. This contribution was approved.

Companies requested more time to check selected aspects of the MU values derivations and corrections for the FR1 and FR2 TX requirements. Therefore [4] and [5] were not approved and will have to be resubmitted for the next e-meeting, with potential improvements.

- Text proposals to the TR 37.941

The following TPs were agreed for the TR 37.941:

- R4-2002434	TP to the TR 37.941: Scope [8]

- R4-2002436	TP to the TR 37.941: measurement types [12]

- R4-2002437	TP to the TR 37.941: OTA measurement systems [13]

- R4-2001814	TP to the TR 37.941: measurement systems calibration [14]

- R4-2002438	TP to the TR 37.941: TX directional requirements [15]

- R4-2001705	TP to TR 37.9xx -Tx MU value derivation sub-clause update [16]

- R4-2001816	TP to the TR 37.941: RX directional requirements [17]

- R4-2002439	TP to the TR 37.941: In-band TRP requirements [18]

- R4-2002440	TP to the TR 37.941: Out-of-band TRP requirements [19]

- R4-2002441	TP to the TR 37.941: Out-of-band blocking requirements [20]

- R4-2002442	TP to TR 37.9xx : Colocation MU value derivation sub-clause updates [21]

- R4-2001820	TP to the TR 37.941: Demodulation performance requirements [22]

- R4-2002443	TP to OTA BS TR on EMC [24]

- R4-2001704	TP to TR 37.9xx - OTA BS testing summary clauses 17-18 [25]

- R4-2002444	TP to TR 37.9xx : Test uncertainty annexes [26]

- R4-2001822	TP to the TR 37.941: annex D, E, F [27]

In order to implement the agreed TPs into the first draft of the TR 37.941 for Rel-15, Big TP for TR 37.941 in [28] was on e-mail approval.

RAN4#94-e-bis (20 – 30 April, 2020)

Documents [29 - 62] were submitted to RAN4#94-e-bis meeting. Summary of the email discussion on [94e Bis][210] OTA_BS_testing topic during the RAN4#94-e-bis meeting was captured in [33].

The following topics were discussed during the electronic meeting:

- General issues

An updated TR 37.941 v0.1.0 was agreed in [29]. Cleanup TP was agreed in [31].

- OTA calibration and test method procedures

Justification for additional Tx test cases for PWS was discussed in [36] with the related TP agreed in [37].

Alignments on the two polarizations testing among test procedures was agreed in TP in [38].

Reverberation chamber description was corrected in TP in [39].

Improvements to the applicability of TRP measurement methods and chambers was agreed in TP in [40].

- Coordinate system

Contributions related to the discussion on the coordinate system were submitted in [41, 42, 43, 44]. Related TP to TR was agreed in [44].

- Conformance testing framework

Conformance testing framework was discussed based on contributions in [45, 46, 47]. The related TP was agreed in [47].

- Measurement uncertainty derivation

Multiple Excel spreadsheets were submitted as the baseline for the MU derivation verification. TX FR1 and FR2 MU derivation tables were submitted in [48, 49]. RX FR1 and FR2 MU derivation tables were submitted in [51, 53].

Related TPs for the MU derivation were agreed in [50, 54, 55, 56, 57, 59].

- Annexes

Annex with the updated MU contributor descriptions was agreed in [60]. Correction of the reference to the RC contributor description was agreed in [61].

- Text proposals to the TR 37.941

The following TPs were agreed for the TR 37.941:

- R4-2003998	TP to TR 37.941: general sections [30]

- R4-2004641	TP to TR 37.941: cleanup [31]

- R4-2004394	TP to TR 37.941 to introduce additional Tx test cases for PWS [37]

- R4-2005502	Ambiguity on two orthogonal polarizations measurements [38]

- R4-2005511	TP to TR 37.941: Correction to reverberation description [39]

- R4-2005512	TP to TR 37.941: Improvement of text in subclause 6.3.2 [40]

- R4-2005503	TP to TR 37.941: Coordinate system [44]

- R4-2005504	TP to TR 37.941: conformance testing framework [47]

- R4-2004531	TP to TR 37.941 -RX MU value derivation sub-clause 10 update (FR1) [50]

- R4-2004535	TP to TR 37.941 Tx TRP emissions MU value derivation sub-clause 11 updates (FR1) [54]

- R4-2004536	TP to TR 37.941: Tx TRP emissions MU value derivation sub-clause 11 updates (FR2) [55]

- R4-2004537	Spurious emissions MU value derivation sub-clause 12 update (FR1) [56]

- R4-2004538	TP to TR 37.941 - Spurious emissions MU value derivation sub-clause 12 update (FR2) [57]

- R4-2005601	TP to TR 37.941 -Tx MU value derivation sub-clause 9 update (FR1) [59]

- R4-2004526	TP to TR 37.941: Test uncertainty annexes – update [60]

- R4-2004643	TP to TR 37.941: reverberation chamber description correction [61]

- R4-2005510	TP to TR 37.941: co-location measurements description [62]

The updated TR 37.941 v0.2.0 implementing the above TPs was agreed during the e-mail approval in [34].

RAN4#95-e (25 May – 5 June, 2020)

Documents [63 - 83] were submitted to RAN4#95-e meeting. Summary of the email discussion on [95e][311] OTA_BS_testing topic during the RAN4#95-e meeting was captured in [63].

Based on the progress during this meeting, the Rel-15 TR was considered to be ready for submission for Approval at RAN#88e (as per WID). Further correction to the Rel-15 TR are to be provided via CRs. The Rel-16 TR (expected to have the same content) is planned to be submitted for Approval at RAN#89e, unless WI extension will be requested by TE vendors for further MU calculations cross-checks.

The following topics were discussed during the electronic meeting:

- General issues

Cleanup TPs were agreed in [65, 83].

- OTA calibration and test method procedures

- Conformance testing framework

Conformance testing framework was further discussed (based on offline coordination) and the related TP capturing further clarifications of the concerns related to the alignment with the legacy TRs was agreed in [73, 74],

- Measurement uncertainty derivation

TP with MU derivations (based on related Excel spreadsheet calculation tables) for additional requirements measurement in PWS (as introduced during previous meeting) was agreed in [68].

Multiple Excel spreadsheets were submitted as the baseline for the MU derivation verification. An updated TX FR2 MU derivation tables were submitted in [69]. An updated RX FR2 MU derivation tables were submitted in [71].

Related TPs for the MU derivation were agreed in [70, 72].

- Text proposals to the TR 37.941

The following TPs were agreed for the TR 37.941:

- R4-2008858	TP to 37.941: MU tables for additional Tx test cases for PWS [68]

- R4-2007911	TP to TR 37.941 FR2 TX directional [70]

- R4-2007913	TP to TR 37.941 FR2 RX directional [72]

- R4-2008856	TP to TR 37.941 MU budget procedure update [73]

- R4-2008857	TP to TR 37.941 EIRP MU budget procedure update [74]

- R4-2008005	TP to TR 37.941 on editorial corrections for PWS references [83]

The updated TR 37.941 v0.3.0 implementing the above TPs was agreed during the e-mail approval in [64].

Due to lack of consensus, the following two TPs were proposed to be further discussed offline before the RAN4#96-e meeting (extract from summary in [63] provided below):

| R4-2008854 | To be noted Ericsson, Nokia, Huawei, MVG (and any other interested companies) to work on this offline before the next meeting. TP in R4-2008854 can be used as baseline for further improvements. |
| --- | --- |
| R4-2008855 | To be noted Ericsson, Nokia, Huawei (and any other interested companies) to work on this offline before the next meeting. TP in R4-2008855 can be used as baseline for further improvements. |

- CRs to legacy TRs

In order to avoid content duplication among the legacy TR/TS and the new TR 37.941, as well as to correct the references to the internal TRs, series of CRs were agreed in [75 – 82] to the following technical reports and technical specifications:

TR 37.842, TR 37.843, TR 38.817-02, TS 37.145-2, TS 38.141-2, TS 37.114.

#### 2.4.2	Remaining Open issues

Based on the progress of email discussions during the meetings, the following items of the TR 37.941 are considered to be open issues for both the Rel-15 and Rel-16:

- Completion and updates of the MU calculation spreadsheets, including possible correction from TE vendors

- cross-TR alignments and the overall cleanup

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1]	R4-2002513	Email discussion summary for RAN4#94e_#84_OTA_BS_testing, Email discussion summary for RAN4#94e_#84_OTA_BS_testing	Moderator (Huawei), RAN4#94-e

[2]	R4-2001806	Work-plan for the OTA BS testing WI, Huawei, RAN4#94-e

[3]	R4-2001807	Skeleton for TR 37.941 on OTA BS testing, Rel-15, Huawei, RAN4#94-e

[4]	R4-2002431	OTA BS testing Tx FR1 MU calculation tables, Huawei, RAN4#94-e

[5]	R4-2002432	OTA BS testing Tx FR2 MU calculation tables, Huawei, RAN4#94-e

[6]	R4-2002433	OTA BS testing RX FR1 MU calculation tables, Huawei, RAN4#94-e

[7]	R4-2001702	OTA BS testing FR1 co-location MU calculation tables, Huawei, RAN4#94-e

[8]	R4-2002434	TP to the TR 37.941: Scope, Huawei, RAN4#94-e

[9]	R4-2001809	TP to the TR 37.941: general sections, Huawei, RAN4#94-e

[10]	R4-2001810	TP to the TR 37.941: Coordinate system, Huawei, RAN4#94-e

[11]	R4-2002435	TP to the TR 37.941: conformance testing framework, Huawei, RAN4#94-e

[12]	R4-2002436	TP to the TR 37.941: measurement types, Huawei, RAN4#94-e

[13]	R4-2002437	TP to the TR 37.941: OTA measurement systems, Huawei, RAN4#94-e

[14]	R4-2001814	TP to the TR 37.941: measurement systems calibration, Huawei, RAN4#94-e

[15]	R4-2002438	TP to the TR 37.941: TX directional requirements, Huawei, RAN4#94-e

[16]	R4-2001705	TP to TR 37.9xx -Tx MU value derivation sub-clause update, Huawei, RAN4#94-e

[17]	R4-2001816	TP to the TR 37.941: RX directional requirements, Huawei, RAN4#94-e

[18]	R4-2002439	TP to the TR 37.941: In-band TRP requirements, Huawei, RAN4#94-e

[19]	R4-2002440	TP to the TR 37.941: Out-of-band TRP requirements, Huawei, RAN4#94-e

[20]	R4-2002441	TP to the TR 37.941: Out-of-band blocking requirements, Huawei, RAN4#94-e

[21]	R4-2002442	TP to TR 37.9xx: Colocation MU value derivation sub-clause updates, Huawei, RAN4#94-e

[22]	R4-2001820	TP to the TR 37.941: Demodulation performance requirements, Huawei, RAN4#94-e

[23]	R4-2001821	TP to the TR 37.941: EMC requirements, Huawei, RAN4#94-e

[24]	R4-2002443	TP to OTA BS TR on EMC, ZTE Corporation, RAN4#94-e

[25]	R4-2001704	TP to TR 37.9xx - OTA BS testing summary clauses 17-18, Huawei, RAN4#94-e

[26]	R4-2002444	TP to TR 37.9xx: Test uncertainty annexes, Huawei, RAN4#94-e

[27]	R4-2001822	TP to the TR 37.941: annex D, E, F, Huawei, RAN4#94-e

[28]	R4-2002430	Big TP for TR 37.941, Rel-15, Huawei, RAN4#94-e

[29]		R4-2003997	Draft TR 37.941 v0.1.0, Huawei, RAN4#94-e-bis

[30]	R4-2003998	TP to TR 37.941: general sections, Huawei, RAN4#94-e-bis

[31]	R4-2004641	TP to TR 37.941: cleanup, Huawei, RAN4#94-e-bis

[32]	R4-2005509	TP to TR 37.941: Improvement to increase readability in Clause 6, Ericsson, Rohde&Schwarz, RAN4#94-e-bis

[33]	R4-2005597	Email discussion summary for [94e Bis][210] OTA_BS_testing, Moderator (Huawei) , RAN4#94-e-bis

[34]	R4-2005609	Draft TR 37.941 v0.2.0, Huawei, RAN4#94-e-bis

[35]	R4-2004038	TP to TR 37.941: Addition of technical background for co-location requirements in subclause 6.4, Ericsson, RAN4#94-e-bis

[36]	R4-2004393	Justification for additional Tx test cases for PWS, ROHDE & SCHWARZ, RAN4#94-e-bis

[37]	R4-2004394	TP to TR 37.941 to introduce additional Tx test cases for PWS, ROHDE & SCHWARZ, RAN4#94-e-bis

[38]	R4-2005502	Ambiguity on two orthogonal polarizations measurements, ROHDE & SCHWARZ, RAN4#94-e-bis

[39]	R4-2005511	TP to TR 37.941: Correction to reverberation description, Ericsson, RAN4#94-e-bis

[40]	R4-2005512	TP to TR 37.941: Improvement of text in subclause 6.3.2, Ericsson LM, RAN4#94-e-bis

[41]	R4-2003999	TP to TR 37.941: coordinate system, Huawei, RAN4#94-e-bis

[42]	R4-2004042	TP to the TR 37.941: Coordinate system, Ericsson, Rohde&Schwarz, RAN4#94-e-bis

[43]	R4-2004525	Discussions on the spherical coordinate system, Nokia, Nokia Shanghai Bell, RAN4#94-e-bis

[44]	R4-2005503	TP to TR 37.941: Coordinate system, Nokia, Nokia Shanghai Bell, RAN4#94-e-bis

[45]	R4-2004040	TP to TR 37.941: MU budget format for RC, Ericsson, RAN4#94-e-bis

[46]	R4-2004041	TP to the TR 37.941: Conformance Framework, Ericsson, RAN4#94-e-bis

[47]	R4-2005504	TP to TR 37.941: conformance testing framework, Huawei, RAN4#94-e-bis

[48]	R4-2004527	OTA BS testing Tx FR1 MU calculation tables, Huawei, RAN4#94-e-bis

[49]	R4-2004529	OTA BS testing Tx FR2 MU calculation tables, Huawei, RAN4#94-e-bis

[50]	R4-2004531	TP to TR 37.941 -RX MU value derivation sub-clause 10 update (FR1), Huawei, RAN4#94-e-bis

[51]	R4-2004532	OTA BS testing Rx FR2 MU calculation tables, Huawei, RAN4#94-e-bis

[52]	R4-2004533	TP to TR 37.941 -RX MU value derivation sub-clause 10 update (FR2), Huawei, RAN4#94-e-bis

[53]	R4-2004534	OTA BS testing RX FR1 MU calculation tables – correction, Huawei, RAN4#94-e-bis

[54]	R4-2004535	TP to TR 37.941 Tx TRP emissions MU value derivation sub-clause 11 updates (FR1), Huawei, RAN4#94-e-bis

[55]	R4-2004536	TP to TR 37.941: Tx TRP emissions MU value derivation sub-clause 11 updates (FR2), Huawei, RAN4#94-e-bis

[56]	R4-2004537	Spurious emissions MU value derivation sub-clause 12 update (FR1), Huawei, RAN4#94-e-bis

[57]	R4-2004538	TP to TR 37.941 - Spurious emissions MU value derivation sub-clause 12 update (FR2), Huawei, RAN4#94-e-bis

[58]	R4-2005505	TP to TR 37.941 -Tx MU value derivation sub-clause 9 update (FR2), Huawei, RAN4#94-e-bis

[59]	R4-2005601	TP to TR 37.941 -Tx MU value derivation sub-clause 9 update (FR1), Huawei, RAN4#94-e-bis

[60]	R4-2004526	TP to TR 37.941: Test uncertainty annexes – update, Huawei, RAN4#94-e-bis

[61]	R4-2004643	TP to TR 37.941: reverberation chamber description correction, Huawei, RAN4#94-e-bis

[62]	R4-2005510	TP to TR 37.941: co-location measurements description, Huawei, RAN4#94-e-bis

[63]		R4-2009045	Email discussion summary for [95e][311] OTA_BS_testing, Moderator (Huawei), RAN4#95-e

[64]	R4-2009064	Draft TR 37.941 v0.3.0, Huawei, RAN4#95-e

[65]	R4-2008853	TP to TR 37.941: editorial cleanup, Huawei, RAN4#95-e

[66]	R4-2008854	TP to TR 37.941: Improvement of technical background information in Clause 6, Ericsson, Rohde & Schwarz, RAN4#95-e

[67]	R4-2008855	TP to TR 37.941: Improvement of the Clause 6.3.3, Ericsson, RAN4#95-e

[68]	R4-2008858	TP to 37.941: MU tables for additional Tx test cases for PWS, ROHDE & SCHWARZ, RAN4#95-e

[69]	R4-2008859	TX directional FR2 MU budget spreadsheet, Huawei, RAN4#95-e

[70]	R4-2007911	TP to TR 37.941 FR2 TX directional, Huawei, RAN4#95-e

[71]	R4-2008860	RX directional FR2 MU budget spreadsheet, Huawei, RAN4#95-e

[72]	R4-2007913	TP to TR 37.941 FR2 RX directional, Huawei, RAN4#95-e

[73]	R4-2008856	TP to TR 37.941 MU budget procedure update, Huawei, RAN4#95-e

[74]	R4-2008857	TP to TR 37.941 EIRP MU budget procedure update, Huawei, RAN4#95-e

[75]	R4-2008861	CR to TR 37.842: internal TR references corrections and content redundancy removal (wrt. TR 37.941 for OTA BS testing), Rel-15, Huawei, RAN4#95-e

[76]	R4-2008862	CR to TR 37.843: internal TR references corrections and content redundancy removal (wrt. TR 37.941 for OTA BS testing), Rel-15, Huawei, RAN4#95-e

[77]	R4-2008863	CR to TR 38.817-02: internal TR references corrections and content redundancy removal (wrt. TR 37.941 for OTA BS testing), Rel-15, Huawei, RAN4#95-e

[78]	R4-2007454	CR to TS 37.145-2: internal TR references corrections (wrt. TR 37.941 for OTA BS testing), Rel-15, Huawei, RAN4#95-e

[79]	R4-2007455	CR to TS 37.145-2: internal TR references corrections (wrt. TR 37.941 for OTA BS testing), Rel-16, Huawei, RAN4#95-e

[80]	R4-2007456	CR to TS 38.141-2: internal TR references corrections (wrt. TR 37.941 for OTA BS testing), Rel-15, Huawei, RAN4#95-e

[81]	R4-2007457	CR to TS 38.141-2: internal TR references corrections (wrt. TR 37.941 for OTA BS testing), Rel-16, Huawei, RAN4#95-e

[82]	R4-2007458	CR to TS 37.114: internal TR reference corrections, Rel-15, Huawei, RAN4#95-e

[83]	R4-2008005	TP to TR 37.941 on editorial corrections for PWS references, ROHDE & SCHWARZ, RAN4#95-e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template