3GPP TSG RAN WG4 #95-e	R4-2007440

Electronic Meeting, 25 May – 05 June, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0736 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on RRC parameter referenced for MIMO layer adaptation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-Core |  |  |  |  |  |  | Date: |  | 2020-5-14 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The agreed CR in RAN4#94-e for 38.133 introduced the “maxMIMO-Layers " into Table 8.2.1.2.7-2 and Table 8.2.2.2.5-2 to reflect the newly introduced interutpion for MIMO layer adaptation which the requirement is in-line with R15 BWP switching. However, the newly IE introduced in Rel-16 was defined as “maxMIMO-Layers-r16” in PDSCH-Config IE which is used to configure the UE specific PDSCH parameters according to 38.331 V16.0.0. This indicates the maximum MIMO layer configuration for a DL BWP, whereas the original Rel-15 “maxMIMO-Layers” is in the IE PDSCH-ServingCellConfig which is used to configure UE specific PDSCH parameters that are common across the UE's BWPs of one serving cell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise “maxMIMO-Layers” to “maxMIMO-Layers-r16” according to latest 38.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IE which reference from 38.331 is not consistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.7, 8.2.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Resubmission of endorsed CR R4-2004061 |  |  |  |  |  |  |  |  |

< Start of changes >

Table 8.2.1.2.7-2: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [2] |
| nrofSRS-Ports |  |
| maxMIMO-Layers-r16 |  |
| Editor’s note: More parameters can be added if identified |  |

< Next Changed Section >

Table 8.2.2.2.5-2: Parameters which cause interruption other than SCS

| Parameters | Comment |
| --- | --- |
| locationAndBandwidth | From TS 38.331 [2] |
| nrofSRS-Ports |  |
| maxMIMO-Layers-r16 |  |
| Editor’s note: More parameters can be added if identified |  |

< End of changes >