3GPP TSG-RAN2 Meeting # 110bis electronic 	R2-2004822

# 1 June - 12 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.306 | CR | 1761 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on L1 feature of NGEN-DC and NE-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-18 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to RAN1 LS rely in R1-1814106, Agreement: The LS is agreed with the following replacements for the answers to questions Q4 and Q5. - Answer for Q4: There are no physical layer features that are dependent on whether NGEN-DC or EN-DC is deployed. From RAN1 perspective, assuming there is no difference in which features are mandatory/optional between NGEN-DC and EN-DC and not considering IODT aspects, the UE capabilities for NGEN-DC can re-use all reported EN-DC capabilities. If IODT aspects are considered, it is possible that different features may be deployed for NGEN-DC and EN-DC and the band combinations or deployed functionality in the set of band combinations that are used/tested may be different, in which case, some IODT differentiation may be necessary. Decisions regarding IODT aspects should be made at the RAN plenary. - Answer for Q5: There are no physical layer features, other than dynamic power sharing, that are dependent on whether NE-DC or EN-DC is deployed. From RAN1 perspective, not considering IODT aspects, the UE capabilities for NE-DC can re-use all reported EN-DC capabilities, except that for dynamic power sharing. If IODT aspects are considered, it is possible that different features may be deployed for NE-DC and EN-DC and the band combinations or deployed functionality in the set of band combinations that are used/tested may be different, in which case, some IODT differentiation may be necessary. Decisions regarding IODT aspects should be made at the RAN plenary. Furthermore, according to RAN1 LS rely in R1-2002792, “RAN1 confirms that the features of dl-1024QAM-TotalWeightedLayers and fd-MIMO-TotalWeightedLayers can be signaled to the LTE part of NE-DC case.”. Therefore, this CR is to clarify the support of NE-DC for related IEs. Therefore, this CR is to clarify the support of NGEN-DC and NE-DC for related IEs for L1 features. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the support of NGEN-DC and NE-DC in field description of related capabilities. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current issue remains, i.e., the UE supporting NGEN-DC and NE-DC cannot indicate the support of related L1 features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.5.31, 4.3.28.13, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

4.3.5.31	dl-1024QAM-r15

This field defines whether the UE supports 1024QAM in DL on this band or on this band within the band combination as described in TS 36.331 [5]. This field is only applicable for UEs of DL category 20, 22 and onwards.

When dl-1024QAM-ScalingFactor-r15 and dl-1024QAM-TotalWeightedLayers-r15 are included, the UE supports 1024QAM in a set of CCs in a band combination if the CCs belong to bands indicated to support 1024QAM in that band combination, and the 1024QAM processing capability condition described by equation 4.3.5.31-1 is satisfied.

w.l1024QAM+lnon1024QAM≤y

where:

-	w is the scaling factor for processing a CC configured with 1024QAM with respect to a CC not configured with 1024QAM as indicated by dl-1024QAM-ScalingFactor-r15,

-	l1024QAM is the total number of DL layers across all CCs configured with 1024QAM,

-	lnon1024QAM is the total number of DL layers acoss all CCs not configured with 1024QAM, and

-	y is total number of weighted layers the UE can process for 1024QAM. Value of y is indicated by dl-1024QAM-TotalWeightedLayers-r15 for all band combinations except for those (NG)EN-DC/NE-DC band combinations for which dl-1024QAM-TotalWeightedLayers is included in ca-ParametersEUTRA (see TS 38.306 [32] and TS 38.331 [35]).

Equation 4.3.5.31-1: 1024QAM processing capability condition.

NOTE:	The 1024QAM processing capability condition described by equation 4.3.5.31-1 applies only when at least one of the CCs in a band combination is configured with 1024QAM.

NEXT CHANGE

4.3.28.13	totalWeightedLayers-r13

This field indicates total number of weighted layers the UE can process for FD-MIMO, as described in equation 4.3.28.13-1 below and TS 36.331 [5] clause 6.3.6, NOTE 8 in UE-EUTRA-Capability field descriptions.

The FD-MIMO processing capability condition is satisfied if:

i ∈ configured CCswi∙li≤y

where:

-	y is total number of weighted layers the UE can process for FD-MIMO. Value of y is indicated by totalWeightedLayers-r13 for all band combinations except for those (NG)EN-DC/NE-DC band combinations for which fd-MIMO-TotalWeightedLayers is included in ca-ParametersEUTRA (see TS 38.331 [35] and TS 38.306 [32]),

-	li is the maximum number of DL layers configured for CC i, and

-	wi=relWeightTwoLayers,  if CC i is configured with FD-MIMO and li=2relWeightFourLayers,  if CC i is configured with FD-MIMO and li=4relWeightEightLayers,  if CC i is configured with FD-MIMO and li=81,   if CC i  is not configured with FD-MIMO.

Equation 4.3.28.13-1: FD-MIMO processing capability condition.

CHANGE END