3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2011088

# Electronic Meeting, 17 August – 28 August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6933 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on serving cell measurement on non-anchor carrier for NB-IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - According to TS 36.304, the NRSRP on non-anchor carrier is translated to the NRSRP on anchor carrier as: Qrxlevmeas = QrxlevmeasNonAnchor - nrs-PowerOffsetNonAnchor. Thus, the inequality for comparing the measurement form anchor carrier and non-anchor carrier shall be ¦ NRSRPanchor – (NRSRPnon-anchor - nrs-PowerOffsetNonAnchor) ¦ ≤ 10 dB - Rephrase the conditon of the presence of NRS on non-anchor carrier |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change the “+” to “-” in the equality for comparing the measurement form anchor carrier and non-anchor carrier - Rephrase the conditon of the presence of NRS on non-anchor carrier |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2.1, 4.6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage

The UE shall measure the NRSRP and NRSRQ level of the serving NB-IoT cell on the anchor carrier and evaluate the cell selection criterion S defined in clause 5.2.3.2 in [1] for the serving NB-IoT cell on the anchor carrier at least every DRX cycle.

If the UE is configured for receiving paging on the non-anchor carrier then the UE shall evaluate the cell selection criterion S defined in clause 5.2.3.2a in [1] for the serving NB-IoT cell on non-anchor carrier at least every DRX cycle.

The UE is allowed to measure NRSRP level of the serving NB-IoT cell, assuming that nrs-NonAnchor-config is enabled indicated by higher layer defined in clause 10.2.6 TS 36.211 [16], on non-anchor carrier provided that:

-	The relaxed monitoring criteria defined in TS 36.304 clause 5.2.4.12 are met,

-	Transmit power difference of the signals/channels between anchor- and non-anchor carriers is signalled to the UE, via the existing parameter nrs-PowerOffsetNonAnchor, and

-	UE is not configured with any positioning measurements.

The UE shall filter the NRSRP and NRSRQ measurements of the NB-IoT serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

When UE measures the NRSRP on non-anchor carrier, UE shall compare the measurements from anchor carrier and non-anchor carrier at least once every one hour by the following inequality:

| NRSRPanchor – (NRSRPnon-anchor - nrs-PowerOffsetNonAnchor) | ≤ 10 dB

where NRSRPanchor is the NRSRP measurement on anchor carrier and NRSRPnon-anchor is the NRSRP measurement on non-anchor carrier. The measurement for comparison shall use at least 2 measurements for filtering. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2. If the measurement for comparison satisfy the inequality, UE is allowed to perform RRM measurements on the non-anchor carrier until the next comparison takes place or until the relaxed monitoring conditions are no longer met. UE shall perform NRSRP measurement on anchor carrier if the inequality is not satisfied until the next comparison takes place.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.6.2.1-1 in Nserv_NB-IoT-NC consecutive DRX cycles that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities. If the UE is configured with eDRX_IDLE cycle and has evaluated according to Table 4.6.2.1-2 in Nserv_NB-NC consecutive DRX cycles within a single PTW that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency and inter-frequency information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in [1], where T=40 s if the UE is not configured with eDRX_IDLE cycle, and T=MAX(40 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle.

Table 4.6.2.1-1: Nserv_NB--NC

| DRX cycle length [s] | Nserv_NB-IoT-NC [number of DRX cycles] |
| --- | --- |
| 0.32 | 2 |
| 0.64 | 2 |
| 1.28 | 2 |
| 2.56 | 2 |
| 5.12 | 2 |
| 10.24 | 2 |

Table 4.6.2.1-2: Nserv_NB-NC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Nserv_NB-IoT-NC [number of DRX cycles] |
| --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥2.56 (1) | 2 |
|  | 0.64 | ≥2.56 (1) | 2 |
|  | 1.28 | ≥5.12 (2) | 2 |
|  | 2.56 | ≥7.68 (3) | 2 |
|  | 5.12 | ≥12.8 (5) | 2 |
|  | 10.24 | ≥23.04 (9) | 2 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

<End of Change 1>

<Start of Change 2>

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage

The UE shall measure the NRSRP and NRSRQ level of the serving NB-IoT cell on the anchor carrier and evaluate the cell selection criterion S defined in clause 5.2.3.2 in [1] for the serving NB-IoT cell on the anchor carrier at least every DRX cycle.

If the UE is configured for receiving paging on the non-anchor carrier then the UE shall evaluate the cell selection criterion S defined in clause 5.2.3.2a in [1] for the serving NB-IoT cell on non-anchor carrier at least every DRX cycle.

The UE is allowed to measure NRSRP level of the serving NB-IoT cell, assuming that nrs-NonAnchor-config is enabled indicated by higher layer defined in clause 10.2.6 TS 36.211 [16], on non-anchor carrier provided that:

-	The relaxed monitoring criteria defined in TS 36.304 clause 5.2.4.12 are met,

-	Transmit power difference of the signals/channels between anchor- and non-anchor carriers is signalled to the UE, via the existing parameter nrs-PowerOffsetNonAnchor,

-	UE is not configured with any positioning measurements.

-	nB configured by higher layer is not equal to 4T.

The UE shall filter the NRSRP and NRSRQ measurements of the serving NB-IoT cell using at least 4 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least DRX cycle/2.

If the UE is not configured with eDRX_IDLE cycle and has evaluated according to Table 4.6.2.3-1 in Nserv_NB_EC consecutive DRX cycles that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities. If the UE is configured with eDRX_IDLE cycle and has evaluated according to Table 4.6.2.3-2 in Nserv_NB-IoT-EC consecutive DRX cycles within a single PTW that the serving NB-IoT cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving NB-IoT cell, regardless of the measurement rules currently limiting UE measurement activities.

If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency and inter-frequency information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in [1], where T= 80 s if the UE is not configured with eDRX_IDLE cycle, and T=MAX(80 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle.

Table 4.6.2.3-1: Nserv_NB_EC

| DRX cycle length [s] | Nserv_NB-IoT-EC [number of DRX cycles] |
| --- | --- |
| 0.32 | 4 |
| 0.64 | 4 |
| 1.28 | 4 |
| 2.56 | 4 |
| 5.12 | 4 |
| 10.24 | 4 |

Table 4.6.2.3-2: Nserv_NB_EC for UE configured with eDRX_IDLE cycle

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 2.56s periods) | Nserv_NB_EC [number of DRX cycles] |
| --- | --- | --- | --- |
| 20.48 ≤ eDRX_IDLE cycle length ≤ 10485.76 | 0.32 | ≥ 2.56 (1) | 4 |
|  | 0.64 | ≥ 5.12 (2) | 4 |
|  | 1.28 | ≥ 7.68 (3) | 4 |
|  | 2.56 | ≥ 12.8 (5) | 4 |
|  | 5.12 | ≥ 23.04 (9) | 4 |
|  | 10.24 | ≥ 43.52 (17) | 4 |
| NOTE 1: The number of DRX cycles in this table is given for the DRX cycles within PTWs. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section X of TS 24.008 [34]. |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, being configured with eDRX_IDLE cycle, changing eDRX_IDLE cycle length, or changing PTW configuration, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE has to meet the requirement corresponding to the second state.

<End of Change 2>