| 3GPP TR 38.809 V1.0.0(2020-09) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Background for Integrated access and backhaul radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners’ Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles – Sophia Antipolis Valbonne – FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	General aspect	8

4.1	Relation with other core specification	8

4.2	RF Requirements reference points	9

4.3	IAB classification	9

4.4	Regulatory aspects	9

4.5	IAB architecture	9

5   Operating band and channel arrangement	11

5.1	General	11

5.2	Operating bands	11

5.3	Channel bandwidth	12

5.4	Channel arrangement	12

6   Co-existence study	12

6.1	System layout and scenario	12

6.1.1 Layout for co-existence study	12

6.1.2 Scenarios for co-existence study	14

6.1.3	Co-location	16

6.2	Simulation assumption	17

6.2.1 Propagation model	17

6.2.2	Antenna configuration	17

6.2.2.1	General	17

6.2.2.2	FR1	17

6.2.2.3            FR2	19

6.2.3 Other simulation assumption	21

6.3	Simulation result	22

6.4	Summary on co-existence study	22

7	Conducted transmitter characteristics	22

7.1	General	22

7.2	IAB output power	22

7.3	Output power dynamics	22

7.3.1	Power control	22

7.4	Transmit ON/OFF power	23

7.5	Transmitted signal quality	23

7.5.1 IAB-DU transmitted signal quality	23

7.5.2.1 Frequency error	23

7.5.2.2 Error Vector Magnitude	23

7.6	Unwanted emissions	23

7.7	Transmitter intermodulation	24

8	Conducted receiver characteristics	24

8.1	General	24

8.2	Reference sensitivity level	24

8.2.1	IAB-DU Reference sensitivity	24

8.2.2	IAB-MT Reference sensitivity	24

8.3	Dynamic range	26

8.4	In-band selectivity and blocking	26

8.5	Out-of-band blocking	26

8.6	Receiver spurious emissions	26

8.6.1 IAB-MT Receiver spurious for IAB type 1-H	26

8.7	Receiver intermodulation	27

8.8	In-channel selectivity	27

9	Radiated transmitter characteristics	27

9.1	General	27

9.2	Radiated transmit power	27

9.3	IAB OTA output power	28

9.4	OTA output power dynamics	28

9.5	OTA transmit ON/OFF power	28

9.6	OTA transmitted signal quality	28

9.6.1 IAB-DU OTA transmitted signal quality	28

9.6.2.1 Frequency error	28

9.6.2.2 Error Vector Magnitude	28

9.7	OTA unwanted emissions	28

9.8	OTA transmitter intermodulation	29

9.8.1 IAB DU intermodulation	29

9.8.2 IAB MT intermodulation	29

9.9	Beam correspondence for IAB-MT	29

10	Radiated receiver characteristics	29

10.1	General	29

10.2	OTA sensitivity	29

10.2.1	IAB-DU OTA sensitivity	29

10.2.2	IAB-MT OTA sensitivity	30

10.3	OTA reference sensitivity level	30

10.3.1	IAB-DU OTA reference sensitivity	30

10.3.2	IAB-MT OTA reference sensitivity	30

10.3.2.1	FR1	30

10.3.2.1	FR2	31

10.4	OTA Dynamic range	32

10.5	OTA in-band selectivity and blocking	32

10.5.1 ACS for IAB MT of IAB type 2-O	32

10.5.2 In-band blocking for IAB-MT	32

10.6	OTA out-of-band blocking	33

10.7	OTA receiver spurious emissions	33

10.7.1 IAB-MT OTA Receiver spurious for IAB type 1-O and 2-O	33

10.8	OTA receiver intermodulation	33

10.9	OTA in-channel selectivity	33

11	IAB RRM requirements	33

12	IAB EMC requirements	33

12.1 IAB EMC Emission requirements	33

12.2 IAB EMC immunity requirements	34

12.2.1 Radiated Immunity requirements	34

12.2.1.1 One enclosure case	34

12.2.1.2 Different enclosure case	34

12.2.1.4 Summary	34

12.2.2 Other Immunity requirements	34

13	IAB performance requirements	34

14	IAB conformance testing aspect	34

Annex A (normative): Co-existence simulation results	35

Annex B (informative):  Change history	36

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

Y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

Should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

Can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

Will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is the Technical Report for the Work Item on Integrated Access and Backhaul for NR. The present document captures the background and the decisions on IAB requirements.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.104: “NR; Base Station (BS) radio transmission and reception”

[3]	3GPP TS 38.101-1: “NR User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

[4]	3GPP TS 38.101-2: “NR User Equipment (UE) radio transmission and reception: Part 2: Range 2 Standalone”

[5]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios"

[6]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing".

[7]                       3GPP TS 38.817-02: "General aspects for Base Station (BS) Radio Frequency (RF) for NR".

[8]                       3GPP TS 38.174: "NR; Integrated Access and Backhaul (IAB) radio transmission and reception"

[9]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain".

[10]	Recommendation ITU-R SM.1539-1: "Variation of the boundary between the out-of-band and spurious domains required for the application of Recommendations ITU-R SM.1541 and ITU-R SM.329".

[11]	CISPR 32: "Electromagnetic compatibility of multimedia equipment - Emission requirements".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

Example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

4	General aspect

4.1	Relation with other core specification

IAB (Integrated Access and backhaul) node is a RAN node that supports wireless access to Ues and wirelessly backhauls the access traffic. Direct application of only Base station RF requirements is not appropriate enabler for both the wireless backhaul and wireless access and thus a new TS spec 38.174 will be specified.

In R16 IAB WI, the wireless backhaul technology is based on NR Uu so the 3GPP terminology will be reused to define the wireless backhaul RF specification. In Figure 6.3.1-1 reference diagram of architecture in 38.874, IAB node is functional splitted with IAB-DU and IAB-MT. IAB-DU has interface with UE and IAB-MT so it is representing the Base Station functionality. IAB-MT has interface with IAB-DU only and thus is representing the UE functionality.

3GPP TS 38.174 is a Single RAT NR IAB specification. The single RAT IAB means the same RAT will apply to both IAB-DU and IAB-MT. It is expected to capture IAB-DU and IAB-MT requirements for the following aspects:

Tx, Rx and demodulation core requirements for NR IAB-DU and NR IAB-MT

-	Conducted and radiated sets of core requirements for the above listed categories (i.e. Tx, Rx and IAB demodulation),

-	Requirements for FR1 and FR2 frequency ranges

-	FR1: Both conducted and OTA requirements will be required for Range 1. The applicability may depend on the requirements.

-		Requirement set 1-H: Conducted requirements and OTA requirements for FR1 hybrid IAB-DU and IAB-MT (which includes antenna functionality).

-	Requirement set 1-O: OTA requirements for FR1 OTA IAB-DU and OTA IAB-MT (which includes antenna functionality).

-	FR2: Only OTA requirements are defined for FR2.

-	Requirement set 2-O: OTA requirements for FR2 OTA IAB-DU and OTA IAB-MT.

3GPP TS 38.104 [3] is a Single RAT NR BS specification. IAB-DU shall reuse the relevant requirements from spec in TS 38.104. Where applicable, the IAB-MT may also re-use requirements from 38.104.

3GPP TS 38.101-1/2 is a Single RAT NR UE specification. Where applicable, IAB-MT may reuse requirements from the spec in TS 38.101-1/2.

4.2	RF Requirements reference points

The requirement reference points for BS is defined as below:

Table 4.2-1: BS requirement sets

| BS type / Requirement set | BS Description | Additional information |
| --- | --- | --- |
| 1-H | A BS operating at FR1 with a requirement set holding requirements defined at the TAB and OTA requirements defined at RIB. | The requirement set is like the one defined for Hybrid AAS BS. Following the approach used in 3GPP TS 37.105 |
| 1-O | A BS operating at FR1 with a requirement set consisting only OTA requirements defined at the RIB. | Following the approach developed in Eaas and documented in 3GPP TR 37.843. |
| 2-O | A BS operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB. | This requirement set is relevant for AAS BS and does not require access to RF connectors. |

The test and reference point of RF requirement for BS should be reused and thus the test spec will not be impacted.

The IAB-Du and IAB_MT may share the same hardware implementation, of so the reference points for the IAB-DU and the IAB-MT may be different.

Table 4.2-2: IAB requirement sets

| IAB type / Requirement set | IAB Description | Additional information |
| --- | --- | --- |
| 1-H | IAB-MT and IAB-DU operating at FR1 with a requirement set holding requirements defined at the respective TAB and OTA requirements defined at the respective RIB | The requirement set for both IAB-DU and IAB-MT. |
| 1-O | IAB-MT and IAB-DU operating at FR1 with a requirement set consisting only of OTA requirements defined at the respective RIB. | The requirement set for both IAB-DU and IAB-MT. |
| 2-O | IAB-MT and IAB-DU operating at FR2 with a requirement set consisting only of OTA requirements defined at the respective RIB. | The requirement set for both IAB-DU and IAB-MT. |

4.3	IAB classification

IAB-MT classification and IAB-DU classification will be defined separately.

For IAB-DU the classification will be the same as NR BS classification in TS 38.104 [x] to support Wide Area IAB-DU, Medium Range IAB-DU and Local Area IAB-DU with the same deployment scenarios for each class as for BS. The same criteria and exactly the same parameter of NR BS classification, e,g, BS to UE minimum distance on the ground for BS without connector, IAB-DU to UE minimum coupling loss for BS with connector, will be applied for IAB-DU with modification as IAB-DU to UE minimum distance on the ground for IAB-DU type 2-O/1-O and IAB-DU minimum coupling loss depends on IAB-DU type 1-H.

For IAB-MT multiple classes will be defined based on deployment scenario as well.

4.4	Regulatory aspects

<Text will be added>

4.5	IAB architecture

The logical architecture (considering the RF interfaces) of the IAB is as follows:

Figure 4.5-1: Logical IAB functions showing RF interfaces

In the RF specification these logical functions have been separated into the HW entities the IAB-DU and the IAB-MT IAB-DU. The HW entities may be implemented in the same radio hardware as shown in figure 4.2-2 or separate radio hardware as shown in figure 4.5-3, the diagrams show the OTA architecture but the same applies for the hybrid architecture.

Figure 4.5-2: Shared IAB hardware

Figure 4.5-3: Separate IAB hardware

The figures show the hardware as either completely shared or completely separate, there are of course many levels of possible integration in between these 2 extremes.

When considering the OTA architecture and specifications the nature of HW is not relevant as the node can be treated as a black box, however in deriving the requirements for the IAB nodes it is important to consider both implementations so that both can be implemented if required.

5   Operating band and channel arrangement

5.1	General

The Frequency range definitions for IAB will be the same as for the NR BS in TS 38.104 [x].

5.2	Operating bands

Operating bands will be added to the IAB specification as they are approved, no bands are precluded at this stage and can be added based on consensus. Currently the FR1 bands shown in table 5.2-1 and all Rel-15 FR2 bands shown in table 5.2-2 are to be included in the IAB specification.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n41 | 2496 MHz – 2690 MHz | TDD |
| n77 | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | TDD |

Table 5.2-2: NR operating bands in FR2

| NR operating band | Uplink (UL) and Downlink (DL) operating band BS transmit/receive UE transmit/receive FUL,low – FUL,high FDL,low – FDL,high | Duplex mode |
| --- | --- | --- |
| n257 | 26500 MHz – 29500 MHz | TDD |
| n258 | 24250 MHz – 27500 MHz | TDD |
| n260 | 37000 MHz – 40000 MHz | TDD |
| n261 | 27500 MHz – 28350 MHz | TDD |

5.3	Channel bandwidth

Channel bandwidth covers definitions on the transmission bandwidth configuration, minimum guard band, RB alignment and channel bandwidth per operating band. For the transmission bandwidth configuration and minimum guard band, existing NR definition for BS and UE for each frequency range is the same. And they should be applied for IAB as it is. For BR alignment the IAB-DU should follow NR BS definition, And IAB-MT should follow NR UE definition. Regarding Channel bandwidth per operating band, it is agreed to incorporate only the bands and associated channel bandwidth to be supported for IAB in IAB specification.

5.4	Channel arrangement

Channel arrangement includes channel spacing, channel raster and sync raster. The introduction of IAB node should be transparent to UE from IAB-DU perspective. And it is not expected to impact on existing fundamental NR design on global channel raster and sync raster as well. Hence the channel raster, channel raster and sync raster definition in existing NR specification, which is identical in BS and UE, will be applied for IAB.

6   Co-existence study

6.1	System layout and scenario

6.1.1 Layout for co-existence study

For Layout 1 considered in IAB co-existence study the details are provided in Table 6.1.1-1 and figure 6.1.1-1 as below.

Figure 6.1.1-1: Layout 1

Table 6.1.1-1: Parameters for Layout 1

| Parameters | Values and remarks |
| --- | --- |
| Layout for nodes | IAB network: Two layer Macro layer: Hex. Grid (3 sector) (all macro BSs are IAB-donors) 19 sites Micro layer: 1 micro BSs per macro BS 1. Random drop (All micro BSs are all outdoor and are IAB-nodes) 2. Drop micro nodes in a circle with center at 40m and radius of 20m Victim network 1. Macro layer: Hex grid 3 sector coordinated layout (0% grid shift) – Rel.15 legacy network 2. Same as aggressor Orientation: Highest gain towards donor |
| Inter-BS distance | Macro layer: 200m FR2, 500m FR1 |
| Minimum distance between Micro BS | 40m,50m, 60m |
| Minimum distance between Macro BS and UE | 10m |
| Minimum distance between Micro BS and UE | 10m |

For Layout 2 considered in IAB co-existence study the details are provided in Table 6.1.1-2 and figure 6.1.1-2 as below.

Figure 6.1.1-2: Layout 2

Table 6.1.1-2: Parameters for Layout 2

| Parameters | Values and remarks |
| --- | --- |
| Layout for nodes | IAB network: Micro layer: Hex tri-sectorial Number of IAB-donors (Ndonor): 1 Number of IAB-nodes is 19 – Ndonor Victim network 1. Micro layer: Hex tri-sectorial – grid shift derived from minimum distance (20meters and 40 meters) 2. same as aggressor IAB-node and Micro BS are assumed to have 3 panels with 120 degree shift relative to each other. |
| Inter-BS distance | Micro layer: 200m |

- Layout 1: All micro IAB nodes are IAB-MTs and study will investigate at least interference on UL to the macro gNB receivers.

- Layout 2: IAB nodes transmitting BH in UL slots or DL slots

For Victim system

- Legacy Rel.15 NR (no IAB nodes) : All micro IAB nodes are IAB-MTs and study will investigate at least interference on UL to the macro gNB receivers

- Same layout as aggressor with IAB nodes deployed: study will investigate at least IAB-MTs DL reception

6.1.2 Scenarios for co-existence study

There are two scenarios considered in co-existence study as shown in table 6.1.2-1.

Table 6.1.2-1: IAB co-existence scenarios

| Scenario | Details |
| --- | --- |
| Scenario 1 | In case of TDM operation the following mapping of IAB node transmission/reception to a TDD pattern (DL/UL/F resources) is assumed: - IAB DU transmission / IAB MT reception: DL time slots - IAB MT transmission / IAB DU reception: UL time slots |
| Scenario 2 for Layout 2 Only | In case of TDM operation the following mapping of IAB node transmission/reception to a TDD pattern (DL/UL/F resources) is assumed: - IAB DU transmission / IAB MT reception: DL time slots - IAB MT transmission / IAB DU reception: DL time slots |

The following half duplex constraints will apply to all the IAB nodes of the network:

- MT and DU operations (resource usage) are TDM-ed at each site

- Each site is only Tx-ing or Rx-ing on different interfaces – Layout 2 and Scenario 2

- DU is able to support multiple cells (and therefore multiple sectors) only if the cells are synchronized, i.e. different sectors of the same site cannot be simultaneously transmitting and receiving.

For IAB operation mode it is agreed as table 6.1.2-2, 6.1.2-3 and 6.1.2-4, with the same TDD pattern is assumed between two adjacent channel operators.

Table 6.1.2-2: Aggressor: IAB Network, Victim: NR Network

| Layout | Scenario | Aggressor system | Victim system |
| --- | --- | --- | --- |
| 1 | 1 | UL: (IAB-MT) -> (IAB-Donor) | UL: UE -> gNB |
| 2 | 1 | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) | UL: UE -> gNB |
|  |  | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE | DL: gNB->UE |
|  | 2 | DL: (IAB-MT) ->(IAB-DU/IAB-Donor)& DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) | DL: gNB->UE |
| Assumption for all scenarios |  | 1 MT and 3 DUs per site (1 DU per sector) |  |

Table 6.1.2-3: Aggressor: NR Network, Victim: IAB Network

| Layout | Scenario | Aggressor system | Victim system |
| --- | --- | --- | --- |
| 1 | 1 | UL: UE -> gNB | UL: (IAB-MT) -> (IAB-Donor) |
|  |  | DL: gNB->UE | DL: (IAB-Donor)->(IAB-MT) |
| 2 | 1 | UL: UE -> gNB | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) |
|  |  | DL: gNB->UE | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE |
|  | 2 | DL: gNB->UE | DL: (IAB-MT) ->(IAB-DU/IAB-Donor) & DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) |
| Assumption for all scenarios |  | 1 MT and 3 DUs per site (1 DU per sector) |  |

Table 6.1.2-4: Aggressor: NR Network, Victim: IAB Network

| Layout | Scenario | Aggressor system | Victim system |
| --- | --- | --- | --- |
| 1 | 1 | UL: (IAB-MT) -> (IAB-Donor) | UL: (IAB-MT) -> (IAB-Donor) |
| 2 | 1 | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) | UL: (IAB-MT) & UE ->(IAB-DU/IAB-Donor) |
|  |  | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE | DL: (IAB-DU/IAB-Donor)->(IAB-MT) & UE |
|  | 2 | DL: (IAB-MT) ->(IAB-DU/IAB-Donor)& DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) | DL: (IAB-MT) ->(IAB-DU/IAB-Donor)& DL: (IAB-DU/IAB-Donor)->(IAB-MT & UE) |
| Assumption for all scenarios: |  | 1 MT and 3 DUs per site (1 DU per sector) |  |

6.1.3	Co-location

An IAB node is capable of transmitting in the DL (IAB-DU) or the UL (IAB-MT). When acting as an IAB-MT there are 2 possible co-location interference scenarios between the IAB-MT and a BS.

- Aggressor IAB-MT transmitting in UL, victim BS receiving in UL

- Aggressor BS transmitting in DL, victim IAB-MT receiving in DL

For co-location, the interference is given by:

Pinterference =10*log1010PACLR10+10PACS10

Where:

PACLR = Ptx_aggressor – ACLRaggressor – coupling

PACS = Ptx_aggressor – ACSvictim – coupling

A conservative estimate for the coupling between two co-located systems is; 30Db for FR1 and 45Db for FR2.

Note: this figure is used only for this analysis. It is not an agreed FR2 isolation figure.

For a micro BS scenario:

Table 6.1.3-1: Co-location interference between BS and IAB-MT for FR1 and FR2

| Parameters |  | IAB |  | BS |  |
| --- | --- | --- | --- | --- | --- |
|  | unit | FR1 | FR2 | FR1 | FR2 |
| Ptx | dBm | 30 | 30 | 33 | 33 |
| ACLR | dB | 45 (Note1) | 28 (Note1) | 45 | 28 |
| Sensitivity (FR2 approx. equivalent conducted sensitivity) | dBm | -96.5 (4.5MHz) (Note2) | approx. -85 (50MHz) (Note2) | -96.5 (4.5MHz) | approx. -85 (50MHz) |
| ACS | dB | 45 | 24 | 45 | 24 |
| Coupling | dB | 30 | 45 (Note3) | 30 | 45 (Note3) |
| IAB to BS interference (UL) | dBm | - | - | -42.0 | -37.5 |
| BS to IAB interference (DL) | dBm | -41.9 | -34.5 | - | - |
| Note1: the ACLR figures used are BS values, it has not been agreed to use BS figure for IAB, and however UE figures will result in worse interference. Note 2: sensitivity values based on NF assumption in co-location simulation see sub-clause 8.2 and 10.2. Note 3: coupling figures for FR2 are not formally agreed, assumption used only for this example |  |  |  |  |  |

Note for FR2 there are no conducted requirements so the coupling and the sensitivity are estimated to a virtual conducted point for the purposes of comparison.

It can be seen that for both FR1 and FR2 significant additional isolation (50 to 60Db) is required if the systems are to be co-located.

The issue exists for both scenario 1 and scenario 2 (see sub-clause 6.1.2) as it occurs in both the UL and the DL.

6.2	Simulation assumption

6.2.1 Propagation model

RAN4 study on IAB co-existence reuse the pathloss model between parent/donor IAB node DU and child IAB node MT agreed in RAN1 in TR 38.874.

The path loss for links between the IAB-node and candidate serving IAB-nodes/donors is determined based on N independent large-scale channel realizations (taking into account LOS/NLOS probability and shadow fading). The realization that results in the minimum pathloss between the IAB-node and the associated serving IAB-node/donor is selected.

- N=3 intra-operator serving cell

- N=1 for all others

6.2.2	Antenna configuration

6.2.2.1	General

Since some parameters required by the array antenna model are not independent, arbitrary parameter values are not supported. If parameters are selected arbitrary the model will produce incorrect gain characteristics.

We define arrays using the number of columns, rows, the separation between them as well as the definition of the element radiation pattern and its gain.

Clearly the element cannot be physically larger than the space between the elements. The element beam width parameters are directly related to the available unit area for the element. Also, the element gain is directly related to the element directivity via the selected beam widths. Therefore, parameters for GEmax, 3Db and 3Db cannot be selected arbitrary.

The array spacing, the element gain and the element beam width must therefore all be aligned.

The element peak gain will be determined by the available physical area as:

GE,max≤10log104πAλ2-LE

, where A is the area available for a single element. The area can be expressed as:

A=dhdv

Also, the maximum achieved peak element gain for given wide symmetrical beam with can be expressed as [3]:

GE,max≈10log1052525φ3dBθ3dB-LE

6.2.2.2	FR1

The FR1 antenna is defined as:

Table 6.2.2.2-1 FR1 IAB antenna model for macro scenario

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in Db | AAθ,φ=AE"θ,φ+10log101+ρ*n=1Nm=1Mwn,m*vn,m2-1 the steering matrix components are given by vn,m=expi*2πm-1*dMλcosθ+n-1*dNλsinθsinφ n=1,2,…N, m=1,2,…M the weighting factor is given by wn,m=1NMexp1*2πm-1dMλsinθ-n-1dNλcosθsinφ n=1,2,…N, m=1,2,…M |
| Antenna element vertical radiation pattern (Db) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=65°,SLAV=30dB |
| Antenna element horizontal radiation pattern (Db) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (Db) | AE''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| Antenna loss /Efficiency | 1.8 Db |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 8, 1) Note 1,2 |
| (dv, dh) | (0.8λ, 0.5λ) |
| Mechanical down tilt | 10° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

The element spacing is and hence the maximum element size is 0.8λ, 0.5λ, this corresponds to an element gain or approx.:

GANT_element≈10*log104π*dv,*dhλ2-Loss≈10*log104π*0.8λ,*0.5λλ2-1.8≈5dBi

The radiation pattern for the 0.8λ, 0.5λ element has a beam width of approx. 65° in elevation and 130° in azimuth.

6.2.2.3 FR2

The FR2 BS antenna is defined as:

Table 6.2.2.3-1. FR2 IAB antenna model for macro scenario

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in Db | AAθ,φ=AE"θ,φ+10log101+ρ*n=1Nm=1Mwn,m*vn,m2-1 the steering matrix components are given by vn,m=expi*2πm-1*dMλcosθ+n-1*dNλsinθsinφ n=1,2,…N, m=1,2,…M the weighting factor is given by wn,m=1NMexp1*2πm-1dMλsinθ-n-1dNλcosθsinφ n=1,2,…N, m=1,2,…M |
| Antenna element vertical radiation pattern (Db) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=30dB |
| Antenna element horizontal radiation pattern (Db) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=30dB |
| Combining method for 3D antenna element pattern (Db) | A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8Db loss) |
| Antenna loss /Efficiency | 1.8 Db |
| BS antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 1) Note 1,2 |
| (dv, dh) | (0.5λ, 0.5λ) |
| Mechanical down tilt | 10° |
| Note 1: Mg = number of antenna panels in elevation, Ng – number of antenna panels in azimuth, M = number of antenna elements/subarrays in elevation, N= number of antenna elements/subarrays in azimuth, P = number of polarizations. Note 2: single polarization simulated under the assumption of polarization match. |  |

In this case the element spacing is and hence the maximum element size is 0.5λ, 0.5λ, this corresponds to an element gain or approx.:

GANT_element≈10*log104π*dv,*dhλ2-Loss≈10*log104π*0.5λ,*0.5λλ2-1.8≈3dBi

The radiation pattern for the 0.5λ, 0.5λ element has a beam width of approx. 130° in elevation and 130° in azimuth.

The UE antenna is defined as:

Table 6.2.2.3-2. FR2 UE antenna model

| Parameter | Values |
| --- | --- |
| Composite Array radiation pattern in Db | AAθ,φ=AE"θ,φ+10log101+ρ*n=1Nm=1Mwn,m*vn,m2-1 the steering matrix components are given by vn,m=expi*2πm-1*dMλcosθ+n-1*dNλsinθsinφ n=1,2,…N, m=1,2,…M the weighting factor is given by wn,m=1NMexp1*2πm-1dMλsinθ-n-1dNλcosθsinφ n=1,2,…N, m=1,2,…M |
| Antenna element vertical radiation pattern (Db) | AE,Vθ"=-min12θ"-90°θ3dB2,SLAV,θ3dB=130°,SLAV=25dB |
| Antenna element horizontal radiation pattern (Db) | AE,Hφ"=-min12φ"φ3dB2,Am,φ3dB=130°,Am=25dB |
| Combining method for 3D antenna element pattern (Db) | A''θ'',φ''=-min-AE,Vθ''+AE,Hφ'',Am |
| Maximum directional gain of an antenna element GE,max | 3 dBi (assuming 1.8Db loss) |
| Antenna loss /Efficiency | 1.8 Db |
| UE antenna configuration | (Mg, Ng, M, N, P) = (1, 1, 2, 2, 1) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

The element definition is the same as that of the BS but the array is smaller.

By combining the element and array patterns this gives a composite gain of:

GANT_composite≈10*log104π*dv,*dhλ2-Loss≈10*log10M*N+GANTelement6+3≈9dBi

6.2.3 Other simulation assumption

The remaining simulation assumptions such as link level assumption, system level assumption, and simulation methodology are captured in this sub-clause.

Table 6.2.3-1: Link level assumptions

| Parameter | Details |
| --- | --- |
| Target SNR | IAB node-MT: - SNR target: 22dB [upper limit of shannon curve] - γ = 1 Legacy NR UE: - SNR target: 15 dB - γ = 1 |
| Power control | MT for UL transmissions: Yes DU for DL transmission: No |
| Throughput mapping | Map SINR into throughput with the shannon equation |

Table 6.2.3-2: System level assumptions

| Parameter | Details |
| --- | --- |
| Duplex mode | TDD |
| Frequency range | FR1: 4.9GHz – FR2: 30GHz |
| Beamforming | FR1: Yes – FR2: Yes |
| Simulation bandwidth | 100MHz for FR1 200MHz for FR2 |
| Number of UEs in the network | FR2: 1 active UE/sector FR1: 3 active UEs/sector |
| gNB Tx power | 33 dBm for FR2 macro and micro 46 dBm for FR1 |
| IAB node Tx power | 33 dBm for FR2, PC is TBD for Scenario 2 for MT link 38dBm for FR1 (medium range power limit) |
| IAB MT min TX power | -10dBm, 0dBm, 10dBm, 20dBm TRP |
| gNB antenna height | 25m for macro cells and 10m for micro cells |
| IAB node antenna height | 25m for macro cells and 10m for micro cells |
| gNB receiver noise figure | 10dB for FR2 5dB for FR1 |
| IAB node receiver noise figure | 10dB for FR2 5dB for FR1 |
| UE Tx power (dBm) | FR2: 22.4dBm EIRP (13.4dBm conducted) FR1: 23dBm (conducted) |
| UE noise figure (dB) | 10 |

Table 6.2.3-3: Simulation methodology

| Parameter | Details |
| --- | --- |
| Layout 1 | Optimum orientation between parent and child |
| Layout 2 | Antenna orientation based on planned macro layout |
| Topology | based on RSRP (based on pathloss and element antenna gain) |
| Activity factor | Up to company |

6.3	Simulation result

Simulation result provided for IAB co-existence study is summarized in Annex A.

6.4	Summary on co-existence study

<Text will be added>

7	Conducted transmitter characteristics

7.1	General

Conducted transmitter characteristics were defined for IAB-MT type 1-H and IAB-DU type 1-H. Requirements were agreed not to be defined for type 1-C, as type 1-H can also support only one TAB connector. Another reason is that, the IAB expects directional antennas, therefore, only AAS types were considered. The general principles of type 1-H requirements are the same as defined for NR base stations, i.e. full set of transceiver units are used in evaluating the conducted transmitter characteristics, unless stated otherwise, and when calculating conducted TX emission limits, the requirements defined as basic limits can be scaled up to 8x depending on the number of active transmitter units and number of cells.

7.2	IAB output power

IAB output power limits are defined for each IAB-DU and IAB-MT class. For each class the rated power per carrier at TAB connector and sum of rated power per carrier over all TAB connectors need to be aligned with specified limits.

For IAB-DU the limits are exactly the same as defined for NR BS type 1-H.

For IAB-MT only two classes, local area and wide area IAB-MT, were agreed to be specified. The IAB-MT output power limits for local area IAB-MT are the same as defined for local area IAB-DU and limits for wide area IAB-MT are the same as defined for wide area IAB-DU. Therefore, no upper power limit is specified for wide area IAB-MT.

7.3	Output power dynamics

7.3.1	Power control

The power control requirements are not defined to the wide area IAB-MT, as the tolerances for power control are large compared to the minimum requirement for dynamic range.

For the local area IAB-MT, the absolute power tolerance was not defined as the dynamic range defined for the local area IAB-MT is not large enough compared to the tolerance value from UE requirement.

The relative and aggregate power tolerance requirements are taken into use with the requirement values to be modified compared to the UE specification.

7.4	Transmit ON/OFF power

Transmit ON/OFF power requirements include two parts: transmitter off power and transmitter transient period. For IAB-DU, it’s straightforward to reuse BS’s both requirements because IAB-DU behaviour is the same as BS.

For IAB-MT, reusing BS or UE requirements for both OFF power and transient period are the candidates. Comparing BS OFF power and UE OFF power requirements, BS FR1 requirements are more stringent than UE but FR2 visa verse. It is unlikely there will be many IAB nodes in close proximity so a relaxed requirement compared to UEs should not cause a rise in overall interference in the system. Therefore, IAB-MT can reuse BS off power. Regarding transient period requirements, the general profiles for BS and UE are similar but FR2 BS transient period length is shorter than UE. UE has many specific profiles requirements to explicitly define the transient period location for different physical layers and different test cases. As IAB-MT implementation has less restriction than commercial UE, it is expected that IAB-MT can meet BS more stringent transient period length requirements. Thus IAB-MT general profile requirements can reuse BS requirements. For the UE specific profiles, they are not needed for IAB-MT because IAB-MT should know UE behaviour when different physical channels are transmitted.

Based the above analysis, both IAB-DU and IAB-MT reuse BS ON/OFF power requirements including the OFF power and transient period.

7.5	Transmitted signal quality

7.5.1 IAB-DU transmitted signal quality

As the IAB-DU behavior is very similar with BS, all of the transmitted signal quality requirements can be imported from BS. Therefore, the frequency error, modulation quality and time alignment error requirements in clause 6.5.1, 6.5.2, 6.5.3 for BS type 1-H in TS 38.104 [TBD] apply to IAB-DU type 1-H.

7.5.2.1 Frequency error

As IAB-MT function is more like a UE, UE frequency error correction can be a reference. When BS transmits DL signal to UE, UE does frequency error correction algorithm to make sure UE follows BS with a relative low residual frequency error.The residual frequency error after compensation should be less than one percent of the subcarrier interval.

If 15KHz SCS is used, one percent is 150 Hz which is 0.1 ppm for 1.5 GHz and less than 0.1 ppm of the higher carrier frequency. For higher modulation such as 256 QAM, the residual frequency error should be much smaller. Therefore, in order to support high modulation, UE frequency offset correction algorithm should make the residual frequency error less than +/- 0.1ppm. The UE frequency offset correction algorithm follows BS carrier frequency through DL signals that’s why the UE frequency error should be defined as relative frequency error not absolute frequency error.

When IAB-MT receives BS DL signal, the similar frequency error correction algorithm should be used to make high modulation support possible. IAB-MT frequency error requirement should reuse UE requirement which is defined as relative requirement as +/-0.1 PPM relative to received signal from parent node.

Therefore, IAB-MT frequency error requirement is defined to reuse UE requirements as ± 0.1 PPM compared to the carrier frequency received from the parent node

7.5.2.2 Error Vector Magnitude

EVM performance is the SNR performance of the transmitted signal. In order to have the same link performance, IAB-MT output signal quality should have the same performance as UE then guarantee the link quality. UE requirements are reused by IAB-MT EVM requirements. The difference is that BPSK EVM requirement is removed considering BPSK modulation is not likely to be used by the backhaul link.

7.6	Unwanted emissions

Unwanted emissions cover the definitions and requirements for OOB boundary, occupied bandwidth, ACLR including also absolute ACLR, OBUE, and spurious emissions.

For IAB-DU type 1-H all unwanted emissions requirements, except for the protection of the BS receiver of own or different BS, are the same as specified for NR BS type 1-H. The background for these requirements can be found from TR 38.817-02 [X]. The requirement for the own or other receiver is not specified, as for NR BS the requirement applies only for FDD operation and no FDD band is defined for IAB.

All the requirements are the same for IAB-MT and IAB-DU based on class so that wide area IAB-MT requirements were agreed to be the same as defined for wide area IAB-DU and requirements for local area IAB-MT were agreed to be the same as defined for local area IAB-DU.

7.7	Transmitter intermodulation

For conducted transmitter intermodulation, it is agreed to reuse the base station framework for both IAB-DU and IAB-MT.

8	Conducted receiver characteristics

8.1	General

<Text will be added>

8.2	Reference sensitivity level

8.2.1	IAB-DU Reference sensitivity

The IAB-DU conducted reference sensitivity requirement is the same as the BS conducted sensitivity requirement.

8.2.2	IAB-MT Reference sensitivity

The IAB-MT uses similar assumptions for antenna architecture and gain as the IAB-DU, it is also assumed that the front end HW is similar and has the same NF.

As such the IAB-MT reference sensitivity will be derived using the same assumptions as the BS. The IAB-MT sensitivity is given by:

Where:

-	BW is the maximum transmission bandwidth for the FRC

-	NF is the noise figure

-	IM is the implementation margin.

-	SNR is the SNR value for which we reach 95% throughput. Each company provided simulation results, and average will be done for each BW.

The NF and the IM margin are hardware dependent and taken from the BS:

NF = 5dB for wide area IAB-MT and 13dB for local area IAB-MT

IM is 2dB

As the IAB-MT operated on the DL the FRC’s and the associated SNR requirements must be based on the UE FRCs. There are many more FRC’s for the UE for each of the channel BW’s however it is sufficient to specify a limited number of FRC’s in the same way as the BS. For each BS FRC there is a UE FRC of the same transmission BW and hence these can be used for the IAB-MT as shown in table 8.2.2-1.

The UE FRC definition is more complex than the BS and includes some parameters which require communication between eth UE and the BS test emulator. The method for conformance for the IAB-MT has not yet been agreed and hence the definition of the FRC is simplified in order to avoid any test implications. As such we keep the MCS, PRB allocation, SCS and CHBW information in FRC for core requirements; further discuss other detailed parameters in conformance phase

In addition for the current IAB FR1 bands only TDD are specified so there is no need for 15 kHz SCS at this release.

Table 8.2.2-1: FRC’s for the FR1 IAB-MT

| BS Reference channel | equivalent UE reference channel (TS 38.101-1 [3], Annex A3.3) |
| --- | --- |
| G-FR1-A1-2 | Table A.3.3.2-2, 5MHz CBW |
| G-FR1-A1-3 | Table A.3.3.2-3, 10MHz CBW |
| G-FR1-A1-5 | Table A.3.3.2-2, 20MHz CBW |
| G-FR1-A1-6 | Table A.3.3.2-3, 20MHz CBW |

The SNR for the BS varies between -0.8dB to -1.2dB for each of the BS FRC’s, however the UE uses a figure of -1dB for all FRC’s. As the SNR is dependent on the modulation the UE figure is used.

Applying these number to the equation gives:

Table 8.2.2-2: NR Wide Area IAB-MT reference sensitivity levels

| IAB-MT channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | signal BW | IM | SNR | NF | IAB-MT reference sensitivity power level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | dB | dB | dB | dBm |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | 3.96 | 2 | -1 | 5 | -102.0 |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | 7.92 | 2 | -1 | 5 | -99.0 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | 18.36 | 2 | -1 | 5 | -95.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | 17.28 | 2 | -1 | 5 | -95.6 |

Table 8.2.2-3: NR Local Area IAB-MT reference sensitivity levels

| IAB-MT channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | signal BW | IM | SNR | NF | IAB-MT reference sensitivity power level |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MHz | dB | dB | dB | dBm |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | 3.96 | 2 | -1 | 13 | -94.0 |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | 8.92 | 2 | -1 | 13 | -91.0 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | 18.36 | 2 | -1 | 13 | -87.4 |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | 17.28 | 2 | -1 | 13 | -87.6 |

The simplified FRCs are defined as follows:

Table 8.2.2-4: FRC parameters for FR1 reference sensitivity level for IAB-MT.

| Reference channel | G-FR1-A1-22 | G-FR1-A1-23 | G-FR1-A1-25 | G-FR1-A1-26 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 30 | 60 | 30 | 60 |
| Allocated resource blocks | 11 | 11 | 51 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 | 1/3 |
| NOTE 1: DL-DMRS-config-type = 1 with DL-DMRS-max-len = 1, DL-DMRS-add-pos = pos2 with = 2, = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size for receiver sensitivity |  |  |  |  |

8.3	Dynamic range

The IAB-DU receiver dynamic range requirement is the same as the BS.

There is no IAB-MT receiver dynamic range requirement.

8.4	In-band selectivity and blocking

Detailed structure of the subclause is TBD.

8.5	Out-of-band blocking

Detailed structure of the subclause is TBD.

8.6	Receiver spurious emissions

Detailed structure of the subclause is TBD.

8.6.1 IAB-MT Receiver spurious for IAB type 1-H

To protect the coexisting service, the spurious level generated from receiver should be limited. For example, for the synchronized protected service, this could be the case of protecting IAB-MT receiving from IAB node#1 as shown in Figure 8.6-1. The RX spurious generated from IAB-MT receiver at IAB node #2 will impact the UE receiving camped to the parent IAB node#1 cell but not on the UE camped to IAB-DU cell of IAB node#2. This is because IAB-DU and IAB-MT will be operated in TDM manner and thus there is no IAB-DU transmitting in IAB node#2 when IAB-MT receiver is enabled. Compared with the traditional coexisting distance of 5m for outdoor deployment for UE to UE interference analysis, the distance between IAB-MT to victim UE at parent IAB cell is larger as it will be installed at site similar to BS. Therefore, there is not necessary to reuse the UE Rx spurious requirement.

For the unsynchronized co-existing service, IAB-MT spurious level does not need to be tighter than IAB-DU receiver spurious requirement.

From protection of both synchronized or unsynchronized coexisting service, the IAB-DU receiver spurious for receiver spurious could be reused for IAB-MT.

Figure 8.6-1: coexisting service interference when IAB-MT receiving from parent IAB

8.7	Receiver intermodulation

For IAB-MT type 1-H, as IBB requirement is agreed to reuse BS requirements and interfering signal power of RX IMD requirement is close to that of IBB requirements in general, therefore it’s agreed to reuse BS RX IMD requirement for IAB-MT.

8.8	In-channel selectivity

For IAB-MT, similar as legacy NR UE, there is no RX in-channel selectivity requirement defined in TS 38.101, therefore it is agreed that no in-channel selectivity will be defined for IAB-MT.

9	Radiated transmitter characteristics

9.1	General

Radiated transmitter characteristics were defined for IAB-MT and IAB-DU types 1-O and 2-O. Additionally IAB-MT and IAB-DU type 1-H needs to meet the radiated transmit power requirement. IAB-DU type 1-O and type 2-O requirements are the same as defined for NR BS. For IAB-MT type 1-O no minimum number of transceivers have been defined, allowing type 1-O IAB-MT to have any number of transceivers. Therefore, IAB-MT type 1-O requirements are defined using basic limits and scaling of basic limits following the same principles as type 1-H requirements.

9.2	Radiated transmit power

For configured maximum power it was discussed how the UE requirement is adapted to fit the characteristic of IAB-MT. The factors like MPR/A-MPR are included in the UE requirements, but they are not specified for the IAB-MT. Therefore, they do not need to be defined for the IAB-MT PCMAX. Similarly, as IAB-MT Tx power is declared by the manufacturer using same framework as BS Tx power declaration, hence power class related factors PPowerClass and ΔPPowerClass are not included in PCMAX definition. Other factors like PEMAX,c, and the factors related to Interband CA, SUL, and SRS are not included in the PCMAX definition.

The PCMAX requirement was agreed to be aligned with the output power declaration, which includes also the declared back off power. Using TRP or EIRP was discussed, and EIRP was agreed to be used, as it is defined for all IAB-MT types and the relevant metric for link budget.

Radiated transmit power is the EIRP level for a declared beam at a specified beam peak direction. For each declared beam, the manufacturer declared EIRP level needs to be achieved within a specified accuracy.

9.3	IAB OTA output power

IAB OTA output power is a TRP requirement, defined for a RIB. For IAB-MT and IAB-DU type 1-O, the TRP limits are different for each class. IAB-DU type 1-O limits were defined to be the same as for NR BS. For local area IAB-MT, due to no minimum required number of transmitters, the rater carrier TRP limits can be scaled from 24 dBm to 33 dBm, depending on the number of active transmitter units. No upper limit for OTA output power is specified for wide area IAB-MT and IAB-DU.

For IAB-MT and IAB-DU type 2-O no absolute power limits are associated with IAB OTA output power requirements and only accuracy requirement towards to declared value is specified.

9.4	OTA output power dynamics

Detailed structure of the subclause is TBD.

9.5	OTA transmit ON/OFF power

According to the analysis in 7.4, both IAB-DU and IAB-MT reuse BS radiated ON/OFF power requirements including the OFF power and transient period.

9.6	OTA transmitted signal quality

9.6.1 IAB-DU OTA transmitted signal quality

As the IAB-DU behavior is very similar with BS, all of the transmitted signal quality requirements can be imported from BS. Therefore, the frequency error, modulation quality and time alignment error requirements in clause 9.6.1, 9.6.2 and 9.6.3 for BS type 1-O and  type 2-O in TS 38.104 [TBD] apply to IAB-DU type 1-O and type 2-O respectively.

9.6.2.1 Frequency error

IAB-MT OTA transmitted signal quality requirement analysis is the same as the conducted requirements in 7.5.2.1 that IAB-MT should follow parent node’s center frequency. IAB-MT type 1-O and type 2-O OTA frequency error reuses UE requirements to be +/-0.1 PPM relative to received signal from parent node.

9.6.2.2 Error Vector Magnitude

IAB-MT OTA EVM requirement analysis is the same as the conducted requirement in 7.5.2.2. IAB-MT type1-O EVM requirements should be the same with conducted requirements. IAB-MT type2-O EVM requirement reuses UE FR2 EVM requirements with the exception that BPSK requirement is removed.

9.7	OTA unwanted emissions

OTA Unwanted emissions cover the definitions and requirements for OOB boundary, occupied bandwidth, ACLR including also absolute ACLR, OBUE, and spurious emissions.

For IAB-DU all unwanted emissions requirements, except for the protection of the BS receiver of own or different BS, are the same as specified for NR BS. The background for these requirements can be found from TR 38.817-02 [X]. The requirement for the own or other receiver is not specified, as for NR BS the requirement applies only for FDD operation and no FDD band is defined for IAB.

Wide area IAB-MT requirements were agreed to be the same as defined for wide area IAB-DU both in FR1 and FR2. In FR1 also local area IAB-MT requirements were agreed to be the same as defined for local area IAB-DU but in FR2 it was agreed that 24 dBc ACLR is sufficient, otherwise requirements for local area IAB-MT were agreed to be the same as defined for local area IAB-DU.

9.8	OTA transmitter intermodulation

9.8.1 IAB DU intermodulation

For FR1 the IAB node could be co-located if the IAB TDD pattern for transmission and receiving is the same for both IAB-DU and IAB-MT of both co-located nodes, i.e. just coordinating the UL/DL timeslots is not sufficient but the actual Tx and Rx time instants of both IAB-DU and IAB-MT need to be aligned. IAB DU intermodulation requirement shall be reused from BS spec.

Therefore IAB-DU type 1-O will reuse the framework of OTA TX intermodulation requirement defined in TS 38.104 section 9.8 for NR BS type 1-O.

There is no OTA TX intermodulation requirement for IAB-DU type 2-O as there is no TX IMD requirement for BS type 2-O.

    - 9.8.2 IAB MT intermodulation

The co-location scenario of IAB is listed below as:

-co-located with another IAB-Node in same band

This scenario shall be further considered as if the IAB-DU and IAB-MT of one IAB-Node don’t transmit simultaneously, there will exist the scenario that during UL timeslot one is transmitting in the backhaul link and the other is receiving in the access link, and correspondingly during DL timeslot one can be transmitting in the access link and another is receiving in the backhaul link. In this case, the excessive interference will prevent the two IAB nodes to be co-located. Hence co-location scenario can only be considered for the following case:

-co-located as IAB-DU and IAB-MT of one IAB-Node transmit simultaneously in the same band with same TDD pattern as 	the other co-located BS and/or IAB-Node.

- In case of two co-located IAB-Nodes, all IAB-MTs and IAB-Dus need to follow same TDD pattern.

In this case, the co-location scenario is similar to a base station co-located with another base station in same band, hence the intermodulation requirement shall be reused from BS spec.

-co-located with another IAB-DU/BS in another band

With the above co-location scenarios, it is proposed to use the framework of base stationTX intermodulation requirement for IAB MT with certain co-location scenarios. Considering the large coupling loss, there is no TX IMD requirement for IAB-MT type 2-O.

9.9	Beam correspondence for IAB-MT

For IAB-MT, it is agreed no explicitly RF core requirement will be defined for beam correspondence.

10	Radiated receiver characteristics

10.1	General

10.2	OTA sensitivity

10.2.1	IAB-DU OTA sensitivity

The IAB-DU OTA sensitivity for IAB-DU type 1-H and 1-O is the same as the BS.

There is no OTA sensitivity requirement for IAB-DU type 2-O.

10.2.2	IAB-MT OTA sensitivity

As with the conducted reference sensitivity requirements as the IAB-MT antenna and receiver front end is similar to the BS the BS OTA sensitivity requirement is derived using the same assumptions as the BS.

The FR1 OTA sensitivity requirement is declared over a declared RoAoA without any boundaries and is used as both a minimum sensitivity requirement and reference level for some of the interference requirements (ACS, in-band blocking, oob blocking, colocations blocking, RX IMD) as such the requirement is almost the same as the BS requirement (with the same declarations required from TS 38.141-2 [6]). However as the IAB-MT receives the DL the FRC’s used are from the UE specification as described in table 8.2.2-1.

There is no OTA sensitivity requirement for IAB-MT type 2-O.

10.3	OTA reference sensitivity level

10.3.1	IAB-DU OTA reference sensitivity

The IAB-DU OTA sensitivity for IAB-DU type 1-H, 2-O and 2-O is the same as the BS.

10.3.2	IAB-MT OTA reference sensitivity

10.3.2.1	FR1

The IAB-MT antenna and front end is similar to that of a BS so the OTA reference sensitivity requirements will be based upon the same assumptions as the BS. As with the other IAB-MT sensitivity requirements however the FRC’s and the associated SNR requirement will be taken from the UE. The UE FRC’s will be of the same signal BW as the associated BS FRC’s for the requirement.

The FR1 OTA reference sensitivity level is calculated based on the required equivalent passive antenna gain if a receiver were to meet the conducted reference sensitivity requirements and cover a declared OTA reference sensitivity RoAoA (see TS 38.141-2 [6]).

The FR1 IAB-MT reference sensitivity therefore uses the same set of declarations as the BS, however the calculation is modified to correspond to the DL FRC’s and their associated SNR values. The OTA reference sensitivities are hence offset from the conducted reference sensitivity values given in tables 8.2.2-1 and 8.2.2-2 as shown in tables 10.3.2.1-1 and 10.3.2.1-2.

Table 10.3.2-1: Wide Area IAB-MT reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | -102.0 - ΔOTAREFSENS |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | -99.0 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | -95.4 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | -95.6 - ΔOTAREFSENS |

Table 10.3.2-2: Local Area IAB-MT reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel Ref TS 38.101-1 [3] Annex A | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 30 | Table A.3.3.2-2, 5MHz CBW | -94.0 - ΔOTAREFSENS |
| 10, 15 | 60 | Table A.3.3.2-3, 10MHz CBW | -91.0 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | Table A.3.3.2-2, 20MHz CBW | -87.4 - ΔOTAREFSENS |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | Table A.3.3.2-3, 20MHz CBW | -87.6 - ΔOTAREFSENS |

The FRCs are the same as those defined for conducted reference sensitivity in sub-clause 8.2.2.

10.3.2.1	FR2

The FR2 IAB-MT antenna and front end is similar to that of a BS so the OTA reference sensitivity requirements will be based upon the same assumptions as the BS.The BS FR2 OTA reference sensitivity level is based on a declared EIS value within a specified range, the range is calculated based on the useful range of antenna gains expected for the BS class. And is calculated as follows:

Hence for a wide area BS

Where: BW is the noise BW of the FRC, NF is the noise figure, IM is implantation margin not related to antenna array, SNR is the required SNR for demodulation and G is the antenna gain and RF losses.

The expected gain range is the same as the BS, however as there is no medium range IAB-MT the local area gain range is expanded to cover the medium range also:

Table 10.3.2.1-1: G assumptions for calculating FR2 WA and LA OTA REFSENS range

| IAB-MT class | G |  |
| --- | --- | --- |
|  | 30 GHz (24.25 – 33.4 GHz) | 45GHz (37 – 52.6 GHz) |
| WA | 10 to 33 dBi | 12 to 35 dBi |
| LA | 0 to 28 dBi | 2 to 30 dBi |

The FRC’s and the associated SNR values are also based on the Dl values used for the UE. There are UE FRC’s defined which have the same signal BW as the BS FRC’s as shown un table 10.3.2.1-2

Table 10.3.2.1-2 FR2 equivalent IAB-MT FRC’s

| BS Reference channel | equivalent UE reference channel (TS 38.101-2, Annex A3.3) |
| --- | --- |
| G-FR2-A1-1 | Table A.3.3.2-1, 50MHz CBW |
| G-FR2-A1-2 | Table A.3.3.2-2, 50MHz CBW |
| G-FR2-A1-3 | Table A.3.3.2-3, 100MHz CBW |

The SNR for the BS FRCs range from -1.1 to -1.2 dB , the UE SNR assumption is -1dB, however as the declaration ranges and declared values are rounded to integer values this makes no difference to the final range. As such the FR2 IAB-MT range is the same as the BS, as follows:

For Wide Area IAB-MT, EISREFSENS_50M is an integer value in the range -96 to -119 dBm. The specific value is declared by the vendor.

For Local Area IAB-MT, EISREFSENS_50M is an integer value in the range -86 to -114 dBm. The specific value is declared by the vendor.

The FR2 IAB-MT FRC definitions in the core specification in a similar way to the FR1 RFCs as explained in subclause 8.2.2. The simplified FR2 FRCs are defined as follows:

Table 10.3.2.1-3: FRC parameters for FR2 reference sensitivity level for IAB-MT.

| Reference channel | G-FR2-A1-21 | G-FR2-A1-22 | G-FR2-A1-23 |
| --- | --- | --- | --- |
| Subcarrier spacing (kHz) | 60 | 120 | 120 |
| Allocated resource blocks | 66 | 32 | 66 |
| CP-OFDM Symbols per slot (Note 1) | 9 | 9 | 9 |
| Modulation | QPSK | QPSK | QPSK |
| Code rate (Note 2) | 1/3 | 1/3 | 1/3 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS, additional DM-RS position = pos2 with l0 = 2, l = 6 and 9 as per Table 7.4.1.1.2-3 of TS 38.211 [3]. NOTE 2: MCS index 4 and target coding rate = 308/1024 are adopted to calculate payload size. |  |  |  |  |

10.4	OTA Dynamic range

The IAB-DU receiver dynamic range requirement is the same as the BS.

There is no IAB-MT receiver dynamic range requirement.

10.5	OTA in-band selectivity and blocking

10.5.1 ACS for IAB MT of IAB type 2-O

The ACS requirement for IAB-MT is agreed as 24 dB in 24.25 – 33.4 GHz BS and 23 dB in 37 – 52.6 GHz. IAB-MT is expected to have better adjacent carrier selectivity than UE ACS due to the limited physical separation distance to the aggressor BS. Generically BS has higher ACS selectivity compared to UE thus BS type of ACS number can be used for IAB-MT.

10.5.2 In-band blocking for IAB-MT

Traditionally for UTRA the in-band blocker is set according to the 99.99% probability of interferer cdf curve. It is recommended in [7] that between 99% and 99.9% could be considered due to the NR OFDMA scheme does not suffer greatly as for WCDMA.

As the interfere level to IAB-MT receiver relates to the distance between victim IAB and aggressor BS, coexisting simulation is done assuming the 40m, 50m , 60m and 80m and companies result is captured in Table 10.5.3-1.

Table 10.5.3-1: Coexisting simulation result for In-band blocking level comparison

| Company | Blocker level @99 Percentile point (dBm) | Blocker level @ 99.9 perncentile point (dBm) | Physical distance to Agressor BS (m) | Simulation scenario (Acc. to R4-1907825) | RAN4 contribution |
| --- | --- | --- | --- | --- | --- |
| Ericsson | -67.5 | -65.5 | 40 | Layout 2 for FR2 | R4-2001873 |
|  | -75 | -66 | 50 |  |  |
|  | -81 | -75 | 60 |  |  |
|  | -55 | -48 | 40 | Layout 2 for FR1 |  |
| Nokia | -52 | -45 | 40 | Layout 2 for FR2 | R4-2001432 |
|  | -55 | -54 | Max grid shift |  |  |
| QUALCOMM incorporation | -45 |  | 40 | Layout 2 for FR2 | R4-2001282 |
|  | -49 |  | 60 |  |  |
|  | -53 |  | 80 |  |  |
| Huawei | -64 |  | 40 | Layout 2 for FR2 (element OTA result) | R4-1914757 |
|  | -69 |  | 60 |  |  |
| ZTE | -42 |  | 40 | Layout 2 for FR1(element OTA result) | R4-2000977 |

It is concluded from the coexisting simulation results in Table 10.5.3-1 that IAB-MT can reuse the OTA BS blocking requirement with CP-OFDM interferer signal. Therefore, the inband blocking level for wide area IAB-MT of type 2-O is specified with 33 dB higher than OTA REFSENS power level with inteferer signal of CP-OFDM waveform. The inband blocking level for local area IAB-MT of type 2-O is specified with 35.5 higher than OTA REFSENS power level with inteferer signal of CP-OFDM waveform for bands n257, n258, n261. The inband blocking level for local area IAB-MT of type 2-O is specified with 34.5 higher than OTA REFSENS power level with CP-OFDM waveform of inteferer signal for bands n260.

10.6	OTA out-of-band blocking

Detailed structure of the subclause is TBD.

10.7	OTA receiver spurious emissions

10.7.1 IAB-MT OTA Receiver spurious for IAB type 1-O and 2-O

The UE receiver spurious requirement can be relaxed as the distance to the protected service will be increased and this follows the same analysis in chapter 8.6.1. For protecting unsynchronized coexisting service, IAB-MT OTA spurious level does not need to be tighter than IAB-DU receiver requirement and as such the BS specification for OTA Receiver spurious can be specified for IAB-MT for IAB type 1-O and 2-O.

10.8	OTA receiver intermodulation

For IAB-MT type 2-O, similar as legacy FR2 NR UE, there is no RX intermodulation requirement defined in TS 38.101-2, therefore it is agreed that no RX intermodulation is defined for FR2 IAB-MT.

For IAB-MT type 1-O similar as IAB-MT type 1-H, as IBB requirement is agreed to reuse BS requirements and interfering signal power of RX IMD requirement is close to that of IBB requirements in general, therefore it’s agreed to reuse BS RX IMD requirement for IAB-MT.

10.9	OTA in-channel selectivity

For IAB-MT, similar as legacy NR UE, there is no RX in-channel selectivity requirement defined in TS 38.101, therefore it is agreed that no in-channel selectivity will be defined for IAB-MT.

11	IAB RRM requirements

Detailed structure of the subclause is TBD.

12	IAB EMC requirements

12.1 IAB EMC Emission requirements

EMC emissions testing covers both conducted and radiated emissions.  . Test methods and levels for conducted emissions, harmonic and voltage fluctuations and flicker testing defined by IEC/CISPR are independent of the product (IAB) characteristics and features, including the operating frequency or the Radio Access Technology (RAT). It is agreed that the applicable requirements for EMC conducted emissions of IAB nodes are the ones defined for NR BS in TS 38.113.

EMC radiated emissions test involves measuring the electromagnetic field strength of the emissions that are unintentionally generated by a product (an IAB node in this case). Emissions are inherent to the switching voltages and currents within any digital circuit. In this case, the radiated emission requirements as well as the test methods that apply are defined by IEC and CISPR subcommittee in the Standard CISPR 32 [11]. Testing and limits of emissions intentionally generated by the BS are covered by ITU-R recommendations SM.329 [9] and SM.1539 [10].

3GPP has agreed that for IAB type 1-O and IAB type 2-O, the radiated emission is covered by radiated spurious emission requirement in TS 38.174 [8], conforming to the test requirement in TS 38.174 [8].

  - 12.2 IAB EMC immunity requirements

12.2.1 Radiated Immunity requirements

  - 12.2.1.1 One enclosure case

The IAB node is a network node with similar outside radio frequency electromagnetic environment as base station. So in this case, it is reasonable to apply base station test levels to the IAB node.

  - 12.2.1.2 Different enclosure case

For different enclosure cases, the radiated immunity requirement apply for each enclosure. Still the test level as reusing the base station requirement applies as the electromagnetic environment is similar to BS.

  - 12.2.1.4 Summary

The base station requirement as 3V/m from 80MHz to 6000MHz is used as a starting point and the exclusion band for Radiated immunity test is FFS.

12.2.2 Other Immunity requirements

It is agreed that similar working environment as for a base station applies to an IAB node. So it is agreed to reuse the BS requirements for IAB node, for the following immunity requirements as listed below.

- ESD test

- Fast transient common mode

- RF common mode 0.15 – 80 MHz

- Voltage dips and interruption

- Surge

13	IAB performance requirements

Detailed structure of the subclause is TBD.

14	IAB conformance testing aspect

Detailed structure of the subclause is TBD.

Annex A (normative): Co-existence simulation results

This section contains references to contributions submitted during Rel-16 IAB WI containing co-existence evaluation results for IAB mainly based on the simulation assumptions provided in Section 6. Details of the simulation parameters and observations can be found in the corresponding referenced contributions:

- R4-1908843	FR1 IAB co-existence study, CMCC

- R4-1908075	Co-existence simulation result for layout 1, Samsung

- R4-1908076	Co-existence simulation result for layout 2, Samsung

- R4-1908586	simulation results for FR1 IAB coexistence study, ZTE

- R4-1908587	simulation results for FR2 IAB coexistence study, ZTE

- R4-1908733	IAB simulation results for Layout 1 Huawei, HiSilicon

- R4-1908872	Simulation results for the homogeneous scenario in FR2, Qualcomm Incorporated

- R4-1908873	Simulation results for the heterogeneous scenario in FR2, Qualcomm Incorporated

- R4-1909272	Initial IAB-Node coexistence simulation results, Nokia, Nokia Shanghai Bell

- R4-1909389	coexistence simulation results for IAB network, Ericsson

- R4-1910838	FR1 IAB co-existence study, CMCC

- R4-1911332	     Simulation results on the minimum Tx power of IAB MT and ACIR, Huawei, HiSilicon

- R4-1911394	     Updated simulation result for layout 1, Samsung

- R4-1911395 	Updated simulation result for layout 2, Samsung

- R4-1911638		IAB-Node coexistence and dynamic range requirements, Nokia, Nokia Shanghai Bell

- R4-1911639		IAB-Node coexistence simulation results, Nokia, Nokia Shanghai Bell

- R4-1911735		simulation results for FR1 IAB coexistence study, ZTE Corporation

- R4-1911736		simulation results for FR2 IAB coexistence study, ZTE Corporation

- R4-1912891	simulation results for FR2 IAB coexistence study, ZTE Corporation

- R4-1912008	Coexistence simulation results updates for FR2 - layout 1, Ericsson

- R4-1912009	Coexistence simulation results for FR1 - layout 1,  Ericsson

- R4-1912010	Coexistence simulation results for FR2 - layout 2,  Ericsson

- R4-1912011		Coexistence simulation results for FR1 - layout 2, Ericsson

- R4-1913334	Further simulation on IAB-MT ACLR and TX min power, Samsung

- R4-1914162	Simulation results for IAB in FR2, Huawei, HiSilicon

- R4-1914163	Simulation results for IAB in FR1, Huawei, HiSilicon

- R4-1914222	Min Tx power simulation results updates- layout 1, Ericcson

- R4-1914235	simulation results for FR2 IAB coexistence study, ZTE Corporation

- R4-1914236	simulation results for FR1 IAB coexistence study, ZTE Corporation

- R4-1914260	IAB-Node coexistence simulation results, Nokia, Nokia Shanghai Bell

- R4-2000972	simulation results for FR1 IAB coexistence study, ZTE Corporation

- R4-2000973	simulation results for FR2 IAB coexistence study, ZTE Corporation

- R4-2000977	In-band blocking for FR1 IAB MT, ZTE Corporation

- R4-2000978	In-band blocking for FR2 IAB MT, ZTE Corporation

Annex B (informative): 
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-04 | RAN4#94bis-e | R4-2005458 |  |  |  | Initial TR skeleton endorsed in R4-1912889 Updated TR to incorporate below TPs R4-1916003 [IAB] TP to TR 38.xxx Co-location scenarios R4-1916166 [IAB] TP to TR 38.xxx System parameter R4-1914761 [IAB] TP to TR 38.xxx Architecture R4-2002487 TP for TR _RF Requirements reference points R4-2002488 TP for TR _Spec organization R4-2002493 [IAB] TP to TR 38.xxx Antenna assumptions R4-2002497 TP for TR _9 IAB OTA transmitter intermodulation R4-2005480 Update on section 4 and section 5 based on latest agreement R4-2003758 TP to TR 38.xyz: Addition of antenna model and parameters in sub-clause 6.2 R4-2005476 [IAB EMC] TP to TR IAB EMC immunity requirements | 0.1.0 |
| 2020-06 | RAN4#95e | R4-2006378 |  |  |  | Updated TR to incorporate below TPs approved during meeting: R4-2008768 TP to TR 38.809 – Correction of IAB-DU and IAB-MT permutation in subclause 4.8. R4-2008777 TP for TR 38.809: Transmit ON/OFF power R4-2006274 TP for TR 38.809: IAB-DU Transmitted signal quality R4-2008787 TP to TR : IAB RX IM requirement (section 8.7 and 10.8) R4-2008789 TP to TR: IAB ICS requirement (section 8.8 and 10.9) R4-2009062 [IAB RF] TP to TR 38.809 IAB TX IMD R4-2008730 TP to TR 38.809 on IAB EMC Emissions R4-2008731 [IAB EMC]TP to TR IAB EMC immunity requirements | 0.2.0 |
| 2020-08 | RAN4#96e | R4-2010223 |  |  |  | Updated TR to incorporate below TPs approved during meeting: R4-2010146 TP for TR38.809: IAB co-existence simulation R4-2012617 TP for TR 38.809: IAB-MT Pcmax and power control R4-2012619 TP to TR 38.809 Completing IAB-MT power related requirements R4-2012623 TP for TR 38.809: IAB-MT Transmit signal quality R4-2012625 TP to TR 38.809 IAB-MT unwanted emission requirements R4-2010148 TP for TR38.809: conclusion on IAB-MT BC requirement R4-2012629 TP to TR 38.809 -IAB RX sensitivity R4-2012632 TP to TR 38.809: ACS and IBB R4-2012756 TP to TR 38.809 IAB-MT RX IMD R4-2012634 TP to TR 38.809: RX spurious R4-2012637 TP to TR 38.809 on IAB EMC emission requirements | 0.3.0 |
| 2020-09 | RAN#89e | RP-201638 |  |  |  | Editorial update Present to RAN-P for approval | 1.0.0 |