3GPP TSG RAN meeting #89e	RP-201670

# Electronic Meeting, September 14-18, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1002 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding FR1 PDCCH Aggregation Level in Annex C.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-09-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This is the replacement of RAN5 agreed CR R5-204712, in which the general configuration of aggregation level is set to 4 in C.3.0/C.3.1 for all RF test cases except for ACS. It's later found that setting AL=4 will impact some TCs since there maybe collision for the PDCCH of SIB1 and PDCCH of UL_grant, so this RANP CR is submitted to undo the general changes of aggregation level. The reason for remaining changes in C.3.1 is as below: As per LS R4-2005205, the NR UE receiver ACS test requirements for PDCCH/DCI could be tighter than PDSCH due to that the PDCCH/DCI may not occupy the entire channel bandwidth where the signal power is lower than that of PDSCH as defined in ACS core specifications. The configuration of aggregation level for ACS testing needs to be specified to ensure the PDCCH performance would not become the bottleneck of ACS testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Table C.3.1-2 for PDCCH Aggregation Level which applies to only ACS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE might fail ACS testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Discussion paper is in R5-204721 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Replacement of RAN5 CR R5-204712. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

# C.3	Connection

## C.3.0	Measurement of Transmitter Characteristics

Unless otherwise stated, Table C.3.0-1 is applicable for measurements on the Transmitter Characteristics (clause 6).

Table C.3.0-1: Downlink Physical Channels transmitted during a connection (FDD and TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of PDSCH to SSS | dB | 0 |
| EPRE ratio of PDSCH to PDSCH DMRS (Note 1) | dB | -3 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of PTRS to PDSCH | dB | Test specific |
| EPRE ratio of OCNG DMRS to SSS | dB | 0 |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) | dB | 0 |
| Note 1: No boosting is applied to any of the channels except PDSCH DMRS. For PDSCH DMRS, 3 dB power boosting is applied assuming DMRS Type 1 configuration when DMRS and PDSCH are TDM’ed and only half of the DMRS REs are occupied. Note 2: Number of DMRS CDM groups without data for PDSCH DMRS configuration for OCNG is set to 1. |  |  |

## C.3.1	Measurement of Receiver Characteristics

Unless otherwise stated, Table C.3.1-1 is applicable for measurements on the Receiver Characteristics (clause 7). For Adjacent channel selectivity testing, Table C.3.1-2 is applied.

Table C.3.1-1: Downlink Physical Channels transmitted during a connection (FDD and TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of PDSCH to SSS | dB | 0 |
| EPRE ratio of PDSCH to PDSCH DMRS (Note 1) | dB | -3 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of PTRS to PDSCH | dB | Test specific |
| EPRE ratio of OCNG DMRS to SSS | dB | 0 |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) | dB | 0 |
| Note 1: No boosting is applied to any of the channels except PDSCH DMRS. For PDSCH DMRS, 3 dB power boosting is applied assuming DMRS Type 1 configuration when DMRS and PDSCH are TDM’ed and only half of the DMRS REs are occupied. Note 2: Number of DMRS CDM groups without data for PDSCH DMRS configuration for OCNG is set to 1. |  |  |

Table C.3.1-2: PDCCH Aggregation Level for ACS testing

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Aggregation level | CCE | 1 | CBW=10MHz when SCS=60kHz |
|  |  | 2 | CBW=15MHz when SCS=60kHz |
|  |  | 4 | CBW=5MHz when SCS=15kHz CBW=10,15MHz when SCS=30kHz CBW=20,25,30MHz when SCS=60kHz |
|  |  | 8 | CBW=10,15MHz when SCS=15kHz CBW=20,25,30MHz when SCS=30kHz CBW=40,50,60,70MHz when SCS=60kHz |
|  |  | 16 | CBW>15 MHz when SCS=15kHz CBW>30 MHz when SCS=30kHz CBW>70 MHz when SCS=60kHz |

<End of modified section 1>