3GPP TSG RAN meeting #89e									 RP-201780

Electronic Meeting, September 14-18, 2020

## Status Report to TSG

Agenda item:			9.7.3

| WI / SI Name | NR Positioning Enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_Pos_Enh |  |  |  |  |
| Unique ID | 860034 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-200928 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 12/2020 | Core part: NA | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: 50% | Core part: NA | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Ren DA |
|  | Company | CATT |
|  | Email | renda@catt.cn |
| Rapporteur | Name | Yi GUO |
|  | Company | Intel Corporation |
|  | Email | yi.guo@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Agreements (RAN1#101-e)

Agreement:

- InF-SH and InF-DH models in TR 38.901 are adopted as the baseline scenarios for defining the channel models, parameters and modelling techniques for performance evaluations in the Rel. 17 positioning enhancements at least for IIoT use cases

- Note: Modifications to parameters in the InF-DH models will be discussed separately.

- Note: Target performance and performance gap identification will be discussed separately.

- Note: Individual companies may consider additional InF models in TR 38.901 as complementary evaluation scenarios in their simulation investigation and the evaluation results can be considered to be captured in the TR 38.857.

- Note: Target positioning requirements may not necessarily be reached for all scenarios.

Agreement (Proposal 4.1-1, Revision #2, in Section 4.1 of R1-2004868):

- Adopt the parameters defined in Table below as the baseline parameters for all scenarios in the evaluation of the positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

- Note: Optional scenarios and assumptions will be discussed separately and can be included

Table: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: FFS |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Agreement:

Optional: The following UE antenna configuration can be considered

- 4 UE panels:

  - The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- FFS: Other details

Agreement:

Absolute-time-of arrival model defined in TR 38.901 without modification is considered in the evaluation of all scenarios.

Agreement:

Blockage model is not considered in the evaluation of all scenarios

Agreement: (Proposal 5.1-4, Revision 3, in Section 5.1 of R1-2004961)

- Adopt the parameters defined in the Table below as the baseline parameters for all InF scenarios in the evaluation of positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

Table: Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: See Proposal 5.1-7 |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

Agreement:

Optional: For evaluating vertical positioning performance, UE antenna height can be uniformly distributed within [0.5, X2]m, where X2 = 2m for InF-SH and X2= for InF-DH defined in TR 38.901.

Agreement:

Clutter parameters {density , height ,size } for high clutter density are set as follows:

- (Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height

- (Optional): {40%, 3m, 5m}

- (Optional): {60%, 6m, 2m}

Agreement:

It will be left to companies to define the configurations for DL PRS and UL SRS for the evaluation of positioning performance.

- Note: Configurations of DL PRS and UL SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Agreement:

CDFs of positioning errors are used as performance metrics in NR positioning evaluation with at least the following percentiles 50%, 67%, 80%, 90%.

- Note: In addition to overall positioning accuracy performance, companies are encouraged to report the estimation accuracy of UE/gNB measurements (e.g., RSTD) for performance comparison.

Agreement:

For TR 38.857, the template used in TR 38.855 for the inclusion of simulation results is reused. In addition, the following parameters should be provided for each scenario together with the simulation results.

| Parameter | [Source 1, scenario, FRx] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) |  |
| Reference signal (type of sequence, number of ports, …) |  |
| Number of sites |  |
| Number of symbols used per slot per positioning estimate |  |
| Number of slots per positioning estimate |  |
| Power-boosting level |  |
| Uplink power control (applied/not applied) |  |
| interference modelling (ideal muting, or other) |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) |  |
| Network synchronization assumptions |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) |  |
| Additional notes, if any |  |

Agreement:

CDF values for positioning accuracy for IIoT scenarios are derived based on:

- Case 1 (Required): The UEs inside the convex hull of the horizontal BS deployment area.

- Case 2 (Optional): All the UEs

Agreement:

Optional: For evaluating vertical positioning performance, gNB antenna height can also be set to two fixed heights, which is either {4, 8} m, or {max(4,), 8}.

Agreement:

Network efficiency and UE efficiency can be evaluated at least in an analytical manner.

- FFS: the definition of efficiency metric (e.g., the positioning performance (accuracy, latency) vs. PRS/SRS resource utilization etc.)

- Note: It will be up to each company on whether to use other methods (e.g., numerical simulation) for the evaluation.

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

Agreement:

Optional: UE mobility can be considered in evaluation with the consideration of the spatial consistency procedure defined in TR 38.901.

- FFS: the details of the mobility models

Agreement:

- UE power consumption for NR positioning can be optionally evaluated in the SI.

- Note: It is up to each company on how to evaluate the power consumption for positioning. The UE power consumption models developed in TR38.840 can be considered as the starting point for defining the UE power consumption model for the evaluation for NR positioning

Agreement:

The TR skeleton in R1-2004948 is endorsed.

Agreement:

Optional: The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1:

        T1:  [X] ns for gNB and [Y] ns for UE

- FFS: X, Y

        Note: RX and TX timing errors are generated per panel independently

        FFS: how the Rx and Tx timing errors are applied

Agreement:

- In Rel-17 SI, for the evaluation of positioning enhancements for commercial use cases, no baseline scenario is defined. UMi, UMa and IOO scenario(s) defined in TR 38.855 can be considered as optional scenarios without modifications to existing configuration parameters.

- FFS: absolute time of arrival model for UMi, UMa and IOO scenarios

Agreement:

Physical layer latency can be evaluated through analysis and, optionally, numerical evaluation.

Agreement:

Higher layer positioning latency can be evaluated in this SI.

- FFS: how to evaluate higher-layer positioning latency

- FFS: which higher-layers should be included in the evaluation

Agreements (RAN1#102-e)

Agreement:

Physical Layer Latency Start and End times are defined as follows:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives. | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

Conclusion:

RAN1 will not define additional optional values for UE and gNB antenna heights for evaluations.

Conclusion:

RAN1 will not define additional details for the optional UE antenna configuration of 4 UE panels for evaluations.

Conclusion:

For power consumption evaluation, it is up to each company to detail their methodology (including power model) for evaluation.

Agreement:

Apply the timing errors as follows:

- For each UE drop,

  - For each panel (in case of multiple panels)

    - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution.

- For each gNB

  - For each panel (in case of multiple panels)

    - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution.

- Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report.

- For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms.

Conclusion:

For UE mobility, the details of the optional mobility model are left to companies.

Agreement:

PRS/SRS resource utilization is the metric used to evaluate network efficiency

- FFS: what is included in resource utilization, e.g. PRS/SRS/MG configurations, beam sweeping assumptions

Agreement:

For the absolute time of arrival modelling in IOO, UMa, Umi, companies may provide the details of their model, if any

Agreement:

Text proposal for LS to RAN WG2 and CC SA WG2 and RAN WG3 for analysis of latency of NR positioning protocols defined in Rel.16:

RAN1 evaluates physical layer latency and its potential reduction for NR Rel-17 positioning solutions. In order to evaluate End-To-End latency of NR positioning solutions the input from RAN2 is needed on latency components of NR/NG-RAN/5GC higher layer positioning protocols. RAN1 respectfully asks if RAN2 can provide a list of latency components with corresponding range of values for the existing and any potential enhanced NR positioning solutions, keeping in mind the End-To-End latency described as desired in the study item description (RP-200928)

Final LS approved in R1-2007264

Agreement:

Physical layer latency for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches are separately studied

Agreement:

Capture the following in TR as an observation:

- Performance analysis of baseline I-IoT InF scenarios shows that InF-SH scenario is characterized by high probability of LOS links. In InF-DH the probability of LOS links is reduced substantially while probability of NLOS links is increased accordingly.

Conclusion:

- Evaluations show that high probability of NLOS links and propagation delay offset imposed by NLOS links may cause performance degradation of positioning accuracy, that was especially observed in InF-DH scenario

- Initial evaluations have also shown that under certain ideal assumptions (e.g., synchronization error, Rx/Tx calibration error) the effective LOS/NLOS classification/detection, outlier determination/rejection techniques may be beneficial to improve NR positioning accuracy

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Conclusion:

- It is observed that calibration errors of UE/gNB Tx/Rx timing may negatively impact accuracy of timing-based methods of Rel.16 positioning solutions when precise UE positioning is targeted.

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Conclusion:

- Evaluations show that network synchronization errors may cause accuracy degradation of the DL-TDOA or UL-TDOA Rel-16 positioning solutions

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Agreement:

- FFS whether Rel.16 granularity of timing measurement reports is enough to avoid degradation in I-IoT scenarios and meet positioning requirements

Agreement:

- At least the following information is provided for positioning physical layer latency analysis:

  - Source initiating request for positioning measurements/location for a given UE (UE, Network)

  - Destination awaiting for positioning measurements/location for a given UE (UE, Network)

  - Start and end triggers/events for physical layer latency evaluation

    - For Rel.16 solutions, it is based on specification for each solution

  - Initial and final RRC State of positioned UE (RRC IDLE, INACTIVE, CONNECTED) at the start and end time for the physical layer latency evaluation

  - Positioning

    - technique (enumeration): (1) DL-TDOA, (2) DL AoD, (3) UL-TDoA, (4) UL-AoA, (5) Multi-RTT, (6) E-CID

    - type: DL, UL, DL+UL

    - mode: UE-based, UE-assisted

  - Latency component w/ value range and description, including information on any parallel (simultaneous) components

  - Total latency value

- Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one:

| Source [UE, NW]/Destination [UE, NW] Positioning technique [DL-TDOA, E-CID, …], type [DL, UL, DL+UL], mode [UE-A, UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  |
| Name of component 1 |  |  |
| Name of component 2 |  |  |
| Name of last component |  |  |
| End trigger |  |  |
| Total values |  |  |

Agreement:

Partial staggering and non-staggering RE mapping of SRS for positioning with different combinations of comb-factors and symbol lengths will be investigated in Rel-17.

- The methods/signalling for addressing potential time-domain aliasing due to the partial/non-staggering RE mapping will be included in the study

Agreement:

- Semi-persistent and a-periodic transmission and reception of DL PRS will be investigated in Rel-17.

  - FFS: the details on when and how to enable semi-persistent and a-periodic DL PRS

  - FFS: to be supported for which positioning methods, e.g.,

    - UE-assisted and/or UE-based positioning

    - DL positioning and/or Multi-RTT

- On-demand transmission and reception of DL PRS will be investigated in Rel-17.

  - FFS: the details on when and how to enable on-demand DL PRS

  - FFS: to be supported for which positioning methods, e.g.,

    - UE-assisted and/or UE-based positioning

    - DL positioning and/or Multi-RTT

- Notes:

  - Semi-persistent means MAC-CE triggered

  - Aperiodic would correspond to DCI-triggered

  - On-demand corresponds to the UE-initiated or network-initiated request of PRS and/or SRS. So, it is NOT the same as whether PRS is DCI-triggered or MAC-CE triggered. It is about UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc.

Agreement:

- Multipath mitigation techniques will be investigated in this SI for improving positioning accuracy, which may include, but not limited to the following:

  - The applicable scenarios and performance benefits of multipath mitigation techniques

  - The methods/measurement/signaling for the LOS/NLOS detection and identification

  - The measurements for supporting the multipath mitigation/utilization

  - The procedure and signaling for supporting the multipath mitigation/utilization

  - Implementation-based solutions (e.g., outlier rejection) without the need of any additional specified method/measurements/procedures/signaling.

- Note: The above study applies to DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted positioning.

Agreement:

- NR positioning for UEs in RRC_IDLE state and UEs in RRC_INACTIVE state will be investigated in Rel-17, including the benefits on latency, network/UE efficiency and UE power consumption

- FFS: which positioning methods to be supported, e.g., DL positioning, UL positioning, DL+UL positioning and/or Multi-RTT

- FFS: the details of how to enable the UE positioning in RRC_IDLE state and RRC_INACTIVE state

  - Reference signals (e.g., based on DL PRS signals, UL SRS signals, both of them, etc.)

  - Signaling and procedures (e.g., based on PRACH procedure, paging triggered UL SRS transmission, etc.)

Agreement:

- For reducing NR positioning latency, more efficient signaling & procedures will be investigated to enable a device to request and report positioning information, which may include, but not limited to, the following aspects:

  - DL PRS/UL SRS configuration, activation or triggering.

  - The request for positioning information (the assistance data, etc.).

  - The report of positioning information (the measurement report, etc.).

- Note: It is not within RAN1 scope to analyze positioning architecture enhancements to enable such more efficient signaling & procedures.

- Note: RAN1 does not make any assumptions on whether the LCS architecture specified in TS 23.273 is enhanced or not.

Agreement:

- Aggregating multiple DL positioning frequency layers of the same or different bands for improving positioning performance for both intra-band and inter-band scenarios will be investigated in Rel-17, which may take into account at least the following

- The scenarios and performance benefits of aggregating multiple DL positioning frequency layers

- The impact of channel spacing, timing offset, phase offset, frequency error, and power imbalance among CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- UE complexity considerations

- Note: What is captured in the TR will be discussed separately.

Agreement:

Simultaneous transmission by the UE and reception by the gNB of the SRS for positioning across multiple CCs and multiple slots can be investigated in Rel-17, which may consider

- The scenarios and performance benefits of the enhancement

- The impact of channel spacing, TA and timing offset, phase offset, frequency error, and power imbalance across slots or CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

Agreement:

- The scenario, benefits, and methods for improving the accuracy of the UL AoA and DL-AoD methods for both UE-based and network-based (including UE-assisted) positioning can be investigated in Rel-17.

Agreement:

The scenario, benefits, methods and signaling for improving positioning accuracy in the presence of the UE Rx/Tx transmission delays, and/or and gNB Rx/Tx transmission delays, will be investigated for UE-based and network-based (including UE-assisted) positioning in Rel-17.

#### 2.1.2	Remaining Open issues

RAN1#101-e is the first RAN1 meeting of this SI. According to the meeting arrangement, RAN1 started working on the following objectives in SID:

Defining the additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

In RAN1#101-e, RAN1 has defined the additional scenarios and completed the baseline simulation assumptions and some optional simulation assumptions for the additional scenarios.

In RAN1#102-e, RAN1 has continued working on above 1st objective and completed it.

In addition, RAN1 has continued working on above 2nd objective in RAN1#102-e. The evaluation is expected to be completed in RAN1#103-e.

Furthermore, RAN1 has started discussing of the following objective in RAN1#102-e. RAN1’s work is expected to be completed in RAN1#103-e.

Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

## 2.2	RAN2

#### 2.2.1	Agreements

Agreements (RAN2#111-e)

Enhancements for commercial use cases:

Agreement:

RAN2 to study positioning in idle/inactive mode, on-demand PRS and latency analysis in the study phase.

Note: RAN2 will continue the discussion on E2E latency based on email discussion;

Integrity-KPIs and use cases:

Agreement:

Agreements:

- Start from the definitions of the four candidate KPIs.  Additional definitions can be added when needed.

Agreements:

1.	Agree to adopt the Target Integrity Risk (TIR), Alert Limit (AL) and Time-to-Alert TTA) as the Integrity KPIs.

2.	Agree to the following definitions of the KPIs:

Target Integrity Risk (TIR)

The probability that the positioning error exceeds the Alert Limit (AL) without warning the user within the required Time-to-Alert (TTA).

NOTE: The TIR is usually defined as a probability rate per some time unit (e.g. per hour, per second or per independent sample).

Alert Limit (AL)

The maximum allowable positioning error such that the positioning system is available for the intended application. If the positioning error is beyond the AL, operations are hazardous and the positioning system should be declared unavailable for the intended application to prevent loss of integrity.

NOTE: When the AL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Alert Limit (HAL) or Vertical Alert Limit (VAL) respectively.

Time-to-Alert (TTA)

The maximum allowable elapsed time from when the positioning error exceeds the Alert Limit (AL) until the function providing position integrity annunciates a corresponding alert.

3.	Agree to include the PL integrity definition with the following baseline; FFS if updates are needed.

Protection Level:

The PL is a statistical upper-bound of the positioning error that ensures that, the probability per unit of time of the true error being greater than the AL and the PL being less than or equal to the AL, for longer than the TTA, is less than the required TIR.

NOTE: When the PL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Protection Level (HPL) or Vertical Protection Level (VPL) respectively.

4.	The additional definitions are FFS on a ‘need-to-define’ basis.

5.	Agree to study the Automotive, IIoT and Rail use cases as illustrative examples.

6.	Agree to the Skeleton for Section 9 of TR 38.857.

Note: RAN2 will continue the discussion via email discussion to capture any additional integrity use cases and open issues on integrity, and draft a TP incorporating the existing agreements and any further progress.

Integrity-Error sources:

Agreement:

Proposal 2: Error source for RAT-dependent positioning methods should be studied under RAN1. Send an LS to RAN1 to trigger the study on error sources for RAT-dependent positioning methods for positioning integrity

Proposal 3: RAN2 can independently study the error sources for RAT-independent positioning methods.

Proposal 5: RAN2 confirms that 4 possible sources of feared events are applicable for RAT-independent positioning in 3GPP system.

1.	Faults in the correction data e.g.

a.	Incorrect computation by the provider

b.	External feared event impacting the provider

2.	Faults in transmitting the data to the UE, e.g.

a.	Data integrity faults

3.	External feared events, e.g.

a.	Satellite feared events

b.	Atmospheric feared events

c.	Multipath

4.	UE faults

#### 2.2.2	Remaining Open issues

Commercial use cases:

-	Continue the study on E2E latency, positioning in idle/inactive mode and on demand PRS;

Integrity-KPIs and use cases:

-	To identify any additional use cases, definitions;

Integrity-error sources:

-	To identify any additional error sources for RAT independent positioning methods;

Integrity- Methodologies for network-assisted and UE-assisted integrity:

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2005095	Session notes for 8.2 (Study on NR positioning enhancements)	Ad-Hoc Chair (Ericsson)

- R1-2003638	Work plan for Study on NR Positioning Enhancements	CATT, Intel, Ericsson

- R1-2004649	TR skeleton for TR 38.857	Ericsson

- R1-2004948	TR skeleton for TR 38.857	Ericsson

- R1-2004701	FL Summary for NR Positioning Enhancements	Moderator (CATT)

- R1-2004868	FL Summary #2 for NR Positioning Enhancements	Moderator (CATT)

- R1-2004961	FL Summary #3 for NR Positioning Enhancements	Moderator (CATT)

- R1-2005049	FL Summary #4 for NR Positioning Enhancements	Moderator (CATT)

- R1-2005049	FL Summary #4 for NR Positioning Enhancements	Moderator (CATT)

- R1-2005102 	Summary of Email Discussion [101-e-Post-NR-Pos-Enh] Moderator (CATT)

- R1-2005188 	Summary #2 of Email Discussion [101-e-Post-NR-Pos-Enh] Moderator (CATT)

- R1-2003639	Summary of discussion on IIoT Scenarios for NR Positioning Enhancements	CATT

- R1-2003906	Additional scenarios for evaluation	Samsung

- R1-2003963	Discussions on IIoT scenarios for positioning	CMCC

- R1-2004141	Discussion on additional scenarios for evaluation	LG Electronics

- R1-2004490	Considerations on Additional Scenarios for Evaluation	Qualcomm Incorporated

- R1-2003284	IIOT Scenarios for Positioning	Futurewei

- R1-2003295	Discussion on scenarios and evaluation methodology for Rel-17 positioning	Huawei, HiSilicon

- R1-2003427	Discussion on additional scenarios for NR positioning evaluation	vivo

- R1-2003479	Additional scenarios for evaluation on positioning enhancements	ZTE

- R1-2003640	IIoT use cases and scenarios for evaluation of NR Positioning Enhancements	CATT

- R1-2003719	Additional scenarios for evaluation of NR positioning	Nokia, Nokia Shanghai Bell

- R1-2003767	I-IoT scenarios for NR positioning evaluations	Intel Corporation

- R1-2004063	Discussion on Scenarios for Evaluation	OPPO

- R1-2004190	Considerations on Scenarios for Evaluations of IIoT Positioning	Sony

- R1-2004199	View on scenarios and evaluation parameters for Rel 17 positioning enhancement	CEWiT

- R1-2004517	Additional scenarios and considerations for NR positioning	Fraunhofer IIS, Fraunhofer HHI

- R1-2004650	Additional scenarios for performance evaluations	Ericsson

- R1-2003296	Performance evaluation for Rel-17 positioning	Huawei, HiSilicon

- R1-2003428	Evaluation of achievable accuracy and latency for NR positioning enhancements	vivo

- R1-2003480	Evaluation results of additional scenarios for positioning	ZTE

- R1-2003668	Evaluation of DL-AoD technique under IIOT scenario	MediaTek Inc.

- R1-2003768	Initial analysis of NR positioning performance in I-IoT scenarios	Intel Corporation

- R1-2004064	Evaluation of NR positioning in IIOT scenario	OPPO

- R1-2004651	Evaluation of Achievable Positioning Accuracy and Latency	Ericsson

- R1-2003547	Evaluation of Rel-16 Positioning for IIOT	Futurewei

- R1-2003641	Discussion of evaluation of NR positioning performance	CATT

- R1-2003720	Views on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

- R1-2003907	Evaluation of achievable positioning accuracy and latency	Samsung

- R1-2003964	Discussions on evaluation methodology of latency	CMCC

- R1-2004191	Considerations on Evaluation of Positioning Accuracy and Latency	Sony

- R1-2004491	Initial Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

- R1-2004518	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

- R1-2003285	IIOT Positioning techniques Consideration	Futurewei

- R1-2003297	Discussion of positioning enhancement	Huawei, HiSilicon

- R1-2003429	Discussion on potential positioning enhancements	vivo

- R1-2003481	Potential NR positioning enhancements for Rel.17	ZTE

- R1-2003642	Discussion of NR positioning enhancements	CATT

- R1-2003669	Views on Positioning enhancement for Rel-17	MediaTek Inc.

- R1-2003701	Potential positioning enhancements	BUPT

- R1-2003721	Initial views on potential positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2003769	Potential NR positioning enhancements	Intel Corporation

- R1-2003908	Potential positioning enhancements	Samsung

- R1-2003965	Discussions on potential positioning enhancements	CMCC

- R1-2003977	Positioning enhancements for RRC IDLE and RRC INACTIVE state UE	Beijing Xiaomi Mobile Software

- R1-2003988	Discussion on potential positioning enhancements	Spreadtrum Communications

- R1-2004065	Discussions on NR Positioning Enhancements	OPPO

- R1-2004142	Discussion on potential positioning enhancements	LG Electronics

- R1-2004175	On Potential Rel-17 NR Positioning Enhancements	Lenovo, Motorola Mobility

- R1-2004192	Potential Techniques for Positioning Enhancements	Sony

- R1-2004420	Discussion on potential techniques for NR Positioning Enhancements	NTT DOCOMO, INC.

- R1-2004492	Initial thoughts on Potential Positioning Enhancements	Qualcomm Incorporated

- R1-2004652	Potential enhancements for NR positioning in release 17	Ericsson

- R1-2003430	Discussion on power consumption model for NR positioning enhancements	vivo

- R1-2003643	Discussion of  NLOS IIoT channel modelling for NR positioning enhancement	CATT

- R1-2003909	Uplink Transmission Based Relative Positioning	Samsung

- R1-2004066	Analysis of NR Positioning for IIOT Scenarios	OPPO

- R1-2004609	Discussion of sidelink positioning	Huawei, HiSilicon

- R1-2004653	PRS separation based on Cyclic shifts	Ericsson

- R1-2007386	Session notes for 8.5 (Study on NR Positioning Enhancements)	Ad-Hoc Chair (Ericsson)

- R1-2006913	TP for additional scenarios and channel models in TR 38.857	Ericsson

- R1-2005251	Additional consideration on evaluation methodology	Huawei, HiSilicon

- R1-2005283	Remaining Issues on Evaluation Assumptions	FUTUREWEI

- R1-2005379	Discussion on additional scenarios for NR positioning evaluation	vivo

- R1-2005462	Evaluation assumptions for NR positioning enhancements	ZTE

- R1-2005577	Remaining Issues on Scenarios for Evaluation of NR Positioning	Sony

- R1-2005710	Remaining issues on additional scenarios for evaluation of NR Positioning Enhancements	CATT

- R1-2005877	Remaining details on additional scenarios for NR positioning evaluations	Intel Corporation

- R1-2005990	Discussion on Scenarios for Evaluation	OPPO

- R1-2006066	Additional scenarios for evaluation	BUPT

- R1-2006148	Additional scenarios for evaluation	Samsung

- R1-2006214	Remaining issues on target performance requirement of IIoT use case	CMCC

- R1-2006238	UE mobility model for evaluation	InterDigital, Inc.

- R1-2006374	Discussion on additional scenarios for evaluation for NR positioning	LG Electronics

- R1-2006427	Additional scenarios for evaluation of NR positioning	Nokia, Nokia Shanghai Bell

- R1-2006458	Additional scenarios for evaluation	Fraunhofer IIS, Fraunhofer HHI

- R1-2006808	Considerations on Additional Scenarios for Evaluation	Qualcomm Incorporated

- R1-2006914	Remaining details on additional scenarios for evaluation	Ericsson

- R1-2007103	FL summary for additional scenarios for evaluation of NR positioning enhancements	Moderator (Ericsson)

- R1-2007209	FL summary#2 for additional scenarios for evaluation of NR positioning enhancements	Moderator (Ericsson)

- R1-2007285	FL summary#3 for additional scenarios for evaluation of NR positioning enhancements	Moderator (Ericsson)

- R1-2005252	Performance evaluation for Rel-17 positioning	Huawei, HiSilicon

- R1-2005380	Evaluation of achievable positioning accuracy and latency	vivo

- R1-2005463	Evaluation results based on NR Rel-16 positioning	ZTE

- R1-2005578	Initial Views on Evaluation of Positioning Accuracy and Latency	Sony

- R1-2005711	Discussion of evaluation of NR positioning performance	CATT

- R1-2005878	NR Positioning Performance in I-IoT Scenarios	Intel Corporation

- R1-2005991	Evaluation of NR positioning in IIOT scenario	OPPO

- R1-2006067	Evaluation of achievable positioning accuracy and latency	BUPT

- R1-2006149	Evaluation of achievable positioning accuracy and latency	Samsung

- R1-2006197	Evaluation of DL-TDOA and DL-AoD techniques under IIOT scenarios	MediaTek Inc.

- R1-2006215	Discussion on achievable positioning latency	CMCC

- R1-2006239	Discussion on evaluation of latency	InterDigital, Inc.

- R1-2006323	Considerations for Positioning Latency Evaluation	Lenovo, Motorola Mobility

- R1-2006375	Discussion on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

- R1-2006428	Initial results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

- R1-2006459	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

- R1-2006623	Positioning evaluation results for additional commercial use cases	CEWiT

- R1-2006809	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

- R1-2006915	Evaluation of achievable positioning accuracy and latency	Ericsson

- R1-2006970	Discussion of evaluation of NR positioning performance	CATT

- R1-2007105	Feature lead summary #1 for email discussion [102-e-NR-Pos-Enh-Eval-Acc-Lat]	Moderator (Intel)

- R1-2007262	Feature lead summary #2 for email discussion [102-e-NR-Pos-Enh-Eval-Acc-Lat]	Moderator (Intel Corporation)

- R1-2007263	Draft LS on Latency of NR Positioning Protocols	Intel Corporation

- R1-2007264	LS on Latency of NR Positioning Protocols	RAN1, Intel Corporation

- R1-2007358	Feature lead summary #3 for email discussion [102-e-NR-Pos-Enh-Eval-Acc-Lat]	Moderator (Intel Corporation)

- R1-2007359	Template for collection of NR positioning evaluation results	Moderator (Intel Corporation)

- R1-2005253	Positioning enhancement in Rel-17	Huawei, HiSilicon

- R1-2005284	Positioning Enhancements	FUTUREWEI

- R1-2005381	Discussion on potential positioning enhancements	vivo

- R1-2005464	Discussion on potential NR positioning enhancements	ZTE

- R1-2005579	Discussion on Positioning Enhancements	Sony

- R1-2005712	Discussion of NR positioning enhancements	CATT

- R1-2005769	Potential positioning enhancements	TCL Communication Ltd.

- R1-2005879	Potential Enhancements of NR Positioning Design	Intel Corporation

- R1-2005992	Discussions on NR Positioning Enhancements	OPPO

- R1-2006068	Potential positioning enhancements	BUPT

- R1-2006150	Potential positioning enhancements	Samsung

- R1-2006194	Views on positioning enhancement for Rel-17	MediaTek Inc.

- R1-2006216	Discussion on potential positioning enhancements	CMCC

- R1-2006240	Discussion on potential positioning enhancements	InterDigital, Inc.

- R1-2006250	Discussion on potential positioning enhancements	Spreadtrum Communications

- R1-2006324	On Potential NR Positioning Enhancements	Lenovo, Motorola Mobility

- R1-2006376	Discussion on potential enhancements for NR positioning	LG Electronics

- R1-2006429	Views on potential positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2006460	Potential positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

- R1-2006522	Initial Views on Potential Positioning Enhancements	Apple

- R1-2006547	Potential positioning enhancements	Beijing Xiaomi Electronics

- R1-2006621	Discussion on positioning enhancements for Rel 17	CEWiT

- R1-2006732	Discussion on potential techniques for NR Positioning Enhancements	NTT DOCOMO, INC.

- R1-2006810	Potential Positioning Enhancements for NR Rel-17 Positioning	Qualcomm Incorporated

- R1-2006859	Discussion on Potential positioning enhancements	CAICT

- R1-2006916	Potential positioning enhancements	Ericsson

- R1-2006972	FL Summary for Potential Positioning Enhancements	Moderator (CATT)

- R1-2007111		FL Summary#2 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2007172	FL Summary#3 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2007210	FL Summary#4 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2007343	FL Summary#5 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2005382	Discussion on power consumption model for NR positioning enhancements	vivo

- R1-2005465	Channel state estimation based on prior channel information	ZTE

- R1-2005713	Discussion of NLOS channel modelling and network time synchronization for NR positioning	CATT

- R1-2005993	Analysis of NR Positioning for IIOT Scenarios	OPPO

- R1-2006151	Uplink Transmission Based Relative Positioning	Samsung

- R1-2006241	Discussion on positioning during idle/inactive mode	InterDigital, Inc.

- R1-2006398	Analysis of positioning service latency	Huawei, HiSilicon

- R1-2006523	Analysis for L1 Positioning Latency	Apple

- R1-2006917	PRS with cyclic shifts	Ericsson

- R2-2006670	Updated Work Plan for R17 SI NR Positioning Enhancements	CATT, Intel Corporation, Ericsson

- R2-2006958	skeleton for TR38857  	Ericsson

- R2-2006671	Skeleton proposals for TR38.857	CATT

- R2-2006542	Proposed table of contents - Section 9 (positioning integrity) - TR 38.857	Swift Navigation, Ericsson, Intel Corporation

- R2-2006749	Handling on Rel-16 leftover issue in Rel-17	Intel Corporation

- R2-2006669	Summary on Rel-17 positioning enhancement discussion in RAN1	CATT, Intel Corporation, Ericsson

- R2-2006672	Discussion on ehancements for commercial use cases	CATT

- R2-2008261	[AT111-e][612][POS] Assumptions for analysis of commercial use cases (Ericsson)	Ericsson

- R2-2006578	Discussion on R17 positioning enhancement	Huawei, HiSilicon

- R2-2006956	Enhancements for commercial use cases	Ericsson

- R2-2006567	Discussion on potential positioning enhancement	vivo

- R2-2007049	Discussion on positioning enhancements for commercial use cases	Spreadtrum Communications

- R2-2007629	NR Positioning Enhancements	Qualcomm Incorporated

- R2-2006750	Consideration on the support of low latency requirement	Intel Corporation

- R2-2007587	End-to-end latency reduction for DL/UL positioning	InterDigital, Inc.

- R2-2007128	On-demand PRS transmission and dynamic PRS resource allocation	Nokia, Nokia Shanghai Bell

- R2-2007159	Discussion on on-demand DL-PRS	OPPO

- R2-2007170	Discussion on PRS enhancements	Beijing Xiaomi Electronics

- R2-2007157	Positioning for UE in RRC Idle and Inactive state		OPPO

- R2-2007173	Positioning enhancements for RRC IDLE and RRC INACTIVE state UE	Beijing Xiaomi Electronics

- R2-2006541	TP for Study on Positioning Integrity and Reliability	Swift Navigation, Deutsche Telekom, u-blox, Ericsson, Mitsubishi Electric, Intel Corporation, CATT, UIC

- R2-2008256	[AT111-e][607][POS] Summary of email discussion on Integrity definitions, KPIs, and use cases (Swift)	Swift Navigation

- R2-2008262	[AT111-e][607][POS] Summary of email discussion on Integrity definitions, KPIs, and use cases (Swift)	Swift Navigation

- R2-2006754	Consideration on positioning integrity	Intel Corporation

- R2-2006673	Discussion on integrity KPIs and use cases	CATT

- R2-2006564	Identify positioning integrity use case and KPIs	vivo

- R2-2006579	Discussion on positioning integrity KPIs and relevant use cases	Huawei, HiSilicon

- R2-2006954	Positioning integrity KPIs and support for RAT dependent use cases	Ericsson

- R2-2007050	Discussion on positioning integrity KPIs and use cases	Spreadtrum Communications

- R2-2007646	Discussion on use cases and KPIs for position integrity		ESA

- R2-2007102	Discussion on Positioning Integrity	Apple

- R2-2007158	Discussion on the KPIs of integrity	OPPO

- R2-2007936	Discussion of the positioning integrity definition	ZTE Corporation, Sanechips

- R2-2007073	Discussion on integrity and reliability for positioning based on an IIoT use case	Sumitomo Elec. Industries, Ltd

- R2-2007187	Discussion on Integrity of positioning information	Sony

- R2-2007937	Discussion of the integrity events and integrity failure	ZTE Corporation, Sanechips

- R2-2008263	[AT111-e][613][POS] Integrity Error Sources (Huawei)	Huawei, HiSilicon

- R2-2008613	LS on the error source for RAT-dependent positioning	Huawei, HiSilicon

- R2-2006580	Discussion on positioning integrity validation and reporting	Huawei, HiSilicon

- R2-2006674	Discussion on error sources, threat models, occurrence rates and failure modes	CATT

- R2-2006565	Identify Error sources for positioning integrity	vivo

- R2-2006955	Factors impacting positioning integrity 	Ericsson

- R2-2007647	Discussion on GNSS position integrity error sources	ESA

- R2-2007938	Discussion of the positioning error sources, threat models and failure modes	ZTE Corporation, Sanechips

- R2-2006566	Discussion on positioning integrity methodologies	vivo

- R2-2006675	Discussion on methodologies for network-assisted and UE-assisted integrity	CATT

- R2-2006581	Discussion for network-assisted and UE-assisted integrity	Huawei, HiSilicon

- R2-2006957	LPP signalling for integrity support of RAT dependent positioning	Ericsson

- R2-2007160	Discussion on methodologies for UE-based and UE-assisted integrity	OPPO

- R2-2007238	Reporting movement model	Fraunhofer IIS, Fraunhofer HHI

- R2-2007246	Reporting the situational quality of RAT and RAT-independent technologies	Fraunhofer IIS, Fraunhofer HHI

- R2-2007588	Methodologies for network-assisted and UE-assisted integrity	InterDigital, Inc.

- R2-2007656	Discussion on methodologies for position integrity	ESA

- R2-2007939	Discussion of the methodologies for network-assisted and UE-assisted integrity	ZTE Corporation, Sanechips

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template