3GPP TSG RAN Meeting #89e										RP-201782

Electronic Meeting, September 14 - 18, 2020

## Status Report to TSG

Agenda item:		9.2.5

| WI / SI Name | UE Power Saving in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_UE_pow_sav_perf |  |  |  |  |
| Unique ID | 830175 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-200494 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 06/2020 | Performance part: 12/2020 | Testing part: NA |  |
| Overall Completion level | Study Item: | Core part: 100% | Performance Part: 55% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Fang-Chen Cheng |
|  | Company | CATT |
|  | Email | fcc@catt.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### Agreements

RAN1 completed

    - Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 completed

#### 2.2.2	Remaining Open issues

None

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

Agreement: (from R4-2012645)

RAN4 agree to introduce demodulation test cases for PDCCH-WUS in Rel-16.

The purpose of such test case was to verify UE supporting PDCCH-WUS feature/function designed in RAN1/RAN2.

Further discuss among option 1/option2 needed considering test time issue.

RAN4 also aware that with option 1/option 2, UE behaviour following PDCCH-WUS indication “0” not verified.

- Test solutions

Option 1/2 listed in Slides#3 will be used for power saving test.

Make a decision in next RAN4 meeting.

- Test complexity aspects

  - DRX cycle

Use short DRX cycle of 10ms to reduce test time.

It should be noted that  using 10ms is only for test purpose and has no relation to real deployment.

  - Number of error samples

TBD error samples will be used.

Note: Currently RAN5 uses 1000 error samples which is time consuming. Need feedback from TE vendors whether 100 error samples are feasible.

Simulation assumptions as captured in R4-2012645.

Agreements in R4-2012124

The following has been agreed for the RRM test cases for power saving.

- Develop test cases only for power saving Scenario 1 (low mobility) and Scenario 2 (not at cell edge).

- No specific test cases for RRM requirements at transition period.

- No test cases for BWP switching delay due to MIMO layer adaption

- No test cases for Scenario 3 (both low mobility and not-at-cell-edge)

- No test cases for EMR carriers while T331 is running and Carrier specific threshold for inter-frequency measurements.

- No additional test cases on RLM/BFD with PDCCH-WUS configured is needed

- Develop the following test cases

| No. | Test case | Remarks |
| --- | --- | --- |
| 1 | Cell reselection to FR1 intra-frequency NR case | Depending on RAN4#97e discussion, this test case may need to be split into 1-a for mobility and 1-b for not-at-cell-edge |
| 2 | Cell reselection to FR1 inter-frequency NR case | Depending on RAN4#97e discussion, this test case may need to be split into 2-a for mobility and 2-b for not-at-cell-edge |
| 3 | Cell reselection to FR1 inter-RAT E-UTRA | Low mobility |
| 4 | Cell reselection to FR1 inter-RAT E-UTRA | Not-at-cell-edge |
| 5 | Cell reselection to FR2 intra-frequency NR case | Depending on RAN4#97e discussion, this test case may need to be split into 5-a for mobility and 5-b for not-at-cell-edge |
| 6 | Cell reselection to FR2 inter-frequency NR case | Depending on RAN4#97e discussion, this test case may need to be split into 6-a for mobility and 6-b for not-at-cell-edge |

#### 2.4.2	Remaining Open issues

Demodulation performance:

Decide the demodulation solutions and agree simulation assumptions.

Complete the demodulation performance evaluation and test cases.

RRM:

- How to combine the criteria (low mobility/not-at-cell-edge) with each test.

Option 1: Include both criteria in the same test in different T period

Option 2: Have separate test for each criterion

- How to consider high/low priority frequency layers for inter-frequency/inter-RAT

- How to reflect the low mobility and not-at-cell-edge criteria by threshold setting

- Whether to exclude the cell search process from test repetition or not

- Complete the test cases and agree CRs.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTx

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#96-e

- R4-2012045	Email discussion summary for [96e][214] NR_UE_pow_sav_RRM	Moderator (CATT)

- R4-2012214	Email discussion summary for [96e][214] NR_UE_pow_sav_RRM	Moderator (CATT)

- R4-2012122	Reply LS on RRM relaxation in power saving	RAN4

- R4-2010359	On the higher priority inter frequency layer relaxation indicator for the UE power savings	vivo

- R4-2010360	CR for IDLE state measurement relaxation for UE power saving	vivo

- R4-2010704	Discussion on RRM requirements maintenance of measurement relaxation for UE power saving	OPPO

- R4-2010705	CR on RRM Measurement relaxation requirements for UE power saving (TS 38.133)	OPPO

- R4-2011111		Discussion on measurement relaxation in power saving	Huawei, Hisilicon

- R4-2011112		CR on measurement relaxation for power saving	Huawei, Hisilicon

- R4-2011211	Correction CR to Rel-16 UE power saving requirements	Ericsson

- R4-2012123	Correction CR to Rel-16 UE power saving requirements	Ericsson

- R4-2012268	Correction CR to Rel-16 UE power saving requirements	Ericsson

- R4-2009764	CR on measurement relaxation requirements for UEs under power saving mode	Xiaomi

- R4-2009808	Discussion on RRM requirements for power saving	CATT

- R4-2009809	CR for RRM requirements for power saving	CATT

- R4-2012124	WF on RRM test cases for NR Power Saving	CATT

- R4-2011113		Test case list for measurement relaxation in power saving	Huawei, Hisilicon

- R4-2009765	Discussion on test cases for power saving RRM	Xiaomi

- R4-2009985	Features for performance tests in power saving WI	Qualcomm

- R4-2010337	Discussion on RRM test cases for R16 UE power saving	vivo

- R4-2011210	Discussions on test cases for Rel-16 UE power saving	Ericsson

- R4-2009810	Discussion on RRM Test cases for power saving	CATT

- R4-2009898	Discussion on RRM measurement relaxation for RRC_IDLE/INACTIVE	MediaTek inc.