3GPP TSG-RAN WG1 Meeting #102-e	R1-2007229

e-Meeting, August 17th – 29th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0126 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify starting slot within DCI 2_5 indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (AT&T) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In subclause 16 of TS 38.213, the specification text “An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol availability in each slot for a number of slots starting from a slot where the IAB-node detects the DCI format 2_5,” does not specifically mention whether the slot is defined based on the DU or MT resource configuration/timing and configured SCS for the AI index file value in DCI Format 2_5. If the SCS is different for the IAB-DU and IAB-MT, different slots may be identified for that reference slot. Given the soft resources are only defined for slots within the resource configuration of an IAB-DU, it seems straightforward that the starting slot is relative to the IAB-DU, however since DCI Format 2_5 is received by an IAB-MT, there appears to be a need to define the starting slot also relative to the slot where the IAB-MT detected DCI Format 2_5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proposed text clarifies that an AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol availability in each slot for a number of slots starting from the earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-MT detects the DCI format 2_5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity at the parent and child IAB nodes of which slot is the starting slot for a given AI index field in a DCI format 2_5 if the SCS is different for the child IAB-DU and IAB-MT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact Analysis: Impact to IAB-node behavior when determining starting slot location for a given AI index filed value in DCI format 2_5. No impact to access UEs. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change>>

# 14	Integrated access-backhaul operation

------- Unchanged text is omitted -------

An AI index field value in a DCI format 2_5 indicates to an IAB-DU a soft symbol availability in each slot for a number of slots starting from athe earliest slot of the IAB-DU which overlaps in time with the slot of the IAB-MT where the IAB-nodeMT detects the DCI format 2_5. The number of slots is equal to or larger than a PDCCH monitoring periodicity for DCI format 2_5 as provided by SearchSpace. The AI index field includes  bits where maxAIindex is the maximum of the values provided by corresponding availabilityCombinationId. An availability for a soft symbol in a slot is identified by a corresponding value resourceAvailability as provided in Table 14-3.

------- Unchanged text is omitted -------

<< End of change>>