3GPP TSG RAN meeting #90e										RP-202209

Electronic Meeting, December 7-11, 2020

Status Report to TSG

Agenda item:			9.7.7

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_SL_Relay |  |  |  |  |
| Unique ID | 860038 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201474 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2021 | Core part: mm/yyyy | Performance part: mm/yyyy | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: 80 % | Core part: xx% | Performance Part: xx% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Qianxi Lu |
|  | Company | OPPO |
|  | Email | qianxi.lu@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

Agreement for “8.7.2	Scope requirements and scenarios”

Agreements:

Proposal 1	[easy]Confirm for L2 U2N Relay that both Case1.1 and Case 1.2 are supported in this SI, i.e.

-	Case 1.1: Before remote connection via relay UE, relay UE and remote UE are in same cell;

-	Case 1.2: Before remote connection via relay UE, relay UE and remote UE are in different cells.

Proposal 2	[easy]Confirm for L2 U2N Relay that Case 2.1 is supported in this SI as baseline, i.e. after remote UE connection via relay UE, relay UE and remote UE are controlled by the relay UE’s serving cell;

Proposal 3	For L3 U2N Relay, relay UE and remote UE can be in the cell same or different cells, after remote UE connection via Relay UE.

Proposal 1	[easy]For L2 U2N Relay, RRC_INACTIVE state is supported for remote UE

Proposal 2	[easy]For L2 U2N Relay, RRC_INACTIVE state is supported for relay UE

Proposal 3	[easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_CONNECTED and relay UE in RRC_IDLE is excluded

Proposal 4	[easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_CONNECTED and relay UE in RRC_INACTIVE is excluded

Proposal 6	[easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_INACTIVE and relay UE in RRC_CONNECTED is supported

Proposal 8	[easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_INACTIVE and relay UE in RRC_INACTIVE is supported

Proposal 9	[easy]For L2 U2N Relay, the RRC states combination of remote UE in RRC_IDLE and relay UE in RRC_INACTIVE is supported

Agreement for “8.7.3.1	Protocol stacks and procedures”

Agreements:

Proposal-1: [Easy] agree the following description for L2 UE-to-NW relay

For L2 UE-to-NW relay, the Uu adaptation layer at Relay UE supports UL bearer mapping between ingress PC5 RLC channels for relaying and egress Uu RLC channels over the Relay UE Uu path.

Proposal-2: [Easy] agree the following description for L2 UE-to-NW relay

The different RBs of the same Remote UE and/or different Remote UEs can be subject to N:1 mapping and data multiplexing over Uu RLC channel

Proposal-3: [Easy] agree the following description for L2 UE-to-NW relay

For L2 UE-to-NW relay, Uu adaptation layer is used to support Remote UE identification for the UL traffic (multiplexing the data coming from multiple Remote UE).

Proposal-6: [Easy] agree the following description for L2 UE-to-NW relay

The Uu adaptation layer can be used to support DL bearer mapping at gNB to map end-to-end Radio Bearer (SRB, DRB) of Remote UE into Uu RLC channel over Relay UE Uu path

Proposal-15: [Easy] agree the following description for L2 UE-to-UE relay

For L2 UE-to-UE relay, the second hop PC5 adaptation layer can be used to support bearer mapping between the ingress RLC channels over first PC5 hop and egress RLC channels over second PC5 hop at Relay UE.

Proposal-25 [Easy]: agree the following description for L2 UE-to-NW relay

gNB implementation can handle the QoS breakdown over Uu and PC5 for the end-to-end QoS enforcement of a particular session established between Remote UE and network in case of L2 based UE to Network relaying.  Details of handling in case PC5 RLC channels with different e2e QoS are mapped to the same Uu RLC channel can be discussed in WI phase.

Proposal-26 [Easy]: agree the following description for L2 UE-to-UE relay

QoS handling for L2 UE-to-UE Relay is subject to upper layer, e.g. solution 31 within TR23.752 studied by SA2.

Proposal-32 [Easy] [merging P31]: agree the following description for L2 UE-to-NW relay

Relay UE can forward the system information to Remote UE via broadcast, groupcast, or dedicated PC5-RRC signalling. The detailed mechanisms of broadcast, groupcast and PC5-RRC signalling design can be discussed in WI stage.

Proposal-35 [Easy]: agree the following access control check principles for L2 UE-to-NW relay

	The Relay UE may provide UAC parameters to Remote UE

	The access control check is performed at Remote UE using the parameters of the cell it intends to access.

	The UE-to-Network Relay UE does not perform access control check for the Remote UE's data.

Agreements:

Proposal-5 (merging P4): agree the following description for L2 UE-to-NW relay

The identity information of Remote UE Uu Radio Bearer and Remote UE is included in the Uu adaptation layer at UL in order for gNB to correlate the received data packets for the specific PDCP entity associated with the right Remote UE Uu Radio Bearer of a Remote UE.

Proposal-7: agree the following description for L2 UE-to-NW relay

The Uu adaptation layer can be used to support DL N:1 bearer mapping and data multiplexing between multiple end-to-end Radio Bearers (SRBs, DRBs) of a Remote UE and/or different Remote UEs and one Uu RLC channel over the Relay UE Uu path

Proposal-8: agree the following description for L2 UE-to-NW relay

The Uu adaptation layer needs to support Remote UE identification for Downlink traffic

Proposal-10 (merging P9): agree the following description for L2 UE-to-NW relay

The identity information of Remote UE Uu Radio Bearer and the identity information of Remote UE needs be put into the Uu adaptation layer by gNB at DL in order for Relay UE to map the received data packets from Remote UE Uu Radio Bearer to its associated PC5 RLC channel.

Proposal-21: agree the following description for L2 UE-to-UE relay

Support the N:1 mapping by first hop PC5 adaptation layer between Remote UE SL Radio Bearers and first hop PC5 RLC channels for relaying.

Proposal-22: agree the following description for L2 UE-to-UE relay

Support the adaptation layer over first hop PC5 between Source Remote UE and Relay UE in order to identify traffic destined to different Destination Remote UEs.

Agreement:

Proposal-27: agree the following description for connection establishment procedure of L2 UE-to-NW relay:

Step 1. The Remote and Relay UE perform discovery procedure, and establish PC5-RRC connection using the legacy Rel-16 procedure as a baseline.

Step 2. The Remote UE sends the first RRC message (i.e. RRCSetupRequest) for its connection establishment with gNB via the Relay UE, using a default L2 configuration on PC5.  The gNB responds with an RRCSetup message to Remote UE. The RRCSetup delivery to the Remote UE uses the default configuration for L2 on PC5. If the relay UE had not started in RRC_CONNECTED, it would need to do its own connection establishment as part of this step. The details for Relay UE to forward the RRCSetupRequest/RRCSetup message for Remote UE at this step can be discussed in WI phase.

Step 3. The gNB and Relay UE perform relaying channel setup procedure over Uu. According to the configuration from gNB, the Relay/Remote UE establishes an RLC channel for relaying of SRB1 towards the Remote UE over PC5. This step prepares the relaying channel for SRB1.

Step 4. Remote UE SRB1 message (e.g. an RRCSetupComplete message) is sent to the gNB via the Relay UE using SRB1 relaying channel over PC5. Then the Remote UE is RRC connected over Uu.

Step 5. The Remote UE and gNB establish security following legacy procedure and the security messages are forwarded through the Relay UE.

Step 6. The gNB sets up additional RLC channels between the gNB and Relay UE for traffic relaying. According to the configuration from gNB, the Relay/Remote UE sets up additional RLC channels between the Remote UE and Relay UE for traffic relaying. The gNB sends an RRCReconfiguration to the Remote UE via the Relay UE, to set up the relaying SRB2/DRBs. The Remote UE sends an RRCReconfigurationComplete to the gNB via the Relay UE as a response.

Proposal 1a: Capture both the protocol stacks with and without PC5 adaptation layer for L2 UE-to-Network relay as candidate solutions in the TR, leave the down selection to WI phase (assuming down-selection first before studying too much on the detailed PC5 adaptation layer functionalities).

Proposal 1b: In the TR sec. 4.5.1.1, remove the Editor Note: “It is FFS if the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE.”. Add normal text “Whether the adaptation layer is also supported at the PC5 interface between Remote UE and Relay UE is left to WI phase.”

Proposal 2a: For L2 UE-to-UE relay, adaptation layer support the N:1 bearer mapping between multiple ingress PC5 RLC channels over first PC5 hop and one egress PC5 RLC channel over second PC5 hop and support the Remote UE identification function.

Proposal 2b: In the TR sec. 5.5.1, remove the Editor Note: “It is FFS on the details to support the N-to-1 mapping between the ingress RLC channels from multiple transmitting Remote UEs to egress RLC channels (going to the same Destination UE) at Relay UE.”

Proposal 2c: For L2 UE-to-UE relay, the identity information of Remote UE end-to-end Radio Bearer is included in the adaptation layer in first and second PC5 hop.

Proposal 2d: In addition, the identity information of Source Remote UE and/or the identity information of Target Remote UE are candidate information to be included in the adaptation layer, which is decided in WI phase.

Proposal 3: For L2 UE-to-UE relay connection establishment procedure, capture in the TR that “R2 consider the SA2 solution in TR 23.752 as baseline”. Further R2 impacts can be discussed in WI phase, if any.

Proposal 4: For L2 UE-to-NW relay, relay UE can support the relaying of the system information to the Remote UE(s) and what system information can be relayed to Remote UEs can be discussed at normative phase. On-demand SI request is supported for Remote UE for all RRC states (Idle/Inactive/Connected state).

Proposal 5: In L2 U2N relay, the paging relaying solution apply to both CN paging and RAN paging via option 2.

Proposal 6a: For L2 UE-to-Network relay, the RRC reconfiguration and RRC connection release procedures can reuse the legacy RRC procedure, with the message content/configuration design left to WI phase.

Proposal 6b: For L2 UE-to-Network relay, the RRC connection re-establishment and RRC connection resume procedures can reuse the legacy RRC procedure as baseline, by considering the agreed “connection establishment procedure of L2 UE-to-NW relay” to handle the relay specific part, with the message content/configuration design left to WI phase.

Proposal 7: In the TR sec. 4.5.5.1, remove the Editor Note: “It is FFS if this PC5 L2 configuration is a default configuration that can be overridden.”

Proposal 8: In the TR sec. 5.5.1, remove the Editor Note: “It is FFS if the adaptation layer is also supported over the first PC5 link (i.e. the PC5 link between the transmitting Remote UE and Relay UE).”

Proposal 9: In the TR sec. 4.5.1.2, remove the Editor Note: “It is FFS if N-to-1 bearer mapping from PC5 RLC channels to Uu interface RLC channel is supported for this case.”

Proposal 1: In L3 relay N3IWF solution (solution#23 in TR 23.752), RAN2 understanding is that remote UE’s NAS is sent over PC5/Uu-DRB. Include it in the status report LS to SA2 (discussed in email discussion [601]).

Proposal 2: If any AS impact of NAS transport in solution#23 is identified by SA2, RAN2 can further discuss it in WI phase.

Proposal 3: For the IP header overhead of L3 U2N relay with N3IWF, RAN2 conclude that outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact could be evaluated by SA2.

Proposal 4: Include conclusion of IP header overhead of L3 U2N relay with N3IWF in the status report LS to be sent to SA2 (discussed in email discussion [601]).

Proposal 5: No AS impact is identified for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure can be reused. Capture this conclusion in Section 4.6.2 of TR 38.836, and remove the following 2 Editor Notes:

Editor note: RAN2 can discuss AS impacts related to SA2 specified QoS solutions.

Editor note: RAN2 further discuss whether it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation.

ENs above can be replaced in the TR by an agreement “RAN2 can consider in WI phase SA2 conclusions on QoS solutions, including whether it is sufficient to enforce E2E QoS via legacy PC5-RRC reconfiguration of SLRB and resource allocation”.

Proposal 6: For L3 relay QoS management, RAN2 don’t intend to study the forward compatibility solution for multi-hop support.

Proposal 7: Both relay and remote UE can be in RRC_INACTIVE state.

Proposal 8: For security of L3 U2N relay:

1)	Capture “Solution#23 of TR 23.752 with N3IWF is feasible to meet end-to-end security requirements.” in Section 4.6.3 of TR 38.836.

2)	Remove the below Editor notes in Section 4.6.3 of TR 38.836

“Editor note: whether other security solution is introduced depends on SA2.”

“Editor note: RAN2 will evaluate any impact in RAN2 scope from these solutions”.

Proposal 9: For control plane procedure of L3 U2N relay, remove the following two Editor Notes in Section 4.6.5 of TR 38.836. Further AS impacts (if any) can be discussed in WI phase.

“Editor note: FFS if there is RAN2 impact to support the related control plane procedures.”

“Editor note: RAN2 will further consider procedures with RAN2 impact.”

Proposal 10: No RAN2 impact of SA2 solutions on L3 U2U relay (Solution#10/ Solution#31/ Solution#32) is identified and the design is in the scope of SA2

Proposal 11: For L3 U2U relay, capture below conclusions in TR 38.836:

1)	In Section 5.6.2, capture that “No RAN2 impact of the solution captured in SA2 TR 23.752 (solution#31) is identified and the design is in the scope of SA2”

2)	In Section 5.6.3, capture below conclusions in TR 38.836:

a.	Capture that “security protection of L3 U2U relay is in the scope of SA2 and SA3. No RAN2 impact is identified”;

b.	Capture an Editor-note: “whether the SA2 captured solutions can satisfy the security requirement depends on SA3.”

3)	In Section 5.6.4, capture “No RAN2 impact of the solutions captured in SA2 TR 23.752 (e.g. solution#10 and solution#32) is identified and the design is in the scope of SA2”

Agreement for “8.7.3.2	Service continuity”

Proposal 1-1 (20/22): The requirement of service continuity is only for U2N relay, but not for U2U relay, during mobility in this release.

Proposal 1-2 (22/22): R2 should study the mobility scenario of “between direct (Uu) path and indirect (via the relay) path” for U2N relay.

Proposal 1-3 (22/22): R2 deprioritize work specific to the mobility scenario of “between indirect (via a first relay UE) and indirect (via a second relay UE)” for path switching in the SI phase, which can be studied in the WI phase, if needed.

Proposal 1-5 (13+/22): R2 deprioritize the group mobility scenario in the SI phase, which may be discussed in WI phase, if needed.

Proposal 2-1 (19+/22): L2 U2N relay uses the RAN2 aspects of the R15 NR HO procedure as the baseline AS layer solution to guarantee service continuity (i.e. gNB hands over the remote UE to a target cell or target relay UE, including HO preparation type of procedure between gNB and relay UE (if needed), RRCReconfiguration to remote UE, remote UE switching to the target, and HO complete message, similar to the legacy procedure).  Exact content of the messages (e.g. HO command) can be discussed in WI phase.  This does not imply that we will send INM over Uu.

Proposal 1-4 (18+/22): R2 focus on the mobility scenarios of intra-gNB cases in the study phase. R2 assume the inter-gNB cases will also be supported. For the inter-gNB cases, compared to the intra-gNB cases, potential different parts on R2 Uu interface in details can be studied either in SI phase or in WI phase.

Proposal 2-2 (21/22): For service continuity of L2 U2N relay, the following baseline procedure is used, in case of remote UE switching to direct Uu cell.

	Step 1: Measurement configuration and reporting

	Step 2: Decision of switching to a direct cell by gNB

	Step 3: RRC Reconfiguration message to remote UE

	Step 4: Remote UE performs RA to the gNB

	Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in the RRC Reconfiguration message.

	Step 6: RRC Reconfiguration to relay UE

	Step 7: The PC5 link is released between remote UE and the relay UE, if needed.

	Step 8: The data path switching.

The order of step 6/7/8 is not restricted. Followings are further discussed in WI phase, including:

	Whether Remote UE suspends data transmission via relay link after step 3.

	Whether Step 6 can be before or after step 3 and its necessity.

	Whether Step 7 can be after step 3 or step 5, and its necessity/replaced by PC5 reconfiguration.

	Whether Step 8 can be after step 5.

Capture Figure 2-2 in the TR.

Figure 2-2: Procedure for remote UE switching to direct Uu cell

Proposal 2-3 (21/22): For service continuity of L2 U2N relay, the following baseline procedure is used, in case of remote UE switching to indirect relay UE:

	Step 1: Remote UE reports one or multiple candidate relay UE(s), after remote UE measures/discoveries the candidate relay UE(s).

	Remote UE may filter the appropriate relay UE(s) meeting higher layer criteria when reporting, in step 1.

	The reporting may include the relay UE’s ID and SL RSRP information, where the measurement on PC5 details can be left to WI phase, in step 1.

	Step 2: Decision of switching to a target relay UE by gNB, and target (re)configuration on relay UE   optionally (like preparation).

	Step 3: RRC Reconfiguration message to remote UE

	Step 4: Remote UE establishes PC5 connection with target relay UE, if the connection has not been setup yet.

	Step 5: Remote UE feedback the RRCReconfigurationComplete to gNB via target path, using the target configuration provided in RRCReconfiguration.

	Step 6: The data path switching.

Following are further discussed in WI phase, including:

	Whether Step 2 should be after relay UE connects to the gNB (e.g. after step 4), if not yet before.

	Whether Step 4 can be before step 2/3.

Capture Figure 2-3 in the TR.

Figure 2-3: Procedure for remote UE switching to indirect relay UE

Proposal 2-4 (21/22): For L2 U2N relay, following information may be included in the RRC Reconfiguration message from gNB to remote UE, in case remote UE switching to indirect relay UE: 1) Identity of the target relay UE; 2) Target Uu and PC5 configuration.

Proposal 3-1 (16+/22): Working Assumption: For service continuity in L3 U2N relay, R2 assume no AS layer solution will be studied to guarantee the service continuity, and leave it to the upper layer (e.g. application layer) solution. This does not exclude studying some enhancements in mobility scenario for other purposes.

Agreement for “8.7.3.3	Relay selection”

Proposal 1 [Easy]: Radio measurements at PC5 interface are considered as part of relay (re)selection criteria.

Proposal 2 [Easy]: Remote UE at least use “Radio signal strength measurements of Sidelink Discovery Messages” to evaluate whether PC5 link quality of a relay UE satisfies relay selection and reselection criterion.

Proposal 3: Remote UE may also use SL-RSRP measurements on the SIdelink unicast link to evaluate whether PC5 link quality with a relay UE satisfies relay reselection criterion.  Details e.g. in case of no transmission on the unicast link can be discussed in WI phase.

Proposal 4 [Easy]: For relay (re)selection, remote UE compares the PC5 radio measurements of a relay UE with the threshold which is configured by gNB or preconfigured.

Proposal 5 [Easy]:  “higher layer criteria” needs to be considered by remote UE for relay (re)selection, but details can be left to SA2 to decide.

Proposal 6 [Easy]:  Relay (re)selection can be triggered by upper layers of remote UE.

Proposal 7 [Easy]:  Relay reselection should be triggered if the NR Sidelink signal strength of current Sidelink relay is below a (pre)configured threshold.

Proposal 8: Relay reselection may be triggered if RLF of PC5 link with current relay UE is detected by remote UE.

Proposal 9 [Easy]: P1-P8, as a baseline for relay (re)selection,  apply to both U2N and U2U scenarios, and for both Layer 2 and Layer 3 solutions.

Proposal 10: For CONNECTED remote UE in Layer 2 U2N scenario, gNB decision on relay selection/reselection is considered in WI phase under the above baseline (P1-P9).

Proposal 12 [Easy]: Additional AS layer criteria can be considered in WI phase for both Layer 2 and layer 3 U2N relay solutions.

Proposal 14 [Easy]: Additional AS layer criteria can be considered in WI phase for both Layer 2 and layer 3 U2U relay solutions.

Proposal 15 [Easy]: For relay selection and reselection, when remote UE has multiple suitable relay UE candidates which meet all AS-layer & higher layer criteria and remote UE need to select one relay UE by itself, it is up to UE implementation to choose one relay UE.  This does not exclude gNB involvement in service continuity for U2N.

Agreement for “8.7.4	Discovery model and procedure for sidelink relaying”

Proposal1: To send a LS to SA2 to consult whether discovery message could be taken as PC5-S signalling or other new signalling in upper layer.  This can be included in R2-2010862 (offline discussion [601]).

Proposal2: Solution to differentiate discovery message in AS layer is also applicable for U2U relay

Proposal3: Both solutions of separate and shared resource pool (compared to data transmission resource pool) are captured in TR. They can be discussed in WI phase.

Proposal4: Discovery messages should be treated equally in terms of channel prioritization in LCP within the separate resource pool.

Proposal 5: For shared resource pool, to introduce a new LCID for discovery message i.e. it is taken as a new SL SRB

Proposal 9: L3 U2N relay UE is allowed to transmit discovery message based on at least pre-configuration when it is connected to a non_SL Relay_Capable gNB whose serving carrier is not shared with SL carrier. Detailed definition of non_SL Relay_Capable gNB can be left for WI phase but at least should include the case that the gNB does not provide SL relay configuration, e.g. no discovery configuration.

Proposal 10: L2 U2N relay UE should be always connected to a SL Relay Capable gNB for relay operation including transmission of discovery message

Proposal11: Remote UE supporting L2 relay is allowed to transmit discovery message (at least by preconfiguration) when it is directly connected to a non_SL Relay_Capable gNB whose serving carrier is not shared with SL carrier.

Proposal 12: Remote UE supporting L3 relay is allowed to transmit discovery message on its own based on at least pre-configuration when it is connected to a non-SL Relay_Capable gNB whose serving carrier is not shared with SL carrier. Detailed definition of non_SL Relay_Capable gNB can be left for WI phase.

Proposal 14: for L3 solution, it is not feasible for serving gNB to configure an out of coverage remote UE with radio configuration for transmission of discovery message

Proposal15: No additional network configuration is needed for measurement by remote UE in RRC_IDLE or RRC_INACTIVE.

Proposal16: For U2U relay operation, relay UE or remote UE is allowed to transmit discovery message when it is triggered by upper layer.

Proposal 17: Both remote UE and relay UE in U2U relay can rely on pre-configuration unless relevant radio configuration is provided by network, either via system information or dedicated signalling

2.2.2	Remaining Open issues

- Analysis/comparison for layer-2 based and layer-3 based relay architecture;

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA2

3.1.1	Agreements with cross-TSG impacts

For Key Issue #3 (Support of UE-to-Network Relay), the followings are taken as interim conclusions:

-	UE-to-Network Relay conclusions are subject to confirmation from RAN WG2 and SA WG3 for normative work.

-	The final decision on whether or not to proceed with Layer-2 and/or Layer-3 into normative work will be made in cooperation with other WGs.

The following is taken as interim conclusions for the L3 UE-to-Network Relay solution:

-	No showstopper has been identified by SA WG2 for L3 UE-to-Network solution. SA WG2 recommends L3 UE-to-Network Relay proceed into normative work, subject to RAN WG2 and SA WG3 conclusion:.

-	Solution #6 is taken as baseline in case the UE-to-Network Relay is a trusted entity by the Remote UE. Solution#23 is agreed to be used to provide end-to-end security for the Remote UE in case the UE-to-Network Relay is not a trusted entity by the Remote UE.

-	For L3 Relay discovery procedure, it is proposed to adopt the standalone discovery procedure (i.e. Model A and Model B), and, the additional information advertised by Relay UE as described in Sol#28 as the basis for normative work.

-	For the L3 relay operation support, Remote UE uses URSP rules to route the traffic on suitable communication path (as described in Sol#26).

-	Security aspects require confirmation from SA WG3.

-	For the Remote UE to use the network resources (e.g. PDU Session and Network Slice) of the Relay UE's serving network, the network-controlled authorization procedures will be determined in the normative phase with coordination with SA WG3. The alignment with the associated security procedures to authenticate the Remote UE and Relay UE will be carried out in normative phase via coordination with SA WG3.

-	The secondary authentication for a Remote UE will be determined by SA WG3. The alignment with the associated security procedures for secondary authentication of the Remote UE will be carried out in normative phase via coordination with SA WG3.

-	For QoS handling, following aspects in Solution #24 and Option #2 of Solution #25 are selected as basis for normative work:

-	L3 Relay can be configured with the 5QIs and PQIs mapping. Based on the mapping or, in case of a non-configured mapping of a requested QoS parameter, based on its implementation, the L3 relay translates the Uu QoS parameters to PC5 QoS parameters and vice versa.

-	To support the dynamic QoS handling, relay UE determines the Uu QoS parameters and PC5 QoS parameters by taking into account the end-to-end QoS requirements provided by remote UE based on its configured QoS mapping information or, in case of a non-configured mapping of a requested QoS parameter, based on its implementation, and initiates PDU session modification procedure and L2 link modification procedure to setup corresponding QoS Flows over Uu and PC5.

-	The SMF of the L3 Relay provides the corresponding QoS rules and flow level QoS parameters to the L3 Relay as part of the PDU session establishment or modification procedures as defined in TS 23.502 [8], clause 4.3.2 and 4.3.3. Alternatively, reflective QoS control over Uu as defined in TS 23.501 [6], clause 5.7.5.3 can be leveraged for dynamic QoS handling of Remote UE to save on signalling between SMF and L3 Relay.

-	Based on signalled QoS rules (via SMF) or derived QoS rules (Uplink Uu via reflective QoS), the UE-to-Network Relay may use the L2 Link Modification procedures as defined in TS 23.287 [5], clause 6.3.3.4 to either move the corresponding ProSe service(s) to the mapped existing PC5 QoS flow or to set up a new PC5 QoS flow.

The followings are taken as interim conclusions for the L2 UE-to-Network Relay solution:

-	No showstopper has been identified by SA WG2 for L2 UE-to-Network Relay solution. SA WG2 recommends L2 UE-to-Network Relay solution proceed into normative work, subject to RAN WG2 and SA WG3 conclusion.

-	Adopt Control and User Plane Protocols as described in Annex A, with confirmation from RAN WGs.

-	The Remote UE has a NAS connection with 5GC and Remote UE Registration and Connection establishment/management, the related procedure in solution #7 can be taken as baseline.

-	For Relay discovery, the standalone discovery is used, and both Model A and Model B are supported.

-	For paging the concluded solution in clause 6.6.2 of TR 23.733 [26] can be reused based on the assumption captured in clause 4.5.5.2 of TR 38.836 [32] adopted by RAN WG2.

For Key Issue #4 (Support of UE-to-UE Relay), the followings are taken as interim conclusions:

- UE-to-UE Relay conclusions are subject to confirmation from RAN2 and SA3 for normative work.

- The final decision on whether or not to proceed with Layer-2 and/or Layer-3 into normative work will be made in cooperation with other WGs.

The followings are taken as interim conclusion for Layer-3 UE-to-UE relay:

-	No showstopper has been identified by SA2 for L3 UE-to-UE solution. SA2 recommends L3 UE-to-UE Relay proceed into normative work, subject to RAN2 and SA3 conclusion.

- 	L3 UE-to-UE relay solution can support relaying of IP and non-IP traffic. For IP traffic, the IP addresses of the UEs can be either assigned by the relay (as described in sol#10) or self-assigned (as described in sol#32). For Non-IP traffic, it can be either handled via IP encapsulation or without IP encapsulation (as described in sol#49).

-	UE-to-UE Relay discovery and selection are supported by:

- Model A discovery (as described in sol#11);

- Model B discovery (as described in sol#8); and

- Integrated PC5 unicast link establishment procedure (as described in sol#8).

-    UE-to-UE relay reselection

The relay reselection can be viewed just like redoing the relay selection as described in Sol#8 or be performed as described in Sol#50. The reselection criteria are to be coordinated with RAN2 WG.

-    QoS support

End-to-end QoS support for Remote UE is provided by splitting the QoS between the two PC5 links between the source UE and target UE. QoS splitting configuration can be provided from PCF as part of policy to both Remote UE and Relay UE or the QoS splitting can be managed by the Relay UE based on the end-to-end QoS needs. For QoS handling, Sol#31 can be considered as the starting point for the normative work.

The following are taken as interim conclusions for the L2 UE-to-UE Relay:

-	No showstopper has been identified by SA WG2 for L2 UE-to-UE solution. SA WG2 recommends L2 UE-to-UE Relay proceed into normative work.

-	For UE-to-UE Relay discovery, both Model A and Model B are supported. It is recommended that Relay discovery is integrated into the PC5 unicast link establishment procedure.

-	For QoS handling, Sol#31 can be taken as baseline.

-	For Relay reselection, the negotiated UE-to-UE Relay reselection in Sol#50 and the Relay selection in Sol#8 can be used under different conditions. Both Sol#50 and Sol#8 can be taken as baseline.

3.1.2	Remaining Open issues with cross-TSG impacts

There are following editor note in TR 23.752 which indicate the open issues with RAN dependency.

For Key Issue #4 (Support of UE-to-UE Relay): for L2 UE-to-UE Relay

NOTE 1:	The operation procedures for supporting the L2 UE-to-UE Relay need coordination with RAN2 to decide how the UE-to-UE Relay performs the data/signalling routing.

NOTE 2:	It is left to RAN2 to support the QoS enforcement in AS layer.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R2-2008760	LS on Direct Discovery and Relay in SA2 (S2-2006587; contact: Oppo)	SA2

- R2-2008777	Left issues on CP procedure for L2 U2N Relay	OPPO

- R2-2008778	Left issues on QoS, Security and L23 comparison	OPPO

- R2-2008779	Left issues on Scenarios for sidelink relay	OPPO

- R2-2008780	Left issues on Service continuity for L2 U2N relay	OPPO

- R2-2008802	Discussion on AS layer protocol of discovery message for SL relay	OPPO

- R2-2008815	Summary of  [Post111-e][623][Relay]Remaining issues on relay discovery (rapporteur)	OPPO

- R2-2008921	Further Clarification on the Scenarios for NR Sidelink Relay	CATT

- R2-2008922	On-demand SI Delivery for Remote UE	CATT

- R2-2008923	Further Clarification on the L2 Service Continuity	CATT

- R2-2008924	Further Discussion on NR Sidelink Relay Selection and Reselection	CATT

- R2-2008925	Discussion on discovery message	CATT

- R2-2008926	[Draft] Reply LS on Direct Discovery and Relay	CATT

- R2-2008939	Work planning of R17 SL relay	OPPO

- R2-2008962	Discussion on remaining issues of L3 relay	Qualcomm Incorporated

- R2-2008964	Discussion on remaining issues of L2 relay	Qualcomm Incorporated

- R2-2008965	Remaining issues on discovery  and relay (re)selection 	Qualcomm Incorporated

- R2-2008966	RRC state and essential RRC procedures in L2 U2N relay	Qualcomm Incorporated

- R2-2008967	Remaining issues on the mobility procedures for L2 relay	Qualcomm Incorporated

- R2-2008977	Further details on SL discovery for relaying	Intel Corporation

- R2-2008983	Open aspects of L2 relaying	Intel Corporation

- R2-2008987	Further details on relay reselection	Intel Corporation

- R2-2009029	Discussion on Relay initiation and (re-)selection	ZTE Corporation, Sanechips

- R2-2009030	Discussion on remaining issues on L2 relay architecture	ZTE Corporation, Sanechips

- R2-2009031	Discussion on Service continuity	ZTE Corporation, Sanechips

- R2-2009032	Discussion on relay discovery and link management	ZTE Corporation, Sanechips

- R2-2009033	Discussion on Remaining issues on L3 relay	ZTE Corporation, Sanechips

- R2-2009068	L3 relay enhancements to improve path switching	Nokia, Nokia Shanghai Bell

- R2-2009069	Discussion on relay selection and reselection	Nokia, Nokia Shanghai Bell

- R2-2009122	Email Report of Post111-e 627 Relay Remaining issues on L2 architecture	MediaTek Inc.

- R2-2009123	Adaptation layer  for PC5 at L2 UE-to-Network Relay	MediaTek Inc.

- R2-2009124	Overhead in N3IWF based L3 relaying architecture	MediaTek Inc.

- R2-2009125	Service Continuity for L2 Relay and L3 Relay	MediaTek Inc.

- R2-2009144	Remaining issues on the adaptation layer for Layer-2 Relay	Spreadtrum Communications

- R2-2009145	Discussion on service continuity for Layer-2 UE-to-Network Relay	Spreadtrum Communications

- R2-2009148	Discussion on relay selection and reselcetion	Spreadtrum Communications

- R2-2009149	Discussion on remaining issues on relay discovery	Spreadtrum Communications

- R2-2009171	Service continuity via L3 UE-to-Network relaying	Samsung Electronics

- R2-2009172	Consideration on relay reselection criteria	Samsung Electronics

- R2-2009173	Sidelink relay discovery open issue	Samsung Electronics

- R2-2009176	Relay (re)selection enhancement	MediaTek Inc.

- R2-2009177	Service Continuity Scenarios and AS-Layer Procedures	Fraunhofer HHI, Fraunhofer IIS

- R2-2009202	Control Plane Aspects for UE to NW Relays	InterDigital

- R2-2009203	Connection Establishment and Maintenance for L2 Relays	InterDigital

- R2-2009204	Discovery Procedure for SL Relays	InterDigital

- R2-2009205	Relay Selection and Reselection	InterDigital

- R2-2009206	Discussion on L2 Relay Architecture and QoS	InterDigital

- R2-2009228	Remaining aspects for discovery	Ericsson

- R2-2009229	Remaining aspects for relay selection and reselection	Ericsson

- R2-2009230	RAN2 impacts introduced by Layer 2 SL relay	Ericsson

- R2-2009271	Further details on Service Continuity for Relaying	Intel Corporation

- R2-2009301	Service Continuity with Sidelink Relay	Futurewei

- R2-2009302	QoS Control with Sidelink Relay	Futurewei

- R2-2009476	Discussion on service continuity for layer 2 UE to NW relay	Apple

- R2-2009523	Summary Report of [Post111-e][622][Relay] Relay selection and reselection	Apple

- R2-2009524	Discussion on remaining issues on NR Sidelink Relay discovery	Apple

- R2-2009525	Discussion on data forwarding mechanisms for Layer 2 UE-to-UE Relay	Apple

- R2-2009526	Discussion on RRC_INACTIVE remote UE	Apple

- R2-2009584	Further discussion on scope and scenarios of SL relay	vivo

- R2-2009585	Open issues on Layer-2 relay	vivo

- R2-2009586	Service continuity for L2 and L3 relay	vivo

- R2-2009587	Remaining issues of sidelink relay discovery procedure	vivo

- R2-2009588	SL-RSRP and SD-RSRP comparioson and additional criterion for relay (re-)selection	vivo

- R2-2009633	Considerations on discovery for sidelink relay	KT Corp.

- R2-2009634	Considerations on relay selection and reselection	KT Corp.

- R2-2009638	Discussion on differentiation of discovery message	SHARP Corporation

- R2-2009650	View on Paging Option 2 in L2 relay	ITL

- R2-2009660	L2 relaying open issues	Samsung Electronics GmbH

- R2-2009661	Need for relaying of on-demand SI	Samsung Electronics GmbH

- R2-2009693	Coverage Extension using Relays	Lenovo, Motorola Mobility

- R2-2009694	QoS support when using Relays	Lenovo, Motorola Mobility

- R2-2009720	Discussion on L3 UE-to-NW relay architecture	Ericsson

- R2-2009721	Service continuity procedure and scenarios for sidelink relay	Ericsson

- R2-2009857	Relay reselection in the failure case	Lenovo, Motorola Mobility

- R2-2009858	Considerations on the UE-to-Network relay and UE-to-UE relay case	Lenovo, Motorola Mobility

- R2-2009891	SL L2 architectrure	Sony

- R2-2009892	SL Relay selection	Sony

- R2-2009901	Protocol stack design for U2N relay and U2U relay case	Lenovo, Motorola Mobility

- R2-2009938	Service Continuity for UE2UE Relay	Nokia, Nokia Shanghai Bell

- R2-2009939	Discussion on L2 based UE-to-Network	Nokia, Nokia Shanghai Bell

- R2-2009970	NR Sidelink Relaying Discovery	Fraunhofer IIS, Fraunhofer HHI

- R2-2009972	NR Sidelink Relay (Re-)Selection Criterion and Procedure	Fraunhofer IIS, Fraunhofer HHI

- R2-2009994	Discovery resources for sidelink relaying 	Kyocera

- R2-2010005	Relay reselection based on discovery 	Kyocera

- R2-2010046	Discussion on relay discovery model and procedure	Nokia, Nokia Shanghai Bell

- R2-2010104	Release procedure for SL Relaying support  	Intel Corporation

- R2-2010129	Needed Information in Adaptation Layer Header for L2 UE-to-UE Relay	Convida Wireless

- R2-2010329	Clarification of remote UE mobility	ETRI

- R2-2010331	On relay discovery	MediaTek Inc.

- R2-2010344	Remaining issues on protocol stacks and procedures for L2 relay	Huawei, HiSilicon

- R2-2010345	NAS transmission and QoS management in L3 U2N relay	Huawei, HiSilicon

- R2-2010346	Summary email discussion [621][Relay] of Service continuity	Huawei, HiSilicon

- R2-2010347	Remaining issues on relay selection and reselection	Huawei, HiSilicon

- R2-2010348	Remaining issues on relay discovery	Huawei, HiSilicon

- R2-2010349	Discussion on the discovery aspects related to SA2 LS S2-2006587	Huawei, HiSilicon

- R2-2010467	Discussion on scenario regarding non SL relay capable gNB	Xiaomi communications

- R2-2010469	Discussion on service continuity	Xiaomi communications

- R2-2010588	Service continuity for SL relay	LG Electronics Inc.

- R2-2010652	PC5 link failure handling for NR sidelink relay	LG Electronics Inc.

- R2-2010658	Scenarios for NR sidelink relay	LG Electronics Inc.

- R2-2010659	Service continuity for Remote UE	LG Electronics Inc.

- R2-2010660	Remaining issues on discovery for NR sidelink relay	LG Electronics Inc.

- R2-2010661	Summary of  [Post111-e][623][Relay]Remaining issues on relay discovery (rapporteur)	OPPO

- R2-2010676	[Draft] Reply LS on Direct Discovery and Relay	OPPO

- R2-2010693	LS on SA2 progress on UE-to-Network Relay and UE-to-UE Relay (S2-2007945; contact: OPPO)	SA2

- R2-2010984	Summary for AI_8.7.2 Scope requirements and scenarios	vivo

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template