3GPP TSG RAN meeting #90e										RP-202264

Electronic Meeting, December 7-11, 2020

## Status Report to TSG

Agenda item:			13.5.9

| WI / SI Name | UE Conformance Test Aspects for Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: No |  | Testing part: Yes |
| Acronym | NR_eMIMO-UEConTest |  |  |  |  |
| Unique ID | 880070 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-200895 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: N/A | Performance part: N/A | Testing part: Dec. 2021 |  |
| Overall Completion level | Study Item: N/A | Core part: N/A | Performance Part: N/A | Testing part: 8% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG5 |
| --- | --- | --- |
| Rapporteur | Name | Chunying GU |
|  | Company | Huawei Technologies Co., Ltd |
|  | Email | guchunying@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

WP is provided as [1].

SIG

Test case 7.1.1.3.10 for Correct Handling of UL HARQ process / Multiple CORESETPoolIndex is completed [4, 5]. Necessary PICS are specified in 38.508-2 [3].

It’s discussed to test RANK 4 PMI reporting implicitly instead of a separate test case. Detailed way forward needs further discussion.

RF

For Full power transmission, it was discussed and agreed to test both Rank 2 and Rank 1 with full power transmission for capable UEs. Test cases are introduced in [7-11]. TP analysis are in [15, 16]. To enable scheduling of full power transmission, the default message of DCI_0_1 is added in 38.508-1 [2].

For Low PAPR DMRS sequence, the test point of additional MPR requirements are analysed [17] and implemented in MPR/SEM/ACLR test cases [12-14].

#### 2.5.2	Remaining Open issues

See details in WP [1].

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

## 4.	References

General documents

[1]	R5-205909	WP - Enhancements on MIMO for NR	Huawei, HiSilicon

38.508-1 documents

[2]	R5-205932	Addition of IE configuration for ULFPTx to clause 5	Huawei, HiSilicon	38.508-1

38.508-2 documents

[3]	R5-206414	Adding UE capabilities for eMIMO	Lenovo, Motorola Mobility	38.508-2

38.523-1 documents

[4]	R5-206415	Addition of eMIMO MAC Test Case	Lenovo, Motorola Mobility	38.523-1

38.523-2 documents

[5]	R5-206416	Addition of applicability for eMIMO Test Cases	Lenovo, Motorola Mobility	38.523-2

38.521-1 documents

[6]	R5-205557	Updating abbreviation list	Ericsson	38.521-1

[7]	R5-205796	Update of MOP for UL MIMO with ULFPTx in 6.2D.1	Huawei, HiSilicon	38.521-1

[8]	R5-206768	Addition of new test case 6.5D.3_1.1 general spurious emissions for Rel-16 UL MIMO	Huawei, HiSilicon	38.521-1

[9]	R5-205799	Addition of new test case 6.5D.3_1.2 UE co-existence spurious emissions for Rel-16 UL MIMO	Huawei, HiSilicon	38.521-1

[10]	R5-205800	Addition of new test case 6.5D.3_1.3 additional spurious emissions for Rel-16 UL MIMO	Huawei, HiSilicon	38.521-1

[11]	R5-206894	Addition of test case 6.5D.1_1, Occupied bandwidth for UL MIMO (Rel-16 onward)	Ericsson	38.521-1

[12]	R5-205791	Update of MPR for PC3 half Pi BPSK DMRS in 6.2.2	Huawei, HiSilicon	38.521-1

[13]	R5-206766	Update of SEM for PC3 half Pi BPSK DMRS in 6.5.2.2	Huawei, HiSilicon	38.521-1

[14]	R5-206767	Update of NR ACLR for PC3 half Pi BPSK DMRS in 6.5.2.4.1	Huawei, HiSilicon	38.521-1

38.905 documents

[15]	R5-206858	Update of test point analysis for MOP in FR1	Huawei, HiSilicon	38.905

[16]	R5-206769	Update of test point analysis for Tx spurious emissions in NR FR1	Huawei, HiSilicon	38.905

[17]	R5-206857	Update of test point analysis for MPR, SEM and ACLR in NR FR1	Huawei, HiSilicon	38.905

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template