3GPP TSG RAN meeting #90e									        	RP-202344

Electronic Meeting, December 7 - 11, 2020

## Status Report to TSG

Agenda item:			9.8.10

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB_enh |  |  |  |  |
| Unique ID | 860050 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201293 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 06/2021 | Performance part: 12/2021 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 9% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #103-e

Agreement

The Rel-16 IAB-DU resource types (Soft/Hard/NA) are the starting point for supporting resource multiplexing for simultaneous operation cases in Rel-17.

- FFS: Whether resource type definitions need to be extended to frequency domain resources

- FFS: Coexistence of simultaneous operation resources and TDM resources

- FFS: Whether new rules governing cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT in case of simultaneous operation are necessary

Agreement

Further consider different applicability restrictions/conditions for simultaneous operation multiplexing cases:

- FFS: Whether a given case is only applicable for certain resource types or combinations: e.g. DL access, DL backhaul, UL access, UL backhaul

- FFS: Network (including parent node) awareness of a child IAB node’s ability to support simultaneous operation due to short-term and long-term factors including panel selection, interference, timing, transmit power, capability indication etc.

- FFS: Necessary differentiation for paired spectrum vs. unpaired spectrum

- FFS: Whether specific enhancements are defined for full-duplex cases vs. being left to implementation (as in Rel-16)

- Note: There should not be any impact on legacy UE behavior

Agreement

The Rel-16 explicit indication of soft resources by DCI Format 2_5 is supported for simultaneous operation cases in Rel-17.

- FFS: Whether/how to extend DCI Format 2_5 to frequency domain resources and/or paired spectrum

- FFS: Coexistence of simultaneous operation resources and TDM resources

Agreement

From a RAN1 perspective, at least intra-donor multi-parent operation is supported in Rel-17

- FFS: Inter-donor operation pending additional input from RAN2/RAN3

Agreement

The explicit indication of soft resources by DCI Format 2_5 is supported for multi-parent scenarios in Rel-17.

- FFS: Whether additional enhancements over the Rel-16 solution are needed

Agreement

From a RAN1 perspective, resource multiplexing and coordination is supported for the following DC scenarios in Rel-17.

- Inter-carrier, inter-band

- Inter-carrier, intra-band is additionally supported at least for FR2

  - At least to the extent it reuses solutions for supporting Inter-carrier, inter-band

  - FFS: whether specific enhancements for inter-carrier, intra-band DC are introduced in Rel-17

Agreement

Select one or both of the following modes of operation for Case 7 timing in RAN1#104-e:

- symbol level alignment without slot level alignment

- slot level alignment

Agreement

Interference management for the following IAB interference scenarios should be discussed:

- Inter-IAB scenarios, including:

  - MT to MT, DU to DU, DU to MT, and MT to DU.

- Interference to non-IAB nodes, including:

  - IAB-DU to non-IAB-DU

  - IAB-MT to non-IAB-DU

- Intra-IAB-node (self-interference) scenarios (Interference between a DU and MT of an IAB-node).

This agreement does not necessarily mean that specification support is needed for any of the scenarios.

Agreement

Consider resource and beam coordination techniques to mitigate/avoid interference, including (not an exhaustive list):

- FFS: whether or not to support IAB‐node (MT) transmissions in DL access slots

  - FFS: if this has RAN1 impact or it can be handled by implementation.

  - FFS: network coordination impact

- FFS: whether Rel-16 resource management framework is sufficient.

Agreement

An IAB-node can rely on an OTA timing synchronization mechanism to enable/maintain Case 6 timing mode

- FFS whether the Rel-16 OTA synchronization mechanism is sufficient or enhancements are required

  - If required, details of enhancements including the uplink timing(s) required to support different timing alignment cases

Agreement

An IAB-node, when operating in Case 7 timing mode, can enable a child node to set its DL Tx timing based on Rel-16 OTA timing synchronization mechanism.

- FFS whether Rel-16 OTA synchronization mechanism enhancements are required

  - FFS details of enhancements, if required

Agreement

Case 6 timing mode operation at an IAB-node is controlled by the parent node to which the UL transmission is intended for.

Agreement

Use the Rel-16 interference management frameworks (e.g. CLI, RIM) to handle IAB interference scenarios, and discuss if any of the following enhancements are needed (not an exhaustive list):

- FFS: extend the information exchange (e.g. the resource configuration, result of CLI measurements, etc.) among different entities (e.g. between parent-child nodes, adjacent IAB nodes, between network and IAB-node, etc.)

- FFS: required enhancements on CLI measurement accuracy (e.g. via timing adjustment, etc.)

- FFS: required enhancements on CLI measurements (e.g. introducing short-term measurements, multi-beam measurements, etc.)

Agreement

Further study requirement of enhanced DL and UL Tx power control mechanism considering the following:

- DL/UL power control with assistance information from the child node.

- DL/UL power control with assistance information from the parent node.

- Central (e.g. by CU) power control coordination (e.g. semi-static max DL/UL Tx power limits).

- Coexistence of different power control mechanisms within an IAB node and in the network.

Note. Any power control mechanism should consider the following aspects:

- Existing base station design principles (e.g. power control and dynamic range capability, etc.) related to transmission power.

- Network constraints in regard to transmitted reference signals with constant power.

#### 2.1.2	Remaining Open issues

25% of the items defined in the RAN1 WID objectives have been accomplished. The following items defined in the WID objectives require further work:

Duplexing enhancements:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #112-e

Agreements on Enhancements to improve topology-wide fairness multi-hop latency and congestion mitigation:

- R2 assumes Rel-17 IAB work will not define any new end-user QoS metrics on top of the existing 5G QoS framework.

- Rel-17 IAB work will comprise agreeing on a definition of topology-wide fairness.

- Topology-wide fairness provides mechanisms for the management of QoS so that the required QoS is met across the topology, regardless of where a UE attaches to the IAB network. Variants of this definition is not precluded. FFS how the success of such mechanisms is evaluated.

- RAN2 will not discuss enhancements to DL E2E flow control without input from RAN3

- FFS if RAN2 will deprioritize splitting data of a radio bearer into two or more paths (RAN3 agreements to deprioritize Multi-Route Support with data split in IAB)

Agreements on Topology adaptation enhancements:

- Consider enhancements to topology adaptation that improve:

  - Robustness, e.g., to rapid shadowing,

  - service-interruption,

  - load balancing among different IAB-nodes, IAB-donor-DUs and IAB-donor-CUs, and

  - reduction in signaling load.

- RAN2 to discuss enhancements to RLF indication/handling with the focus on the reduction of service interruption after BH RLF.

- CHO and potential IAB-specific enhancements of CHO is on the table.

- DAPS and potential IAB-specific enhancements of DAPS is not precluded for now (but as there is no PDCP it is not clear how to support DAPS).

- For message bundling, RAN2 at least wait for more progress to be made in RAN3 on topology adaptation procedures.

- RAN2 to discuss local rerouting, including the benefits over central route determination, and on how topology-wide objectives can be addressed.

#### 2.2.2	Remaining Open issues

3% of the items defined in the RAN2 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #110-e

Agreements on topology adaptation enhancements:

- For IAB nodes connected to a single donor, IAB-MT migration between IAB-donors can support robustness and load balancing; the Xn handover preparation procedure is taken as baseline.

- For IAB nodes connected to 2 donors, robustness and load balancing can be supported by using simultaneous connectivity

- It is not precluded for an IAB node to have simultaneous F1 interfaces to 2 donor CUs using the concept of separate logical IAB-DUs in the same physical node

- Given that the IAB-DU cells can only be configured by one donor at a time, the timing for the switching of such cells with respect to the migration of the collocated IAB-MT are FFS

- As a consequence of adopting the Xn HO prep procedure as BL, the new IAB-donor needs to have an F1AP association with the IAB-DU holding the target cell before responding to the initiating message of the UE migration procedure

- UE-migration to the new IAB-donor requires security context/key change

- For IAB-MT migration, continue to discuss full and gradual sequences to migrate IAB-MT, UEs and descendent nodes

Agreements on CHO and DAPS:

- Discuss how to support simultaneous connectivity with 2 donors, to reduce service interruption; potential solutions may include dual-protocol-stack solutions (“DAPS-like”); FFS whether the same solution also applies to descendant nodes

- The simultaneous connectivity dual-protocol-stack solutions (“DAPS-like”) of an IAB node should allow at least DL simultaneous transmission of BH traffic carried on BH RLC channels, on the paths to both donors.

- Rel-16 CHO can be considered as baseline for the discussion of CHO for IAB; further analysis is expected

- R3-207184C:\Users\ghampel\AppData\Local\Temp\Temp1_RAN3_110-e_agenda_with_Tdocs20201112_EOM.zip\Inbox\R3-207184.zip: LS to RAN2 on UL simultaneous transmission for dual-protocol-stack solutions (“DAPS-like”)

Agreement on reduction of service interruption:

For intra-Donor case:

- The RRCReconfiguration to the descendant IAB can be transferred via the source path, i.e. before the migrating IAB detach from source parent cell.

- Study the packet loss mitigation in intra-donor migration, e.g. further clarify the scenario for packet loss and possible solutions.

- Discuss the avoidance of unnecessary transmissions in intra-donor migration (including the scenario of RLF recovery), with focus on RAN3 impact.

For inter-Donor case:

- Study the solution for the baseline RLF scenario, where IAB node experiencing RLF can connect only to 1 donor at a time.

- An RRC indication is provided to the migrating IAB node on whether it is undergoing inter- or intra-donor migration. This indication also applies to RLF recovery. FFS on the content of the indication.

- The issue on Reduction of Service Interruption for inter-Donor case will be discussed after the basic migration procedure is determined.

Agreements on topology redundancy:

CP-UP separation:

- In Rel-17 eIAB, the following two scenarios are supported for CP-UP separation:

  - Scenario 1: F1-C uses NR access link via M-NG-RAN node (non-donor node) + F1-U uses backhaul link via S-NG-RAN node (donor node)

  - Scenario 2: F1-U uses backhaul link via M-NG-RAN node (donor node) + F1-C uses NR access link via S-NG-RAN node (non-donor node)

- R3-207198: LS to RAN2 on the support of CP-UP separation

Inter-donor topology redundancy:

- WA: In Rel-17, RAN3 agrees to support the following scenarios for inter-donor topology redundancy with the principle that an IAB-DU only has F1 interface with one Donor-CU:

  - Scenario 1: the IAB node is multi-connected with 2 Donors.

  - Scenario 2: the IAB node’s parent/ancestor node is multi-connected with 2 Donors.

- Agree LS to RAN1 (cc to RAN2)

- The inter-donor topology redundancy is applicable for F1-U traffic:

  - FFS on how to support data transmission of UE bearers via 2 donors.

  - FFS on the granularities of the load balancing for F1-U traffic.

- The inter-donor topology redundancy is applicable for F1-C traffic. FFS on granularities for F1-C traffic.

- As a starting point, the F1 interface of the boundary IAB node and descendant IAB node(s) terminate to the same donor. The following open issues need further discussion:

  - FFS at which of the two donors these F1 interfaces terminate

  - FFS if boundary and descendent IAB-nodes can have their F1 interfaces terminate at different donors.

- In inter-donor topology redundancy, the traffic may be sent from one donor CU directly to the donor DU of another donor and further towards the IAB node, without passing through additional donor CU(s).

- R3-207199: LS to RAN1 (cc RAN2) on the support inter-donor topological redundancy

Agreements on congestion mitigation:

- An IAB node at the parent side of a congested backhaul link may send a congestion indication to the IAB-donor-CU-CP.

- So far the following solutions for IAB DL end-to-end flow control are on the table:

  - Highest PDCP SN received from parent node;

  - Bitmap of PDUs transmitted to lower layers out of sequence;

  - Packet marking;

  - Received volume and Receiving data rate.

  - “do nothing” option, i.e. use current DDDS as it is

Agreements on multi-hop performance: QoS, latency, fairness:

- Topology-wide fairness can be discussed in RAN2 first.

- Local re-routing in other scenarios, e.g. congestion mitigation, load balancing can be discussed in RAN2 first.

- Inter-donor-DU local re-routing in Rel-17 IAB should be supported; details are FFS

#### 2.3.2	Remaining Open issues

8% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.4	RAN4

#### 2.4.1	Agreements

None. Work has not started.

#### 2.4.2	Remaining Open issues

All objectives from WID on enhancements to IAB for NR.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #103-e

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2007594 | Resource multiplexing between backhaul and access for IAB duplexing enhancements | Huawei, HiSilicon |
| R1-2007595 | Enhancements for simultaneous operation of MT and DU | Huawei, HiSilicon |
| R1-2007684 | Enhancement to resource multiplexing between child and parent links | vivo |
| R1-2007685 | Other enhancements for simultaneous operation of child and parent links | vivo |
| R1-2007686 | Other enhancements to Rel-17 IAB nodes | vivo |
| R1-2007786 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | Fujitsu |
| R1-2008029 | Discussion on resource multiplexing in the simultaneous operation | CMCC |
| R1-2008030 | Discussion on the enhancement for simultaneous operation | CMCC |
| R1-2008184 | Enhancements to Resource Multiplexing for NR IAB | Samsung |
| R1-2008185 | Enhancements to Timing, Power Control and CLI for NR IAB | Samsung |
| R1-2008312 | Enhancements for IAB resource multiplexing | AT&T |
| R1-2008313 | Enhancements for IAB interference management | AT&T |
| R1-2008314 | Performance of simultaneous operation of IAB-node’s child and parent links | AT&T |
| R1-2008327 | Other enhancements for IAB | Huawei, HiSilicon |
| R1-2008406 | Discussions on IAB resource multiplexing enhancements | LG Electronics |
| R1-2008407 | Discussions on other enhancements for simultaneous operation | LG Electronics |
| R1-2008408 | Discussion on RACH procedure for IAB enhancement | LG Electronics |
| R1-2008816 | Discussion on simultaneous operation of IAB-node’s child and parent links | CEWiT |
| R1-2008858 | Enhancements to the IAB resource multiplexing | ZTE, Sanechips |
| R1-2008859 | Enhancements for simultaneous operation of child and parent links | ZTE, Sanechips |
| R1-2008860 | Discussion on multiplexing capability for IAB | ZTE, Sanechips |
| R1-2008863 | Enhancements for resource multiplexing among IAB backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-2008864 | Other enhancements for simultaneous operation of IAB Parent and Child | Nokia, Nokia Shanghai Bell |
| R1-2008995 | Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node | Intel Corporation |
| R1-2008996 | Power Control Enhancements for Simultaneous Operations | Intel Corporation |
| R1-2009018 | Enhancements to resource multiplexing between child and parent links of an IAB node | ETRI |
| R1-2009019 | Discussion on simultaneous operation enhancements | ETRI |
| R1-2009020 | Other enhancements to IAB | ETRI |
| R1-2009108 | Enhancements to resource multiplexing for IAB | Lenovo, Motorola Mobility |
| R1-2009109 | Enhancements for simultaneous operation in IAB systems | Lenovo, Motorola Mobility |
| R1-2009137 | Power Control: Subsequent Contribution | Sharp |
| R1-2009190 | Resource multiplexing between child and parent links of an IAB node | NTT DOCOMO, INC. |
| R1-2009191 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | NTT DOCOMO, INC. |
| R1-2009220 | Discussion on IAB resource multiplexing enhancements | ETRI |
| R1-2009221 | Discussions on enhancements to resource multiplexing between child and parent links of an IAB node | CEWiT |
| R1-2009269 | Resource management for enhanced duplexing | Qualcomm Incorporated |
| R1-2009270 | On enhancements for simultaneous operation of IAB-node's child and parent links | Qualcomm Incorporated |
| R1-2009301 | Resource multiplexing and DC in enhanced IAB | Ericsson |
| R1-2009302 | Timing, CLI and power control in enhanced IAB | Ericsson |
| R1-2009303 | Refined beam calibration in enhanced IAB | Ericsson |
| R1-2009567 | Summary #1 of [103-e-NR-eIAB-02] | Moderator (Qualcomm) |
| R1-2009604 | Summary #1 of [103-e-NR-eIAB-01] | Moderator (AT&T) |
| R1-2009709 | Summary #2 of [103-e-NR-eIAB-02] | Moderator (Qualcomm) |
| R1-2009734 | Summary #2 of [103-e-NR-eIAB-01] | Moderator (AT&T) |
| R1-2009765 | Summary #3 of [103-e-NR-eIAB-02] | Moderator (Qualcomm) |
| R1-2009837 | Session notes for 8.10 (Enhancements to Integrated Access and Backhaul) | Ad-hoc Chair (Samsung) |

RAN2 #112-e

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2008848 | Consideration on topology-wide fairness, multi-hop latency and congestion mitigation | CATT |
| R2-2008849 | Consideration on Topology adaptation enhancements | CATT |
| R2-2009006 | Discussion on the fairness enforcement for IAB | Fujitsu |
| R2-2009007 | CHO for UE or IAB-MT on migration | Fujitsu |
| R2-2009073 | Report from email discussion [Post111-e][902][eIAB] | Samsung Electronics GmbH |
| R2-2009089 | On topology-wide fairness | Samsung Electronics GmbH |
| R2-2009090 | Enhancements to multi-hop latency and congestion mitigation | Samsung Electronics GmbH |
| R2-2009091 | Views on duplexing enhancements | Samsung Electronics GmbH |
| R2-2009200 | Discussion on Topology-wide fairness and flow control enhancement | Intel Corporation |
| R2-2009201 | Enhancements to establish efficient topologies and backhaul failure recovery | Intel Corporation |
| R2-2009261 | On multi-hop latency and congestion mitigation | InterDigital |
| R2-2009262 | On IAB Topology Adaptation | InterDigital |
| R2-2009291 | Updated workplan for Rel-17 IAB | Qualcomm Incorporated (WI Rapporteur) |
| R2-2009292 | Report of email discussion on topology adaptation enhancements RAN2 scope | Qualcomm Incorporated |
| R2-2009293 | Simulations on fairness support in IAB topology | Qualcomm Incorporated |
| R2-2009329 | Discussion on congestion, RLF, latency and fairness handling | vivo |
| R2-2009330 | Consideration of Inter-CU IAB Migration | vivo |
| R2-2009332 | Multi-hop scheduling and local routing enhancements for IAB | AT&T |
| R2-2009387 | Considerations on topology adaptation enhancements | ZTE, Sanechips |
| R2-2009388 | Discussion on topology-wide fairness multi-hop latency and congestion mitigation | ZTE, Sanechips |
| R2-2009389 | Discussion on duplexing enhancement | ZTE, Sanechips |
| R2-2009422 | On topology adaptation enhancements | Nokia, Nokia Shanghai Bell |
| R2-2009508 | Better Cell Selection for eIAB nodes for improved topology adaptation | Apple |
| R2-2009509 | Fairness metrics in multi-hop eIAB networks | Apple |
| R2-2009610 | Topology optimization in IAB | NEC |
| R2-2009651 | Enhancements for topology-wide fairness, multi-hop latency and congestion mitigation | Huawei, HiSilicon |
| R2-2009652 | Consideration of topology adaptation enhancement for R17-IAB | Huawei, HiSilicon |
| R2-2009653 | Duplexing enhancements for R17 IAB | Huawei, HiSilicon |
| R2-2009667 | Discussion on topology-wide fairness, multi-hop latency and congestion mitigation | LG Electronics |
| R2-2009798 | Hop-by-hop flow control in uplink | Nokia, Nokia Shanghai Bell |
| R2-2009886 | Topology-wide fairness enhancements | Sony |
| R2-2009887 | Topology adaptation enhancements in IAB | Sony |
| R2-2010099 | Rel. 17 IAB enhancements for fairness, multi-hop latency reduction, and congestion mitigation | Futurewei Technologies |
| R2-2010137 | Consideration on avoiding RLF recovery at former descendent nodes | Sharp |
| R2-2010158 | On WI scope and solutions for topology adaptation and inter-CU migration | Ericsson |
| R2-2010159 | On Topology-wide Fairness, Multi-hop Latency and Congestion Mitigation | Ericsson |
| R2-2010233 | Consideration of topology adaptation enhancements for eIAB | Kyocera |
| R2-2010441 | BAP Packet Duplication and BH RLF Indication Enhancements | LG Electronics France |
| R2-2010489 | Discussion on congestion mitigation enhancements | ETRI |
| R2-2010490 | RAN2 impacts of Rel.17 IAB topology adaptation enhancements | Futurewei Technologies |
| R2-2010670 | On topology adaptation enhancements | Nokia, Nokia Shanghai Bell |
| R2-2010671 | On topology adaptation enhancements | Nokia, Nokia Shanghai Bell |
| R2-2011040 | Report from [AT112-e][031][eIAB] Topology Adaptation – PART A | Qualcomm (Moderator) |
| R2-2011060 | Discussion summary - [AT112-e][030][eIAB] | nnSamsung |
| R2-2011061 | Report from email discussion [Post111-e][902][eIAB] Enhancements to improve topology-wide fairness, multi-hop latency and congestion mitigation (Samsung) | Samsung Electronics GmbH |
| R2-2011125 | Report from [AT112-e][031][eIAB] Topology Adaptation – PART B | Qualcomm (Email discussion rapporteur) |
| R2-2011142 | Discussion summary - [AT112-e][030][eIAB] | nnSamsung |

RAN3 #110-e

| TDoc | Title | Source |
| --- | --- | --- |
| R3-205961 | Considerations on IAB-donor indicator | KDDI Corporation |
| R3-205981 | Considerations on UE migration procedure during Inter-donor IAB node migration | KDDI Corporation |
| R3-205999 | Discussion on inter-donor IAB node migration procedure | Samsung |
| R3-206000 | Discussion on service interruption reduction for IAB | Samsung |
| R3-206001 | Discussion on mitigation on packet loss and unnecessary transmission | Samsung |
| R3-206002 | Discussion on inter-donor topology redundancy for IAB | Samsung |
| R3-206003 | Discussion on CP-UP separation for IAB | Samsung |
| R3-206004 | Discussion on UP-based congestion mitigation in IAB | Samsung |
| R3-206072 | Open Issues on topological redundancy for IAB | LG Electronics |
| R3-206107 | Discussion on inter-donor IAB migration | Fujitsu |
| R3-206208 | Inter-Donor IAB Node Migration Enhancement | Intel Deutschland GmbH |
| R3-206209 | Mitigation of Packet Loss | Intel Deutschland GmbH |
| R3-206210 | Congestion Indication to CU-CP | Intel Deutschland GmbH |
| R3-206255 | Updated workplan for IAB enhancements | Qualcomm Incorporated (WI Rapporteur) |
| R3-206256 | Inter-donor IAB-node migration | Qualcomm Incorporated |
| R3-206257 | Interruption time reduction for Intra-donor IAB-node migration | Qualcomm Incorporated |
| R3-206258 | IAB enhancements for inter-donor topological redundancy | Qualcomm Incorporated |
| R3-206259 | Inter-donor coordination of IAB-DU resource management | Qualcomm Incorporated |
| R3-206287 | Discussion on Inter-Donor IAB Node Migration | Nokia, Nokia Shanghai Bell |
| R3-206288 | discussion on Reduction of Service Interruption during Topology Adaptation | Nokia, Nokia Shanghai Bell |
| R3-206289 | discussion on Inter-CU topology redundancy | Nokia, Nokia Shanghai Bell |
| R3-206290 | support F1-C Traffic Transfer over Xn interface | Nokia, Nokia Shanghai Bell |
| R3-206292 | Discussion on Inter-donor migration | Google Inc. |
| R3-206294 | Inter IAB donor-CU topology adaptation | CATT |
| R3-206295 | Discussion on Simultaneous Connection to two IAB-donors | CATT |
| R3-206296 | Reducing the Service Interruption for IAB | CATT |
| R3-206297 | Discuss the improvements to DDDS for IAB UP-based congestion mitigation | CATT |
| R3-206298 | Consider on Multi-Hop Performance: QoS, Latency, Fairness | CATT |
| R3-206332 | Principles of Group Mobility for Inter-donor IAB-node Migration | AT&T |
| R3-206558 | Discussion on inter-CU BH RLF recovery | ZTE, Sanechips |
| R3-206559 | Discussion on inter-Donor IAB Node Migration procedure | ZTE, Sanechips |
| R3-206560 | Consideration on supporting R16 CHO and DAPS in IAB | ZTE, Sanechips |
| R3-206561 | Discussion on reduction of service interruption in intra-donor IAB node migration | ZTE, Sanechips |
| R3-206562 | Discussion on inter-donor redundancy | ZTE, Sanechips |
| R3-206563 | Discussion on DL E2E flow and congestion control in R17-IAB | ZTE, Sanechips |
| R3-206564 | Discussion on IAB duplexing enhancements | ZTE, Sanechips |
| R3-206586 | Inter-donor Load Balancing in IAB Networks | Ericsson |
| R3-206587 | On RLF Recovery Support for IAB Nodes | Ericsson |
| R3-206588 | On the Applicability of CHO for RLF Recovery in IAB Networks | Ericsson |
| R3-206589 | Downlink End-to-End Flow Control in IAB Networks | Ericsson |
| R3-206590 | CR TS 38.425 IAB End-to-End Flow Control Feedback | Ericsson |
| R3-206647 | Discussion on using DDDS for IAB congestion mitigation | Nokia, Nokia Shanghai Bell |
| R3-206649 | Discussion on CP-based approach for DL congestion mitigation | LG Electronics |
| R3-206665 | Inter-CU migration procedure | Huawei |
| R3-206666 | CHO and DAPS in R17 IAB | Huawei |
| R3-206667 | Inter-CU RLF recovery procedure | Huawei |
| R3-206668 | Inter-CU BH RLF recovery procedure for IAB node | Huawei |
| R3-206669 | Discussion on IAB topological redundancy | Huawei |
| R3-206670 | Routing enhancement for IAB | Huawei |
| R3-206671 | Discussion on IAB E2E flow control | Huawei |
| R3-206854 | CB: # 11_IABinterDonorMigration - Summary of email discussion | Qualcomm - moderator |
| R3-206855 | CB: # 12_IABinterDonorMigration_CHOandDAPS - Summary of email discussion | Huawei - moderator |
| R3-206856 | CB: # 13_IABreducingSvcInterruption - Summary of email discussion | Nokia - moderator |
| R3-206857 | CB: # 14_IABtopoRed - Summary of email discussion | Samsung - moderator |
| R3-206858 | CB: # 15_IABcongestionMitigation - Summary of email discussion | Ericsson - moderator |
| R3-206859 | CB: # 16_IABmultiHop - Summary of email discussion | CATT - moderator |

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template