3GPP TSG-RAN WG1 Meeting # 103-e	R1-2009382

# e-Meeting, October 26th – November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0143 | rev | - | Current version: | 15.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on HARQ-ACK Determination for SPS Release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Regarding type-2 HARQ-ACK codebook for SPS release, the PUCCH slot for the corresponding HARQ-ACK transmission is determined by K1 and K0 should not be part of it. As shown in the following figure, the PDCCH providing SPS release is transmitted in slot#0 with K1=2, then the PUCCH carrying HARQ-ACK for SPS release is to be transmitted in slot#2. However, the current TS38.213 implies that the PUCCH slot for HARQ-ACK of the SPS release should be determined by both K1 and K0, which is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the PUCCH slot for HARQ-ACK of the SPS release should be determined by K1 and K0 should not be part of it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect description on how to determine the PUCCH slot for HARQ-ACK of the SPS release. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR is based on RAN1’s common understanding, which has no impact on UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | The corresponding draft CR was first submitted in R1-2007723. |  |  |  |  |  |  |  |  |

#### 9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

A UE determines monitoring occasions for PDCCH with DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release on an active DL BWP of a serving cell , as described in Clause 10.1, and for which the UE transmits HARQ-ACK information in a same PUCCH in slot  based on

-	PDSCH-to-HARQ_feedback timing indicator field values for PUCCH transmission with HARQ-ACK information in slot  in response to PDSCH receptions or SPS PDSCH release

-	slot offsets  [6, TS 38.214] provided by time domain resource assignment field in DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions and by pdsch-AggregationFactor, when provided.

The set of PDCCH monitoring occasions for DCI format 1_0 or DCI format 1_1 for scheduling PDSCH receptions or SPS PDSCH release is defined as the union of PDCCH monitoring occasions across active DL BWPs of configured serving cells, ordered in ascending order of start time of the search space set associated with a PDCCH monitoring occasion. The cardinality of the set of PDCCH monitoring occasions defines a total number  of PDCCH monitoring occasions.

A value of the counter downlink assignment indicator (DAI) field in DCI format 1_0 or DCI format 1_1 denotes the accumulative number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI format 1_0 or DCI format 1_1 is present, up to the current serving cell and current PDCCH monitoring occasion, first in ascending order of serving cell index and then in ascending order of PDCCH monitoring occasion index , where .

The value of the total DAI, when present [5, TS 38.212], in DCI format 1_1 denotes the total number of {serving cell, PDCCH monitoring occasion}-pair(s) in which PDSCH reception(s) or SPS PDSCH release associated with DCI format 1_0 or DCI format 1_1 is present, up to the current PDCCH monitoring occasion  and is updated from PDCCH monitoring occasion to PDCCH monitoring occasion.

< ------------------------- Other parts are omitted ------------------------- >