3GPP TSG RAN WG1 Meeting #103-e R1-2009589 e-Meeting, 26th October – 3rd November, 2020 CR-Form-v12.0. CHANGE REQUEST. 38.214; CR; 0143; rev; -; Current version:; 16.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Correction on L1-RSRP and Minimum scheduling offset. Source to WG:; Moderator (CATT), MediaTek. Source to TSG:; R1. Work item code:; NR_UE_pow_sav-Core; Date:; 2020-11-05. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; In Clause 5.2.1.5.1a, the minimumSchedulingOffsetK0-r16 is configured for DL BWP and minimumSchedulingOffsetK2-r16 is configured for UL BWP. In Clause 5.2.2.5, the interference measurement is not required if the UE is configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP during the time duration indicated by drx-onDurationTimer outside DRX active time.. -. Summary of change:; Adding “minimumSchedulingOffsetK2-r16 for UL BWP in Clause 5.2.1.5.1a Remove CSI-RS and CSI-IM occasions for interference measurements for L1-RSRP during the time indicated by drx-onDurationTimer in Clause 5.2.2.5. Consequences if not approved:; This CR is to correct the CSI-RS timing for UL BWP and L1-RSRP measurements indicated by drx-onDuratinTimer. Clauses affected:; 5.2.1.5.1a and 5.2.2.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:; Isolated impact analysis: Current specification of CSI-RS timing with minimum scheduling offset configured for cross-slot scheduling have one RRC parameter minimumSchedulingOffsetK0-r16 for both DL and UL BWP, which is incorrect. Additional minimumSchedulingOffsetK2-r16 is added for UL BWP. Current specification for L1-RSRP measurements indicated by drx-onDurationTimer include the CSI-RS and CSI-IM occasion for interference measurements, which are not required. The CR is to remove the unnecessary requirements for L1-RSRP when it indicated by drx-onDurationTimer.. This CR's revision history:; This is the first version for this CR..

##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies

<Unchanged parts are omitted>

Aperiodic CSI-RS timing:

-	When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL BWP or minimumSchedulingOffsetK2-r16 for any UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states. The CSI-RS triggering offset has the values of {0, 1, …, 31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where

-	n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16,

-	μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively,

-	Nslot, offset, PDCCHCA and μoffset,PDCCHare the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDCCH respectively, Nslot, offset, CSIRSCA and  μoffset,CSIRS are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell transmitting the CSI-RS respectively, as defined in [4, TS 38.211] clause 4.5

<Unchanged parts are omitted>

#### 5.2.2.5 CSI reference resource definition

<Unchanged parts are omitted>

When DRX is configured, the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitOtherPeriodicCSI to report CSI with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to quantities other than 'cri-RSRP' and 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report CSI during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in Clause 5.2.1.4 if receiving at least one CSI-RS transmission occasion for channel measurement and CSI-RS and/or CSI-IM occasion for interference measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise. When the UE is configured to monitor DCI format 2_6 and if the UE configured by higher layer parameter ps-TransmitPeriodicL1-RSRP to report L1-RSRP with the higher layer parameter reportConfigType set to 'periodic' and reportQuantity set to 'cri-RSRP' or 'ssb-Index-RSRP' when drx-onDurationTimer is not started, the UE shall report L1-RSRP during the time duration indicated by drx-onDurationTimer also outside active time according to the procedure described in clause 5.2.1.4 and when reportQuantity set to 'cri-RSRP' if receiving at least one CSI-RS transmission occasion for channel measurement during the time duration indicated by drx-onDurationTimer outside DRX active time or in DRX Active Time no later than CSI reference resource and drops the report otherwise.

<Unchanged parts are omitted>