3GPP TSG-RAN WG1 Meeting #103-e	R1-2009443

# E-meeting,  October 26 – November 13, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1365 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | PUR configuration of transmission mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Sierra Wireless), Ericsson, Huawei, HiSilicon, Qualcomm, Lenovo/Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Alignment to the agreement made in RAN1 #103-e where “PUR supports only TM 1 and 2” Since there is no RRC configuration of TM, it is unclear how the TM is configured for PUR transmissions. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed transmission mode column and rows corresponding to TM 2, 6 and 9 in table 7.1-9. Single-antenna port or Transmit diversity configuration is based on the number of PBCH antenna ports: If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used, otherwise Transmit diversity The term “PUR C-RNTI” has been replaced by “PUR-RNTI”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unsupported PUR TM modes are specified. There is no way to configure the TM for PUR transmissions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TS 36.213 Clause 7.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------- Text start (TS 36.213 Clause 7.1) --------------------------------------------

If a BL/CE UE is configured by higher layers to decode MPDCCH with CRC scrambled by the PUR-RNTI, the UE shall decode the MPDCCH and any corresponding PDSCH according to the respective combinations defined in Table 7.1-9. The scrambling initialization of PDSCH corresponding to these MPDCCHs is by PUR-RNTI.

Table 7.1-9: MPDCCH and PDSCH configured by PUR-RNTI

|  | DCI format | Search Space | Transmission scheme of PDSCH corresponding to MPDCCH |
| --- | --- | --- | --- |
|  | 6-1A or 6-1B | UE specific by PUR-RNTI | If the number of PBCH antenna ports is one, Single-antenna port, port 0 is used (see Subclause 7.1.1), otherwise Transmit diversity (see Subclause 7.1.2). |

----------------------------------- Text end (TS 36.213 Clause 7.1)  -----------------------------------------------------