# 3GPP TSG RAN WG1 #103-e															R1-2009840

# e-Meeting, October 26th – November 13th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 1368 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for multi-TB early termination |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), Qualcomm Incorporated, ZTE, Ericsson, Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Current early termination for eMTC with multi-TB has the following issues: - For explicit HARQ-ACK feedback, current text is limited to a single TB. - For implicit HARQ-ACK feedback, current text is limited to a single TB, and the conditions only mention a single HARQ ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For explicit HARQ-ACK feedback, clarify that it also applies to multiple TBs. - For implicit HARQ-ACK feedback, clarify that it also applies to multiple TBs and introduce the conditions on HARQ processes / NDI and timing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Early termination is not supported for multi-TB scheduling |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 36.212 CR 0353 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

A BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B intended for the UE in the UE-specific search space indicating HARQ-ACK(s) corresponding to transport block(s) associated to HARQ process(es) in the most recent PUSCH transmission with N>1, drop the remaining PUSCH transmission(s) (if any) corresponding to the transport block(s) no later than subframe n+k, where

- subframe n is the last subframe in which the MPDCCH is transmitted; and

- for FDD, k = 4;

- for TDD the value of k is given in Table 8-2 for the corresponding TDD UL/DL configuration; If the value of k is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure;

- value of is determined by the repetition number field in the corresponding DCI associated with the most recent PUSCH transmission;

- if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI associated with the most recent PUSCH transmission is using uplink resource allocation type 5,  where  is defined in [3] and  is determined according to procedure in subclause 8.1.6,  otherwise.

For a BL/CE UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig, if the UE detects a first MPDCCH with DCI format 6-0A/6-0B intended for the UE scheduling PUSCH in subframes {s0,…,sN-1}, and if the UE detects a second MPDCCH with DCI format 6-0A/6-0B intended for the UE scheduling PUSCH in subframes {q0,…,qL-1} with M≤q0≤sN-1, where

-	for any HARQ ID that is indicated in both the first MPDCCH and the second MPDCCH, the NDI indicated in the second MPDCCH is toggled with respect to the NDI indicated in the first MPDCCH

-	for each HARQ ID i indicated in the first MPDCCH, the first subframe M in which the second MPDCCH is transmitted meets M≥s0,i+k, where  s0,i is the first subframe in which the HARQ ID i is transmitted

-	for FDD, k = 4,

-	for TDD the value of k is given in Table 8-2 for the corresponding TDD UL/DL configuration; If the value of k is not given in Table 8-2 for subframe n, denote subframe n' as the first downlink/special subframe which has a value in Table 8-2 after subframe n, and substitute n with n' in the above procedure

the UE shall

-	drop the remaining PUSCH transmission(s) of the transport block(s) scheduled by the first MPDCCH starting from subframe K, where M<K≤q0, and

-	deliver HARQ-ACK feedback corresponding to the transport block(s) scheduled by the first MPDCCH to higher layers, and

-	transmit the PUSCH scheduled by the second MPDCCH in subframes {q0,…,qL-1}

<Unchanged parts are omitted>