3GPP TSG-RAN WG4 Meeting # 97-e                                                             R4-2015017

Electronic Meeting, 2-13 November, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0527 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1[R16]: Clarification of non-simultaneous Rx/Tx operation for CA_n77-n79 and CA_n78-n79 in TS 38.101-1. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2020-11-12 |
| Category: | A |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It is unclear whether it is synchronous operation or asynchronous operation when proposing new configuration that include CA_n77-n79 or CA_n78-n79. Also, it is not good to have to mention this every time we propose a higher order configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - NOTE5 is revised for CA_n78-n79 in Table 5.2A.2.1-1. This indicates that if UE supports band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. Also, this restriction is extended to higher order configurations. - NOTE7 is added for CA_n77-n79 in Table 5.2A.2.1-1. This indicates that the minimum requirements apply only when there is non-simultaneous Tx/Rx operation between n77-n79 NR carriers. Also, this restriction is extended to higher order configurations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | When proposing a new higher order band combo, there is no mention of whether it is synchronous or asynchronous, or it must be mentioned of these every time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of change >>

5.2A	Operating bands for CA

5.2A.0	General

CA operating bands including Band n90 are defined by the corresponding CA operating bands including Band n41 with Band n90 replacing Band n41. For brevity the said CA operating bands including Band n90 are not listed in the tables below but are covered by this specification.

5.2A.1	Intra-band CA

NR intra-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.1-1 and Table 5.2A.1-2, where all operating bands are within FR1.

Table 5.2A.1-1: Intra-band contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1 | n1 |
| CA_n7 | n7 |
| CA_n40 | n40 |
| CA_n41 | n41 |
| CA_n461 | n46 |
| CA_n48 | n48 |
| CA_n66 | n66 |
| CA_n71 | n71 |
| CA_n77 | n77 |
| CA_n78 | n78 |
| CA_n79 | n79 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. |  |

Table 5.2A.1-2: Intra-band non-contiguous CA operating bands in FR1

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n3(*) | n3 |
| CA_n7(*) | n7 |
| CA_n25(*) | n25 |
| CA_n41(*) | n41 |
| CA_n48(*) | n48 |
| CA_n66(*) | n66 |
| CA_n77(*) | n77 |
| CA_n78(*) | n78 |
| NOTE 1: The minimum requirements only apply for non simultaneous Tx/Rx between all carriers for TDD combinations. NOTE 2: The notation CA_nX(*) in this table indicates intra-band non-contiguous CA for band nX. The configurations for each band are in 5.5A.2. |  |

5.2A.2	Inter-band CA

NR inter-band carrier aggregation is designed to operate in the operating bands defined in Table 5.2A.2.1-1, 5.2A.2.2-1 and Table 5.2A.2.3-1, where all operating bands are within FR1.

Table 5.2A.2-1: Void

Table 5.2A.2-2: Void

Table 5.2A.2-3: Void

5.2A.2.1	Inter-band CA (two bands)

Table 5.2A.2.1-1: Inter-band CA operating bands involving FR1 (two bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3 | n1, n3 |
| CA_n1-n7 | n1, n7 |
| CA_n1-n8 | n1, n8 |
| CA_n1-n28 | n1, n28 |
| CA_n1-n40 | n1, n40 |
| CA_n1-n41 | n1, n41 |
| CA_n1-n77 | n1, n77 |
| CA_n1-n78 | n1, n78 |
| CA_n1-n79 | n1, n79 |
| CA_n2-n5 | n2, n5 |
| CA_n2-n48 | n2, n48 |
| CA_n2-n66 | n2, n66 |
| CA_n2-n77 | n2, n77 |
| CA_n2-n78 | n2, n78 |
| CA_n3-n7 | n3, n7 |
| CA_n3-n8 | n3, n8 |
| CA_n3-n28 | n3, n28 |
| CA_n3-n38 | n3, n38 |
| CA_n3-n40 | n3, n40 |
| CA_n3-n41 | n3, n41 |
| CA_n3-n771 | n3, n77 |
| CA_n3-n781 | n3, n78 |
| CA_n3-n791 | n3, n79 |
| CA_n5-n7 | n5, n7 |
| CA_n5-n66 | n5, n66 |
| CA_n5-n77 | n5, n77 |
| CA_n5-n78 | n5, n78 |
| CA_n5-n79 | n5, n79 |
| CA_n7-n25 | n7, n25 |
| CA_n7-n28 | n7, n28 |
| CA_n7-n66 | n7, n66 |
| CA_n7-n78 | n7, n78 |
| CA_n8-n391 | n8, n39 |
| CA_n8-n40 | n8, n40 |
| CA_n8-n41 | n8, n41 |
| CA_n8-n751 | n8, n75 |
| CA n8-n781 | n8, n78 |
| CA_n8-n791 | n8, n79 |
| CA_n20-n282 | n20, n28 |
| CA_n20-n75 | n20, n75 |
| CA_n20-n78 | n20, n78 |
| CA_n25-n41 | n25, n41 |
| CA_n25-n466 | n25, n46 |
| CA_n25-n66 | n25, n66 |
| CA_n25-n71 | n25, n71 |
| CA_n25-n78 | n25,n78 |
| CA_n28-n40 | n28, n40 |
| CA_n28-n41 | n28, n41 |
| CA_n28-n50 | n28, n50 |
| CA_n28-n752 | n28, n75 |
| CA_n28-n77 | n28, n77 |
| CA_n28-n781 | n28, n78 |
| CA_n29-n66 | n29, n66 |
| CA_n29-n70 | n29, n70 |
| CA_n38-n66 | n38, n66 |
| CA_n38-n781 | n38, n78 |
| CA_n39-n40 | n39, n40 |
| CA_n39-n41 | n39, n41 |
| CA_n39-n791 | n39, n79 |
| CA_n40-n41 | n40, n41 |
| CA_n40-n78 | n40, n78 |
| CA_n40-n791,4 | n40, n79 |
| CA_n41-n501 | n41, n50 |
| CA_n41-n66 | n41, n66 |
| CA_n41-n711 | n41, n71 |
| CA_n41-n78 | n41, n78 |
| CA_n41-n791,3 | n41, n79 |
| CA_n46-n486 | n46, n48 |
| CA_n46-n666 | n46, n66 |
| CA_n48-n66 | n48, n66 |
| CA_n50-n78 | n50, n78 |
| CA_n66-n70 | n66, n70 |
| CA_n66-n71 | n66, n71 |
| CA_n66-n77 | n66, n77 |
| CA_n66-n78 | n66, n78 |
| CA_n70-n71 | n70, n71 |
| CA_n75-n781 | n75, n78 |
| CA_n76-n781 | n76, n78 |
| CA_n77-n797 | n77, n79 |
| CA_n78-n795 | n78, n79 |
| CA_n78-n92 | n78, n92 |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. NOTE 2: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. NOTE 3: The frequency range below 2506 MHz for Band n41 is not used in this combination. NOTE 4: Applicable for frequency range above 4800 MHz for Band n79 in this combination. NOTE 5: For UEs supporting band n77, the minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n78-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. NOTE 6: The PCell is allocated in the licensed band in this combination. NOTE 7: The minimum requirements apply only when there is non-simultaneous Rx/Tx operation between n77-n79 NR carriers. This restriction applies also for these carriers when applicable NR CA configuration is part of a higher order configuration. |  |

5.2A.2.2	Inter-band CA (three bands)

Table 5.2A.2.2-1: Inter-band CA operating bands involving FR1 (three bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n7 | n1, n3, n7 |
| CA_n1-n3-n8 | n1, n3, n8 |
| CA_n1-n3-n28 | n1, n3, n28 |
| CA_n1-n3-n41 | n1, n3, n41 |
| CA_n1-n3-n78 | n1, n3, n78 |
| CA_n1-n7-n28 | n1, n7, n28 |
| CA_n1-n7-n78 | n1，n7, n78 |
| CA_n1-n8-n78 | n1, n8, n78 |
| CA_n1-n28-n78 | n1, n28, n78 |
| CA_n1-n40-n78 | n1, n40, n78 |
| CA_n3-n7-n28 | n3, n7, n28 |
| CA_n3-n7-n78 | n3, n7, n78 |
| CA_n3-n8-n78 | n3, n8, n78 |
| CA_n3-n28-n77 | n3, n28, n77 |
| CA_n3-n28-n78 | n3, n28, n78 |
| CA_n3-n40-n41 | n3, n40, n41 |
| CA_n3-n41-n79 | n3, n41, n79 |
| CA_n5-n66-n78 | n5, n66, n78 |
| CA_n7-n25-n66 | n7, n25, n66 |
| CA_n7-n28-n78 | n7, n28, n78 |
| CA_n7-n66-n78 | n7, n66, n78 |
| CA_n8-n39-n41 | n8, n39, n41 |
| CA_n8-n41-n79 | n8, n41, n79 |
| CA_n20-n28-n78 | n20, n28, n78 |
| CA_n25-n41-n66 | n25, n41, n66 |
| CA_n25-n41-n71 | n41, n66, n71 |
| CA_n25-n66-n71 | n25, n66, n71 |
| CA_n25-n66-n78 | n25, n66, n78 |
| CA_n28-n40-n78 | n28, n40, n78 |
| CA_n28-n41-n78 | n28, n41, n78 |
| CA_n29-n66-n70 | n29, n66, n70 |
| CA_n39-n41-n79 | n39, n41, n79 |
| CA_n40-n41-n791,2 | n40, n41, n79 |
| CA_ n41-n66-n71 | n41, n66, n71 |
| CA_n66-n70-n71 | n66, n70, n71 |
| NOTE 1: The frequency range below 2506 MHz for Band n41 is not used in this band combination. NOTE 2: Applicable for frequency range above 4800 MHz for Band n79 in this band combination. |  |

5.2A.2.3	Inter-band CA (four bands)

Table 5.2A.2.3-1: Inter-band CA operating bands involving FR1 (four bands)

| NR CA Band | NR Band (Table 5.2-1) |
| --- | --- |
| CA_n1-n3-n7-n28 | n1, n3, n7, n28 |
| CA_n1-n3-n7-n78 | n1, n3, n7, n78 |
| CA_n1-n3-n8-n78 | n1, n3, n8, n78 |
| CA_n1-n3-n28-n78 | n1, n3, n28, n78 |
| CA_n3-n7-n28-n78 | n3, n7, n28, n78 |
| CA_n7-n25-n66-n78 | n7, n25, n66, n78 |

<< End change >>