3GPP TSG-RAN WG4 Meeting # 97-e 	R4-2017444

# Electronic Meeting, 2-13 Nov, 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR | 0003 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.175 on Voltage dips and interruptions, Release 16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2020-11-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Performance criteria is updated to reflect considerations on the test levels. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR updates the performance criteria according to the test levels defined by IEC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB EMC specification would be mis-aligned with the source IEC specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2; 9.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of new section ------------------------------

### 9.6.2	Test method and level

.

The test method shall be in accordance with IEC 61000-4-11 [17], where

the test levels shall be:

-	Voltage dip: 0 % residual voltage for 0.5 cycle;

-	Voltage dip: 0 % residual voltage for 1 cycle;

-	Voltage dip: 70 % residual voltage for 25/30 cycles (at 50/60 Hz);

-	Voltage interruption: 0 % residual voltage for 250/300 cycles (at 50/60 Hz).

### 9.6.3	Performance criteria

For a 0 % residual voltage dip test, the performance criteria for transient phenomena shall be applied:

-	Criteria 6.2 for IAB node

-	Criteria 6.4 for ancillary equipment

For a 70% residual voltage dip test and for voltage interruption test, the following applies:

1.	In the case where the equipment is fitted with or connected to a battery back-up, the following performance criteria shall be applied:

-	Criteria 6.2 for IAB node

-	Criteria 6.4 for ancillary equipment

2.	In the case where the equipment is powered solely from the AC mains supply (without the use of a parallel battery back-up) volatile user data may have been lost and if applicable the communication link need not to be maintained and lost functions should be recoverable by user or operator:

-	No unintentional responses shall occur at the end of the test, when the voltage is restored to nominal

-	In the event of loss of communications link or in the event of loss of user data, this fact shall be recorded in the test report

----------------------------- End of proposed text ------------------------------