3GPP TSG-RAN4 Meeting #97-e	R4-2016451

# Online, , 2nd Nov 2020 - 13th Nov 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0562 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to for 38.101-1: CA uplink power clarification |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | T-Mobile USA |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some of the wording on UE maximum output power for carrier aggregation is unclear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the wording for UE maximum output power for carrier aggregation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wording remains unclear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<First Changed Section>

#### 6.2A.1.3	UE maximum output power for Inter-band CA

For inter-band downlink carrier aggregation with one uplink carrier assigned to one NR band, the transmitter power requirements in clause 6.2 apply.

For inter-band uplink carrier aggregation with uplink assigned to two NR bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1 ms). The maximum output power is specified in Table 6.2A.1.3-1.

For PC3 inter-band carrier aggregation with one uplink component carrier assigned to one NR band in NR band n41, n77, n78, and n79, the requirements for power class 2 are not applicable and the corresponding requirements for a power class 3 UE shall apply.

Table 6.2A.1.3-1 UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n3A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n1A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n5A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n48A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n2A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3-n38A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n3A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n5A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n25A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n7A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n39A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n8A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n20A-n28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n20A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n25A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n50A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n28A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n38A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n38A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n39A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n40A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n79A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n41A-n50A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n48A-n66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n50A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n77A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n66A-n78A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n70A-n71A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_n78A-n92A |  |  |  |  | 23 | +2/-32 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: Power class 3 is the default power class unless otherwise stated |  |  |  |  |  |  |  |  |

<End of Changes>