3GPP TSG-RAN Meeting #90-e	RP-202590

# Electronic Meeting, 7th Dec. 2020 – 11th Dec. 2020	    		       (Revision of RP-201483)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Revised Rel-17 WID on DC of 4 bands LTE inter-band CA (4DL1UL) and 1 NR band (1DL1UL)

# Document for:	Approval

Agenda Item:	9.9.17.4

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: Rel-17 Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL)

## Acronym: DC_R17_4BLTE_1BNR_5DL2UL

## Unique identifier: 	881001

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

## Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| DC_R17_4BLTE_1BNR_5DL2UL | RAN4 | 881001 | Rel-17 Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) |

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 881101 | Core part: Rel-17 Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | Child WID |
| 881201 | Perf. part: Rel-17 Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | Child WID |

## 3	Justification

All new EN-DC and NE-DC configurations consisting of 5 different bands DL with 2 different bands UL (4 different LTE bands and 1 NR band) will be defined under this WI. New configurations still emerge from exiting bands and whenever new band is specified, it will create a potential for several new EN-DC and NE-DC configurations consisting of 5 different bands DL with 2 different bands UL (4 different LTE bands and 1 NR band).

The EN-DC and NE-DC configurations will be introduced in a release independent manner from Rel-15 based on TS38.307, which will be updated depending on newly introduced configurations.

The preconditions to propose 5 different bands DL with 2 different bands UL (4 different LTE bands and 1 NR band) in rel-17 are as follows.

- Constituent LTE inter band CA including intra band CA for 4 different bands DL with 1 band UL shall be completed and specified in advance.

- Each of the four paired EN-DC configurations of 1 LTE band including intra band CA + 1 NR band used in a certain 5 different bands DL with 2 different bands UL shall be completed and specified in advance.

Example 1: If the following configurations is proposed,

| Downlink DC configuration | Uplink DC configuration |
| --- | --- |
| EN-DC_1A-2A-3A-4A_n5A | DC_1A_n5A DC_2A_n5A DC_3A_n5A DC_4A_n5A |

- LTE CA of CA_1A-2A-3A-4A shall be completed and specified in advance.

- EN-DC of DC_1A_n5A, DC_2A_n5A, DC_3A_n5A and DC_4A_n5A shall be completed and specified in advance.

Example 2: If the following configurations is proposed,

| Downlink DC configuration | Uplink DC configuration |
| --- | --- |
| NE-DC_n5A_1A-2A-3A-4A | DC_n5A_1A DC_n5A_2A DC_n5A_3A DC_n5A_4A |

- LTE CA of CA_1A-2A-3A-4A shall be completed and specified in advance.

- NE-DC of DC_n5A_1A, DC_n5A_2A, DC_n5A_3A and DC_n5A_4A shall be completed and specified in advance.

Example 3: If the following configurations is proposed,

| Downlink DC configuration | Uplink DC configuration |
| --- | --- |
| EN-DC_1C-2A-3A-4A_n5C | DC_1C_n5C DC_2A_n5C |

- LTE CA of DL_CA_1C-2A-3A-4A_UL_CA_1C shall be completed and specified in advance.

- EN-DC of DC_1C_n5C and DC_2A_n5C shall be completed and specified in advance.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

- Specify the band-combination specific RF requirements for all listed EN-DC configurations consisting of 5 different bands DL with 2 different bands UL (4 different LTE bands and 1 NR band) including at least

  - Applicable frequencies if necessary

  - Applicable bandwidths and bandwidth sets if necessary

- Analyze combinations that have self-desensitization due to following reasons:

  - TX Harmonic and/or intermodulation overlap of receive band

  - TX signal overlap of receiver harmonic frequency

  - TX frequency being in close proximity of one of the receive bands

  - Any other identified reasons such that insufficient cross band isolation, harmonic mixing

- For the combination where self-desensitization exists, specify at least needed

  - ∆TIB, c and ∆RIB, c

  - Reference sensitivity exceptions including MSD test cases

- Add conformance testing in RAN5 specifications (to follow at a later stage)

of all REL-17 EN-DC configurations consisting of 5 different bands DL with 2 different bands UL (4 different LTE bands and 1 NR band) that fall into the category defined by the WI title.

An overview of these EN-DC and NE-DC configurations is provided in the appended Excel sheet.

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

- Required changes to be added to release independence TS 38.307.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | TR 37.717-41-11 | Rel-17 Dual Connectivity (DC) of 4 bands LTE inter-band CA (4DL/1UL) and 1 NR band (1DL/1UL) | RAN#92 | RAN#93 | Johannes Hejselbaek, Nokia, johannes.hejselbaek@nokia.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 38.101-3 | Add new DC band combinations and related RF core requirements | TSG#93 | Core part |
| TS 38.307 | Release independent manner will be applied to all new DC band combinations according to each DC band combination | TSG#93 | Perf. Part |

## 6	Work item Rapporteur(s)

Johannes Hejselbaek, Nokia, johannes.hejselbaek@nokia.com

## 7	Work item leadership

RAN WG4

## 8	Aspects that involve other WGs

None

## 9	Supporting Individual Members

Supporting IM name

Nokia

Nokia Shanghai Bell

LGE

Softbank

Samsung

Ericsson

AT&T

Huawei

Vodafone

CHTTL

Telstra

SK Telecom

Rogers