3GPP TSG RAN meeting #90e									 RP-202718

Electronic Meeting, December 7-11, 2020

## Status Report to TSG

Agenda item:			9.7.3

| WI / SI Name | NR Positioning Enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_Pos_Enh |  |  |  |  |
| Unique ID | 860034 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-202094 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2021 (changed from 12/2020) | Core part: NA | Performance part: NA | Testing part: NA |  |
| Overall Completion level | Study Item: 90% | Core part: NA | Performance Part: NA | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Ren DA |
|  | Company | CATT |
|  | Email | renda@catt.cn |
| Rapporteur | Name | Yi GUO |
|  | Company | Intel Corporation |
|  | Email | yi.guo@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

Agreements (RAN1#101-e)

Agreement:

- InF-SH and InF-DH models in TR 38.901 are adopted as the baseline scenarios for defining the channel models, parameters and modelling techniques for performance evaluations in the Rel. 17 positioning enhancements at least for IIoT use cases

- Note: Modifications to parameters in the InF-DH models will be discussed separately.

- Note: Target performance and performance gap identification will be discussed separately.

- Note: Individual companies may consider additional InF models in TR 38.901 as complementary evaluation scenarios in their simulation investigation and the evaluation results can be considered to be captured in the TR 38.857.

- Note: Target positioning requirements may not necessarily be reached for all scenarios.

Agreement (Proposal 4.1-1, Revision #2, in Section 4.1 of R1-2004868):

- Adopt the parameters defined in Table below as the baseline parameters for all scenarios in the evaluation of the positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

- Note: Optional scenarios and assumptions will be discussed separately and can be included

Table: Common scenario parameters applicable for all scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 3.5GHz | 28GHz |
| Bandwidth, MHz | 100MHz | 400MHz |
| Subcarrier spacing, kHz | 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Baseline: Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU Optional: FFS |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns (Optional) |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Agreement:

Optional: The following UE antenna configuration can be considered

- 4 UE panels:

  - The antenna elements of the same polarization of the same panel is virtualized into one TXRU

- FFS: Other details

Agreement:

Absolute-time-of arrival model defined in TR 38.901 without modification is considered in the evaluation of all scenarios.

Agreement:

Blockage model is not considered in the evaluation of all scenarios

Agreement: (Proposal 5.1-4, Revision 3, in Section 5.1 of R1-2004961)

- Adopt the parameters defined in the Table below as the baseline parameters for all InF scenarios in the evaluation of positioning performance in Rel-17.

- Note: Individual companies may consider additional parameter values or different parameter settings in their simulation investigation

Table: Parameters common to InF scenario(s)

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-SH, InF-DH |  | InF-SH, InF-DH |
| Layout | Hall size | InF-SH: (baseline) 300x150 m (optional) 120x60 m InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Peneteration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be at least the convex hull of the horizontal BS deployment. It can also be the whole hall area if the CDF values for positioning accuracy is obtained from whole hall area. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): FFS |  |  |
| UE mobility |  | 3km/h (Optional): FFS |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): FFS |  |  |
| Clutter parameters: {density , height ,size } |  | Low clutter density: {20%, 2m, 10m} High clutter density: See Proposal 5.1-7 |  |  |
| Note 1: According to Table A.2.1-7 in 3GPP TR 38.802 |  |  |  |  |

Agreement:

Optional: For evaluating vertical positioning performance, UE antenna height can be uniformly distributed within [0.5, X2]m, where X2 = 2m for InF-SH and X2= for InF-DH defined in TR 38.901.

Agreement:

Clutter parameters {density , height ,size } for high clutter density are set as follows:

- (Baseline): {40%, 2m, 2m} for fixed UE antenna height and gNB antenna height

- (Optional): {40%, 3m, 5m}

- (Optional): {60%, 6m, 2m}

Agreement:

It will be left to companies to define the configurations for DL PRS and UL SRS for the evaluation of positioning performance.

- Note: Configurations of DL PRS and UL SRS supported by Rel-16 specifications are used for evaluation of the achievable performance based on Rel-16 positioning technologies.

Agreement:

CDFs of positioning errors are used as performance metrics in NR positioning evaluation with at least the following percentiles 50%, 67%, 80%, 90%.

- Note: In addition to overall positioning accuracy performance, companies are encouraged to report the estimation accuracy of UE/gNB measurements (e.g., RSTD) for performance comparison.

Agreement:

For TR 38.857, the template used in TR 38.855 for the inclusion of simulation results is reused. In addition, the following parameters should be provided for each scenario together with the simulation results.

| Parameter | [Source 1, scenario, FRx] |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) |  |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) |  |
| Reference signal (type of sequence, number of ports, …) |  |
| Number of sites |  |
| Number of symbols used per slot per positioning estimate |  |
| Number of slots per positioning estimate |  |
| Power-boosting level |  |
| Uplink power control (applied/not applied) |  |
| interference modelling (ideal muting, or other) |  |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) |  |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) |  |
| Network synchronization assumptions |  |
| Beam-related assumption (beam sweeping / alignment assumptions at the tx and rx sides) |  |
| Precoding assumptions (codebook, nrof antenna elements used, etc) |  |
| Additional notes, if any |  |

Agreement:

CDF values for positioning accuracy for IIoT scenarios are derived based on:

- Case 1 (Required): The UEs inside the convex hull of the horizontal BS deployment area.

- Case 2 (Optional): All the UEs

Agreement:

Optional: For evaluating vertical positioning performance, gNB antenna height can also be set to two fixed heights, which is either {4, 8} m, or {max(4,), 8}.

Agreement:

Network efficiency and UE efficiency can be evaluated at least in an analytical manner.

- FFS: the definition of efficiency metric (e.g., the positioning performance (accuracy, latency) vs. PRS/SRS resource utilization etc.)

- Note: It will be up to each company on whether to use other methods (e.g., numerical simulation) for the evaluation.

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for [90%] of UEs

  - Vertical position accuracy (< [2 or 3] m) for [90%] of UEs

  - End-to-end latency for position estimation of UE (< [100 ms])

  - FFS: Physical layer latency for position estimation of UE (< [10 ms])

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< X m) for [90%] of UEs

    - X = [0.2 or 0.5] m

  - Vertical position accuracy (< Y m) for [90%] of UEs

    - Y = [0.2 or 1] m

  - End-to-end latency for position estimation of UE (< [10ms, 20ms, or 100ms])

  - FFS: Physical layer latency for position estimation of UE (< [10ms])

- Note: Target positioning requirements may not necessarily be reached for all scenarios

Agreement:

Optional: UE mobility can be considered in evaluation with the consideration of the spatial consistency procedure defined in TR 38.901.

- FFS: the details of the mobility models

Agreement:

- UE power consumption for NR positioning can be optionally evaluated in the SI.

- Note: It is up to each company on how to evaluate the power consumption for positioning. The UE power consumption models developed in TR38.840 can be considered as the starting point for defining the UE power consumption model for the evaluation for NR positioning

Agreement:

The TR skeleton in R1-2004948 is endorsed.

Agreement:

Optional: The UE/gNB RX and TX timing error, in FR1/FR2, can be modeled as a truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1:

        T1:  [X] ns for gNB and [Y] ns for UE

- FFS: X, Y

        Note: RX and TX timing errors are generated per panel independently

        FFS: how the Rx and Tx timing errors are applied

Agreement:

- In Rel-17 SI, for the evaluation of positioning enhancements for commercial use cases, no baseline scenario is defined. UMi, UMa and IOO scenario(s) defined in TR 38.855 can be considered as optional scenarios without modifications to existing configuration parameters.

- FFS: absolute time of arrival model for UMi, UMa and IOO scenarios

Agreement:

Physical layer latency can be evaluated through analysis and, optionally, numerical evaluation.

Agreement:

Higher layer positioning latency can be evaluated in this SI.

- FFS: how to evaluate higher-layer positioning latency

- FFS: which higher-layers should be included in the evaluation

Agreements (RAN1#102-e)

Agreement:

Physical Layer Latency Start and End times are defined as follows:

| Method | Start | End |
| --- | --- | --- |
| UE assisted DL-only & DL-ECID & Multi-RTT | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information message | Successful decoding of the PUSCH carrying the LPP Provide Location Information message |
| UL-only method & UL ECID & Multi-RTT | Reception by the gNB of the NRPPa measurement request message | The transmission by the gNB of the NRPPa measurement response message |
| UE-based | Transmission of the PDSCH from the gNB carrying the LPP Request Location Information if applicable, otherwise, - Alt. 1: transmission of the PUSCH carrying the MG Request from the UE. - Alt. 2: Transmission of the PDSCH from the gNB carrying the LPP message containing the assistance data - Alt. 3: Start of the Reception of DL PRS Note: Suggest to downselect this at the next meeting. Note: The high layers latency components may be subject to adjustment for different alternatives. | Successful decoding of the PUSCH at gNB carrying the LPP Provide Location Information message if applicable, otherwise Calculation of Location Estimate at the UE |

Conclusion:

RAN1 will not define additional optional values for UE and gNB antenna heights for evaluations.

Conclusion:

RAN1 will not define additional details for the optional UE antenna configuration of 4 UE panels for evaluations.

Conclusion:

For power consumption evaluation, it is up to each company to detail their methodology (including power model) for evaluation.

Agreement:

Apply the timing errors as follows:

- For each UE drop,

  - For each panel (in case of multiple panels)

    - Draw a random sample for the Tx error according to [-2*Y,2*Y] and another random sample for the Rx error according to the same [-2*Y,2*Y] distribution.

- For each gNB

  - For each panel (in case of multiple panels)

    - Draw a random sample for the Tx error according to [-2*X,2*X] and another random sample for the Rx error according to the same [-2*X,2*X] distribution.

- Any additional Time varying aspects of the timing errors, if simulated, can be left up to each company to report.

- For UE evaluation assumptions in FR2, it is assumed that the UE can receive or transmit at most from one panel at a time with a panel activation delay of 0ms.

Conclusion:

For UE mobility, the details of the optional mobility model are left to companies.

Agreement:

PRS/SRS resource utilization is the metric used to evaluate network efficiency

- FFS: what is included in resource utilization, e.g. PRS/SRS/MG configurations, beam sweeping assumptions

Agreement:

For the absolute time of arrival modelling in IOO, UMa, Umi, companies may provide the details of their model, if any

Agreement:

Text proposal for LS to RAN WG2 and CC SA WG2 and RAN WG3 for analysis of latency of NR positioning protocols defined in Rel.16:

RAN1 evaluates physical layer latency and its potential reduction for NR Rel-17 positioning solutions. In order to evaluate End-To-End latency of NR positioning solutions the input from RAN2 is needed on latency components of NR/NG-RAN/5GC higher layer positioning protocols. RAN1 respectfully asks if RAN2 can provide a list of latency components with corresponding range of values for the existing and any potential enhanced NR positioning solutions, keeping in mind the End-To-End latency described as desired in the study item description (RP-200928)

Final LS approved in R1-2007264

Agreement:

Physical layer latency for DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted approaches are separately studied

Agreement:

Capture the following in TR as an observation:

- Performance analysis of baseline I-IoT InF scenarios shows that InF-SH scenario is characterized by high probability of LOS links. In InF-DH the probability of LOS links is reduced substantially while probability of NLOS links is increased accordingly.

Conclusion:

- Evaluations show that high probability of NLOS links and propagation delay offset imposed by NLOS links may cause performance degradation of positioning accuracy, that was especially observed in InF-DH scenario

- Initial evaluations have also shown that under certain ideal assumptions (e.g., synchronization error, Rx/Tx calibration error) the effective LOS/NLOS classification/detection, outlier determination/rejection techniques may be beneficial to improve NR positioning accuracy

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Conclusion:

- It is observed that calibration errors of UE/gNB Tx/Rx timing may negatively impact accuracy of timing-based methods of Rel.16 positioning solutions when precise UE positioning is targeted.

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Conclusion:

- Evaluations show that network synchronization errors may cause accuracy degradation of the DL-TDOA or UL-TDOA Rel-16 positioning solutions

- Note: Additional evaluations need to be performed before deciding whether and how to capture the above in the TR

Agreement:

- FFS whether Rel.16 granularity of timing measurement reports is enough to avoid degradation in I-IoT scenarios and meet positioning requirements

Agreement:

- At least the following information is provided for positioning physical layer latency analysis:

  - Source initiating request for positioning measurements/location for a given UE (UE, Network)

  - Destination awaiting for positioning measurements/location for a given UE (UE, Network)

  - Start and end triggers/events for physical layer latency evaluation

    - For Rel.16 solutions, it is based on specification for each solution

  - Initial and final RRC State of positioned UE (RRC IDLE, INACTIVE, CONNECTED) at the start and end time for the physical layer latency evaluation

  - Positioning

    - technique (enumeration): (1) DL-TDOA, (2) DL AoD, (3) UL-TDoA, (4) UL-AoA, (5) Multi-RTT, (6) E-CID

    - type: DL, UL, DL+UL

    - mode: UE-based, UE-assisted

  - Latency component w/ value range and description, including information on any parallel (simultaneous) components

  - Total latency value

- Latency components are recommended to be captured in table and ordered consequently in time starting from the earliest one:

| Source [UE, NW]/Destination [UE, NW] Positioning technique [DL-TDOA, E-CID, …], type [DL, UL, DL+UL], mode [UE-A, UE-B], Initial and Final RRC States [IDLE, INACTIVE, CONNECTED] |  |  |
| --- | --- | --- |
| Latency Component | Value Range | Description of Latency Component |
| Start trigger |  |  |
| Name of component 1 |  |  |
| Name of component 2 |  |  |
| Name of last component |  |  |
| End trigger |  |  |
| Total values |  |  |

Agreement:

Partial staggering and non-staggering RE mapping of SRS for positioning with different combinations of comb-factors and symbol lengths will be investigated in Rel-17.

- The methods/signalling for addressing potential time-domain aliasing due to the partial/non-staggering RE mapping will be included in the study

Agreement:

- Semi-persistent and a-periodic transmission and reception of DL PRS will be investigated in Rel-17.

  - FFS: the details on when and how to enable semi-persistent and a-periodic DL PRS

  - FFS: to be supported for which positioning methods, e.g.,

    - UE-assisted and/or UE-based positioning

    - DL positioning and/or Multi-RTT

- On-demand transmission and reception of DL PRS will be investigated in Rel-17.

  - FFS: the details on when and how to enable on-demand DL PRS

  - FFS: to be supported for which positioning methods, e.g.,

    - UE-assisted and/or UE-based positioning

    - DL positioning and/or Multi-RTT

- Notes:

  - Semi-persistent means MAC-CE triggered

  - Aperiodic would correspond to DCI-triggered

  - On-demand corresponds to the UE-initiated or network-initiated request of PRS and/or SRS. So, it is NOT the same as whether PRS is DCI-triggered or MAC-CE triggered. It is about UE or LMF request/suggesting/recommending specific PRS pattern, ON/OFF, periodicity, BW, etc.

Agreement:

- Multipath mitigation techniques will be investigated in this SI for improving positioning accuracy, which may include, but not limited to the following:

  - The applicable scenarios and performance benefits of multipath mitigation techniques

  - The methods/measurement/signaling for the LOS/NLOS detection and identification

  - The measurements for supporting the multipath mitigation/utilization

  - The procedure and signaling for supporting the multipath mitigation/utilization

  - Implementation-based solutions (e.g., outlier rejection) without the need of any additional specified method/measurements/procedures/signaling.

- Note: The above study applies to DL only, UL only, DL+UL positioning solutions for UE-based and UE-assisted positioning.

Agreement:

- NR positioning for UEs in RRC_IDLE state and UEs in RRC_INACTIVE state will be investigated in Rel-17, including the benefits on latency, network/UE efficiency and UE power consumption

- FFS: which positioning methods to be supported, e.g., DL positioning, UL positioning, DL+UL positioning and/or Multi-RTT

- FFS: the details of how to enable the UE positioning in RRC_IDLE state and RRC_INACTIVE state

  - Reference signals (e.g., based on DL PRS signals, UL SRS signals, both of them, etc.)

  - Signaling and procedures (e.g., based on PRACH procedure, paging triggered UL SRS transmission, etc.)

Agreement:

- For reducing NR positioning latency, more efficient signaling & procedures will be investigated to enable a device to request and report positioning information, which may include, but not limited to, the following aspects:

  - DL PRS/UL SRS configuration, activation or triggering.

  - The request for positioning information (the assistance data, etc.).

  - The report of positioning information (the measurement report, etc.).

- Note: It is not within RAN1 scope to analyze positioning architecture enhancements to enable such more efficient signaling & procedures.

- Note: RAN1 does not make any assumptions on whether the LCS architecture specified in TS 23.273 is enhanced or not.

Agreement:

- Aggregating multiple DL positioning frequency layers of the same or different bands for improving positioning performance for both intra-band and inter-band scenarios will be investigated in Rel-17, which may take into account at least the following

- The scenarios and performance benefits of aggregating multiple DL positioning frequency layers

- The impact of channel spacing, timing offset, phase offset, frequency error, and power imbalance among CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

- UE complexity considerations

- Note: What is captured in the TR will be discussed separately.

Agreement:

Simultaneous transmission by the UE and reception by the gNB of the SRS for positioning across multiple CCs and multiple slots can be investigated in Rel-17, which may consider

- The scenarios and performance benefits of the enhancement

- The impact of channel spacing, TA and timing offset, phase offset, frequency error, and power imbalance across slots or CCs to the positioning performance for intra-band contiguous/ non-contiguous and inter-band scenarios

Agreement:

- The scenario, benefits, and methods for improving the accuracy of the UL AoA and DL-AoD methods for both UE-based and network-based (including UE-assisted) positioning can be investigated in Rel-17.

Agreement:

The scenario, benefits, methods and signaling for improving positioning accuracy in the presence of the UE Rx/Tx transmission delays, and/or and gNB Rx/Tx transmission delays, will be investigated for UE-based and network-based (including UE-assisted) positioning in Rel-17.

Agreements (RAN1#103-e)

R1-2009842 is endorsed.

TR draft in R1-2009418 endorsed with the following modifications:

- Remove all instances of “can be investigated in Rel-17”

- Add agreement on DL PRS aggregation into Section 7

R1-2009431 is endorsed as v0.1.1

R1-2009665 is endorsed as v0.1.2 with the following modifications:

- Change “For the issues related to LOS/NLOS issues in positioning” to “For the issues related to mitigating effects of multipath/NLOS for positioning” in Section 8.4

- Remove company names

- Move tables in Section 8.4 to an Appendix

- Remove “(as per the optional model)” in “For issues related to gNB/UE TX/RX timing errors (as per the optional model)” in Section 8.4

Endorse TR draft in R1-2009743 in principle with the following changes:

- Merge 10.8, 10.9 and 10.10 as per Qualcomm’s comments

- Remove individual company observations as per Nokia’s comments

- Update per agreements made in the last GTW

Conclusion:

When reporting network efficiency, the methodology is left up to each company and details should be provided.

Conclusion:

For SRS bandwidth stitching and PRS bandwidth stitching, when modelling the effect of phase offset between two PFLs (for 2 CCs in the UL), a uniformly distributed phase offset is applied to the 2nd PFL (2nd CC in the UL) with respect to the 1st PFL (1st CC in the UL).

- If phase offset modeled, companies to provide how the phase offset is modeled in their evaluations.

Agreement:

- In Rel-17 target positioning requirements for commercial use cases are defined as follows:

  - Horizontal position accuracy (< 1 m) for 90% of UEs

  - Vertical position accuracy (< 3 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100 ms)

  - Physical layer latency for position estimation of UE (< 10 ms)

- In Rel-17 target positioning requirements for IIoT use cases are defined as follows:

  - Horizontal position accuracy (< 0.2 m) for 90% of UEs

  - Vertical position accuracy (< 1 m) for 90% of UEs

  - End-to-end latency for position estimation of UE (< 100ms, in the order of 10 ms is desired)

  - Physical layer latency for position estimation of UE (<10ms)

- Note 1: Target positioning requirements may not necessarily be reached for all scenarios and deployments

- Note 2: For some scenarios the requirement for Horizontal position accuracy can be relaxed to < 0.5 m in IIoT use cases.

- Note 3: All positioning techniques may not achieve the target positioning requirements over all scenarios

Agreement:

Capture the following observations (On horizontal positioning accuracy in InF-SH) in the TR (editorial modifications and addition of references for the sources can be added when incorporating into the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-SH scenario.

- Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is achieved by Rel.16 solutions.

- Results were provided by [12] out of [17] sources for FR1 and by [9] sources out of [17] for FR2

  - For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [3] sources  and is not achieved in contributions from [9] sources

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [7] sources and is not achieved in contributions from [5] sources

  - For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [6] sources and is not achieved in contributions from [3] sources

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [8] sources and is not achieved in contributions from [1] sources

Agreement:

Capture the following observations on horizontal positioning accuracy in InF-DH (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the baseline InF-DH scenario.

- Based on the results provided by a majority of sources, sub-meter level @ 90% of horizontal positioning accuracy is not achieved by Rel.16 based solutions.

- Results were provided by [13] sources (Huawei R1-2007576, BUPT R1-2007720, ZTE R1-2007754, CATT R1-2007859, FUTUREWEI R1-2007908, OPPO R1-2008225, Nokia R1- 2008300, Sony R1-2008364, CEWiT R1-2008720, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665, Intel R1-2007945) out of [17] for FR1 and by [9] sources (Huawei R1-2007576, BUPT R1-2007720, ZTE R1-2007754, CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665, Intel R1-2007945) out of [17] for FR2

  - For NR positioning evaluations in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contribution from [1] source (CATT R1-2007859) and is not achieved in contributions from [12] sources (Huawei R1-2007576, BUPT R1-2007720, ZTE R1-2007754, FUTUREWEI R1-2007908, OPPO R1-2008225, Nokia R1- 2008300, Sony R1-2008364, CEWiT R1-2008720, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665, Intel R1-2007945)

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [4] sources (BUPT R1-2007720, CATT R1-2007859, QC R1-2009361, vivo R1-2007665) and is not achieved in contributions from [9] sources (Huawei R1-2007576, ZTE R1-2007754, FUTUREWEI R1-2007908, OPPO R1-2008225, Nokia R1- 2008300, Sony R1-2008364, CEWiT R1-2008720, Ericsson R1-2008764, Intel R1-2007945)

  - For NR positioning evaluations in FR2 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 0.2m @ 90% is achieved in contributions from [3] sources (BUPT R1-2007720, QC R1-2008618, vivo R1-2007665) and is not achieved in contributions from [6] sources (Huawei R1-2007576, ZTE R1-2007754, CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, Intel R1-2007945)

    - Accuracy of ≤ 0.5m @ 90% is achieved in contributions from [3] sources (BUPT R1-2007720, QC R1-2008618, vivo R1-2007665) and is not achieved in contributions from [6] sources (Huawei R1-2007576, ZTE R1-2007754, CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, Intel R1-2007945)

Agreement:

Capture the following observations on vertical positioning accuracy in InF-SH (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-SH scenario.

- Results were provided by [4] sources (ZTE R1-2007754, CATT R1-2007859, vivo R1-2007665, Intel R1-2007945) out of [17] for FR1 and by [4] sources (ZTE R1-2007754, CATT R1-2007859, QC R1-2008618, Intel R1-2007945) out of [17] for FR2 band

  - For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [2] sources (ZTE R1-2007754, vivo R1-2007665) and is not achieved from [2] sources (CATT R1-2007859, Intel R1-2007945)

  - For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [4] sources (ZTE R1-2007754, CATT R1-2007859, QC R1-2008618, Intel R1-2009390) [and is not achieved by [0] sources]

Agreement:

Capture the following observations on vertical positioning accuracy in InF-DH (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the InF-DH scenario (including evaluations with variable gNB/UE heights).

- Results were provided by [5] sources (ZTE R1-2007754, CATT R1-2007859, vivo R1-2007665, Intel R1-2007945, Huawei R1-2007576) out of [17] for FR1 and by [4] sources (ZTE R1-2007754, CATT R1-2007859, Intel R1-2007945, Huawei R1-2007576) out of [17] for FR2 band

  - For NR positioning evaluations in FR1 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [2] sources (CATT R1-2007859, vivo R1-2007665) and is not achieved from [3] sources (ZTE R1-2007754, Intel R1-2007945, Huawei R1-2007576)

  - For NR positioning evaluations in FR2 band, the following is observed with respect to vertical positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contribution from [1] source (Huawei R1-2007576) and is not achieved from [3] sources (ZTE R1-2007754, CATT R1-2007859, Intel R1-2007945)

Agreement:

Capture the following in the TR:

- Evaluation results for LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms in indoor factory scenarios were provided by [12] sources (OPPO, Futurewei, vivo, Intel, Qualcomm, ZTE, Huawei, CeWiT, Nokia, Sony, Fraunhofer, Ericsson) out of [17] sources

- NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques

- From the evaluations,

  - [9] sources (Futurewei, Intel, ZTE, Huawei, CeWiT, Nokia, Sony, Fraunhofer, Ericsson) evaluated LOS/NLOS identification with additional specification changes relative to Rel.16 solutions

  - [2] sources (vivo, Qualcomm) evaluated outlier rejection algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

  - [1] source (OPPO) evaluated NLOS mitigation using triangle-based inequality algorithm (implementation-based algorithm that can be applied for Rel.16 solutions without specification changes)

- Comparative analysis of LOS/NLOS identification with specification changes vs implementation based methods (outlier rejection algorithms) was done by 6 sources (Intel, Huawei, vivo, Qualcomm, ZTE, Oppo)

  - Three sources (Intel, Huawei, ZTE) observe that NR positioning based on LOS/NLOS identification outperforms NR positioning utilizing outlier rejection

  - Three sources (vivo, Qualcomm, Oppo) observe that NR positioning utilizing outlier rejection outperforms NR positioning utilizing LOS/NLOS identification

Agreement:

- Capture the following in the TR:

  - Evaluation results of gNB/UE TX/RX timing errors (as per the optional model) are provided by [7] sources (Huawei, ZTE, Qualcomm, Intel, CATT, Ericsson, vivo) out of [17] sources)

- Capture summary of results provided in Tables in Section 2.1.2.3 of R1-2009606 in TR with the following modifications

  - Removal of the numbers in brackets after “YES”/”No”.

  - Change “barely” to “NO”

Agreement:

Capture the following in the TR:

- Evaluation results for aggregation of DL positioning frequency layers were provided by [5] sources (Intel, Qualcomm, Huawei, vivo, Ericsson) out of [17].

- Aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments such as: bandwidth and spacing of aggregated layers, timing offset and frequency offset over frequency layers, phase discontinuity and possible amplitude imbalance.

  - One source (Huawei) observes that aggregation with phase continuity can help to improve the positioning accuracy, and discontinuous aggregation can approach the performance of contiguous aggregation with the same frequency span

  - One source (Intel) has shown that  aggregation of frequency layers (without modeling impairements) improves the positioning accuracy for intra-band contiguous configuration and that further study is needed for other cases including impairments

  - One source (Ericsson) has observed that PRS aggregation shows potential gains without modeling phase error, but these gains are lost when the phase error between CCs becomes too large

  - One source (Qualcomm) has analyzed aggregation of 2 and 4 frequency layers for different channel spacings, time and phase offset across frequency layers

  - One source (vivo R1-2007666) has analyzed aggregation of 2 frequency layers for different time offset values and observed that:

    - For the case without impairements modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, performance target [0.2m @ 90%] cannot be achieved in both InF-SH and InF-DH.

    - For the case without impairements modeling, aggregation of multiple DL positioning frequency layers 50MHz+50MHz, the performance is worse than 100MHz but better than 50MHz.

    - The performance of aggregation of frequency layers degrades if timing offset is increased

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Summary table on physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted NR positioning from discussion round #1 in the TR in Section 3.1.1 of R1-2009606

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR1 was provided by [11] sources

- Summary of physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning in FR2 was provided by [4] sources

- For evaluation in FR1,

  - results from [11] sources out of [11] sources (Qualcomm, Huawei, ZTE, vivo, Lenovo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] (ZTE, Intel) sources out of [11] sources (Qualcomm, Huawei, ZTE, vivo, Lenovo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- For evaluation in FR2,

  - results from [4] sources out of [4] sources (ZTE, vivo, Lenovo, OPPO) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 10ms

  - results from [2] (ZTE, vivo) sources out of [4] sources (ZTE, vivo, Lenovo, OPPO) show that minimum estimated physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-assisted NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL TDOA/DL-AOD UE-assisted NR Positioning

  - DL PRS alignment, transmission, measurement (including processing time) and report delay

  - Measurement gap request, configuration and alignment time

  - UE/gNB higher layer (LPP/RRC) processing times

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table in Section 3.2.1 of R1-2009606 on physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR1 was provided by [8] sources (Huawei, vivo, LGE, CATT, Nokia, OPPO, Interdigital, Intel)

- Summary of physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning in FR2 was provided by [2] sources (vivo, OPPO)

- For evaluation in FR1,

  - results from [3] sources (Huawei, CATT, Nokia) out of [8] sources (Huawei, vivo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 10ms

  - results from [8] sources out of [8] sources (Huawei, vivo, LGE, CATT, Nokia, OPPO, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from [2] sources out of [2] sources (vivo, OPPO) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning exceeds 10ms

  - results from [1] (OPPO) sources out of [2] sources (vivo, OPPO) show that minimum estimated physical layer latency for Rel.16 UL-TDOA/UL-AOA NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 UL-TDOA/UL-AOA NR Positioning

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - gNB higher layer processing delays (RRC/ NRPPa processing times)

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table on physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR1 was provided by [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel)

- Summary of physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning in FR2 was provided by [0] sources

- For evaluation in FR1,

  - results from [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel) out of [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning exceeds 10ms

  - results from [4] sources (Qualcomm, Huawei, vivo, Interdigital) out of [6] sources (Qualcomm, Huawei, vivo, LGE, Interdigital, Intel) show that minimum estimated physical layer latency for Rel.16 Multi-RTT UE-assisted NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 Multi-RTT UE-assisted NR positioning

  - DL PRS alignment, transmission, measurement time and report delay

  - Measurement gap request, configuration, alignment time

  - SRS for positioning processing time

  - SRS for positioning alignment time (depends on periodic or aperiodic SRS for positioning)

  - UE/gNB higher layer (LPP/RRC/NRPPa) processing times

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table on physical layer latency for Rel.16 E-CID NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR1 was provided by [3] sources (Huawei, ZTE, LGE)

- Summary of physical layer latency for Rel.16 E-CID NR positioning in FR2 was provided by [0] sources

- For evaluation in FR1,

  - results from [2] sources (ZTE, LGE) out of [3] sources (Huawei, ZTE, LGE) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning exceeds 10ms

  - results from [3] sources (Huawei, ZTE, LGE) out of [3] sources (Huawei, ZTE, LGE) show that minimum estimated physical layer latency for Rel.16 E-CID NR positioning does not exceed 100ms

- The following list provides the major physical layer latency components for Rel.16 E-CID NR positioning

  - Higher layer signaling processing

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- Capture summary table on physical layer latency for Rel.16 DL-only UE-based NR positioning from discussion round #1 in the TR

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR1 was provided by [6] sources (Qualcomm, Huawei, vivo, Lenovo, OPPO, Interdigital)

- Summary of physical layer latency for Rel.16 DL-only UE-based NR positioning in FR2 was provided by [2] sources (vivo, Lenovo)

- For evaluation in FR1,

  - results from [4] sources (Huawei, vivo, OPPO, Interdigital) out of [6] sources (Qualcomm, Huawei, vivo, Lenovo, OPPO, Interdigital) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [6] sources out of [6] sources (Qualcomm, Huawei, vivo, Lenovo, OPPO, Interdigital) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning does not exceed 100ms

- For evaluation in FR2,

  - results from [2] sources out of [2] sources (vivo, Lenovo) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 10ms

  - results from [1] (vivo) sources out of [2] sources (vivo, Lenovo) show that minimum estimated physical layer latency for Rel.16 DL-only UE-based NR positioning exceeds 100ms

- The following list provides the major physical layer latency components for Rel.16 DL-only UE-based NR positioning

  - DL PRS alignment, transmission, measurement time and, if requested, report delay

  - Measurement gap request, configuration, alignment time

  - Higher layer (LPP/RRC) processing times

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

- Results were provided by [2] sources (Ericsson R1-2008764, QC R1-2008618) out of [17] for FR1 band

  - For NR positioning evaluations for UMa scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 80% is achieved for the outdoor UEs in contributions from [1] source (QC, R1-2008618) out of [2] sources (Ericsson R1-2008764, QC R1-2008618) in the scenario without absolute time of arrival modelling. Zero sources met an accuracy of ≤ 1m @ 90%.

    - Accuracy of ≤ 10m @ 90% is achieved for the outdoor UEs in contributions from [2] sources (Ericsson R1-2008764, QC R1-2008618) out of [2] sources in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 10m @ 90% is achieved for the indoor UEs in contributions from [1] source (Ericsson R1-2008764) out of [2] sources in the scenario without absolute time of arrival modelling

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Results were provided by [4] sources (Nokia R1- 2008300, Ericsson R1-2008764, QC R1-2008618, Fraunhofer R1-2009428) out of [17] for FR1 band

  - For NR positioning evaluations for UMi scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contributions from [2] sources (Ericsson R1-2008764, QC R1-2008618) and is not achieved from [2] sources (Nokia R1- 2008300, Fraunhofer R1-2009428) in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 1m @ 90% is not achieved from [2] sources (QC R1-2008618, Fraunhofer) in a scenario with absolute time of arrival modelling

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

- Results were provided by [5] sources (CATT R1-2007859, Nokia R1- 2008300, Sony R1-2008364, Ericsson R1-2008764, vivo R1-2007665) out of [17] for FR1 and [5] sources (CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665) out of [17] for FR2 band

  - For NR positioning evaluations for IOO scenario in FR1 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contributions from [4] sources (CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, vivo R1-2007665) and is not achieved from [1] source (Nokia R1- 2008300) in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 1m @ 90% is achieved from [1] source (vivo R1-2005380) in a scenario with absolute time of arrival modelling

  - For NR positioning evaluations for IOO scenario in FR2 band, the following is observed with respect to horizontal positioning accuracy:

    - Accuracy of ≤ 1m @ 90% is achieved in contributions from [5] sources (CATT R1-2007859, Sony R1-2008364, Ericsson R1-2008764, QC R1-2008618, vivo R1-2007665) in the scenario without absolute time of arrival modelling

    - Accuracy of ≤ 1m @ 90% is achieved from [1] source (vivo R1-2005380) in a scenario with absolute time of arrival modelling

Agreement:

Capture the following observations (Editorial modifications and updates to references to be made when capturing in the TR):

- The results for the UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were analyzed by 2 sources (Huawei/HiSi, vivo) out of 17 sources (assumptions may be different between the different sources)

- In one source (Huawei/HiSi), the following observations were made:

  - RRC_IDLE/RRC_INACTIVE state positioning can save about 7%-40% power consumption compared to C-DRX configuration

- In one source (vivo), the following observations were made:

  - Positioning report in the RRC_IDLE state can provide 44.32 % of power saving gain compared to the report in the RRC_CONNECTED state

  - Positioning measurement and report in the RRC_IDLE state can provide at least 48.38 % of power saving gain compared to the measurement and report in the RRC_CONNECTED state

Conclusion:

Estimated minimum DL PRS measurement time in Rel.16 can be 88.5ms depending on DL PRS configuration settings

- Note: The following assumptions are made

  - One DL PRS frequency layer in FR1

  - CSSF = 1

  - NRxBeam, i = 1,

  - Nsample = 4 (DL PRS RSTD measurements are done across 4 DL PRS periods)

  - Both DL PRS periodicity and MGRP are equal to 20ms

  - Configured DL PRS resources are within UE DL PRS processing capacity (N,T) = (0.5ms, 8ms)

Conclusion:

- SRS for positioning measurement time of 12 ms can be achieved under certain SRS for positioning configuration settings depending on the frame configuration

  - Note: The following assumptions are made

    - SRS for positioning alignement time 0.5 ms

    - SRS for positioning transmission time 0.5ms

    - SRS for positioning processing time 5 ms

    - 30 kHz SCS in FR1

    - Single SRS resource set with single SRS resource

    - Four SRS instances

- Note: Considering UL link budget and interference on SRS for positioning signals, the longer transmission time may be needed that will further increase SRS for positioning measurement time

Agreement:

Capture the following observations in the TR (Editorial modifications and updates to references to be made when capturing in the TR):

  - The results for the PRS resource utilization were analyzed by 3 sources (Huawei/HiSi, vivo, CATT) out of 17 sources

  - In one source (Huawei), the PRS resource utilization was evaluated for the case of 160 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12, 4, and 1 symbol per PRS resource:

    - PRS with 12, 4, and 1 symbol has positioning resource utilization of 2.14 %, 0.714 %, and 0.179 %, respectively

  - In one source (vivo), the PRS resource utilization was evaluated:

    - In FR1, for 20 ms DL PRS periodicity and MG periodicity, 3ms MGL, 30 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 1 beams per site, PRS resource utilization is 3.21% while the MGL/MGRP (UE overhead) is 15%.

    - In FR2, for 20 ms DL PRS periodicity, 20ms for MGL and MGRP, 120 kHz subcarrier spacing, comb 6 and 6 symbols per PRS resource, 18 positioning sites and 64 beams per site, PRS resource utilization is 51.42% while the MGL/MGRP (UE overhead) is 100%

    - It was observed by the source that the network and device efficiency can be improved by on-demand PRS (assuming the same latency) compared to periodic PRS

  - In one source (CATT), the PRS resource utilization was evaluated for the case of 20 ms DL PRS periodicity, 30 kHz subcarrier spacing, and 12 symbols per PRS resource:

    - PRS with 12 symbols has positioning resource utilization of 2.1 %.

Agreement:

- NR positioning for UEs in RRC_INACTIVE state is recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

- The details of how to enable the UE positioning in RRC_ INACTIVE state can be further discussed during normative work. These details may include, but are not limited to the following aspects:

  - UL reference signals (e.g., SRS for positioning, PRACH preambles) for UL measurements

  - Signalling and procedures for support the assistance data delivery, DL-PRS configuration, UL reference signals for positioning resource configuration, measurement reporting), which may be developed based on the enhancements of existing signalling and procedures (e.g., existing 2-step and/or 4-step PRACH procedures, paging procedure, small data transmission).

Agreement:

Capture the following in the TR:

From a physical layer perspective, on-demand transmission and reception of DL PRS, which includes at least the following is recommended

  - UE-initiated request of on-demand DL PRS transmission

  - LMF (network)-initiated request of on-demand DL PRS transmission

- Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

Agreement:

Capture the following in the TR:

Simultaneous transmission by the gNB and reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs can be studied further and if needed, specified during normative work

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, bands and RF architectures, can be further studied

Agreement:

Capture the following in the TR:

Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers can be studied further and if needed, specified during normative work.

- From both gNB and UE perspective, the applicability and feasibility of this enhancement for different scenarios, configurations, particular bands and RF architectures, can be further studied.

Agreement:

Capture the following in the TR:

- The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays are recommended for normative work, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Note: The details of the solutions are left for further discussion in normative work.

Agreement:

Capture the following in the TR:

The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

- UL AoA is recommended for normative work for network-based positioning solutions.

- DL-AoD is recommended for normative work for UE-based and network-based (including UE-assisted) positioning solutions.

Agreement:

Capture the following in the TR:

Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation can be studied further, and if needed, specified during normative work for improving positioning accuracy.

- Note: The details of the enhancements of reporting are left for further discussion in normative work, which may include, but are not limited to the following information associated with multi-path, e.g., LOS/NLOS identification, time of arrival of the multi-path components, signal power and/or relative power, power delay profile, angle, and/or polarization information, coherence bandwidth, etc.

Agreement:

Capture the following in the TR:

Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Aperiodic reception in the above corresponds to DCI-triggered reception

Agreement:

Capture the following in the TR:

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs can be studied further and if needed, specified during normative work.

- Note: Semi-persistent reception in the above corresponds to MAC-CE activated reception

Agreement:

Capture the following in the TR:

- The enhancements of signaling & procedures for reducing NR positioning latency are recommended for normative work, including DL and DL+UL positioning methods

  - The details of the solutions are left for further discussion in normative work, which may include the following aspects:

    - Latency reduction related to the measurement gap

    - Latency reduction related to the reporting and request of the measurements (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules)

    - Latency reduction related to measurement time

- The following enhancements of signaling & procedures for reducing NR positioning latency can be studied and specified, if needed

  - Latency reduction related to the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure)

  - Latency reduction related to the reception of DL PRS (e.g., priority rules for the reception of DL PRS)

- No assumptions are made on whether the LCS architecture specified in TS 23.273 is enhanced or not.

Agreement:

Capture the following in the TR:

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state.

- Note: This does not imply that measurements have to be reported in RRC_IDLE state.

Conclusion:

It is up to RAN2 to decide whether to support the enhancements of NR positioning reporting of DL positioning measurements and/or positioning estimates for RRC_IDLE UEs.

Agreement:

Capture the following for the Conclusions section of the TR (Modifications can be made based on further discussion as necessary):

------------------------------------------------------- Begin Text ------------------------------------------------------

This study focused on the analysis of potential enhancements and solutions necessary to support the high accuracy, low latency, high network efficiency and device efficiency to NR positioning targeting both general commercial and IIOT use cases.

In the study item, Rel-17 target positioning requirements for RAT dependent solutions were discussed and defined for general commercial use cases and IIoT use cases, including horizontal and vertical positioning accuracy, and physical layer and end-to-end positioning latency (see Section 5). Additional scenarios and channel models for evaluating Rel-17 NR positioning enhancements were developed for the evaluation of the achievable positioning performance of the enhancements (see Section 6).

The potential positioning enhancements for improving positioning accuracy, reducing latency, and improving network and device efficiency of NR positioning were studied. The potential positioning enhancements, which were investigated rigorously in this study, as outlined in Section 7.X.

NR positioning accuracy with Rel.16 positioning solutions were evaluated under the condition that gNB time synchronization error and gNB/UE TX/RX timing errors are not modelled for InF-SH scenario and InF-DH scenario for both FR1 and FR2 bands. The evaluation results show:

- For horizontal positioning accuracy,

  - in the InF-SH scenario, based on the results provided [by a majority of sources], sub-meter level @ 90% is achieved in both FR1 and FR2 bands.

  - in the InF-DH scenario, based on the results provided [by a majority of sources], sub-meter level @ 90% is not achieved in both FR1 and FR2 bands.

- For vertical positioning accuracy

  - in the InF-SH scenario,

    - sub-meter level @ 90% is achieved by some sources but not achieved by some other sources in FR1 band

    - sub-meter level @ 90% is achieved by all sources in FR2 band;

  - in the InF-DH scenario,

    - sub-meter level @ 90% is achieved by some sources and is not achieved by some other sources in both FR1 and FR2 bands

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the IOO scenario

- Based on the results provided by a majority of the sources, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in IOO scenario

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMa scenario

- Based on the results provided, 10 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMa scenario

For the case without modeling synchronization and gNB/UE TX/RX timing errors in the UMi scenario

- Based on the results provided by some of the companies, 1 m level @ 90% of horizontal positioning accuracy is achieved by Rel.16 in UMi scenario

The impact of NLOS/multipath on NR positioning accuracy and the resolutions for NLOS/multipath mitigation were investigated. NR positioning utilizing LOS/NLOS identification, outlier rejection, NLOS mitigation based on triangle inequality algorithms improve performance of positioning accuracy with respect to solutions that do not apply these techniques.

The impact of gNB/UE TX/RX timing errors on NR positioning accuracy were investigated. Evaluation results show the gNB/UE TX/RX timing errors have significant impact on positioning accuracy.

Aggregation of NR positioning frequency layers for improving positioning accuracy were investigated. Evaluation results show that aggregation of NR positioning frequency layers improves positioning accuracy under certain scenarios, configurations, and assumptions on modelled impairments as outlined in Section [8.4].

Physical layer latency for Rel.16 DL-TDOA/DL-AOD UE-Assisted, UL-TDOA/UL-AOA, Multi-RTT, E-CID and DL-only UE-based NR positioning were investigated, and the major physical layer latency components for these NR positioning techniques were also identified as shown in Section [8.4].

The UE efficiency (power saving) in the RRC_IDLE/RRC_INACTIVE states were also analysed, and power saving gains are observed with detailed observations related to power savings are outlined in Section [8.4].

The network efficiency in terms of resource utilization was analyzed and benefits of potential positioning enhancements observed are outlined in Section [8.4].

The potential positioning enhancements for improving positioning accuracy, reducing latency, and improving network and device efficiency of NR positioning were studied.

The following enhancements have been recommended for normative work

- NR positioning for UEs in RRC_INACTIVE state, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

  - Support of UE positioning measurements for UEs in RRC_inactive state

    - Options that can be considered include DL-PRS or DL-PRS and SSB

  - Support of gNB positioning measurements for UEs in RRC_inactive state

- On-demand transmission and reception of DL PRS, which includes at least

  - UE-initiated request of on-demand DL PRS transmission

  - LMF (network)-initiated request of on-demand DL PRS transmission

  - Above enhancements are recommended for both DL and DL+UL positioning methods and both UE-based and UE-assisted positioning solutions.

- The methods, measurements, signaling, and procedures for improving positioning accuracy in the presence of the UE Rx/Tx timing delays, and/or and gNB Rx/Tx timing delays, including

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- The enhancements of the procedure, measurements, reporting, and signalling for improving the accuracy of

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions

- The enhancements of signaling & procedures for reducing NR positioning latency related to, including DL and DL+UL positioning methods:

  - the measurement gap

  - the measurement request and reporting (e.g., via RRC signaling, MAC-CE and/or physical layer procedure, and/or priority rules)

  - the measurement time

The following enhancements are considered beneficial for the purpose of improving positioning accuracy, reducing latency, improving network and/or device efficiency and are being recommended to be studied further and if needed, specified during normative work

- Simultaneous transmission by the gNB and aggregated reception by the UE of intra-band one or more contiguous carriers in one or more contiguous PFLs

- Simultaneous transmission by the UE and aggregated reception by the gNB of the SRS for positioning in multiple contiguous intra-band carriers

- Enhancements of information reporting from UE and gNB for supporting multipath/NLOS mitigation

- Aperiodic reception of DL PRS from the TRPs of the serving gNB and aperiodic reception of DL PRS from the TRPs of the neighbouring gNBs

- Semi-persistent reception of DL PRS from the TRPs of the serving gNB and Semi-persistent reception of DL PRS from the TRPs of the neighbouring gNBs

- Enhancements of signaling & procedures for reducing NR positioning latency related to

  - the request and response of positioning assistance data (e.g., via RRC signaling, MAC-CE and/or physical layer procedure)

  - the reception of DL PRS (e.g., priority rules for the reception of DL PRS

From a physical layer perspective, it is feasible for a UE to perform DL positioning measurement in RRC_IDLE state. This does not imply that measurements have to be reported in RRC_IDLE state.

It is recommended to proceed with a normative work to support NR positioning enhancements.

------------------------------------------------------- End Text ------------------------------------------------------

#### 2.1.2	Remaining Open issues

None

## 2.2	RAN2

#### 2.2.1	Agreements

Agreements (RAN2#112-e)

Enhancements for commercial use cases:

Agreement:

Agreements:

1: For latency analysis of Rel.16 solutions, RAN2 only consider the latency of positioning procedure, i.e. step 5 in MO-LR/step 12 in MT-LR (involving RRC, LPP, NRPPa, MAC).  A note is added to the TR and in our response to RAN1 to clarify this is what we covered.  We can clarify to RAN1 that more time would be needed for an end-to-end analysis.

2: For latency analysis of Rel.16 solutions, RAN2 only consider the latency caused by UE, gNB, AMF and LMF.

3: For latency analysis of Rel.16 solutions, RAN2 consider both UE-based and UE-assisted.

Agreements:

Positioning measurement reporting (including location estimates for UE-based) should be supported in RRC_INACTIVE; involvement of SDT is FFS.  Reporting of specific measurements is pending RAN1 decision.

Agreements on on-demand PRS:

RAN2 study on-demand PRS mechanism for DL-based, UL&DL-based methods (e.g. multi-RTT), and UE-Based and UE-assisted positioning methods in this SI.

Summary of latency results has been endorsed in R2-2010872, and has been sent to RAN1 in R2-2010873.

Note: RAN2 will continue the discussion on following issues based on email discussion:

- TP for latency analysis results;

- Evaluation of latency enhancement solutions;

- support of on demand PRS;

- Positioning support in RRC_IDLE/RRC_INACTIVE

Integrity:

Agreement:

Agreements:

1	 RAN2 to agree following additional sub-feared events:

3.	External feared events, e.g.

-	Spoofing

-	Jamming/interference

4.	UE faults

-	GNSS receiver measurement error

-	Hardware faults

2	 RAN2 to confirm the need to capture the table on feared events and corresponding assistance data in the TR; the actual handling of these events is FFS.

Note: RAN2 will continue the discussion to finalise integrity text proposals based on email discussion:

- TP on Integrity KPIs, Concepts, Use Cases;

- TP on Integrity Error Sources;

- TP on Integrity Methodologies;

#### 2.2.2	Remaining Open issues

Commercial use cases:

-	Continue the evaluation on latency enhancement solutions;

-	Continue the study on support of on demand PRS and Positioning support in RRC_IDLE/RRC_INACTIVE;

Integrity:

-	Finalise integrity text proposals, and identify additional issues to be resolved in the SI phase;

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

- R1-2009835	Session notes for 8.5 (Study on NR Positioning Enhancements)	Ad-Hoc Chair (Ericsson)

- R1-2008762	Update of TR 38.857	Ericsson

- R1-2009418	Update of TR 38.857	Ericsson

- R1-2009430	Study on NR Positioning Enhancements, v0.1.0	Ericsson

- R1-2009431	Update of TR 38.857, v0.1.0	Ericsson

- R1-2009665	Update of TR 38.857	Ericsson

- R1-2009670	3GPP TR 38.857 Study on NR Positioning Enhancements, v0.2.0	Ericsson

- R1-2009743 	3GPP TR 38.857 Study on NR Positioning Enhancements, v0.3.0	Ericsson

- R1-2007575	Scenarios and evaluation assumption for IIoT positioning	Huawei, HiSilicon

- R1-2007664	Discussion on additional scenarios for NR positioning evaluation	vivo

- R1-2007753	Remaining issues on evaluation assumptions	ZTE

- R1-2007858	Remaining issues on additional scenarios for evaluation of NR Positioning Enhancements	CATT

- R1-2007907	Remaining Issues on Evaluation Assumptions	FUTUREWEI

- R1-2007944	NR positioning scenarios	Intel Corporation

- R1-2008014	Remaining issues on target performance requirement of IIoT use case	CMCC

- R1-2008224	Discussion on Additional Scenarios for Evaluation of NR Positioning	OPPO

- R1-2008299	Additional scenarios for evaluation of NR positioning	Nokia, Nokia Shanghai Bell

- R1-2008363	Considerations on Additional Scenarios for Evaluation	Sony

- R1-2008415	Discussions on additional scenarios for evaluation for NR positioning	LG Electronics

- R1-2008488	Discussion on evaluation assumptions for latency	InterDigital, Inc.

- R1-2008617	Considerations on Additional Scenarios for Evaluation	Qualcomm Incorporated

- R1-2008763	Remaining details on additional scenarios for evaluation	Ericsson

- [103-e-NR-ePos-01] Email discussion/approval on additional scenarios for evaluation and TR updates until 11/4; address any remaining aspects by 11/12 – Florent (Ericsson)

- Additional checkpoint where agreements can be declared: 11/9

- R1-2009343	FL summary for additional scenarios for evaluation	Moderator (Ericsson)

- R1-2009364	Output#1 of [103-e-NR-ePos-01] Email discussion on additional scenarios for evaluation	Moderator (Ericsson)

- R1-2009503	Output #2 of [103-e-NR-ePos-01] Email discussion on additional scenarios for evaluation	Moderator (Ericsson)

- R1-2007576	Evaluation results for Rel-16 positioning and Rel-17 enhancement	Huawei, HiSilicon

- R1-2007665	Evaluation of NR positioning performance	vivo

- R1-2007720	Evaluation of achievable positioning accuracy	BUPT

- R1-2007754	Evaluation of achievable accuracy and latency	ZTE

- R1-2007859	Discussion of evaluation of NR positioning performance	CATT

- R1-2007908	NLOS Identification and Mitigation	FUTUREWEI

- R1-2007945	NR positioning evaluation	Intel Corporation

- R1-2007997	NR Positioning Latency Evaluations	Lenovo, Motorola Mobility

- R1-2008225	Evaluation of NR positioning in IIOT scenario	OPPO

- R1-2008300	Results on evaluation of achievable positioning accuracy and latency	Nokia, Nokia Shanghai Bell

- R1-2008364	Discussion on Performance evaluation of Rel-17 positioning	Sony

- R1-2008416	Discussions on evaluation of achievable positioning accuracy and latency for NR positioning	LG Electronics

- R1-2008489	Evaluation of achievable positioning latency	InterDigital, Inc.

- R1-2008618	Evaluation of achievable Positioning Accuracy & Latency	Qualcomm Incorporated

- R1-2008709	Evaluation of positioning enhancements	Fraunhofer IIS, Fraunhofer HHI

- R1-2008720	Positioning evaluation results on potential enhancements for additional use cases	CEWiT

- R1-2008764	Evaluation of achievable positioning accuracy and latency	Ericsson

- R1-2009345	Feature lead summary for evaluation of NR Positioning enhancements - AI 8.5.2		Moderator (Intel Corporation)

- R1-2009375	Summary #1 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

- R1-2009376	Summary #2 of RAN WG1 E-mail Discussion [103e-NR-ePos-02]- AI 8.5.2	Moderator (Intel Corporation)

- R1-2009406	Summary #3 of RAN WG1 E-mail Discussion [103e-NR-ePos-02]- AI 8.5.2	Moderator (Intel Corporation)

- R1-2009407	Summary #4 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

- R1-2009606	Summary #5 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

- R1-2009607	Summary #6 of RAN WG1 E-mail Discussion [103e-NR-ePos-02] - AI 8.5.2	Moderator (Intel Corporation)

- R1-2009762	Discussion on DL PRS and SRS for Positioning Measurement Time ([103-e-NR-ePos-02])			Moderator (Intel Corporation)

- R1-2007552	Positioning Enhancements	FUTUREWEI

- R1-2007577	Positioning enhancement in Rel-17	Huawei, HiSilicon

- R1-2007666	Discussion on potential positioning enhancements	vivo

- R1-2007721	Potential positioning enhancements	BUPT

- R1-2007755	Discussion on potential NR positioning enhancements	ZTE

- R1-2007860	Discussion of NR positioning enhancements	CATT

- R1-2007886	Potential positioning enhancements	TCL Communication Ltd.

- R1-2007946	NR positioning enhancements	Intel Corporation

- R1-2007998	Potential NR Positioning Enhancements	Lenovo, Motorola Mobility

- R1-2008015	Discussion on potential positioning enhancements	CMCC

- R1-2008083	Potential positioning enhancements	Xiaomi

- R1-2008168	Potential positioning enhancements	Samsung

- R1-2008226	Discussions on NR Positioning Enhancements	OPPO

- R1-2008301	Views on potential positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2008365	Considerations on potential positioning enhancements	Sony

- R1-2008417	Discussions on potential enhancements for NR positioning	LG Electronics

- R1-2008491	Discussion on potential positioning enhancements	InterDigital, Inc.

- R1-2008519	Views on positioning enhancement for Rel-17	MediaTek Inc.

- R1-2008550	Discussion on potential techniques for NR Positioning Enhancements	NTT DOCOMO, INC.

- R1-2008619	Potential Positioning Enhancements for NR Rel-17 Positioning	Qualcomm Incorporated

- R1-2008710	Potential positioning enhancements	Fraunhofer IIS

- R1-2008718	Discussion on positioning enhancements for Release 17	CEWiT

- R1-2008765	Potential positioning enhancements	Ericsson

- R1-2009314	FL Summary for Potential Positioning Enhancements		Moderator (CATT)

- R1-2009366	FL Summary #2 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009395	FL Summary #3 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009396	FL Summary #4 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009397	FL Summary #5 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009398	FL Summary #6 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009399	FL Summary #7 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009673	FL Summary #8 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009678	FL Summary #9 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2009679	FL Summary #10 for Potential Positioning Enhancements	Moderator (CATT)

- R1-2007667	Discussion on new techniques for NR positioning	vivo

- R1-2007756	Channel state estimation based on prior channel information	ZTE

- R1-2007861	Discussion of NLOS channel modelling and network time synchronization for NR positioning			CATT

- R1-2007910	Polarization-based LOS Detection	FUTUREWEI

- R1-2008169	Uplink Transmission Based Relative Positioning	Samsung

- R1-2008227	Analysis of NR Positioning Requirements	OPPO

- R1-2008302	Additional considerations for positioning enhancements	Nokia, Nokia Shanghai Bell

- R1-2008321	Evaluations on additional channel model features	Huawei, HiSilicon

- R1-2008492	Discussion on positioning failure and mobility modeling	InterDigital, Inc.

- R1-2008766	PRS with cyclic shifts	Ericsson

- R2-2008707	LS on Latency of NR Positioning Protocols (R1-2007264; contact: Intel)	RAN1

- R2-2008766   LS on Requirements on positioning for UAS (S6-200269; contact: InterDigital)   SA6

- R2-2009001	Report of [Post111-e][625][POS] End-to-end latency analysis (Intel)	Intel Corporation

- R2-2010866	Summary of latency results	Intel Corporation

- R2-2010872   Summary of latency results	Intel Corporation

- R2-2010867	Draft Reply LS on Latency of NR Positioning Protocols	Intel

- R2-2010873	Reply LS on Latency of NR Positioning Protocols	RAN2

- R2-2010669	Summary of 8.11.2 Enhancements for commercial use cases	CATT

- R2-2010868	[AT112-e][607][POS]Gathering of latency enhancement solutions (CATT)	CATT

- R2-2010881	TP on the proposed latency enhancements for TR 38.857	CATT

- R2-2008775	Discussion on on-demand DL-PRS	OPPO	discussion	Rel-17

- R2-2008776	Positioning in RRC_IDLE and RRC_INACTIVE state	OPPO

- R2-2008810	Further discussion on ehancements for commercial use cases	CATT

- R2-2008885	Discussion on Positioning in Idle/Inactive mode	InterDigital, Inc.

- R2-2008886	Discussion on End-to-End Latency Reduction for DL/UL Positioning	InterDigital, Inc.

- R2-2008887	Discussion on On Demand Reference Signals for Positioning 	InterDigital, Inc.

- R2-2009002	Support of positioning in idle/inactive mode	Intel Corporation

- R2-2009023	Solution directions to reduce end-to-end latency	Intel Corporation

- R2-2009039	Discussion on positioning enhancement	vivo Mobile Communication Co.,

- R2-2009040	Procedure of on-demand PRS	vivo Mobile Communication Co.,

- R2-2009041	Positioning in RRC idle and inactive state	vivo Mobile Communication Co.,

- R2-2009137	Discussion on positioning enhancements for commercial use cases	Spreadtrum Communications

- R2-2009286	Reporting movement models	Fraunhofer IIS	discussion

- R2-2009287	Reporting the situational quality of RAT and RAT-independent technologies	Fraunhofer IIS, Fraunhofer HHI

- R2-2009574	Discussion on PRS enhancements	Beijing Xiaomi Electronics

- R2-2009577	Positioning enhancements on RRC idle/inactive UE and latency reduction	Beijing Xiaomi Electronics

- R2-2009897	Considerations on potential positioning enhancements	Sony

- R2-2010072	Enhancements for commercial use cases	Ericsson

- R2-2010095	NR Positioning Enhancements	Qualcomm Incorporated

- R2-2010096	NR Positioning Latency Analysis and Enhancements	Qualcomm Incorporated

- R2-2010097	On-Demand PRS	Qualcomm Incorporated	discussion

- R2-2010131	Reporting movement models	Fraunhofer IIS, Fraunhofer HHI	discussion

- R2-2010161	On-demand PRS transmission and dynamic PRS resource allocation	Nokia, Nokia Shanghai Bell

- R2-2010276	Discussion on IDLE INACTIVE pos, on-demand PRS and latency analysis	Huawei, HiSilicon

- R2-2010277	Discussion on R17 positioning enhancement	Huawei, HiSilicon

- R2-2010472	Disucssion on IDLE/INACTIVE mode positioning	ZTE Corporation, Sanechips

- R2-2010473	Discussion on on-demand PRS	ZTE Corporation, Sanechips

- R2-2010627	Discussion on enhancement for commercial use cases	Samsung R&D Institute UK

- R2-2010648	Support for positioning in idle/inactive mode	Samsung R&D Institute UK

- R2-2009129	Summary of [Post111-e][626][POS] Email Discussion on integrity use cases and specification impacts	Swift Navigation

- R2-2010880	Email Summary [AT112-e][614][POS] GNSS Integrity	Swift Navigation

- R2-2010877	TP on Integrity KPIs, Concepts, Use Cases	Swift Navigation

- R2-2010878	TP on Integrity Error Sources	Swift Navigation

- R2-2010879	TP on Integrity Methodologies	Swift Navigation

- R2-2008811	Discussion on integrity service level	CATT

- R2-2009760	Positioning integrity for Industrial IoT use cases	Nokia, Nokia Shanghai Bell

- R2-2009898	Discussion on Integrity of positioning information	Sony

- R2-2010074	Industrial IoT use-case	Ericsson

- R2-2010090	Integrity and reliability for IIoT positioning use cases	Convida Wireless

- R2-2010098	Discussion on including PL Availability as an additional integrity KPI	ESA

- R2-2010475	Discussion on integrity&error source factor transmission	ZTE Corporation, Sanechips

- R2-2010700	Summary of 8.11.3.2 Error sources threat models occurrence rates and failure modes	Intel Corporation

- R2-2008812	Discussion on error sources, threat models, occurrence rates and failure modes	CATT	discussion	Rel-17	FS_NR_pos_enh

- R2-2009282	Error sources, threat models, occurrence rates and failure modes	Fraunhofer IIS

- R2-2009331	Discussion on GNSS Integrity Errors	Swift Navigation, Ericsson, Intel Corporation, u-blox

- R2-2010061	Text Proposal on GNSS position integrity error sources	ESA

- R2-2010073	GNSS position integrity error sources	Ericsson

- R2-2010135	Error sources, threat models, occurrence rates and failure modes	Fraunhofer IIS, Fraunhofer HHI

- R2-2010278	Discussion on threat models and failure modes	Huawei, HiSilicon

- R2-2010642	Introduction of Integrity monitoring for GNSS and its error	Samsung R&D Institute UK

- R2-2010675	Summary of 8.11.3.3: Methodologies for network-assisted and UE-assisted integrity	InterDigital

- R2-2008774	Discussion on  methodology for integrity	OPPO

- R2-2008813	Discussion on methodologies for network-assisted and UE-assisted integrity	CATT

- R2-2008888	Discussion on methodologies for network-assisted and UE-assisted integrity	InterDigital, Inc.

- R2-2009003	Methodologies for network-assisted and UE-assisted integrity	Intel Corporation, Swift Navigation

- R2-2009043	Integrity signaling and procedures	vivo Mobile Communication Co.,

- R2-2009138	Discussion on integrity methodologies for network-assisted and UE-assisted integrity	Spreadtrum Communications

- R2-2009333	TP for GNSS Integrity Methodologies	Swift Navigation, Ericsson, Intel Corporation, u-blox

- R2-2009530	Discussion on Positioning Integrity	Apple

- R2-2009578	Discussion on methodologies for positioning integrity	Beijing Xiaomi Electronics

- R2-2009761	Signalling for Positioning Integrity Support	Nokia, Nokia Shanghai Bell

- R2-2010075	Methodologies for network-assisted and UE-assisted integrity	Ericsson

- R2-2010279	Discussion for network-assisted and UE-assisted integrity	Huawei, HiSilicon

- R2-2010474	Discussion of the methodologies for network-assisted and UE-assisted integrity	ZTE Corporation, Sanechips

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template