3GPP TSG-RAN Meeting #90-e	RP-202602

Electronic Meeting, 7th – 11th Dec, 2020

- Source:	Huawei, HiSilicon

- Title:	New WID proposal: RF requirements enhancement for NR frequency range 1 (FR1)

- Document for:	Approval

Agenda Item:	9.8.24

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	RF requirements enhancement for NR frequency range 1 (FR1)

## Acronym: NR_RF_FR1_enh

## Unique identifier: 890062

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-17

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 Work Item is presented or e.g. the related Study Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. List previous Work Items of earlier releases if relevant.}

{This section is mandatory to be filled out by the rapporteur.}

{Not applicable for a Study Item}

{For a Building Block: list here the parent Feature }

{For a Work Task: list here the parent Building Block }

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {optional free text} |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

This work includes two parts: Rel-17 new topics for RF requirement enhancement and Rel-16 FR1 leftover topics.

Regarding Rel-17 new topics, we would like to consider the further enhancement of Tx switching between carriers with UL-MIMO supported on carrier(s), and support of the 26dBm high power UE for intra-band contiguous UL CA.

In Rel-16 the mechanism and RAN4 requirements are specified for Tx switching between case 1 and case 2. The motivation is to select the better uplink carrier and better transmission mode in a given slot for transmission assuming that there are only two con-current transmitters. In Rel-16 it is assumed that only one transmitter can be supported on one carrier and two transmitters can be supported on the other carrier. But recently two transmitters are enabled for more NR bands. Thus it would be possible for UE to support two transmitter on both carriers. In this case, the Tx switching would be conducted between two carriers which are both capable of 2Tx transmissions. The uplink throughput could be further improved. In the meanwhile, UL-MIMO requirements for interested SUL/TDD/FDD bands are to be further specified to enable UE 2Tx transmissions on both carriers when configured switching with SUL or UL-CA band combinations.

The second topic is to support 26dBm high power UE for intra-band contiguous UL CA. It would also be beneficial to investigate the UE architecture to support high power as well as the large aggregated channel bandwidth. And the supported channel bandwidth for PA also needs be studied. Based on the outcome of discussion on UE architecture, the RF requirements can be specified.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objectives for Rel-17 RF requirement continuation work for FR1 include:

- 1) Enable UL MIMO configuration for SUL band configurations

  - Specify UL MIMO requirements for example SUL configurations with SUL band n80

    - Take SUL_n41A-n80A for the example SUL band configuration

  - Remove the RAN2 and RAN4 restriction on configuring UL MIMO for SUL band configurations

- 2) Specify UE requirements to enable Tx switching between different cases across carriers based on SUL and NR inter-band uplink CA for UE supporting maximum two concurrent transmissions

  - Specify UE requirements to enable Tx switching between cases

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (carrier 1 + carrier 2) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

  - Specify UE requirements to enable Tx switching between cases, where 1 carrier on band A and 2 contiguous aggregated carriers on band B, and band A is for SUL or non-SUL and band B is a non-SUL band

    - The scenarios include

      - For Tx switching based on SUL band combination, or uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |

and

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

      - For Tx switching based on uplink CA band combination

|  | Number of Tx chains in WID (band A + band B) |
| --- | --- |
| Case 1 | 1T+1T |
| Case 2 | 0T+2T |
| Case 3 | 2T+0T |

    - Specify the following RAN4 requirements for above scenarios

      - Length of switching period

      - Time mask RF requirements

      - Uplink interruption and downlink interruption (RRM) requirements, if needed

    - Minimize the impacts on RAN1

      - Update RAN1 uplink switching for carrier aggregation and supplementary uplink

    - Minimize the impacts on RAN2

      - Update the RRC signaling to indicate the switching period location and length

      - Update the UE capabilities

Note 1:  Only addressing the case of co-located and synchronized network deployment for the two UL carriers.

Note 2:  Only addressing the case of single TAG for the two UL carriers for SUL and for UL CA.

Note 3:  The UE is configured with two different uplink carrier frequencies.

- 3) HPUE for TDD intra-band contiguous UL CA

  - Take n41 and n78 intra-band contiguous UL CA for examples

    - The two example intra-band contiguous UL CA configurations are under considerations

      - CA_n41C, CA_n78C

  - Take n77 intra-band non-contiguous UL CA for example

    - One example intra-band non-contiguous UL CA configurations are under considerations: CA_n77(2A)

  - Investigate and specify the 26dBm power class for n41and n78 intra-band contiguous and n77 intra-band contiguous/non-contiguous UL CA

    - Identify the impact of different UE architectures on the requirements

      - Power class relation between single CC and intra-band contiguous/non-contiguous CA on HPUE band is clarified if any

    - Specify the mechanism to meet SAR requirements if necessary

      - Mechanism for HPUE on single carrier can be a start point considering the same UL-DL configuration assumption

    - A-MPR requirement

  - Specify MPR requirements

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

This Perf. Part WI has to standardize the Perf. Part requirements:

- Required changes are to be added to release independence TS 38.307.

- Specify RRM performance requirements if needed.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.101-1 | Add FR1 RF Requirements to User Equipment (UE) radio transmission and reception; Part 1 | #95 | Core part |
| 38.101-3 | Add FR1 RF Requirements to User Equipment (UE) radio transmission and reception; Part 3 | #95 | Core part |
| 38.307 | Add FR1 RF Requirements to User Equipments (UEs) supporting a release-independent frequency band | #97 | Perf. part |
| 38.133 | Add RRM core requirements if needed | #95 | Core part |
| 38.133 | Add RRM performance requirements if needed | #97 | Perf. part |
| 38.306 | Add impacts on 38.306 User Equipment (UE) radio access capabilities, if needed | #95 | Core part |
| 38.331 | Add impacts on 38.331 Radio Resource Control (RRC) Protocol specification, if needed | #95 | Core part |
| 38.213 | Add impacts on 38.213 Physical layer procedures for control, if needed | #95 | Core part |
| 38.214 | Add impacts on 38.214 Physical layer procedures for data, if needed | #95 | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

Qian, Zhang, Huawei, zhangqian67@huawei.com

Jianchi Zhu, China Telecom, zhujc@chinatelecom.cn, responsible to R1 and R2 work

## 7	Work item leadership

Primary WG: R4

Secondary WG: R1

## 8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if specific, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

China Telecom

CMCC

Ericsson

HiSilicon

Huawei

LG Electronics

OPPO

Vivo

Xiaomi

ZTE