3GPP TSG RAN WG1 email discussion [5G-ACIA]

October 12 – 16, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Features and simulation assumption for 5G ACIA URLLC LS response

Document for:		Discussion and Decision

# Introduction

In RAN1#102-e, RAN1 received an LS from 5G-ACIA requesting RAN1 to carry out an URLLC and IIOT performance evaluation, using Rel-16 URLLC/IIOT features as a baseline [R1-2006953]. The LS was treated in RAN Plenary, which agreed on the following way forward:

RP-202069 Way forward and RAN work for 5G ACIA requested simulations Ericsson result of email discussion [89E][03][5GACIA_simulation] proposal: - Start an offline email-based activity to provide evaluation results for 5G-ACIA - One company volunteers as moderator - Proposes a work plan to follow - Ericsson is willing do this - Discussions are on the RAN1_NR reflector - Email activity only during short periods (< week) distributed across the time allocated to the activity - No email activity in weeks before/during/after RAN1 meetings or RAN defined inactive periods - All companies should strive to limit email activity as much as possible - Outcome of the offline discussion will directly go to RAN without need for discussion in RAN1 nor need for LS from RAN1 to RAN - Target completion by RAN#91 - At RAN#91, RAN will decide on a response LS to 5G-ACIA

Afterwards, the moderator (Ericsson) shared the following schedule in the RAN1_NR reflector:

- 12-16 October 2020

  - Discussion on which URLLC features to include in the evaluations and simulation assumptions

- 14-18 December 2020

  - First round of simulation results

- 22-26 February 2021

  - Second round of simulation results

- 8-12 March 2021

  - Finalization of the report to RAN#91

In this contribution, we provide our inputs to the item highlighted above. Section 2 discusses our preferred way forward regarding the simulation assumptions, whereas Section 3 discusses our view on which features should be included for the performance evaluation. Observations and proposals are then summarized in Section 4.

# Simulation Assumptions

5G-ACIA has selected motion control (TS 22.104, clause A.2.2.1) as the main focus for the evaluation. From the three motion control use cases in Table 1, and according to our understanding, 5G-ACIA suggests to prioritize use case #2 with 1 ms transfer interval (periodicity).

Table 1: Communication characteristics of motion control use cases (extracted from R1-2006953)

| Characteristic parameter |  |  |  |  | Influence quantity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Commu-nication service availabil-ity: target value (note 1) | Communi-cation ser-vice relia-bility: mean time between failures | End-to-end latency: maximum (note 2) (note 12a) | Service bit rate: user ex-peri-enced data rate (note 12a) | Mes-sage size [byte] (note 12a) | Transfer interval: target value (note 12a) | Survival time (note 12a) | UE speed (note 13) | # of UEs | Service area (note 3) |
| 99,999 % to 99,99999 % | ~ 10 years | < transfer in-terval value | – | 50 | 500 μs | 500 μs | ≤ 75 km/h | ≤ 20 | 50 m x 10 m x 10 m |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer in-terval value | – | 40 | 1 ms | 1 ms | ≤ 75 km/h | ≤ 50 | 50 m x 10 m x 10 m |
| 99,9999 % to 99,999999 % | ~ 10 years | < transfer in-terval value | – | 20 | 2 ms | 2 ms | ≤ 75 km/h | ≤ 100 | 50 m x 10 m x 10 m |

Proposal 1: Harmonize the basic simulation assumptions focusing only on Use Case #2 (shown in yellow in Table 1) as suggested in the LS.

Besides, detailed simulation assumptions have also been provided building on previous RAN1 Rel-16 eURLLC evaluations documented in TR 38.824. As noted in the LS, remaining parameters are at the discretion of each company or to be agreed in 3GPP. In Table 1 and Table 2, we highlight in blue the key assumptions that we think that should be discussed (and agree, wherever possible) across the companies.

Table 2: Simulation assumptions as recommended in R1-2006953

| Parameters | Values | Reasons |
| --- | --- | --- |
| Factory hall size | 120x50m | See Section 5.2.2 |
| Room height | 10m | See service area in Table 1 |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. | See Section 5.2.2 |
| BS/TRP antenna height | 1.5 m for InF-SL and InF-DL, 8m for InF-SH and InF-DH. | According to Table 1, height of factory is 10m. |
| Layout – BS/TRP deploy-ment | Depending on the number of TRPs | See Section 5.2.2 |
| Channel model | UC-2: InF-DH > InD-DL > InF-SH > InF-SL | See Section 5.2.3 and 5.2.8. |
| Carrier frequency and sim-ulation bandwidth | TDD 4 GHz: 100 MHz 30 GHz: 160 MHz | There are no FDD bands identified at 4GHz. To reduce the simu-lation burden, it is suggested to do the performance evalu-ation only for TDD bands. Carrier frequency and bandwidths are inline with TR 38.824. |
| TDD DL-UL configuration | Simulation company should report the used DL-UL configuration. | Due to symmetric DL/UL traffic, 1:1 DL-UL configuration is recommended. |
| Number of UEs per service area | Up to 50 per service area, e.g., 10, 20, 40, and 50 | See Table 1 and Section 5.2.8 |
| UE distribution | All UEs randomly distributed within the respective service area. | See Table 1 and Section 5.2.4 |
| Message size | 48 bytes | See Section 5.2.8 |
| Traffic model | DL traffic arrival with option-1, option-2, and option-3. | See section 5.2.7 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time rela-tionship with option-1 and option-2 | See section 5.2.7 |
| CSA requirements | UC-#2: 99.9999% | See Table 1 lower bound of CSA re-quirement for UC-#2 is chosen for re-duced simulation burden. |
| Performance metrics | 1) CSA: single CDF of CSA distribution of all UEs in factory hall | See Section 5.2.9 for CSA calculation with non-zero survival time; |
|  | 2) Latency: single CDF of latency distribution of all UEs in factory hall | See Section 5.1.3 for latency metric; |
|  | 3) Percentage of UEs satisfying re-quirements and 4) resource utili-zation | Metric 3) and 4) are low priority. |

## Key Performance Indicators

Four key performance indicators (KPI) have been identified by 5G-ACIA: 1) the CDF of Communication service availability (CSA) distribution of all UEs in the network, 2) single CDF of latency distribution of all UEs in the network, 3) percentage of UEs satisfying the requirements and 4) resource utilization.

Alignment on the value of the latency budget for the air interface seems essential for the evaluation. Use case #2 in Table 1 specifies an E2E latency requirement of 1 ms and, in [R1-2006937], it was proposed that the latency budget of the air-interface is half of the E2E latency budget. However, Section 5.1.3 of the LS mentions that “it is assumed that the CN induced latency can be negligible in this WI. As a result, this WI focuses on the latency performance of the RAN”, thus the following is proposed:

Proposal 2: For the selected use case #2 of motion control, the latency budget available to the air interface corresponds to the entire E2E latency budget of 1 ms.

## Network layout and base station deployment

5G-ACIA has suggested a factory hall size of 120x50x10 m3. The number of BSs/TRPs has been left to the discretion of each company; however, we see convenient to agree on a common assumption in 3GPP to ensure consistency across the contributed performance results (as the number of BSs/TRPs will clearly impact the network performance). In TR 38.824, 12 BSs with 20 meter ISD were simulated in the same 120x50x10 m3 area; we propose to reuse the same assumption for this evaluation.

Proposal 3: For the network layout, 12 BSs are assumed to be deployed in the 120x50x10 m3 area with the same 2D placement as in TR 38.901 and TR 38.824.

- BS height follows 5G-ACIA recommendation: 1.5 m for InF-SL and InF-DL, 8m for InF-SH and InF-DH.

## Traffic model

5G-ACIA has proposed three options for the timing of DL traffic arrival across users:

- Option-1: all UEs’ DL messages arriving at NG-RAN node in the first transfer interval are uniformly random distributed within the TI time window.

- Option-2: all UEs’ DL messages arriving at NG-RAN node in the first transfer interval are in one burst.

- Option-3: All UEs in one service area can be divided into several groups, DL messages of UEs in the same group will arrive at NG-RAN node in one burst with the following assumptions.

  - Number of groups within a service area: 2

  - Number of UEs in a group: all groups have equal number of UEs

  - 3GPP can determine to use either a pre-defined value or a random value for the burst arrival time differences between different groups.

Option 2 provides the highest traffic burstiness and, most likely, the worst-case of latency and reliability performance. In contrast, option 1 uniformly ‘spreads’ the traffic across all available TTIs/mini-slots thus significantly better performance is expected. Option 3 corresponds to a more sophisticated assumption where 2 UE groups are defined per service area, and DL messages arrive simultaneously within each group. Each service area has a size of 50x10m2 (i.e. there are 12 service areas in total) which is different than what was assumed for the eURLLC Rel-16 SI evaluations. To reduce simulation burden and implementation effort, it is prefered to have option 1 (as first priority) and option 2 (as second priority). Options 1 and 2 are sufficient to cover the challenges from RAN point of view, and they are aligned to the more typical RAN1 simulation assumptions. In addition, in our view option 1 is more aligned with the use case of interest for 5G-ACIA and hence should be the priority. It is worth mentioning that option 3 can be seen as a middle ground case, with a performance that lies between option 1 and option 2. By providing performance results with different number of users using option 1 and option 2, the expected performance with option 3 can be to some extent estimated.

Proposal 4: The following priorities are assumed for the timing of the DL and UL traffic arrivals:

- Option 1 (random distributed offset) is mandatory

- Option 2 (simultaneous traffic arrival for all UEs) is optional

- Option 3 (2 groups per service area and aligned traffic arrival per group) is optional with low priority

In addition, the following two options are proposed for UL and DL traffic timing dependency:

- Option A: DL and UL traffic arrival time instants are independent

- Option B: UL traffic arrives at some pre-defined x time duration, where x can be, e.g., half of transfer interval, after the respective DL traffic arrival time.

Even though Option B may seem more sophisticated modeling, no significant difference in performance is expected, especially considering the use case in question assumes traffic to be rather periodic in nature. E.g., if the offset of the DL traffic is uniformly (randomly) distributed (i.e. Option 1 as per Proposal 4), selecting option A or option B for UL traffic timing will anyway result in randomly distributed UL traffic. Same reasoning applies for single-burst / simultaneous DL traffic arrival case. With this in mind, it is preferred to adopt the simpler Option A (DL and UL traffic arrival time instants are independent).

Proposal 5: For the UL and DL traffic timing dependency, adopt the following: DL and UL traffic arrival time instants are independent.

## UE Speed Modeling

Table 1 specifies a UE speed of 75 km/h. Besides, 5G-ACIA mentions in Section 5.2.4 of the LS that “(…) for the sake of simulation simplicity, we propose to only assume linear movement in the simulations”. Obviously, explicit UE linear mobility at 75 km/h speed seems not very realistic assumption, and will considerably complicate the discussion regarding handover modeling, etc. when considering the 120x50m factory hall setup. For simplicity, it is preferred not to include explicit UE mobility in the simulations and only assume 75 km/h speed for the sake of fast fading modeling (i.e. as traditionally assumed in RAN1 evaluations).

Proposal 6: Fast fading is modeled assuming a UE speed of 75 km/h. No explicit UE mobility (nor handovers) are modeled in the evaluations.

# URLLC/IIoT Features to be considered for the evaluation

Rel-15 and Rel-16 NR incorporates a wide range of features and enablers of low-latency and high-reliability. Some of them are specifically tailored for the support of URLLC/IIoT use cases (e.g. those standardized as part of the R16 eURLLC WI), whereas a few others have not been designed only for URLLC/IIoT but can nevertheless be regarded as key enablers for it (e.g. “mini-slots”, configured grants, etc.).

In Table 3 we provide our view on the features which should be considered in the evaluations as well as their mapping to corresponding UE capabilities. It is worth noting that not all URLLC/IIoT enhancements are equally relevant or applicable for the evaluation scope of this contribution, and thus some of them have not been included in Table 3; this includes, among others, some control channel (PDCCH, PUCCH) enhancements since these are generally highly abstracted in system-level simulations, and enhancements related to intra- and inter-UE multiplexing since the evaluation only considers single traffic profile for all UEs in the network (i.e. no mixed eMBB/URLLC scenario). However, it should be noted that such features can be seen as critical for real-life URLLC implementations.

Observation 1: Due to the single-traffic profile assumption in the simulation, benefits of R15 and R16 enhancements related to intra- and inter-UE mixed-service multiplexing are not expected to be visible in the system-level simulations.

In Table 3 we also highlight that, due to the periodic/deterministic nature of the motion control use case, the PDSCH and PUSCH allocations can be proactively scheduled (either dynamically or semi-persistently) ahead of the traffic arrivals. Therefore, frequent PDCCH monitoring occasions and overprovisioning of UL grants (e.g. UL CG with 2 OFDM symbol periodicity) are not seen essential for the evaluated use case, though expected to be useful in real-life deployments. Frequent PDCCH monitoring occasions can still be relevant for fast scheduling of the HARQ retransmissions (if the latency budget allows).

Observation 2: Due to the periodic/deterministic nature of the motion control use case, frequent PDCCH monitoring occasions and overprovisioning of UL grants (e.g. UL CG with 2 OFDM symbol periodicity) do not need to be explicitly modelled in the simulations.

Table 3: Overview of URLLC/IIoT enhancements and features and their corresponding relevance for the evaluation

| Feature | Description/comments | Simulation modeling aspects and expected performance impact (low, medium, high) | Related UE capabilities |
| --- | --- | --- | --- |
| UL/DL mini-slots | Time-domain resource allocations of 2, 4 or 7 OFDM symbols (OS). Key enabler of low-latency | Fixed TTI duration typically assumed in system-level simulations. Companies to select the TTI size taking into account what is possible as per the defined UE capabilities. Note that, since traffic is deterministic, 1 or at most 2 unicast PxSCHs per slot per UE should be sufficient to achieve the desired latency. Performance impact: high | PDSCH mapping type B (5-6a) 2 OS TTI: Up to 7 unicast PDSCHs per slot for UE proc. cap. 2 (5-13a) 4 OS TTI: Up to 4 unicast PDSCHs per slot for UE proc. cap. 2 (5-13c) 7 OS TTI: Up to 2 unicast PDSCHs per slot for UE proc. cap. 2 (5-13d) 2 OS TTI: Up to 7 unicast PUSCHs per slot for UE proc. cap. 2 (5-13e) 4 OS TTI: Up to 4 unicast PUSCHs per slot for UE proc. cap. 2 (5-13f) |
| Configured UL grants (CG) | Semi-static periodic UL grant, thus removing the need for dynamic scheduling request and scheduling grants. | Periodic UL resource allocation with fixed MCS and TB size. CG periodicity and time offset can be aligned with the timing of the traffic generation. Performance impact: high | Type 1/2 Configure UL grant (5-19/5-20) |
| UE processing time capability 2 | Short PDSCH decoding delay and PUSCH preparation delay. | Reduced HARQ retransmission delay and TB tx/rx processing times in line with the specifications. Performance impact: high gNB processing times generally assumed to be equal to UE processing times. | PUSCH processing capability 2 (5-5c) PDSCH processing capability 2 (5-5a) |
| URLLC MCS table | CQI table with associated BLER target of 1E-5. MCS table with low spectral-efficiency entries for PDSCH/PUSCH scheduling | MCS selection according to Table 5.2.2.1-4 in TS 38.214. UE CQI reports associated with a 1E-5 BLER targets. Performance impact: High | New MCS table for PDSCH (5-34) New MCS table for PUSCH (5-34) New CQI table with target BLER of 1E-5 (5-32c) |
| DL Semi-persistent scheduling | Semi-static periodic DL assignment with down to 1 slot periodicity. | Periodic DL resource allocation with fixed MCS and TB size. SPS periodicity and time offset can be aligned with the timing of the traffic generation. Performance impact for PDSCH: Low Performance impact on PDCCH blocking: High | Support of SPS periodicity shorter than 10 ms (12-6) |
| PDCCH monitoring occasions |  | PDSCH and PUSCH allocations can be proactively scheduled ahead of the traffic arrivals. Frequent PDCCH monitoring occasions not needed for the initial transmissions but can still be relevant for the HARQ retransmissions (if the latency budget allows). Performance impact: Low | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot (3-5b) Rel-16 PDCCH monitoring capability (11-2b) |
| DCI format x_2 | DCI format with configurable fields for PDSCH and PUSCH scheduling | It can be assumed that a compact DCI is used for DL/UL scheduling thus reducing the PDCCH overhead that is modeled in the simulations. Performance impact: Low | Monitoring DCI format 1_2 and DCI format 0_2 (11-1) |
| Sub-slot PUCCH config / HARQ-ACK | Sub-slots of 2*7 and 7*2 symbols for finer HARQ-ACK feedback timing indication and multiple PUCCHs per slot containing HARQ-ACK | HARQ-ACK retransmission delay can be reduced under some cirscumtances. With periodic traffic, at most 1 PUCCH with HARQ-ACK is needed per slot. Performance impact: Low | More than one PUCCH for HARQ-ACK transmission within a slot (11-3x) |
| PDCP duplication | Duplication of DL data at PDCP level for transmission on different cells/carriers | Independent transmissions schedule from two or more cells towards a UE. TB is correctly received if at least the transmission from one of the legs is decoded succesfully. For static UEs and in the absence of mobile obstacles, duplication techniques offer minor benefit. Performance impact: Low | PDCP duplication (L2/L3 FG1-6) |
| Joint downlink transmission from multiple transmission points (TRP) | Joint downlink transmission from multiple TRPs using SDMA/FDMA/TDMA schemes. | Fast switching of transmission point or joint transmission from multiple TRPs. For static UEs and in the absence of mobile obstacles, duplication techniques offer minor benefit. Performance impact: Low | Multi-DCI or single-DCI based multi-TRP operation (16-2a/2b) |

Proposal 7: For the performance evaluation of the motion control use case, the following NR features are considered:

- With high priority:

  - UL and DL mini-slots of 2, 4 or 7 OFDM symbols

  - Configured UL grants

  - UE Processing capability 2

- With medium priority:

  - URLLC MCS table and CQI reports with 1E-5 BLER target

  - DL Semi-persistent scheduling

- With low priority:

  - PDCP duplication / joint multi-TRP DL transmissions

Other features can be ‘implicitly’ modeled as follows:

  - Short DCI format x_2 resulting in reduced PDCCH overhead as compared to legacy x_0 and x_1 DCI formats.

  - Sub-slot HARQ-ACK for potentially faster HARQ-ACK retransmission delay

# Conclusion

In this contribution we have presented our view on which URLLC features to include in the evaluations as well as the simulation assumptions for the 5G-ACIA performance evaluation. The discussion can be summarized with the following observations and proposals:

Proposal 1: Harmonize the basic simulation assumptions focusing only on Use Case #2 (shown in yellow in Table 1) as suggested in the LS.

Proposal 2: For the selected use case #2 of motion control, the latency budget available to the air interface corresponds to the entire E2E latency budget of 1 ms.

Proposal 3: For the network layout, 12 BSs are assumed to be deployed in the 120x50x10 m3 area with the same 2D placement as in TR 38.901 and TR 38.824.

- BS height follows 5G-ACIA recommendation: 1.5 m for InF-SL and InF-DL, 8m for InF-SH and InF-DH.

Proposal 4: The following priorities are assumed for the timing of the DL and UL traffic arrivals:

- Option 1 (random distributed offset) is mandatory

- Option 2 (simultaneous traffic arrival for all UEs) is optional

- Option 3 (2 groups per service area and aligned traffic arrival per group) is optional with low priority

Proposal 5: For the UL and DL traffic timing dependency, adopt the following: DL and UL traffic arrival time instants are independent.

Proposal 6: Fast fading is modeled assuming a UE speed of 75 km/h. No explicit UE mobility (nor handovers) are modeled in the evaluations.

Observation 1: Due to the single-traffic profile assumption in the simulation, benefits of R15 and R16 enhancements related to intra- and inter-UE mixed-service multiplexing are not expected to be visible in the system-level simulations.

Observation 2: Due to the periodic/deterministic nature of the motion control use case, frequent PDCCH monitoring occasions and overprovisioning of UL grants (e.g. UL CG with 2 OFDM symbol periodicity) do not need to be explicitly modelled in the simulations.

Proposal 7: For the performance evaluation of the motion control use case, the following NR features are considered:

- With high priority:

  - UL and DL mini-slots of 2, 4 or 7 OFDM symbols

  - Configured UL grants

  - UE Processing capability 2

- With medium priority:

  - URLLC MCS table and CQI reports with 1E-5 BLER target

  - DL Semi-persistent scheduling

- With low priority:

  - PDCP duplication / joint multi-TRP DL transmissions

Other features can be ‘implicitly’ modeled as follows:

  - Short DCI format x_2 resulting in reduced PDCCH overhead as compared to legacy x_0 and x_1 DCI formats.

  - Sub-slot HARQ-ACK for potentially faster HARQ-ACK retransmission delay

# References

- R1-2006953	LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation, 5G ACIA

- RP-202069	Way forward and RAN work for 5G ACIA requested simulations, Ericsson

- R1-2006937	Discussion on URLLC and IIoT performance evaluation in response to 5G-ACIA, Huawei, HiSilicon

- TS 22.104	Service requirements for cyber-physical control applications in vertical domains, 3GPP