3GPP TSG-RAN Meeting #90-e                                                                 Revision of RP-201653

Electronic meeting, December, 2020

- Source:	vivo, OPPO, CMCC, CAICT, Rohde & Schwarz

- Title:	New WID: NR FR1 UE SA and EN-DC TRP and TRS

- Document for:	Approval

Agenda Item:	9.1.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	NR FR1 UE SA and EN-DC TRP and TRS

## Acronym: NR_FR1_TRP_TRS

## Unique identifier: TBD

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

|  | Feature |
| --- | --- |
| X | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 550022 | Study on Measurements of radio performances for LTE terminals - Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology | RAN4 TR 37.902 will be used as a baseline for NR FR1 TRP TRS |
| 580037 | LTE UE TRP and TRS and UTRA Hand Phantom related UE TRP and TRS Requirements | RAN4 TS 37.144 will be used as a baseline for NR FR1 TRP TRS |
| 630011 | UE Conformance Test Aspects - LTE UE TRP and TRS and UTRA Hand Phantom; | RAN5 TS 37.544 will be used as a baseline for NR FR1 TRP TRS test specification definition |
| 760087 | UE Conformance Test Aspects - 5G system with NR and LTE | Content of TS 38.521-1 and 3 will be used as reference for test configuration |

## 3	Justification

Until now, the UE FR1 transmit power and receiver sensitivity are tested by conducted methodology at the temporary antenna ports and it remains unknown what the actual performance of the UE would be in realistic network conditions with the UE antenna included. Radiated performance based on OTA testing is one of the most important characteristics to verify the entire UE performance under conditions more closely resembling the end user’s interaction with the device.

In order to ensure the good overall system performance, the requirements for NR UE TRP and TRS is important for consistent devices performance in the real NR networks which operate in the OTA manner. Unified requirements in 3GPP will provide authoritative guidance and will greatly promote the development of 5G industries.

Therefore, it is proposed to define the performance requirement and associated verification methodology for SISO NR FR1 SA and EN-DC TRP and TRS in a new Work Item.

## 4	Objective

### 4.1	Objective of Core part WI

The objective of this Work Item is to extend SISO OTA methodology defined in TR37.902 to SISO NR FR1 SA and EN-DC modes and to specify SISO NR FR1 OTA performance requirements for both SA and EN-DC UEs.

Investigate and specify the following aspects:

- General aspects

-	Considering the following device types:

-	Smartphone

- Considering UEs with antenna configurations of 1Tx, 2Tx, 2 Rx and 4 Rx

-	Tablet

-	Laptop embedded equipment (LEE)

-	Laptop mounted equipment (LME)

-	Test scenarios:

-	For smartphone, head/hand phantoms testing configuration is the first priority

-	For other device types,

-	Free space (FS) testing configuration is the first priority

-	OTA performance requirements with head/hand/Laptop ground plane phantoms are second priority

-	Environmental conditions:

-	Normal temperature and voltage test conditions

- SISO OTA Test methodology enhancement

-	Specify necessary enhancements of the SISO OTA test methodology for NR FR1 TRP and TRS, e.g.

-	Using the test methodology defined in TR37.902 as well as the associated aspects related to measurement uncertainty in TR25.914 as the basis for NR FR1

-	Support UE operating frequency in the range of 410 MHz – 7125 MHz (i.e., test methods will cover all the NR FR1 bands)

-	Support up to 100 MHz CBW

-	Define the configured power settings for EN-DC (1 CC LTE with 1 CC NR)

-	Develop the Measurement Uncertainty (MU) assessment [RAN5]

- Measurement Uncertainty (MU) aspects will be handled by RAN5 and the conclusions can be captured in a separate section of TR

-	Consider UE with multi-antenna under SISO OTA Test Methodology, e.g.

-	Study whether a test procedure for UL Transmit Diversity of SA, if this feature is supported by UE, is needed

- This task shall not start until RAN4 concludes on all of the corresponding requirements related to UL Transmit Diversity of SA

-	Consider how to treat the UE with Tx switching and ensure predictable verification of TRP results

-	Consider how to treat the UE with multiple antenna receivers and ensure predictable verification of TRS results

-	Consider whether exceptional requirements to be tested for EN-DC TRS is needed, this will be treated as second priority

-    Example: NSA TRS requirements for potential UE self-interference due to IMD3 in EN-DC

-	Consider the testing time reduction for TRP and TRS among the bands and EN-DC band combinations that UE support

-    Example: Alternative Single Point Offset TRP/TIS Test is not precluded

During the course of this work item, ongoing communication with 3GPP RAN WG5, CTIA OTA Working Group, CCSA TC9 WG1, GCF, ETSI MSG TFES and PTCRB shall be maintained to ensure industry coordination on this topic.

<Whether we need the note and how to rephrase the note (if needed) is up to RAN, RAN4 and RAN5 leaderships’ decision >

### 4.2	Objective of Performance part WI

- Performance part framework

- Define a brand-new framework on how to handle requirements task for SA and EN-DC TRP and TRS before collecting trustable UE measurement results, the requirements task will follow the framework strictly, e.g.

-	Main actions in the framework in sequence:

- Requirements task should be a step-by-step approach, bands selected as first priority in the WID will be defined for the first step

- Decide the minimum number of devices (e.g., at least [15 or 20]) for defining requirements

- Start lab alignment activity among volunteered labs (prefer certified labs) before collecting measurement results

- Select sufficient devices those are commercially available in the market, and the measurement results of these devices from the aligned labs should be submitted for data processing

- Specify the requirements based on the measurement results with a per-band approach

-	Start with one type of device requirement which is most efficient to collect enough results

-	Only specify 4Rx requirement for n41, n78, n79

-	Specifying requirements of SA with 1 CC is the first priority

-	Define clear process of submitting and processing the measurement results

-	Theoretical analysis including network deployment and UE antenna design aspects can be considered

-	Discussion of simulation results of TRP and TRS is not precluded

- Specify final requirements (started after making conclusion of detailed framework)

- Specify the NR FR1 SISO SA TRP and TRS requirements and tolerance:

-	Band n41, n78 and n79 for PC3 and/or PC2 UEs are the first priority

-	Define the detailed requirements of the selected bands based on the conclusion of above requirement definition framework

- Specify the FR1 EN-DC TRP and TRS requirements and tolerance:

-	For EN-DC, only NR requirements will be specified and no additional LTE requirements will be introduced.

-	Only consider EN-DC combinations of 1 CC LTE with 1 CC NR

-	Band n41, n78 and n79 related EN-DC band combinations for PC3 UEs are the first priority

-	Further limiting the number of EN-DC band combinations

-	Define the detailed requirements of the selected bands based on the conclusion of above requirement definition framework

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| Internal TR | 38.8xx | Measurements of User Equipment (UE) Over-the-Air (OTA) performance for NR FR1; Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS) test methodology |  | RAN#93 September 2021 | Related to Core part; Leading WG: RAN4 |
| TS | 38.1xx | NR; User Equipment (UE) Over-the-Air (OTA) performance requirements; Range 1 Standalone and Range 1 Interworking operation with other radios | RAN#96 June 2022 | RAN#97 September 2022 | Related to Perf. part; Leading WG: RAN4 |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

## 6	Work item Rapporteur(s)

Company:

Email:

## 7	Work item leadership

RAN WG4

Secondary responsibility: RAN WG5

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

Apple

CAICT

China Telecom

China Unicom

CMCC

ETS-Lindgren

Genius technology

Huawei

HiSilicon

Intel

KDDI

MVG

OPPO

Orange

Qualcomm

Rohde & Schwarz

Samsung

Sanechips

Spirent

SAICT

vivo

Xiaomi

ZTE