3GPP TSG RAN Meeting #91-e									RP-210045

Electronic Meeting, March 16 - 26, 2021

Status Report to TSG

Agenda item:			9.6.8

| WI / SI Name | Study on NR QoE management and optimizations for diverse services |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: Yes | Core part: No | Performance part: No |  | Testing part: No |
| Acronym | FS_NR_QoE |  |  |  |  |
| Unique ID | 860061 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-193256 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: 03/2021 | Core part: N/A | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: 100% | Core part: N/A | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Xiang Pan |
|  | Company | China Unicom |
|  | Email | panx14@chinaunicom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2#113-e (Jan 2021)

NR QoE SI is closed from RAN2 point of view, and the TP for TR 38.890 update [36] is endorsed.

36 contributions ([1] ~ [36]) were submitted in this meeting. Two email discussions were assigned as below:

-	[AT113-e][039]:

- Phase 1: progress on key issues

- Phase 2: progress on the TP capturing RAN2 agreements

- The email report is captured in [32]

-	[AT113-e][040]: progress on the handling of the incoming LS R2-2100079 [5].

The potential NR QoE solutions and procedures have been discussed and the following agreements were made.

- General Principle

NR QoE takes LTE QoE solution as baseline and details can be discussed during the WI phase. LTE QoE solution includes the following key parts:

-	Both signalling-based and management-based initiated cases are allowed

-	The LTE QoE feature is activated by Trace Function

-	Application layer measurement configuration received from OAM or CN can be encapsulated in a transparent container, which is forwarded to UE in a downlink RRC message. Application layer measurements received from UE's higher layer can be encapsulated in a transparent container and sent to network in an uplink RRC message

- QoE measurement Configuration and Reporting

-	Management-based QoE configuration should not override signalling-based QoE configuration. Details can be discussed during the WI phase.

-	QoE reports are sent via a separate SRB (separate from current SRBs) in NR, as this reporting is lower priority than other SRB transmissions.

-	Configuration and Reporting for multiple simultaneous QoE measurements for a UE can be supported (can determine whether there is AS impact in the WI phase).

-	The details of pause/resume mechanism need to be resolved in potential WI phase, e.g. is pause/resume for all QoE reports or per QoE configuration, how long can the UE store the reports, limit for stored reports size etc. (these points can be captured in TR 38.890).

-	RRC signaling is used by the gNB to indicate the UE to pause or resume the QoE reporting.

-	RAN may need to release an ongoing QoE measurements configuration or QoE reporting configuration, e.g. if handing over to a network that doesn’t support this.

-	R2 assumes that RRC segmentation may be needed for transmission of QoE reports and the details can be discussed during the WI phase.

- Support for Mobility

-	RAN2 will address in the WI the details of Area Handling at mobility.

-	For the Area Handling at mobility there are three main options on the table.

- Option 1, where the network is responsible to keep track of whether the UE is inside or outside the area and configures / releases configuration accordingly.

- Option 2, where the network is responsible to keep track of whether the UE is inside or outside the area, and the UE responsible to manage start stop of QoE accordingly.

- Option 3, where the UE is responsible for area checking (UE has the area configuration) and to manage start stop of QoE accordingly.

-	RAN2 will address in the WI the details of mobility procedure adaptation for signalling-based QoE vs management-based QoE.

-	QoE measurements in RRC INACTIVE state can be supported, for MBS.

-	QoE measurements in RRC IDLE state can be supported, for MBS.

-	Whether the UE stores its QoE configuration when going to RRC INACTIVE state for potential use when the UE moves back to RRC Connected state will be decided in the WI phase.

- Radio-related measurements and information for QoE

-	Collection of radio related measurements, if needed, should be done by existing methods such as MDT, if UE supports MDT in R17.

- RAN visible QoE

-	Whether any QoE measurements need to be visible to RAN is a RAN3 topic.

- Handling of incoming LSs

RAN2 received the incoming LSs [1] – [5], and all were noted. RAN2 discussed the reply of LS [5], and approved the reply LS to SA5 in R2-2102500 [34].

In [34], RAN2 considered the different features requested to be implemented in Rel-17 for NR QoE and have the following updates:

- Mobility support for QoE measurement: RAN2 acknowledges the need for area handling during the mobility for QMC support. The exact solution will be discussed in the WI phase.

- Simultaneous QMCs in a UE: RAN2 agreed that “Configuration and Reporting for multiple simultaneous QoE measurements for a UE can be supported (can determine whether there is AS impact in the WI phase)”. The means for distinguishing QoE reports by the gNB will be decided in WI.

- Temporary stop and restart at RAN overload: RAN2 agreed to support pause/resume mechanism for QoE reporting during RAN overload with RRC signalling. The exact solution detail is still under discussion.

RAN2 kindly asks SA5 to take the above information into account and take action as necessary.

2.2.2	Remaining Open issues

None

2.3	RAN3

2.3.1	Agreements

RAN3#111-e (Jan 2021)

NR QoE SI is closed in RAN3.

68 contributions ([37] ~ [104]) were submitted in this meeting. The remaining issues were discussed and the following agreements were made.

- Support for Mobility

-	Measurement reporting continuity in intra-RAT mobility scenarios is supported for intra-node mobility for both management-based QoE and signalling-based QoE. At least signalling-based QoE supports this also in case of inter-node mobility, FFS on management-based QoE.

-	Capture requirements of support for mobility in LS from SA4 and SA5 in the TR.

-	Capture requirements of support for mobility in related TS in the TR.

-	Agree the TP for agreements on mobility in R3-211356 [102].

- QoE measurement Configuration and Reporting

-	Support XR as new service type for QoE measurement

-	Support multiple QoE measurements for the same UE, detailed mechanisms on whether and how to deactivate one of them are up to RAN2

-	In case of RAN overload, RAN could take some measures and the detailed mechanisms are up to RAN2

-	Agree the TP on definitions in R3-211357 [103].

-	Agree the TP on open issues of QoE configuration and reporting in R3-211358 [104].

- Per slice QoE measurement

-	Agree the TP for per slice QoE measurement in R3-211341 [99].

- RAN visible QoE

-	Agree the TP for QoE visibility at the RAN in R3-211342 [100].

- Features ranking

-	The following features are recommended by RAN3 to be specified in Rel-17 normative phase:

- Signalling-based activation/deactivation

- Management-based activation/deactivation

- QoE measurement handling at RAN overload

- Multiple QMC support

- QoE continuity for intra-RAT mobility

- QoE measurement in RRC_INACTIVE

- RAN visible QoE

- Per-slice QoE measurement

- Alignment of MDT and QoE measurements

-	The following feature is recommended by RAN3 to be deprioritized in Rel-17 normative phase:

- QoE continuity for Inter-system mobility

- QoE measurement in RRC_IDLE

- QoE continuity for Inter-RAT mobility

- QoE support for MR-DC

-	Agree the TP for feature ranking in R3-211343 [101].

Based on conclusion of NR QoE SI in RAN3, RAN3 agreed on a LS [84] to RAN2, SA4 and SA5, and the LS is to inform them of taking the TR 38.890 v0.3.0 [83] into account.

2.3.2	Remaining Open issues

None

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SA4

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#113-e

- R2-2100034	NR QoE progress in RAN3 (R3-207120; contact: China Unicom)	RAN3	LS in	Action

- R2-2100039	LS on Framework for QoE Measurement Collection (R3-207189; contact: Nokia)	RAN3	LS in	Information

- R2-2100075	LS Reply on New service type of NR QoE (S4-201576; contact: Huawei)	SA4	LS in	Information

- R2-2100076	LS reply on QoE Measurement Collection (S4-201600; contact: Ericsson)	SA4	LS in	Action

- R2-2100079	LS on QoE Measurement Collection (S5-205347; contact: Ericsson)	SA5	LS in	Action

- R2-2100597	Generic requirements for QMC in NR	Nokia, Nokia Shanghai Bell	discussion	Agreement

- R2-2100598	QMC procedures principles	Nokia, Nokia Shanghai Bell	discussion	Agreement

- R2-2100706	Discussion on QoE configuration and reporting	vivo	discussion	Decision

- R2-2100846	Discussion on QoE measurement collection in NR	OPPO	discussion	Approval

- R2-2100879	Discussions on the QoE SI Metrics and Collection Procedures	Apple	discussion	Decision

- R2-2100967	Discussion on NR QoE	CATT	discussion

- R2-2100995	QoE measurements in NR	LG Electronics Inc.	discussion	Decision

- R2-2101189	Discussion on QoE configuration and report aspects	Huawei, HiSilicon	discussion	Decision

- R2-2101190	Discussion on QoE handling during UE mobility	Huawei, HiSilicon	discussion	Decision

- R2-2101191	Discussion on other QoE aspects	Huawei, HiSilicon	discussion	Decision

- R2-2101271	Solution for QoE Management	Ericsson	discussion

- R2-2101272	Mobility Support for NR QoE Management	Ericsson	discussion

- R2-2101273	Analysis of QoE measurements at OAM and RAN	Ericsson	discussion

- R2-2101336	LS reply on QoE Measurement Collection	QUALCOMM INCORPORATED	LS out	Approval

- R2-2101338	Handling of NR QoE measurements	QUALCOMM INCORPORATED	discussion	Decision

- R2-2101339	Handling of NR QoE reporting	QUALCOMM INCORPORATED	discussion	Decision

- R2-2101496	Ranking and prioritization of QoE enhancement features	QUALCOMM Incorporated	discussion	Decision

- R2-2101581	Discussion on the RAN2 related work on NR QoE	China Unicom	discussion	Agreement

- R2-2101806	Discussion on NR QoE management	CMCC	discussion	Discussion

- R2-2101878	Transport of NR QoE report	Samsung	discussion	Decision

- R2-2101879	RRC signaling for NR QoE	Samsung	discussion	Decision

- R2-2101880	Alignment with RAN3 agreements for NR QoE	Samsung	discussion	Decision

- R2-2101917	Miscellaneous discussion on QoE	ZTE Corporation, Sanechips	discussion	Decision

- R2-2101918	Discussion on NR QoE continuity during handover	ZTE Corporation, Sanechips	discussion	Decision

- R2-2101919	Stop an ongoing QoE measurement reporting	ZTE Corporation, Sanechips	discussion	Decision

- R2-2102243	Summary document on AI 8.14 NR QoE SI	China Unicom	discussion	Decision

- R2-2102367	Summary of [AT113-e][039][eQoE] RAN2 conclusions on QoE (China Unicom)	China Unicom	discussion	Decision

- R2-2102368	TP for TR update (RAN2)	China Unicom	pCR	Agreement

- R2-2102500	LS reply on QoE Measurement Collection	RAN2	LS out	Approval

- R2-2102417	LS reply on QoE Measurement Collection	Qualcomm	LS out	Approval

- R2-2102483	TP for TR update (RAN2)	China Unicom	pCR	Agreement

RAN3#111-e

- R3-210041	LS Reply on New service type of NR QoE	SA WG4	LS in	Discussion

- R3-210042	LS reply on QoE Measurement Collection	SA WG4	LS in	Discussion

- R3-210355	QoE report suspending in RAN overload and RRC_IDLE/INACTIVE	Qualcomm Incorporated	discussion	Agreement

- R3-210356	LS reply on QoE Measurement Collection	Qualcomm Incorporated	LS out	Approval

- R3-210357	RAN visible QoE	Qualcomm Incorporated	discussion	Agreement

- R3-210358	Ranking and prioritization of QoE enhancement features	Qualcomm Incorporated	discussion	Agreement

- R3-210507	Open issues in NR QoE solutions	Samsung	discussion	Approval

- R3-210508	Discussion on per slice QoE measurement	Samsung	discussion	Approval

- R3-210509	Discussion on radio related measurements and information in NR QoE	Samsung	discussion	Approval

- R3-210510	Discussion on RAN visible QoE and RAN triggered QMC	Samsung	discussion	Approval

- R3-210527	pCR for TR 38.890: Handling of QoE Measurement and Reporting and Support for New Services	Ericsson	other	Agreement

- R3-210528	pCR for TR 38.890: QoE Visibility at the RAN	Ericsson	other	Agreement

- R3-210529	pCR for TR 38.890: Mobility Support for NR QoE Management	Ericsson	other	Agreement

- R3-210530	pCR for TR 38.890: QoE Support for Network Slicing	Ericsson	other	Agreement

- R3-210658	(TP for TR 38.890) QoE measurement collection continuity vs. reporting continuity	Nokia, Nokia Shanghai Bell	other	Agreement

- R3-210659	(TP for TR 38.890) QoE measurement configuration for specific UEs	Nokia, Nokia Shanghai Bell	other	Agreement

- R3-210660	(TP for TR 38.890) RAN awareness of QoE measurement reports	Nokia, Nokia Shanghai Bell	other	Agreement

- R3-210661	(TP for TR 38.890) Slice scope in QoE measurement activation and reports	Nokia, Nokia Shanghai Bell	other	Agreement

- R3-210770	Discussion on support for NR QoE Mobility	CATT	discussion

- R3-210771	TP for support for NR QoE Mobility	CATT	pCR	Agreement

- R3-210772	Discussion on RAN visible QoE configuration and reporting	CATT	discussion

- R3-210773	TP for RAN visible QoE configuration and reporting	CATT	pCR	Agreement

- R3-210820	Further discussions on the remaining open issues of QoE configuration and reporting	Huawei	pCR	Agreement

- R3-210821	Further discussions on the remaining open issues of QoE report visibility at RAN	Huawei	pCR	Agreement

- R3-210842	Further discussion on scenarios and mechanisms for per slice QoE measurement	China Unicom	discussion

- R3-210845	Draft TR 38.890	China Unicom	draft TR

- R3-210846	Consideration on slice QoE measurement	ZTE	discussion

- R3-210847	More consideration on QOE mobility support	ZTE	discussion

- R3-210848	Further consideration on NR QoE service and procedure	ZTE	discussion

- R3-210849	TP for TR 38890	ZTE	other	Agreement

- R3-210863	Further discussions on the remaining open issues of support QoE during mobility	Huawei	pCR	Agreement

- R3-210864	Further discussions on the remaining open issues of support slice for QoE	Huawei	pCR	Agreement

- R3-210900	Remaining issues on NR QoE management	CMCC	discussion	Discussion

- R3-210901	TP to 38.890 for NR QoE management	CMCC	other	Agreement

- R3-211011	CB: # NRQoE1-TR_update - Summary of email discussion	China Unicom - moderator	discussion	Discussion

- R3-211012	CB: # NRQoE2-Mobility - Summary of email discussion	Nokia - moderator	discussion	Discussion

- R3-211013	CB: # NRQoE3-RANConfig_Report - Summary of email discussion	Huawei - moderator	discussion	Discussion

- R3-211014	CB: # NRQoE4-Slice - Summary of email discussion	ZTE - moderator	discussion	Discussion

- R3-211015	CB: # NRQoE5-RAN_visible - Summary of email discussion	Ericsson - moderator	discussion	Discussion

- R3-211016	CB: # NRQoE6-Features_ranking - Summary of email discussion	Qualcomm - moderator	discussion	Discussion

- R3-211190	TP for Per slice QoE measurement	ZTE	other	Agreement

- R3-211201	pCR for TR 38.890: QoE Visibility at the RAN	Ericsson	other	Agreement

- R3-211208	TP to 38.890 on definitions	Samsung	pCR	Agreement

- R3-211220	TP to 38.890 on TR update	China Unicom	other	Agreement

- R3-211221	Further discussions on the remaining open issues of QoE configuration and reporting	Huawei	pCR	Agreement

- R3-211229	(TP for TR 38.890) RAN3#111-e agreements on mobility	Nokia - moderator	other	Agreement

- R3-211233	TR 38.890 v0.3.0	China Unicom - moderator	other	Agreement

- R3-211234	Progress of NR QoE SI in RAN3	China Unicom	LS out	Agreement

- R3-211235	(TP for TR 38.890) RAN3#111-e agreements on mobility	Nokia - moderator	other	Agreement

- R3-211236	Further discussions on the remaining open issues of QoE configuration and reporting	Huawei	pCR	Agreement

- R3-211237	TP to 38.890 on definitions	Samsung	pCR	Agreement

- R3-211238	TP for Per slice QoE measurement	ZTE	other	Agreement

- R3-211281	(TP for TR 38.890) RAN3#111-e agreements on mobility	Nokia – moderator, Ericsson	other	Agreement

- R3-211304	CB: # NRQoE6-Features_ranking - Summary of email discussion	Qualcomm - moderator	discussion	Discussion

- R3-211305	TP to 38.890 for QoE feature ranking	Qualcomm Incorporated	other	Agreement

- R3-211306	pCR for TR 38.890: QoE Visibility at the RAN	Ericsson	pCR	Agreement

- R3-211307	CB: # NRQoE5-RAN_visible - Summary of email discussion	Ericsson - moderator	discussion	Discussion

- R3-211312	TP for Per slice QoE measurement	ZTE	other	Agreement

- R3-211320	TP to 38.890 on definitions	Samsung	pCR	Agreement

- R3-211328	TP to 38.890 on definitions	Samsung, Ericsson, Huawei	pCR	Agreement

- R3-211339	TP to 38.890 on TR update	China Unicom	other	Agreement

- R3-211340	Further discussions on the remaining open issues of QoE configuration and reporting	Huawei, Samsung, Ericsson	pCR	Agreement

- R3-211341	TP for Per slice QoE measurement	ZTE, Samsung, China Unicom, Ericsson	other	Agreement

- R3-211342	pCR for TR 38.890: QoE Visibility at the RAN	Ericsson, Samsung, Qualcomm, CMCC	pCR	Agreement

- R3-211343	TP to 38.890 for QoE feature ranking	Qualcomm Incorporated, Ericsson, Huawei	other	Agreement

- R3-211356	(TP for TR 38.890) RAN3#111-e agreements on mobility	Nokia - moderator, Ericsson	other	Agreement

- R3-211357	TP to 38.890 on definitions	Samsung, Ericsson, Huawei	pCR	Agreement

- R3-211358	Further discussions on the remaining open issues of QoE configuration and reporting	Huawei, Samsung, Ericsson	pCR	Agreement