3GPP TSG-RAN #91	e																					  RP-210124

16th March – 26th March 2021 Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0206 | rev | 7 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of SFN Offset per cell over Xn |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, China Telecom, ZTE, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2021-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Lack of LTE-NR timing information causes mobility and throughput performance issues due to, e.g. error prone measurements configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The offset in time of SFN0 with respect to an absolute time initialisation is signalled in microseconds, per cell, to inform the receiving node of the exact time start of neighbour cells’ SFN0 Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because this is additional information that may be used by receiving node The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of LTE-NR timing information causes mobility and throughput performance issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1, 8.4.2, 9.2.2.11, 9.2.2.12, , 9.2.2.xx, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.473 CR 0431 TS 36.423 CR 1367 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: resubmission with added procedure text and information exchange for E-UTRA; Rev3: resubmission Rev4: Removal of additions for neighbour information Rev5: change granularity to microseconds Rev6: change SFN Offset length Rev7: Correctinos to cover page, correctinos of the ASN.1 |  |  |  |  |  |  |  |  |

First Change

### 8.4.1	Xn Setup

#### 8.4.1.1	General

The purpose of the Xn Setup procedure is to exchange application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE 1:	If Xn-C signalling transport is shared among multiple Xn-C interface instances, one Xn Setup procedure is issued per Xn-C interface instance to be setup, i.e. several Xn Setup procedures may be issued via the same TNL association after that TNL association has become operational.

NOTE 2:	Exchange of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.1.2	Successful Operation

Figure 8.4.1.2: Xn Setup, successful operation

The NG-RAN node1 initiates the procedure by sending the XN SETUP REQUEST message to the candidate NG-RAN node2. The candidate NG-RAN node2 replies with the XN SETUP RESPONSE message.

The AMF Region Information IE in the XN SETUP REQUEST message shall contain a complete list of Global AMF Region IDs to which the NG-RAN node1 belongs. The AMF Region Information IE in the XN SETUP RESPONSE message shall contain a complete list of Global AMF Region IDs to which the NG-RAN node2 belongs.

The List of Served Cells NR IE and the List of Served Cells E-UTRA IE, if contained in the XN SETUP REQUEST message, shall contain a complete list of cells served by NG-RAN node1 or, if supported, a partial list of served cells together with the Partial List Indicator IE. The List of Served Cells NR IE and the List of Served Cells E-UTRA IE, if contained in the XN SETUP RESPONSE message, shall contain a complete list of cells served by NG-RAN node2 or, if supported, a partial list of served cells together with the Partial List Indicator IE.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the XN SETUP REQUEST message the SUL Information IE and the Supported SUL band List IE for each served cell where supplementary uplink is configured.

If Supplementary Uplink is configured at the NG-RAN node2, the candidate NG-RAN node2 shall include in the XN SETUP RESPONSE message the SUL Information IE and the Supported SUL band List IE for each served cell where supplementary uplink is configured.

If the NG-RAN node1 is an ng-eNB, it may include the Protected E-UTRA Resource Indication IE into the XN SETUP REQUEST. If the XN SETUP REQUEST sent by an ng-eNB contains the Protected E-UTRA Resource Indication IE, the receiving gNB should take this into account for cell-level resource coordination with the ng-eNB. The gNB shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same ng-eNB.

The protected resource pattern indicated in the Protected E-UTRA Resource Indication IE is not valid in subframes indicated by the Reserved Subframes IE, as well as in the non-control region of the MBSFN subframes i.e. it is valid only in the control region therein. The size of the control region of MBSFN subframes is indicated in the Protected E-UTRA Resource Indication IE.

In case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the XN SETUP REQUEST message and the XN SETUP RESPONSE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the Intended TDD DL-UL Configuration NR IE is included in the XN SETUP REQUEST or XN SETUP RESPONSE message, the receiving NG-RAN node should take this information into account for cross-link interference management and/or NR-DC power coordination with the sending NG-RAN node. The receiving NG-RAN node shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of an update of the IE for the same cell(s).

If the TNL Configuration Info IE is contained in the XN SETUP REQUEST message, the NG-RAN node2 shall, if supported, take this IE into account for IPSec establishment.

If the TNL Configuration Info IE is contained in the XN SETUP RESPONSE message, the NG-RAN node1 shall, if supported, take this IE into account for IPSec establishment.

If the Partial List Indicator NR IE or the Partial List Indicator NR IE is set to "partial" in the XN SETUP REQUEST message the candidate NG-RAN node2 shall, if supported, assume that the List of Served Cells NR IE or the List of Served Cells E-UTRA IE in the XN SETUP REQUEST message includes a partial list of cells.

If the Partial List Indicator NR IE or the Partial List Indicator NR IE is set to "partial" in the XN SETUP RESPONSE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, assume that the List of Served Cells NR IE or the List of Served Cells E-UTRA IE in the XN SETUP RESPONSE message includes a partial list of cells.

If the Cell and Capacity Assistance Information NR IE or the Cell and Capacity Assistance Information E-UTRA IE is present in the XN SETUP REQUEST message the candidate NG-RAN node2 shall, if supported, use it when generating the list of NG-RAN served cell information to include in the XN SETUP RESPONSE message.

If the Cell and Capacity Assistance Information NR IE or the Cell and Capacity Assistance Information E-UTRA IE is present in the XN SETUP RESPONSE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

If the CSI-RS Transmission Indication IE is contained in the XN SETUP REQUEST message, the NG-RAN node2 shall, if supported, take this IE into account for neighbour cell’s CSI-RS measurement.

If the CSI-RS Transmission Indication IE in the XN SETUP RESPONSE message, the NG-RAN node1 shall, if supported, take this IE into account for neighbour cell’s CSI-RS measurement.

The initiating NG-RAN node1 may include the PRACH Configuration IE (for served E-UTRA cells) or the NR Cell PRACH Configuration IE (for served NR cells) or the NPRACH Configuration IE (for served NB-IoT cells) in the XN SETUP REQUEST message. The candidate NG-RAN node2 may also include the PRACH Configuration IE (for served E-UTRA cells) or NR Cell PRACH Configuration IE (for served NR cells) or the NPRACH Configuration IE (for served NB-IoT cells) in the XN SETUP RESPONSE message. The NG-RAN node receiving the IE may use this information for RACH optimisation.

The XN SETUP REQUEST message may contain for each cell served by NG-RAN node1 NPN related broadcast information. The XN SETUP RESPONSE message may contain for each cell served by NG-RAN node2 NPN related broadcast information.

If the SFN Offset IE is included in the XN SETUP REQUEST or XN SETUP RESPONSE message, the receiving NG-RAN node shall, if supported, use this information to deduce the SFN0 time offset of the reported cell.The receiving NG-RAN node shall consider the received SFN Offset IE content valid until reception of an update of the IE for the same cell(s).

#### 8.4.1.3	Unsuccessful Operation

Figure 8.4.1.3-1: Xn Setup, unsuccessful operation

If the candidate NG-RAN node2 cannot accept the setup it shall respond with the XN SETUP FAILURE message with appropriate cause value.

If the XN SETUP FAILURE message includes the Time To Wait IE, the initiating NG-RAN node1 shall wait at least for the indicated time before reinitiating the Xn Setup procedure towards the same NG-RAN node2.

If case of network sharing with multiple Cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the XN SETUP REQUEST message and the XN SETUP REQUEST FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the Message Oversize Notification IE is included in the XN SETUP FAILURE, the initiating node shall, if supported, deduce that the failure is due to a too large XN SETUP REQUEST message and ensure that the total number of served cells in following XN SETUP REQUEST message is equal to or lower than the value of the Maximum Cell List Size IE.

#### 8.4.1.4	Abnormal Conditions

If the first message received for a specific TNL association is not an XN SETUP REQUEST, XN SETUP RESPONSE, or XN SETUP FAILURE message then this shall be treated as a logical error.

If the initiating NG-RAN node1 does not receive either XN SETUP RESPONSE message or XN SETUP FAILURE message, the NG-RAN node1 may reinitiate the Xn Setup procedure towards the same NG-RAN node, provided that the content of the new XN SETUP REQUEST message is identical to the content of the previously unacknowledged XN SETUP REQUEST message.

If the initiating NG-RAN node1 receives an XN SETUP REQUEST message from the peer entity on the same Xn interface:

-	In case the NG-RAN node1 answers with an XN SETUP RESPONSE message and receives a subsequent Xn SETUP FAILURE message, the NG-RAN node1 shall consider the Xn interface as non operational and the procedure as unsuccessfully terminated according to sub clause 8.4.1.3.

-	In case the NG-RAN node1 answers with an XN SETUP FAILURE message and receives a subsequent XN SETUP RESPONSE message, the NG-RAN node1 shall ignore the XN SETUP RESPONSE message and consider the Xn interface as non operational.

### 8.4.2	NG-RAN node Configuration Update

#### 8.4.2.1	General

The purpose of the NG-RAN node Configuration Update procedure is to update application level configuration data needed for two NG-RAN nodes to interoperate correctly over the Xn-C interface.

NOTE:	Update of application level configuration data also applies between two NG-RAN nodes in case the SN (i.e. the gNB) does not broadcast system information other than for radio frame timing and SFN, as specified in the TS 37.340 [8]. How to use this information when this option is used is not explicitly specified.

The procedure uses non UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: NG-RAN node Configuration Update, successful operation

The NG-RAN node1 initiates the procedure by sending the NG-RAN NODE CONFIGURATION UPDATE message to a peer NG-RAN node2.

If Supplementary Uplink is configured at the NG-RAN node1, the NG-RAN node1 shall include in the NG-RAN NODE CONFIGURATION UPDATE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells To Add IE and in the Served NR Cells To Modify IE.

If Supplementary Uplink is configured at the NG-RAN node2, the NG-RAN node2 shall include in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message the SUL Information IE and the Supported SUL band List IE for each cell added in the Served NR Cells IE if any.

If the TAI Support List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the receiving node shall replace the previously provided TAI Support List IE by the received TAI Support List IE.

If the Cell Assistance Information NR IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served NR Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Cell Assistance Information E-UTRA IE is present, the NG-RAN node2 shall, if supported, use it to generate the Served E-UTRA Cells IE and include the list in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message.

If the Partial List Indicator NR IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served NR Cells IE in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Partial List Indicator E-UTRA IE is included in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message and set to "partial" the NG-RAN node1 shall, if supported, assume that the Served E-UTRA Cells IE in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message includes a partial list of NR cells.

If the Cell and Capacity Assistance Information NR IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

If the Cell and Capacity Assistance Information E-UTRA IE is present in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message from the candidate NG-RAN node2, the NG-RAN node1 shall, if supported, store the collected information to be used for future NG-RAN node interface management.

Upon reception of the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall update the information for NG-RAN node1 as follows:

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for IPSec establishment.

If the TNL Configuration Info IE is contained in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the NG-RAN node1 shall take this IE into account for IPSec establishment.

If the CSI-RS Transmission Indication IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall take this IE into account for neighbour cell’s CSI-RS measurement.

The NG-RAN NODE CONFIGURATION UPDATE message may contain for each cell served by NG-RAN node1 NPN related broadcast information. The NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message may contain for each cell served by NG-RAN node2 NPN related broadcast information.

Update of Served Cell Information NR:

-	If Served Cells NR To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information NR IE.

-	If Served Cells NR To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old NR-CGI IE according to the information in the Served Cell Information NR IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information NR IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE is contained in the Served Cells NR To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If Served Cells NR To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old NR-CGI IE.

-	If the Intended TDD DL-UL Configuration NR IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 should take this information into account for cross-link interference management and/or NR-DC power coordination with the NG-RAN node1. The NG-RAN node2 shall consider the received Intended TDD DL-UL Configuration NR IE content valid until reception of a new update of the IE for the same NG-RAN node2.

-	If the NR Cell PRACH Configuration IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

- 	If the SFN Offset IE is contained in the Served Cell Information NR IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

Update of Served Cell Information E-UTRA:

-	If Served Cells E-UTRA To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall add cell information according to the information in the Served Cell Information E-UTRA IE.

-	If Served Cells E-UTRA To Modify IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall modify information of cell indicated by Old ECGI IE according to the information in the Served Cell Information E-UTRA IE.

-	When either served cell information or neighbour information of an existing served cell in NG-RAN node1 need to be updated, the whole list of neighbouring cells, if any, shall be contained in the Neighbour Information E-UTRA IE. The NG-RAN node2 shall overwrite the served cell information and the whole list of neighbour cell information for the affected served cell.

-	If the Deactivation Indication IE is contained in the Served Cells E-UTRA To Modify IE, it indicates that the concerned cell was switched off to lower energy consumption.

-	If the Served Cells E-UTRA To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, NG-RAN node2 shall delete information of cell indicated by Old ECGI IE.

-	If the Protected E-UTRA Resource Indication IE is included into the NG-RAN NODE CONFIGURATION UPDATE (inside the Served Cell Information E-UTRA IE), the receiving gNB should take this into account for cell-level resource coordination with the ng-eNB. The gNB shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same ng-eNB. The protected resource pattern indicated in the Protected E-UTRA Resource Indication IE is not valid in subframes indicated by the Reserved Subframes IE (contained in E-UTRA - NR CELL RESOURCE COORDINATION REQUEST messages), as well as in the non-control region of the MBSFN subframes i.e. it is valid only in the control region therein. The size of the control region of MBSFN subframes is indicated in the Protected E-UTRA Resource Indication IE.

-	If the PRACH Configuration IE is contained in the Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

- 	If the NPRACH Configuration IE is contained in the Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE may use this information for RACH optimisation.

-	If the SFN Offset IE is contained in Served Cell Information E-UTRA IE in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node receiving the IE shall, if supported, use this information to update the SFN0 time offset of the reported cell.

Update of TNL addresses for SCTP associations:

If the TNL Association to Add List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall, if supported, use it to establish the TNL association(s) with the NG-RAN node1. The NG-RAN node2 shall report to the NG-RAN node1, in the NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the NG-RAN node1 as follows:

-	A list of successfully established TNL associations shall be included in the TNL Association Setup List IE;

-	A list of TNL associations that failed to be established shall be included in the TNL Association Failed to Setup List IE.

If the TNL Association to Remove List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message the NG-RAN node2 shall, if supported, initiate removal of the TNL association(s) indicated by the received Transport Layer information towards the NG-RAN node1.

If the TNL Association to Update List IE is included in the NG-RAN NODE CONFIGURATION UPDATE message the NG-RAN node2 shall, if supported, update the TNL association(s) indicated by the received Transport Layer information towards the NG-RAN node1.

Update of AMF Region Information:

-	If AMF Region Information To Add IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall add the AMF Regions to its AMF Region List.

-	If AMF Region Information To Delete IE is contained in the NG-RAN NODE CONFIGURATION UPDATE message, the NG-RAN node2 shall remove the AMF Regions from its AMF Region List.

#### 8.4.2.3	Unsuccessful Operation

Figure 8.4.2.3-1: NG-RAN node Configuration Update, unsuccessful operation

If the NG-RAN node2 cannot accept the update it shall respond with the NG-RAN NODE CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the NG-RAN NODE CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the NG-RAN node1 shall wait at least for the indicated time before reinitiating the NG-RAN Node Configuration Update procedure towards the same NG-RAN node2. Both nodes shall continue to operate the Xn with their existing configuration data.

If case of network sharing with multiple cell ID broadcast with shared Xn-C signalling transport, as specified in TS 38.300 [9], the NG-RAN NODE CONFIGURATION UPDATE message and the NG-RAN NODE CONFIGURATION UPDATE FAILURE message shall include the Interface Instance Indication IE to identify the corresponding interface instance.

#### 8.4.2.4	Abnormal Conditions

If the NG-RAN node1 after initiating NG-RAN node Configuration Update procedure receives neither NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE message nor NG-RAN NODE CONFIGURATION UPDATE FAILURE message, the NG-RAN node1 may reinitiate the NG-RAN node Configuration Update procedure towards the same NG-RAN node2, provided that the content of the new NG-RAN NODE CONFIGURATION UPDATE message is identical to the content of the previously unacknowledged NG-RAN NODE CONFIGURATION UPDATE message.

Second Change

#### 9.2.2.11	Served Cell Information NR

This IE contains cell configuration information of an NR cell that a neighbouring NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| NR-PCI | M |  | INTEGER (0..1007, …) | NR Physical Cell ID | – |  |
| NR CGI | M |  | 9.2.2.7 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the NR Cell Identity in the NR CGI IE. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE NR-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  |  |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>DL NR Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>UL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>DL Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>UL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the UL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >>>DL Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the DL Transmission Bandwidth IE shall be ignored. | YES | ignore |
| >TDD |  |  |  |  |  |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>Frequency Info | M |  | NR Frequency Info 9.2.2.19 |  | – |  |
| >>>Transmission Bandwidth | M |  | NR Transmission Bandwidth 9.2.2.20 |  | – |  |
| >>>Intended TDD DL-UL Configuration NR | O |  | 9.2.2.40 |  | YES | ignore |
| >>>TDD UL-DL Configuration Common NR | O |  | OCTET STRING | The tdd-UL-DL-ConfigurationCommon as defined in TS 38.331 [10] | YES | ignore |
| >>>Carrier List | O |  | NR Carrier List 9.2.2.63 | If included, the Transmission Bandwidth IE shall be ignored. | YES | ignore |
| Measurement Timing Configuration | M |  | OCTET STRING | Contains the MeasurementTimingConfiguration inter-node message for the served cell, as defined in TS 38.331 [10]. | – |  |
| Connectivity Support | M |  | 9.2.2.28 |  | – |  |
| Broadcast PLMN Identity Info List NR |  |  | 0..<maxnoofBPLMNs> |  |  |  | This IE corresponds to the PLMN-IdentityInfoList IE in SIB1 as specified in TS 38.331 [8]. All PLMN Identities and associated information contained in the PLMN-IdentityInfoList IE are included and provided in the same order as broadcast in SIB1. |  | YES |  | ignore |  |
| >Broadcast PLMNs |  |  | 1..<maxnoofBPLMNs> |  |  |  | Broadcast PLMNs in SIB1 associated to the NR Cell Identity IE. |  | – |  |  |  |
| >>PLMN Identity | M |  |  |  | 9.2.2.4 |  |  |  | – |  |  |  |
| >TAC | M |  |  |  | 9.2.2.5 |  |  |  | – |  |  |  |
| >NR Cell Identity | M |  |  |  | BIT STRING (SIZE(36)) |  |  |  | – |  |  |  |
| >RANAC | O |  |  |  | RAN Area Code 9.2.2.6 |  |  |  | – |  |  |  |
| >Configured TAC Indication | O |  |  |  | 9.2.2.39a |  | NOTE: This IE is associated with the TAC in the Broadcast PLMN Identity Info List NR IE |  | YES |  | ignore |  |
| >NPN Broadcast Information | O |  |  |  | 9.2.2.71 |  | If this IE is included the content of the Broadcast PLMNs IE in the Broadcast PLMN Identity Info List NR IE is ignored. |  | YES |  | reject |  |
| Configured TAC Indication | O |  |  |  | 9.2.2.39a |  | NOTE: This IE is associated with the TAC on top-level of the Served Cell Information NR IE |  | YES |  | ignore |  |
| NPN Broadcast Information | O |  |  |  | 9.2.2.71 |  | If this IE is included the content of the Broadcast PLMNs IE in the top Served Cell Information NR IE is ignored. |  | YES |  | reject |  |
| SSB Positions In Burst | O |  |  |  | 9.2.2.64 |  |  |  | YES |  | ignore |  |
| NR Cell PRACH Configuration | O |  |  |  | OCTET STRING |  | Containing 9.3.1.139 NR Cell PRACH Configuration as of TS 38.473 [41]. |  | YES |  | ignore |  |
| CSI-RS Transmission Indication | O |  |  |  | ENUMERATED (activated, deactivated, ...) |  | This IE indicates the CSI-RS transmission status of the given cell. |  | YES |  | ignore |  |
| SFN Offset | O |  | 9.2.2.xx |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. Value is 12. |

#### 9.2.2.12	Served Cell Information E-UTRA

This IE contains cell configuration information of an E-UTRA cell that a neighbour NG-RAN node may need for the Xn AP interface.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA PCI | M |  | INTEGER (0..503, …) | E-UTRA Physical Cell ID | – |  |
| ECGI | M |  | E-UTRA CGI 9.2.2.8 |  | – |  |
| TAC | M |  | 9.2.2.5 | Tracking Area Code | – |  |
| RANAC | O |  | RAN Area Code 9.2.2.6 |  | – |  |
| Broadcast PLMNs |  | 1..<maxnoofBPLMNs> |  | Broadcast PLMNs in SIB1 associated to the E-UTRA Cell Identity in the ECGI IE. NOTE: In this version of the specification, it is possible to broadcast only up to 6 PLMN IDs. | – |  |
| >PLMN Identity | M |  | 9.2.2.4 |  | – |  |
| CHOICE E-UTRA-Mode-Info | M |  |  |  | – |  |
| >FDD |  |  |  |  | – |  |
| >>FDD Info |  | 1 |  |  | – |  |
| >>>UL EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NUL in TS 36.104 [25] for E-UTRA operating bands for which it is defined; ignored for E-UTRA operating bands for which NUL is not defined | – |  |
| >>>DL EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NDL in TS 36.104 [25] | – |  |
| >>>UL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.22 | Same as DL Transmission Bandwidth in this release; ignored in case UL EARFCN value is ignored | – |  |
| >>>DL E-UTRA Transmission Bandwidth | M |  | E-UTRA Transmission Bandwidth 9.2.2.22 |  | – |  |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.2.47 | Corresponds to MDL in TS 36.104 [25] | YES | reject |
| >>>Offset of NB-IoT Channel Number to UL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.2.47 | Corresponds to MUL in TS 36.104 [25] | YES | reject |
| >TDD |  |  |  |  | – |  |
| >>TDD Info |  | 1 |  |  | – |  |
| >>>EARFCN | M |  | E-UTRA ARFCN 9.2.2.21 | Corresponds to NDL/NUL in TS 36.104 [25] | – |  |
| >>>E-UTRA Transmission Bandwidth | M |  | 9.2.2.22 |  | – |  |
| >>>Subframe Assignment | M |  | ENUMERATED (sa0, sa1, sa2, sa3, sa4, sa5, sa6, ...) | Uplink-downlink subframe configuration information defined in TS 36.211 [26] | – |  |
| >>>Special Subframe Info |  | 1 |  | Special subframe configuration information defined in TS 36.211 [26] | – |  |
| >>>>Special Subframe Patterns | M |  | ENUMERATED (ssp0, ssp1, ssp2, ssp3, ssp4, ssp5, ssp6, ssp7, ssp8, ssp9, ssp10, ...) |  | – |  |
| >>>>Cyclic Prefix DL | M |  | ENUMERATED (Normal, Extended,…) |  | – |  |
| >>>>Cyclic Prefix UL | M |  | ENUMERATED (Normal, Extended, ...) |  | – |  |
| >>>Offset of NB-IoT Channel Number to DL EARFCN | O |  | Offset of NB-IoT Channel Number to EARFCN 9.2.2.47 | Corresponds to MDL in TS 36.104 [25] | YES | reject |
| >>>NB-IoT UL DL Alignment Offset | O |  | 9.2.2.48 | Corresponds to the TDD-UL-DL-AlignmentOffset-NB in TS 36.331 [14]. | YES | reject |
| Number of Antenna Ports E-UTRA | O |  | 9.2.2.23 |  | – |  |
| PRACH Configuration | O |  | E-UTRA PRACH Configuration 9.2.2.25 |  | – |  |
| MBSFN Subframe Info |  | 0..<maxnoofMBSFN> |  | MBSFN subframe defined in TS 36.331 [14] | – |  |
| >Radioframe Allocation Period | M |  | ENUMERATED (n1, n2, n4, n8, n16, n32, …) |  | – |  |
| >Radioframe Allocation Offset | M |  | INTEGER (0..7, ...) |  | – |  |
| >MBSFN Subframe Allocation E-UTRA | M |  | 9.2.2.26 |  | – |  |
| E-UTRA Multiband Info List | O |  | 9.2.2.24 |  | – |  |
| FreqBandIndicatorPriority | O |  | ENUMERATED (not-broadcast, broadcast, ...) | This IE indicates that the eNodeB supports FreqBandIndicationPriority, and whether FreqBandIndicatorPriority is broadcast in SIB 1 (see TS 36.331 [14]) | – |  |
| BandwidthReducedSI | O |  | ENUMERATED (scheduled, ...) | This IE indicates that the SystemInformationBlockType1-BR is scheduled in the cell (see TS 36.331 [14]) | – |  |
| Protected E-UTRA Resource Indication | O |  | 9.2.2.29 | This IE indicates which E-UTRA control/reference signal resources are protected and are not subject to E-UTRA - NR Cell Resource Coordination. | – |  |
| Broadcast PLMN Identity Info List E-UTRA |  |  |  | 0..<maxnoofEUTRABPLMNs> |  |  |  | This IE corresponds to the cellAccessRelatedInfoList-5GC IE in SIB1 as specified in TS 36.331 [14]. All PLMN Identities and associated information contained in the cellAccessRelatedInfoList-5GC IE are included and provided in the same order as broadcast in SIB1. |  | YES |  | ignore |  |
| >Broadcast PLMNs |  |  |  | 1..<maxnoofEUTRABPLMNs> |  |  |  | Broadcast PLMNs in SIB1 associated to the E-UTRA Cell Identity IE. |  | – |  |  |  |
| >>PLMN Identity |  | M |  |  |  | 9.2.2.4 |  |  |  | – |  |  |  |
| >TAC |  | M |  |  |  | 9.2.2.5 |  |  |  | – |  |  |  |
| >E-UTRA Cell Identity |  | M |  |  |  | BIT STRING (SIZE(28)) |  |  |  | – |  |  |  |
| >RANAC |  | O |  |  |  | RAN Area Code 9.2.2.6 |  |  |  | – |  |  |  |
| NPRACH Configuration |  | O |  |  |  | NPRACH Configuration 9.2.2.74 |  |  |  | YES |  | ignore |  |
| SFN Offset | O |  | 9.2.2.xx |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofBPLMNs | Maximum no. of broadcast PLMNs by a cell. The value is 12. |
| maxnoofMBSFN | Maximum no. of MBSFN frame allocation with different offset. Value is 8. |
| maxnoofEUTRABPLMNs | Maximum no. of PLMN Ids.broadcast in an E-UTRA cell. Value is 6. |

#### Third Change

#### 9.2.2.xx	SFN Offset

This IE contains the time offset between an absolute time reference and the SFN0 start. The IE is calculated assuming that the SFN transmission started at the absolute time reference. The absolute time reference chosen is 1980-01-06 T00:00:19 International Atomic Time (TAI).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| SFN Time Offset | M |  | BIT STRING (SIZE(24)) | Time offset in microseconds between the absolute time reference "1980-01-06 T00:00:19 International Atomic Time (TAI)” and the SFN0 start. The maximum usable value is (1024*10^4-1). Values higher than the maximum are discarded. |  |  |

Fifth Change

### 9.3.5	Information Element Definitions

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

-- ASN1START

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CNTypeRestrictionsForEquivalent,

id-CNTypeRestrictionsForServing,

id-Additional-UL-NG-U-TNLatUPF-List,

id-ConfiguredTACIndication,

id-AlternativeQoSParaSetList,

id-CurrentQoSParaSetIndex,

id-DefaultDRB-Allowed,

id-DLCarrierList,

id-EndpointIPAddressAndPort,

id-ExtendedTAISliceSupportList,

id-FiveGCMobilityRestrictionListContainer,

id-SecondarydataForwardingInfoFromTarget-List,

id-LastE-UTRANPLMNIdentity,

id-IntendedTDD-DL-ULConfiguration-NR,

id-MaxIPrate-DL,

id-SecurityResult,

id-OldQoSFlowMap-ULendmarkerexpected,

id-PDUSessionCommonNetworkInstance,

id-BPLMN-ID-Info-EUTRA,

id-BPLMN-ID-Info-NR,

id-DRBsNotAdmittedSetupModifyList,

id-Secondary-MN-Xn-U-TNLInfoatM,

id-ULForwardingProposal,

id-DRB-IDs-takenintouse,

id-SplitSessionIndicator,

id-NonGBRResources-Offered,

id-MDT-Configuration,

id-TraceCollectionEntityURI,

id-NPN-Broadcast-Information,

id-NPNPagingAssistanceInformation,

id-NPNMobilityInformation,

id-NPN-Support,

id-LTEUESidelinkAggregateMaximumBitRate,

id-NRUESidelinkAggregateMaximumBitRate,

id-ExtendedRATRestrictionInformation,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-DAPSRequestInfo,

id-OffsetOfNbiotChannelNumberToDL-EARFCN,

id-OffsetOfNbiotChannelNumberToUL-EARFCN,

id-NBIoT-UL-DL-AlignmentOffset,

id-TDDULDLConfigurationCommonNR,

id-CarrierList,

id-ULCarrierList,

id-FrequencyShift7p5khz,

id-SSB-PositionsInBurst,

id-NRCellPRACHConfig,

id-Redundant-UL-NG-U-TNLatUPF,

id-Redundant-DL-NG-U-TNLatNG-RAN,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-ExtendedPacketDelayBudget,

id-Additional-Redundant-UL-NG-U-TNLatUPF-List,

id-RedundantCommonNetworkInstance,

id-TSCTrafficCharacteristics,

id-RedundantQoSFlowIndicator,

id-Additional-PDCP-Duplication-TNL-List,

id-RedundantPDUSessionInformation,

id-UsedRSNInformation,

id-RLCDuplicationInformation,

id-CSI-RSTransmissionIndication,

id-UERadioCapabilityID,

id-secondary-SN-UL-PDCP-UP-TNLInfo,

id-pdcpDuplicationConfiguration,

id-duplicationActivation,

id-NPRACHConfiguration,

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated,

id-DL-scheduling-PDCCH-CCE-usage,

id-UL-scheduling-PDCCH-CCE-usage,

id-SFN-Offset,

maxEARFCN,

maxnoofAllowedAreas,

maxnoofAMFRegions,

maxnoofAoIs,

maxnoofBPLMNs,

maxnoofCAGs,

maxnoofCAGsperPLMN,

maxnoofCellsinAoI,

maxnoofCellsinNG-RANnode,

maxnoofCellsinRNA,

maxnoofCellsinUEHistoryInfo,

maxnoofCellsUEMovingTrajectory,

maxnoofDRBs,

maxnoofEPLMNs,

maxnoofEPLMNsplus1,

maxnoofEUTRABands,

maxnoofEUTRABPLMNs,

maxnoofForbiddenTACs,

maxnoofMBSFNEUTRA,

maxnoofMultiConnectivityMinusOne,

maxnoofNeighbours,

maxnoofNIDs,

maxnoofNRCellBands,

maxnoofPDUSessions,

maxnoofPLMNs,

maxnoofProtectedResourcePatterns,

maxnoofQoSFlows,

maxnoofQoSParaSets,

maxnoofRANAreaCodes,

maxnoofRANAreasinRNA,

maxnoofSCellGroups,

maxnoofSCellGroupsplus1,

maxnoofSliceItems,

maxnoofExtSliceItems,

maxnoofSNPNIDs,

maxnoofsupportedTACs,

maxnoofsupportedPLMNs,

maxnoofTAI,

maxnoofTAIsinAoI,

maxnoofTNLAssociations,

maxnoofUEContexts,

maxNRARFCN,

maxNrOfErrors,

maxnoofRANNodesinAoI,

maxnooftimeperiods,

maxnoofslots,

maxnoofExtTLAs,

maxnoofGTPTLAs,

maxnoofCHOcells,

maxnoofPC5QoSFlows,

maxnoofSSBAreas,

maxnoofNRSCSs,

maxnoofPhysicalResourceBlocks,

maxnoofRACHReports,

maxnoofAdditionalPDCPDuplicationTNL,

maxnoofRLCDuplicationstate,

maxnoofBluetoothName,

maxnoofCellIDforMDT,

maxnoofMDTPLMNs,

maxnoofTAforMDT,

maxnoofWLANName,

maxnoofSensorName,

maxnoofNeighPCIforMDT,

maxnoofFreqforMDT,

maxnoofNonAnchorCarrierFreqConfig

FROM XnAP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM XnAP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-Single-Container{},

XNAP-PROTOCOL-EXTENSION,

XNAP-PROTOCOL-IES

FROM XnAP-Containers;

Unchanged Text Skipped

ServedCells-E-UTRA-Item ::= SEQUENCE {

served-cell-info-E-UTRA		ServedCellInformation-E-UTRA,

neighbour-info-NR			NeighbourInformation-NR									OPTIONAL,

neighbour-info-E-UTRA		NeighbourInformation-E-UTRA								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {ServedCells-E-UTRA-Item-ExtIEs} } OPTIONAL,

...

}

ServedCells-E-UTRA-Item-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-SFN-Offset				CRITICALITY ignore EXTENSION SFN-Offset					PRESENCE optional },

...

}

Unchanged Text Skipped

##### -- Served Cells NR IEs

ServedCellInformation-NR ::= SEQUENCE {

nrPCI								NRPCI,

cellID								NR-CGI,

tac									TAC,

ranac								RANAC						OPTIONAL,

broadcastPLMN						BroadcastPLMNs,

nrModeInfo							NRModeInfo,

measurementTimingConfiguration		OCTET STRING,

connectivitySupport					Connectivity-Support,

iE-Extensions						ProtocolExtensionContainer { {ServedCellInformation-NR-ExtIEs} } OPTIONAL,

...

}

ServedCellInformation-NR-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ ID id-BPLMN-ID-Info-NR				CRITICALITY ignore	EXTENSION BPLMN-ID-Info-NR						PRESENCE optional }|

{ ID id-ConfiguredTACIndication			CRITICALITY ignore	EXTENSION ConfiguredTACIndication				PRESENCE optional }|

{ ID id-SSB-PositionsInBurst			CRITICALITY ignore	EXTENSION SSB-PositionsInBurst					PRESENCE optional }|

{ ID id-NRCellPRACHConfig				CRITICALITY ignore	EXTENSION NRCellPRACHConfig						PRESENCE optional }|

{ ID id-NPN-Broadcast-Information		CRITICALITY reject	EXTENSION NPN-Broadcast-Information			PRESENCE optional }|

{ ID id-CSI-RSTransmissionIndication	CRITICALITY ignore	EXTENSION CSI-RSTransmissionIndication		PRESENCE optional } |

{ ID id-SFN-Offset				CRITICALITY ignore EXTENSION SFN-Offset					PRESENCE optional },

...

}

SFN-Offset ::= SEQUENCE {

SFN-Time-Offset					BIT STRING (SIZE(24)),

iE-Extensions		ProtocolExtensionContainer { {SFN-Offset-ExtIEs} } OPTIONAL,

...

}

SFN-Offset-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

Sixth Change

### 9.3.7	Constant definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

XnAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) xnap (2) version1 (1) xnap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM XnAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-handoverPreparation											ProcedureCode ::= 0

id-sNStatusTransfer												ProcedureCode ::= 1

id-handoverCancel												ProcedureCode ::= 2

id-retrieveUEContext											ProcedureCode ::= 3

id-rANPaging													ProcedureCode ::= 4

id-xnUAddressIndication											ProcedureCode ::= 5

id-uEContextRelease												ProcedureCode ::= 6

id-sNGRANnodeAdditionPreparation								ProcedureCode ::= 7

id-sNGRANnodeReconfigurationCompletion							ProcedureCode ::= 8

id-mNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 9

id-sNGRANnodeinitiatedSNGRANnodeModificationPreparation			ProcedureCode ::= 10

id-mNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 11

id-sNGRANnodeinitiatedSNGRANnodeRelease							ProcedureCode ::= 12

id-sNGRANnodeCounterCheck										ProcedureCode ::= 13

id-sNGRANnodeChange												ProcedureCode ::= 14

id-rRCTransfer													ProcedureCode ::= 15

id-xnRemoval													ProcedureCode ::= 16

id-xnSetup														ProcedureCode ::= 17

id-nGRANnodeConfigurationUpdate									ProcedureCode ::= 18

id-cellActivation												ProcedureCode ::= 19

id-reset														ProcedureCode ::= 20

id-errorIndication												ProcedureCode ::= 21

id-privateMessage												ProcedureCode ::= 22

id-notificationControl											ProcedureCode ::= 23

id-activityNotification											ProcedureCode ::= 24

id-e-UTRA-NR-CellResourceCoordination							ProcedureCode ::= 25

id-secondaryRATDataUsageReport									ProcedureCode ::= 26

id-deactivateTrace												ProcedureCode ::= 27

id-traceStart													ProcedureCode ::= 28

id-handoverSuccess												ProcedureCode ::= 29

id-conditionalHandoverCancel									ProcedureCode ::= 30

id-earlyStatusTransfer											ProcedureCode ::= 31

id-failureIndication												ProcedureCode ::= 32

id-handoverReport												ProcedureCode ::= 33

id-resourceStatusReportingInitiation							ProcedureCode ::= 34

id-resourceStatusReporting										ProcedureCode ::= 35

id-mobilitySettingsChange										ProcedureCode ::= 36

id-accessAndMobilityIndication									ProcedureCode ::= 37

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxEARFCN									INTEGER ::= 262143

maxnoofAllowedAreas							INTEGER ::= 16

maxnoofAMFRegions							INTEGER ::= 16

maxnoofAoIs									INTEGER ::= 64

maxnoofBluetoothName						INTEGER ::= 4

maxnoofBPLMNs								INTEGER ::= 12

maxnoofCAGs									INTEGER ::= 12

maxnoofCAGsperPLMN							INTEGER ::= 256

maxnoofCellIDforMDT							INTEGER ::= 32

maxnoofCellsinAoI							INTEGER ::= 256

maxnoofCellsinUEHistoryInfo					INTEGER ::= 16

maxnoofCellsinNG-RANnode					INTEGER ::= 16384

maxnoofCellsinRNA							INTEGER ::= 32

maxnoofCellsUEMovingTrajectory				INTEGER ::= 16

maxnoofDRBs									INTEGER ::= 32

maxnoofEUTRABands							INTEGER ::= 16

maxnoofEUTRABPLMNs							INTEGER ::= 6

maxnoofEPLMNs								INTEGER ::= 15

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofEPLMNsplus1							INTEGER ::= 16

maxnoofForbiddenTACs						INTEGER ::= 4096

maxnoofFreqforMDT							INTEGER ::= 8

maxnoofMBSFNEUTRA							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofMultiConnectivityMinusOne			INTEGER ::= 3

maxnoofNeighbours							INTEGER ::= 1024

maxnoofNeighPCIforMDT						INTEGER ::= 32

maxnoofNIDs									INTEGER ::= 12

maxnoofNRCellBands							INTEGER ::= 32

maxnoofPLMNs								INTEGER ::= 16

maxnoofPDUSessions							INTEGER ::= 256

maxnoofProtectedResourcePatterns			INTEGER ::= 16

maxnoofQoSFlows								INTEGER ::= 64

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofRANAreaCodes							INTEGER ::= 32

maxnoofRANAreasinRNA						INTEGER ::= 16

maxnoofRANNodesinAoI						INTEGER ::= 64

maxnoofSCellGroups							INTEGER ::= 3

maxnoofSCellGroupsplus1						INTEGER ::= 4

maxnoofSensorName							INTEGER ::= 3

maxnoofSliceItems							INTEGER ::= 1024

maxnoofSNPNIDs								INTEGER ::= 12

maxnoofsupportedPLMNs						INTEGER ::= 12

maxnoofsupportedTACs						INTEGER ::= 256

maxnoofTAforMDT								INTEGER ::= 8

maxnoofTAI									INTEGER ::= 16

maxnoofTAIsinAoI 							INTEGER ::= 16

maxnooftimeperiods							INTEGER ::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofUEContexts							INTEGER ::= 8192

maxNRARFCN									INTEGER ::= 3279165

maxNrOfErrors								INTEGER ::= 256

maxnoofslots								INTEGER ::= 5120

maxnoofExtTLAs								INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofCHOcells								INTEGER ::= 8

maxnoofPC5QoSFlows 							INTEGER ::= 2064

maxnoofSSBAreas								INTEGER ::= 64

maxnoofRACHReports							INTEGER ::= 64

maxnoofNRSCSs								INTEGER ::= 5

maxnoofPhysicalResourceBlocks				INTEGER ::= 275

maxnoofAdditionalPDCPDuplicationTNL			INTEGER ::= 2

maxnoofRLCDuplicationstate					INTEGER ::= 3

maxnoofWLANName								INTEGER ::= 4

maxnoofNonAnchorCarrierFreqConfig			INTEGER ::= 15

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-ActivatedServedCells																				ProtocolIE-ID ::= 0

id-ActivationIDforCellActivation																	ProtocolIE-ID ::= 1

id-admittedSplitSRB																					ProtocolIE-ID ::= 2

id-admittedSplitSRBrelease																			ProtocolIE-ID ::= 3

id-AMF-Region-Information																			ProtocolIE-ID ::= 4

id-AssistanceDataForRANPaging																		ProtocolIE-ID ::= 5

id-BearersSubjectToCounterCheck																		ProtocolIE-ID ::= 6

id-Cause																							ProtocolIE-ID ::= 7

id-cellAssistanceInfo-NR																			ProtocolIE-ID ::= 8

id-ConfigurationUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 9

id-CriticalityDiagnostics																			ProtocolIE-ID ::= 10

id-XnUAddressInfoperPDUSession-List																	ProtocolIE-ID ::= 11

id-DRBsSubjectToStatusTransfer-List																	ProtocolIE-ID ::= 12

id-ExpectedUEBehaviour																				ProtocolIE-ID ::= 13

id-GlobalNG-RAN-node-ID																				ProtocolIE-ID ::= 14

id-GUAMI																							ProtocolIE-ID ::= 15

id-indexToRatFrequSelectionPriority																	ProtocolIE-ID ::= 16

id-initiatingNodeType-ResourceCoordRequest															ProtocolIE-ID ::= 17

id-List-of-served-cells-E-UTRA																		ProtocolIE-ID ::= 18

id-List-of-served-cells-NR																			ProtocolIE-ID ::= 19

id-LocationReportingInformation																		ProtocolIE-ID ::= 20

id-MAC-I																							ProtocolIE-ID ::= 21

id-MaskedIMEISV																						ProtocolIE-ID ::= 22

id-M-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 23

id-MN-to-SN-Container																				ProtocolIE-ID ::= 24

id-MobilityRestrictionList																			ProtocolIE-ID ::= 25

id-new-NG-RAN-Cell-Identity																			ProtocolIE-ID ::= 26

id-newNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 27

id-UEReportRRCTransfer																				ProtocolIE-ID ::= 28

id-oldNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 29

id-OldtoNewNG-RANnodeResumeContainer																ProtocolIE-ID ::= 30

id-PagingDRX																						ProtocolIE-ID ::= 31

id-PCellID																							ProtocolIE-ID ::= 32

id-PDCPChangeIndication																				ProtocolIE-ID ::= 33

id-PDUSessionAdmittedAddedAddReqAck																	ProtocolIE-ID ::= 34

id-PDUSessionAdmittedModSNModConfirm																ProtocolIE-ID ::= 35

id-PDUSessionAdmitted-SNModResponse																	ProtocolIE-ID ::= 36

id-PDUSessionNotAdmittedAddReqAck																	ProtocolIE-ID ::= 37

id-PDUSessionNotAdmitted-SNModResponse																ProtocolIE-ID ::= 38

id-PDUSessionReleasedList-RelConf																	ProtocolIE-ID ::= 39

id-PDUSessionReleasedSNModConfirm																	ProtocolIE-ID ::= 40

id-PDUSessionResourcesActivityNotifyList															ProtocolIE-ID ::= 41

id-PDUSessionResourcesAdmitted-List																	ProtocolIE-ID ::= 42

id-PDUSessionResourcesNotAdmitted-List																ProtocolIE-ID ::= 43

id-PDUSessionResourcesNotifyList																	ProtocolIE-ID ::= 44

id-PDUSession-SNChangeConfirm-List																	ProtocolIE-ID ::= 45

id-PDUSession-SNChangeRequired-List																	ProtocolIE-ID ::= 46

id-PDUSessionToBeAddedAddReq																		ProtocolIE-ID ::= 47

id-PDUSessionToBeModifiedSNModRequired																ProtocolIE-ID ::= 48

id-PDUSessionToBeReleasedList-RelRqd																ProtocolIE-ID ::= 49

id-PDUSessionToBeReleased-RelReq																	ProtocolIE-ID ::= 50

id-PDUSessionToBeReleasedSNModRequired																ProtocolIE-ID ::= 51

id-RANPagingArea																					ProtocolIE-ID ::= 52

id-PagingPriority																					ProtocolIE-ID ::= 53

id-requestedSplitSRB																				ProtocolIE-ID ::= 54

id-requestedSplitSRBrelease																			ProtocolIE-ID ::= 55

id-ResetRequestTypeInfo																				ProtocolIE-ID ::= 56

id-ResetResponseTypeInfo																			ProtocolIE-ID ::= 57

id-RespondingNodeTypeConfigUpdateAck																ProtocolIE-ID ::= 58

id-respondingNodeType-ResourceCoordResponse															ProtocolIE-ID ::= 59

id-ResponseInfo-ReconfCompl																			ProtocolIE-ID ::= 60

id-RRCConfigIndication																				ProtocolIE-ID ::= 61

id-RRCResumeCause																					ProtocolIE-ID ::= 62

id-SCGConfigurationQuery																			ProtocolIE-ID ::= 63

id-selectedPLMN																						ProtocolIE-ID ::= 64

id-ServedCellsToActivate																			ProtocolIE-ID ::= 65

id-servedCellsToUpdate-E-UTRA																		ProtocolIE-ID ::= 66

id-ServedCellsToUpdateInitiatingNodeChoice															ProtocolIE-ID ::= 67

id-servedCellsToUpdate-NR																			ProtocolIE-ID ::= 68

id-s-ng-RANnode-SecurityKey																			ProtocolIE-ID ::= 69

id-S-NG-RANnodeUE-AMBR																				ProtocolIE-ID ::= 70

id-S-NG-RANnodeUEXnAPID																				ProtocolIE-ID ::= 71

id-SN-to-MN-Container																				ProtocolIE-ID ::= 72

id-sourceNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 73

id-SplitSRB-RRCTransfer																				ProtocolIE-ID ::= 74

id-TAISupport-list																					ProtocolIE-ID ::= 75

id-TimeToWait																						ProtocolIE-ID ::= 76

id-Target2SourceNG-RANnodeTranspContainer															ProtocolIE-ID ::= 77

id-targetCellGlobalID																				ProtocolIE-ID ::= 78

id-targetNG-RANnodeUEXnAPID																			ProtocolIE-ID ::= 79

id-target-S-NG-RANnodeID																			ProtocolIE-ID ::= 80

id-TraceActivation																					ProtocolIE-ID ::= 81

id-UEContextID																						ProtocolIE-ID ::= 82

id-UEContextInfoHORequest																			ProtocolIE-ID ::= 83

id-UEContextInfoRetrUECtxtResp																		ProtocolIE-ID ::= 84

id-UEContextInfo-SNModRequest																		ProtocolIE-ID ::= 85

id-UEContextKeptIndicator																			ProtocolIE-ID ::= 86

id-UEContextRefAtSN-HORequest																		ProtocolIE-ID ::= 87

id-UEHistoryInformation																				ProtocolIE-ID ::= 88

id-UEIdentityIndexValue																				ProtocolIE-ID ::= 89

id-UERANPagingIdentity																				ProtocolIE-ID ::= 90

id-UESecurityCapabilities																			ProtocolIE-ID ::= 91

id-UserPlaneTrafficActivityReport																	ProtocolIE-ID ::= 92

id-XnRemovalThreshold 																				ProtocolIE-ID ::= 93

id-DesiredActNotificationLevel																		ProtocolIE-ID ::= 94

id-AvailableDRBIDs 																					ProtocolIE-ID ::= 95

id-AdditionalDRBIDs 																				ProtocolIE-ID ::= 96

id-SpareDRBIDs 																						ProtocolIE-ID ::= 97

id-RequiredNumberOfDRBIDs 																			ProtocolIE-ID ::= 98

id-TNLA-To-Add-List																					ProtocolIE-ID ::= 99

id-TNLA-To-Update-List																				ProtocolIE-ID ::= 100

id-TNLA-To-Remove-List																				ProtocolIE-ID ::= 101

id-TNLA-Setup-List																					ProtocolIE-ID ::= 102

id-TNLA-Failed-To-Setup-List																		ProtocolIE-ID ::= 103

id-PDUSessionToBeReleased-RelReqAck																	ProtocolIE-ID ::= 104

id-S-NG-RANnodeMaxIPDataRate-UL																		ProtocolIE-ID ::= 105

id-PDUSessionResourceSecondaryRATUsageList															ProtocolIE-ID ::= 107

id-Additional-UL-NG-U-TNLatUPF-List																	ProtocolIE-ID ::= 108

id-SecondarydataForwardingInfoFromTarget-List														ProtocolIE-ID ::= 109

id-LocationInformationSNReporting																	ProtocolIE-ID ::= 110

id-LocationInformationSN																			ProtocolIE-ID ::= 111

id-LastE-UTRANPLMNIdentity																			ProtocolIE-ID ::= 112

id-S-NG-RANnodeMaxIPDataRate-DL																		ProtocolIE-ID ::= 113

id-MaxIPrate-DL																						ProtocolIE-ID ::= 114

id-SecurityResult																					ProtocolIE-ID ::= 115

id-S-NSSAI																							ProtocolIE-ID ::= 116

id-MR-DC-ResourceCoordinationInfo																	ProtocolIE-ID ::= 117

id-AMF-Region-Information-To-Add																	ProtocolIE-ID ::= 118

id-AMF-Region-Information-To-Delete																	ProtocolIE-ID ::= 119

id-OldQoSFlowMap-ULendmarkerexpected																ProtocolIE-ID ::= 120

id-RANPagingFailure																					ProtocolIE-ID ::= 121

id-UERadioCapabilityForPaging																		ProtocolIE-ID ::= 122

id-PDUSessionDataForwarding-SNModResponse															ProtocolIE-ID ::= 123

id-DRBsNotAdmittedSetupModifyList																	ProtocolIE-ID ::= 124

id-Secondary-MN-Xn-U-TNLInfoatM																		ProtocolIE-ID ::= 125

id-NE-DC-TDM-Pattern																				ProtocolIE-ID ::= 126

id-PDUSessionCommonNetworkInstance																	ProtocolIE-ID ::= 127

id-BPLMN-ID-Info-EUTRA																				ProtocolIE-ID ::= 128

id-BPLMN-ID-Info-NR																					ProtocolIE-ID ::= 129

id-InterfaceInstanceIndication																		ProtocolIE-ID ::= 130

id-S-NG-RANnode-Addition-Trigger-Ind																ProtocolIE-ID ::= 131

id-DefaultDRB-Allowed																				ProtocolIE-ID ::= 132

id-DRB-IDs-takenintouse																				ProtocolIE-ID ::= 133

id-SplitSessionIndicator																			ProtocolIE-ID ::= 134

id-CNTypeRestrictionsForEquivalent																	ProtocolIE-ID ::= 135

id-CNTypeRestrictionsForServing																		ProtocolIE-ID ::= 136

id-DRBs-transferred-to-MN																			ProtocolIE-ID ::= 137

id-ULForwardingProposal																				ProtocolIE-ID ::= 138

id-EndpointIPAddressAndPort 																		ProtocolIE-ID ::= 139

id-IntendedTDD-DL-ULConfiguration-NR																ProtocolIE-ID ::= 140

id-TNLConfigurationInfo																				ProtocolIE-ID ::= 141

id-PartialListIndicator-NR																			ProtocolIE-ID ::= 142

id-MessageOversizeNotification																		ProtocolIE-ID ::= 143

id-CellAndCapacityAssistanceInfo-NR																	ProtocolIE-ID ::= 144

id-NG-RANTraceID																					ProtocolIE-ID ::= 145

id-NonGBRResources-Offered																			ProtocolIE-ID ::= 146

id-FastMCGRecoveryRRCTransfer-SN-to-MN																ProtocolIE-ID ::= 147

id-RequestedFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 148

id-AvailableFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 149

id-RequestedFastMCGRecoveryViaSRB3Release															ProtocolIE-ID ::= 150

id-ReleaseFastMCGRecoveryViaSRB3																	ProtocolIE-ID ::= 151

id-FastMCGRecoveryRRCTransfer-MN-to-SN																ProtocolIE-ID ::= 152

id-ExtendedRATRestrictionInformation																ProtocolIE-ID ::= 153

id-QoSMonitoringRequest																				ProtocolIE-ID ::= 154

id-FiveGCMobilityRestrictionListContainer															ProtocolIE-ID ::= 155

id-PartialListIndicator-EUTRA																		ProtocolIE-ID ::= 156

id-CellAndCapacityAssistanceInfo-EUTRA																ProtocolIE-ID ::= 157

id-CHOinformation-Req																				ProtocolIE-ID ::= 158

id-CHOinformation-Ack																				ProtocolIE-ID ::= 159

id-targetCellsToCancel																				ProtocolIE-ID ::= 160

id-requestedTargetCellGlobalID																		ProtocolIE-ID ::= 161

id-procedureStage																					ProtocolIE-ID ::= 162

id-DAPSRequestInfo																					ProtocolIE-ID ::= 163

id-DAPSResponseInfo-List																			ProtocolIE-ID ::= 164

id-CHO-MRDC-Indicator																				ProtocolIE-ID ::= 165

id-OffsetOfNbiotChannelNumberToDL-EARFCN															ProtocolIE-ID ::= 166

id-OffsetOfNbiotChannelNumberToUL-EARFCN															ProtocolIE-ID ::= 167

id-NBIoT-UL-DL-AlignmentOffset																		ProtocolIE-ID ::= 168

id-LTEV2XServicesAuthorized																			ProtocolIE-ID ::= 169

id-NRV2XServicesAuthorized																			ProtocolIE-ID ::= 170

id-LTEUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 171

id-NRUESidelinkAggregateMaximumBitRate																ProtocolIE-ID ::= 172

id-PC5QoSParameters																					ProtocolIE-ID ::= 173

id-AlternativeQoSParaSetList																		ProtocolIE-ID ::= 174

id-CurrentQoSParaSetIndex																			ProtocolIE-ID ::= 175

id-MobilityInformation 																				ProtocolIE-ID ::= 176

id-InitiatingCondition-FailureIndication															ProtocolIE-ID ::= 177

id-UEHistoryInformationFromTheUE																ProtocolIE-ID ::= 178

id-HandoverReportType																				ProtocolIE-ID ::= 179

id-HandoverCause																					ProtocolIE-ID ::= 180

id-SourceCellCGI																					ProtocolIE-ID ::= 181

id-TargetCellCGI																					ProtocolIE-ID ::= 182

id-ReEstablishmentCellCGI																			ProtocolIE-ID ::= 183

id-TargetCellinEUTRAN																				ProtocolIE-ID ::= 184

id-SourceCellCRNTI																					ProtocolIE-ID ::= 185

id-UERLFReportContainer																				ProtocolIE-ID ::= 186

id-NGRAN-Node1-Measurement-ID																		ProtocolIE-ID ::= 187

id-NGRAN-Node2-Measurement-ID																		ProtocolIE-ID ::= 188

id-RegistrationRequest																				ProtocolIE-ID ::= 189

id-ReportCharacteristics																				ProtocolIE-ID ::= 190

id-CellToReport																								ProtocolIE-ID ::= 191

id-ReportingPeriodicity																					ProtocolIE-ID ::= 192

id-CellMeasurementResult																				ProtocolIE-ID ::= 193

id-NG-RANnode1CellID																						ProtocolIE-ID ::= 194

id-NG-RANnode2CellID																				ProtocolIE-ID ::= 195

id-NG-RANnode1MobilityParameters																	ProtocolIE-ID ::= 196

id-NG-RANnode2ProposedMobilityParameters															ProtocolIE-ID ::= 197

id-MobilityParametersModificationRange																ProtocolIE-ID ::= 198

id-TDDULDLConfigurationCommonNR																		ProtocolIE-ID ::= 199

id-CarrierList																						ProtocolIE-ID ::= 200

id-ULCarrierList																					ProtocolIE-ID ::= 201

id-FrequencyShift7p5khz																				ProtocolIE-ID ::= 202

id-SSB-PositionsInBurst																				ProtocolIE-ID ::= 203

id-NRCellPRACHConfig																				ProtocolIE-ID ::= 204

id-RACHReportInformation																			ProtocolIE-ID ::= 205

id-IABNodeIndication																				ProtocolIE-ID ::= 206

id-Redundant-UL-NG-U-TNLatUPF																		ProtocolIE-ID ::= 207

id-CNPacketDelayBudgetDownlink																		ProtocolIE-ID ::= 208

id-CNPacketDelayBudgetUplink																		ProtocolIE-ID ::= 209

id-Additional-Redundant-UL-NG-U-TNLatUPF-List														ProtocolIE-ID ::= 210

id-RedundantCommonNetworkInstance																	ProtocolIE-ID ::= 211

id-TSCTrafficCharacteristics																		ProtocolIE-ID ::= 212

id-RedundantQoSFlowIndicator																		ProtocolIE-ID ::= 213

id-Redundant-DL-NG-U-TNLatNG-RAN																	ProtocolIE-ID ::= 214

id-ExtendedPacketDelayBudget																		ProtocolIE-ID ::= 215

id-Additional-PDCP-Duplication-TNL-List																ProtocolIE-ID ::= 216

id-RedundantPDUSessionInformation																	ProtocolIE-ID ::= 217

id-UsedRSNInformation																				ProtocolIE-ID ::= 218

id-RLCDuplicationInformation																		ProtocolIE-ID ::= 219

id-NPN-Broadcast-Information																		ProtocolIE-ID ::= 220

id-NPNPagingAssistanceInformation																	ProtocolIE-ID ::= 221

id-NPNMobilityInformation																			ProtocolIE-ID ::= 222

id-NPN-Support																						ProtocolIE-ID ::= 223

id-MDT-Configuration																				ProtocolIE-ID ::= 224

id-MDTPLMNList																						ProtocolIE-ID ::= 225

id-TraceCollectionEntityURI																			ProtocolIE-ID ::= 226

id-UERadioCapabilityID																				ProtocolIE-ID ::= 227

id-CSI-RSTransmissionIndication																		ProtocolIE-ID ::= 228

id-SNTriggered  																					ProtocolIE-ID ::= 229

id-DLCarrierList																					ProtocolIE-ID ::= 230

id-ExtendedTAISliceSupportList																		ProtocolIE-ID ::= 231

id-cellAssistanceInfo-EUTRA																			ProtocolIE-ID ::= 232

id-ConfiguredTACIndication																			ProtocolIE-ID ::= 233

id-secondary-SN-UL-PDCP-UP-TNLInfo																	ProtocolIE-ID ::= 234

id-pdcpDuplicationConfiguration																		ProtocolIE-ID ::= 235

id-duplicationActivation																			ProtocolIE-ID ::= 236

id-NPRACHConfiguration																				ProtocolIE-ID ::= 237

id-QosMonitoringReportingFrequency																	ProtocolIE-ID ::= 238

id-QoSFlowsMappedtoDRB-SetupResponse-MNterminated														ProtocolIE-ID ::= 239

id-DL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 240

id-UL-scheduling-PDCCH-CCE-usage																	ProtocolIE-ID ::= 241

id-SFN-Offset																						ProtocolIE-ID ::= XXX

END

-- ASN1STOP

End of Changes