3GPP TSG-RAN5 Meeting #90-e	R5-210558

# Electronic Meeting, 22 February – 5 March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5257 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding subPRB allocation to 6.6.2.1EC, Spectrum Emission Mask for UE category M2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per the WP, subPRB allocation should be added to test case 6.6.2.1EC |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added subPRB allocation to test configuration table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | efeMTC WI cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.2.1EC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

#### 6.6.2.1EC	Spectrum Emission Mask for UE category M2

##### 6.6.2.1EC.1	Test purpose

To verify that the power of any UE emission shall not exceed specified lever for the specified channel bandwidth.

##### 6.6.2.1EC.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

##### 6.6.2.1EC.3	Minimum conformance requirements

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the assigned E-UTRA channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2.1EC.3-1, the spurious requirements in clause 6.6.3EC are applicable.

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1EC.3-1 for the specified channel bandwidth.

Table 6.6.2.1EC.3-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

The normative reference for this requirement is TS 36.101 [2] clause 6.6.2.1.

##### 6.6.2.1EC.4	Test description

6.6.2.1EC.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.6.2.1EC.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2 respectively. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.2.1EC.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | TDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 0 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 0 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 0 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 0 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 0 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 0 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 0 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 0 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 0 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 0 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 0 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 0 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 0 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 0 |
| 24 | 15MHz |  | QPSK | 16 | 16 | 0 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 0 |
| 26 | 15MHz |  | 16QAM | 16 | 16 | 0 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 0 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 0 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 0 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 0 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 0 |
| High range |  |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 6 | 0 |
| 6 | 3MHz |  | QPSK | 2 | 2 | 1 |
| 7 | 3MHz |  | QPSK | 5 | 5 | 1 |
| 8 | 3MHz |  | QPSK | 12 | 12 | 0 |
| 9 | 3MHz |  | 16QAM | 2 | 2 | 1 |
| 10 | 3MHz |  | 16QAM | 12 | 12 | 0 |
| 11 (Note 4) | 5MHz |  | QPSK | 1 | 1 | 3 |
| 12 (Note 5) | 5MHz |  | QPSK | 3 | 3 | 3 |
| 13 | 5MHz |  | QPSK | 24 | 24 | 0 |
| 14 (Note 4) | 5MHz |  | 16QAM | 1 | 1 | 3 |
| 15 | 5MHz |  | 16QAM | 3 | 3 | 3 |
| 16 (Note 5) | 5MHz |  | 16QAM | 5 | 5 | 3 |
| 17 | 5MHz |  | 16QAM | 24 | 24 | 0 |
| 18 (Note 4) | 10MHz |  | QPSK | 4 | 4 | 7 |
| 19 (Note 5) | 10MHz |  | QPSK | 5 | 5 | 7 |
| 20 | 10MHz |  | QPSK | 24 | 24 | 4 |
| 21 (Note 4) | 10MHz |  | 16QAM | 3 | 3 | 7 |
| 22 | 10MHz |  | 16QAM | 5 | 5 | 7 |
| 23 | 10MHz |  | 16QAM | 24 | 24 | 4 |
| 24 | 15MHz |  | QPSK | 15 | 15 | 9 |
| 25 | 15MHz |  | QPSK | 24 | 24 | 8 |
| 26 | 15MHz |  | 16QAM | 15 | 15 | 9 |
| 27 | 15MHz |  | 16QAM | 24 | 24 | 8 |
| 28 | 20MHz |  | QPSK | 18 | 18 | 13 |
| 29 | 20MHz |  | QPSK | 24 | 24 | 12 |
| 30 | 20MHz |  | 16QAM | 18 | 18 | 13 |
| 31 | 20MHz |  | 16QAM | 24 | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 UE |  |  |  |  |  |  |

Table 6.6.2.1EC.4.1-2: Test Configuration Table, subPRB allocation

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, 15 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 |
| 2 | 5MHz |  | QPSK | ¼ | 0 |
| 3 | 5MHz |  | QPSK | ½ | 0 |
| 4 | 10MHz |  | π/2-BPSK | ¼ | 0 |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 4 |
| 6 | 10MHz |  | QPSK | ¼ | 0 |
| 7 | 10MHz |  | QPSK | ¼ | 4 |
| 8 | 10MHz |  | QPSK | ½ | 0 |
| 9 | 10MHz |  | QPSK | ½ | 4 |
| 10 | 15MHz |  | π/2-BPSK | ¼ | 0 |
| 11 | 15MHz |  | π/2-BPSK | ¼ | 8 |
| 12 | 15MHz |  | QPSK | ¼ | 0 |
| 13 | 15MHz |  | QPSK | ¼ | 8 |
| 14 | 15MHz |  | QPSK | ½ | 0 |
| 15 | 15MHz |  | QPSK | ½ | 8 |
| High range |  |  |  |  |  |
| 16 | 5MHz |  | π/2-BPSK | ¼ | 0 |
| 17 | 5MHz |  | QPSK | ¼ | 0 |
| 18 | 5MHz |  | QPSK | ½ | 0 |
| 19 | 10MHz |  | π/2-BPSK | ¼ | 0 |
| 20 | 10MHz |  | π/2-BPSK | ¼ | 4 |
| 21 | 10MHz |  | QPSK | ¼ | 0 |
| 22 | 10MHz |  | QPSK | ½ | 4 |
| 23 | 10MHz |  | QPSK | ¼ | 0 |
| 24 | 10MHz |  | QPSK | ½ | 4 |
| 25 | 15MHz |  | π/2-BPSK | ¼ | 0 |
| 26 | 15MHz |  | π/2-BPSK | ¼ | 8 |
| 27 | 15MHz |  | QPSK | ¼ | 0 |
| 28 | 15MHz |  | QPSK | ¼ | 8 |
| 29 | 15MHz |  | QPSK | ½ | 0 |
| 30 | 15MHz |  | QPSK | ½ | 8 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.2.1EC.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.2.1EC.4.3.

6.6.2.1EC.4.2	Test procedure

1.	SS sends uplink scheduling information via PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.2.1EC.4.1-1. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.3EC.5-1 to 6.2.3EC.5-3. The period of the measurement shall be at least the continuous duration of one sub-frame (1ms). For TDD slots with transient periods are not under test. For HD-FDD slots with transient periods are not under test. Half-duplex guard sub frame are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to table 6.6.2.1EC.5-1, as applicable. The centre frequency of the filter shall be stepped in continuous steps according to the same table. The measured power shall be recorded for each step. The measurement period shall capture the active TSs.

4.	For UE supporting subPRB allocation, repeat step 1-4 with UL RMC according to Table 6.6.2.1EC.4.1-2.

6.6.2.1EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA.

##### 6.6.2.1EC.5	Test requirements

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.3EC.5-1 to 6.2.3EC.5-3 as appropriate, and the power of any UE emission shall fulfil requirements in Table.6.6.2.1EC.5-1, as applicable.

Table 6.6.2.1EC.5-1: General E-UTRA spectrum emission mask

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -8.5 | -11.5 | -13.5 | -16.5 | -18.5 | -19.5 | 30 kHz |
| 1-2.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | -8.5 | 1 MHz |
| 2.5-2.8 | -23.5 |  |  |  |  |  | 1 MHz |
| 2.8-5 |  |  |  |  |  |  | 1 MHz |
| 5-6 |  | -23.5 | -11.5 | -11.5 | -11.5 | -11.5 | 1 MHz |
| 6-10 |  |  | -23.5 |  |  |  | 1 MHz |
| 10-15 |  |  |  | -23.5 |  |  | 1 MHz |
| 15-20 |  |  |  |  | -23.5 |  | 1 MHz |
| 20-25 |  |  |  |  |  | -23.5 | 1 MHz |
| Note 1: The first and last measurement position with a 30 kHz filter is at ΔfOOB equals to 0.015 MHz and 0.985 MHz. Note 2: At the boundary of spectrum emission limit, the first and last measurement position with a 1 MHz filter is the inside of +0.5MHz and -0.5MHz, respectively. Note 3: The measurements are to be performed above the upper edge of the channel and below the lower edge of the channel Note 4: For the 2.5-2.8 MHz offset range with 1.4 MHz channel bandwidth, the measurement position is at ΔfOOB equals to 3 MHz. |  |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth

<< End of changes>>