3GPP TSG-RAN5 Meeting #90-e	R5-211340

# Online, , 22nd Feb 2021 - 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0205 | rev | 1 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of support for BDS B1C signal |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications, CATT, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | B1C_BDS_pos_UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Additional background information is required to support BDS signal B1C as follows : 1. Addition of B1I and B1C references in References (for consistency – not actually used in the document). 2. Addition of information for GNSS-AuxiliaryInformation in Table 6.1.3.3-1. 3. Addition of KlobucharModel2 in GNSS-IonosphericModel. 4. Addition of information on Model-7 in GNSS-NavigationModel. 5. Addition of GNSS-AcquisitionAssistance for B1C. 6. Addition of information on Model-3 and Model-4 in GNSS-Almanac. 7. Addition of GNSS-AuxiliaryInformation for B1C. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Addition of B1I and B1C references in References. 2. Addition of information for GNSS-AuxiliaryInformation in Table 6.1.3.3-1. 3. Addition of KlobucharModel2 in GNSS-IonosphericModel. 4. Addition of information on Model-7 in GNSS-NavigationModel. 5. Addition of GNSS-AcquisitionAssistance for B1C. 6. Addition of information on Model-3 and Model-4 in GNSS-Almanac. 7. Addition of GNSS-AuxiliaryInformation for B1C. 8. Formatting typos fixed Note that the actual values for many IEs are “FFS” following on-going RAN 5 work to update the GNSS scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Background information to support BDS signal B1C will remain missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.1.3.3, 6.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Similar CR for RF. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | One revision of R5-210053 |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

	For a specific reference, subsequent revisions do not apply.

	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document unless the context in which the reference is made suggests a different Release is relevant (information on the applicable release in a particular context can be found in e.g. test case title, description or applicability, message description or content).

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[3]	Void

[4]	Void

[5]	Void

[6]	3GPP TS 37.571-1: “User Equipment (UE) conformance specification for UE positioning; Part 1: Terminal conformance”.

[7]	3GPP TS 37.571-2: “User Equipment (UE) conformance specification for UE positioning; Part 2: Protocol conformance”.

[8]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[9]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[10]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[11]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[12]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[13]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[14]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[15]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[16]	3GPP TS 25.331: “Radio Resource Control (RRC); Protocol specification”

[17]	STANAG 4294: NATO STANAG 4294. Navstar Global Positioning System (GPS) System Characteristics.

[18]	3GPP TS 36.104: "Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) radio transmission and reception".

[19]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[20]	3GPP TS 36.508: "Common test environments for User Equipment (UE) conformance testing".

[21]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[22]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[23]	Bluetooth Special Interest Group (SIG): “Bluetooth Core Specification v4.2”, December 2014.

[24]	3GPP TS 38.508-1: "User Equipment (UE) conformance specification; Part 1: Common test environment".

[25]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", China Satellite Navigation Office, December 2019.

[26]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December 2017.

## Next changed section

#### 6.1.3.3	Default Assistance Data for TS 37.571-2 subclauses 7 and 9

This subclause defines the GNSS assistance data elements which shall be provided to the UE in certain tests in TS 37.571-2 [7] subclauses 7 and 9 in the LPP Provide Assistance Data messages in the absence of a corresponding LPP Request Assistance Data message. The GNSS assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSS(s) supported by the UE. GNSS assistance data IEs not supported by the UE shall not be sent. GNSS assistance data IEs supported by the UE but not listed below shall not be sent.

Table 6.1.3.3-1: Default GNSS assistance data to be provided to the UE

| GNSS Assistance Data IE supported by the UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted. GNSS-Acquisition Assistance supported by the UE | UE-assisted. GNSS-Acquisition Assistance not supported by the UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes(1) | No | Yes(1) |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes(2) | Yes(2) | Yes(2 |
| GNSS-AuxiliaryInformation | Yes(3) | Yes(3) | Yes(3) |
| Note1: Only if more than one GNSS supported by the UE Note2: Only if GLONASS and at least one other GNSS supported by the UE. Note3: Only if GLONASS supported by the UE, and/or if the UE supports multiple signals per GNSS and/or if BDS B1C supported by the UE. |  |  |  |

#### 6.1.3.4	Assistance Data values for TS 37.571-2 subclauses 7 and 9

Where assistance data is required on a per-satellite basis, or where the values of the data also varies with time it is specified in comma-separated-variable files in the GNSS data zip file specified in Annex B. These files specify the values to be used for each satellite, indexed by satellite SV ID, and, where applicable, the values to be used indexed by both time and satellite SV ID.

Assistance data that is marked as “time varying” and the gnss-TimeOfDay field are only specified and used in 1 second increments. Interpolation between these values shall not be used.

The accuracy of the gnss-TimeOfDay and assistance data that is marked as “time varying” in the provided assistance data shall be within +/- 2 s relative to the GNSS time in the system simulator. In the case that assistance data is required but satellite signals are not required then this clause does not apply.

Assistance data Information Elements and fields that are not specified shall not be used.

The information elements detailed below are fully defined inTS 36.355 [8]

# GNSS REFERENCE TIME:

# GNSS-ReferenceTime: If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 0 (gps) |
| gnss-DayNumber |  | 11683 |
| gnss-TimeOfDay |  | 1860 s. Start time. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | Not present |
| gps-TOW-Assist |  |  |
| satelliteID |  | PRNs: 1, 11, 17, 20, 23, 28. |
| tlmWord |  | 10922 (for all PRNs) |
| antiSpoof |  | 1 (for all PRNs) |
| alert |  | 0 (for all PRNs) |
| tlmRsvdBits |  | 2 (for all PRNs) |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS-ReferenceTime: If GLONASS is the only GNSS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 4 (glonass) |
| gnss-DayNumber |  | 5844 |
| gnss-TimeOfDay |  | 12645 s. Start time. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | 00 |
| gps-TOW-Assist |  | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS-ReferenceTime: If Galileo is the only GNSS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 3 (galileo) |
| gnss-DayNumber |  | 4536 |
| gnss-TimeOfDay |  | 1860 s. Start time. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | Not present |
| gps-TOW-Assist |  | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS-ReferenceTime: If BDS is the only GNSS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-SystemTime |  |  |
| gnss-TimeID |  | 5 (bds) |
| gnss-DayNumber |  | 2191 |
| gnss-TimeOfDay |  | 1847 s. Start time. Add integer number of 1 seconds as required. (Note) |
| gnss-TimeOfDayFrac-msec |  | Not present |
| notificationOfLeapSecond |  | Not present |
| gps-TOW-Assist |  | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Not present |
| Note: gnss-TimeOfDay This is the value of gnss-TimeOfDay when the GNSS scenario is started in the GNSS simulator. The value of gnss-TimeOfDay to be used in the Reference Time IE shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value, rounded up to the next 1 second interval. This “current gnss-TimeOfDay” is then also used to determine the value of any other Information Elements marked as “Time varying” in subclause 6.1.3.4. In the case that the (hardware) GNSS simulator is switched off or not present then the value of gnss-TimeOfDay given above may be used. |  |  |

# GNSS REFERENCE LOCATION:

# GNSS-ReferenceLocation

| Information Element | Units | Value/remark |
| --- | --- | --- |
| threeDlocation |  |  |
| latitudeSign |  | 0 |
| degreesLatitude | degrees | 35.744287 |
| degreesLongitude | degrees | 139.680176 |
| altitudeDirection |  | 0 |
| altitude | m | 300 |
| uncertaintySemiMajor | m | 3000 |
| uncertaintySemiMinor | m | 3000 |
| orientationMajorAxis | degrees | 0 |
| uncertaintyAltitude | m | 500 |
| confidence | % | 68 |

# GNSS IONOSPHERIC MODEL:

# GNSS-IonosphericModel: If GPS or GLONASS or BDS B1I supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-IonosphericModel |  |  |
| klobucharModel |  |  |
| dataID |  | 00 |
| alfa0 | seconds | 4.6566129 10E-9 |
| alfa1 | sec/semi-circle | 1.4901161 10E-8 |
| alfa2 | sec/(semi-circle)2 | -5.96046 10E-8 |
| alfa3 | sec/(semi-circle)3 | -5.96046 10E-8 |
| beta0 | seconds | 79872 |
| beta1 | sec/semi-circle | 65536 |
| beta2 | sec/(semi-circle)2 | -65536 |
| beta3 | sec/(semi-circle)3 | -393216 |
| neQuickModel |  | Not present unless Galileo is also supported by the UE |
| klobucharModel2 |  | Not present unless BDS B1C is also supported by the UE |

# GNSS-IonosphericModel: If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-IonosphericModel |  |  |
| klobucharModel |  | Not present unless GPS or GLONASS or BDS B1I is also supported by the UE |
| neQuickModel |  |  |
| ai0 |  | 64.4 |
| ai1 |  | 0 |
| ai2 |  | 0 |
| ionoStormFlag1 |  | 0 |
| ionoStormFlag2 |  | 0 |
| ionoStormFlag3 |  | 0 |
| ionoStormFlag4 |  | 0 |
| ionoStormFlag5 |  | 0 |
| klobucharModel2 |  | Not present unless BDS B1C is also supported by the UE |

# GNSS-IonosphericModel: If BDS B1C supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-IonosphericModel |  |  |
| klobucharModel |  | Not present unless GPS or GLONASS or BDS B1I is also supported by the UE |
| neQuickModel |  | Not present unless Galileo is also supported by the UE |
| klobucharModel2 |  |  |
| alfa1-r16 |  | FFS |
| alfa2-r16 |  | FFS |
| alfa3-r16 |  | FFS |
| alfa4-r16 |  | FFS |
| alfa5-r16 |  | FFS |
| alfa6-r16 |  | FFS |
| alfa7-r16 |  | FFS |
| alfa8-r16 |  | FFS |
| alfa9-r16 |  | FFS |

# GNSS TIME MODEL:

# GNSS-TimeModelList: If GLONASS and GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-TimeModelRefTime | seconds | 1860 (s) |
| tA0 |  | 0 |
| gnss-TO-ID |  | 1 (GPS) |
| weekNumber |  | 1669 |
| deltaT |  | 15 |

# GNSS-TimeModelList: If Galileo and GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-TimeModelRefTime | Seconds | 1860 (s) |
| tA0 |  | 0 |
| gnss-TO-ID |  | 1 (GPS) |
| weekNumber |  | 1669 |
| deltaT |  | 0 |

# GNSS-TimeModelList: If BDS and GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-TimeModelRefTime | seconds | 1860 (s) |
| tA0 |  | 0 |
| gnss-TO-ID |  | 1 (GPS) |
| weekNumber |  | 1669 |
| deltaT |  | 13 |

# GNSS NAVIGATION MODEL:

# GNSS-NavigationModel (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | PRNs: 1, 11, 17, 20, 23, 28 |
| svHealth |  | 0 |
| iod |  | 0 |
| gnss-ClockModel |  |  |
| nav-ClockModel |  | Model-2 |
| navToc |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navaf2 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navaf1 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navaf0 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navTgd |  | See file: Sig GNSS Navigation Model subtest1.csv |
| gnss-OrbitModel |  |  |
| nav-KeplerianSet |  | Model-2 |
| navURA |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navFitFlag |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navToe |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navOmega |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navDeltaN |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navM0 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navOmegaADot |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navE |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navIDot |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navAPowerHalf |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navI0 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navOmegaA0 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navCrs |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navCis |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navCus |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navCrc |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navCic |  | See file: Sig GNSS Navigation Model subtest1.csv |
| navCuc |  | See file: Sig GNSS Navigation Model subtest1.csv |
| addNAVparam |  |  |
| ephemCodeOnL2 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| ephemL2Pflag |  | See file: Sig GNSS Navigation Model subtest1.csv |
| reserved1 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| reserved2 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| reserved3 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| reserved4 |  | See file: Sig GNSS Navigation Model subtest1.csv |
| ephemAODA |  | See file: Sig GNSS Navigation Model subtest1.csv |

Note: in the case that the UE supports Modernized GPS then the UE may also support the GNSS-NavigationModel (Model-3), however in this case the GNSS-NavigationModel (Model-2) for GPS shall still be used.

# GNSS-NavigationModel (Model-4): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-4): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | Slot Numbers: 3, 4, 9, 10, 18, 20 |
| svHealth |  | 00000000 |
| iod |  | 225 |
| gnss-ClockModel |  |  |
| glonass-ClockModel |  | Model-4 |
| gloTau |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloGamma |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloDeltaTau |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gnss-OrbitModel |  |  |
| glonass-ECEF |  | Model-4 |
| gloEn |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloP1 |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloP2 |  | See file: Sig GNSS Navigation Model subtest2.csv |
| glom |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloX |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloXdot |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloXdotdot |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloY |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloYdot |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloYdotdot |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloZ |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloZdot |  | See file: Sig GNSS Navigation Model subtest2.csv |
| gloZdotdot |  | See file: Sig GNSS Navigation Model subtest2.csv |

# GNSS-NavigationModel (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | SV IDs: 5, 10, 11, 18, 19, 20 |
| svHealth |  | 0 |
| iod |  | 0 |
| gnss-ClockModel |  |  |
| standardClockModelList |  | (SIZE) 1 if the UE supports only Galileo E1, (SIZE) 2 if the UE supports multiple Galileo signals. Model-1 |
| .StandardClockModelElement |  | I/NAV |
| stanClockToc |  | See file: Sig GNSS Navigation Model subtest3.csv |
| stanClockAF2 |  | See file: Sig GNSS Navigation Model subtest3.csv |
| stanClockAF1 |  | See file: Sig GNSS Navigation Model subtest3.csv |
| stanClockAF0 |  | See file: Sig GNSS Navigation Model subtest3.csv |
| stanClockTgd |  | See file: Sig GNSS Navigation Model subtest3.csv. Not present if the UE supports multiple Galileo signals. |
| sisa |  | See file: Sig GNSS Navigation Model subtest3.csv |
| stanModelID |  | 0 (I/NAV). Present only if the UE supports multiple Galileo signals. |
| .StandardClockModelElement |  | F/NAV. Present only if the UE supports multiple Galileo signals. |
| stanClockToc |  | See file: Sig GNSS Navigation Model Gal FNAV.csv |
| stanClockAF2 |  | See file: Sig GNSS Navigation Model Gal FNAV.csv |
| stanClockAF1 |  | See file: Sig GNSS Navigation Model Gal FNAV.csv |
| stanClockAF0 |  | See file: Sig GNSS Navigation Model Gal FNAV.csv |
| stanClockTgd |  | See file: Sig GNSS Navigation Model Gal FNAV.csv |
| sisa |  | See file: Sig GNSS Navigation Model Gal FNAV.csv |
| stanModelID |  | 1 (F/NAV). |
| gnss-OrbitModel |  |  |
| keplerianSet |  | Model-1 |
| keplerToe |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerW |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerDeltaN |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerM0 |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerOmegaDot |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerE |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerIDot |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerAPowerHalf |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerI0 |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerOmega0 |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerCrs |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerCis |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerCus |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerCrc |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerCic |  | See file: Sig GNSS Navigation Model subtest3.csv |
| keplerCuc |  | See file: Sig GNSS Navigation Model subtest3.csv |

# GNSS-NavigationModel (Model-6): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| nonBroadcastFlag |  | 0 |
| gnss-SatelliteList |  | (SIZE) 6 |

# GNSS-NavModelSatelliteElement (Model-6): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | SV IDs: 1, 2, 7, 18, 21, 27 |
| svHealth |  | 0 |
| iod |  | 0 |
| gnss-ClockModel |  |  |
| BDS-ClockModel-r12 |  | (SIZE) 1. Model-6 |
| bdsAODC-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsToc-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsA0-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsA1-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsA2-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsTgd1-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| gnss-OrbitModel |  |  |
| BDS-KeplerianSet-r12 |  | Model-6 |
| bdsAODE-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsURAI-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsToe-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsAPowerHalf-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsE-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsW-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsDeltaN-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsM0-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsOmega0-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsOmegaDot-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsI0-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsIDot-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsCuc-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsCus-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsCrc-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsCrs-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsCic-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |
| bdsCis-r12 |  | See file: Sig GNSS Navigation Model subtest9.csv |

Note: in the case that the UE supports BDS B1C then the UE may also support the GNSS-NavigationModel (Model-7), however in this case the GNSS-NavigationModel (Model-6) for BDS shall still be used.

# GNSS ACQUISITION ASSISTANCE:

In the case that the UE only supports GPS L1 C/A for GPS then the GNSS-AcquisitionAssistance (GPS L1 C/A) shall be used for GPS.

In the case that the UE supports Modernized GPS then the GNSS-AcquisitionAssistance to be used for GPS depends on the GNSS-AcquisitionAssistance(s) supported by the UE for GPS. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (GPS L1 C/A)

GNSS-AcquisitionAssistance (Modernized GPS L5)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_GPS_L1CA and pc_GNSS_AcquAssist_GPS_L5, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (GPS L1 C/A) shall be used.

# GNSS-AcquisitionAssistance (GPS L1 C/A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (GPS L1 C/A) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | PRNs: 1, 11, 17, 20, 23, 28. |
| doppler0 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| doppler1 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| dopplerUncertainty |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| codePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| intCodePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| codePhaseSearchWindow |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| azimuth |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| elevation |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| codePhase1023 |  | Time varying. Calculated from “codePhase”, see file: Sig GNSS Acquisition Assistance subtest1.csv (Note) |
| dopplerUncertaintyExt-r10 |  | Not present |
| Note: This field is “Time varying” and its value depends on the “current gnss-TimeOfDay”. The value of this field to be used shall be determined by taking the “current gnss-TimeOfDay” value and selecting the field value in the Sig GNSS Acquisition Assistance subtest1.csv file corresponding to the value of “current gnss-TimeOfDay”. |  |  |

# GNSS-AcquisitionAssistance (Modernized GPS L5)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 3 (GPS L5) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Modernized GPS L5)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | PRNs: 1, 11, 17, 20, 23, 28. |
| doppler0 |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| doppler1 |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| dopplerUncertainty |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| codePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| intCodePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| codePhaseSearchWindow |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| azimuth |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| elevation |  | Time varying. See file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| codePhase1023 |  | Time varying. Calculated from “codePhase”, see file: Sig GNSS Acquisition Assistance modGPS L5.csv (Note) |
| dopplerUncertaintyExt-r10 |  | Not present |
| Note: This field is “Time varying” and its value depends on the “current gnss-TimeOfDay”. The value of this field to be used shall be determined by taking the “current gnss-TimeOfDay” value and selecting the field value in the Sig GNSS Acquisition Assistance modGPS L5.csv file corresponding to the value of “current gnss-TimeOfDay”. |  |  |

# GNSS-AcquisitionAssistance: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (GLONASS G1) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | Slot Numbers: 3, 4, 9, 10, 18, 20 |
| doppler0 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| doppler1 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| dopplerUncertainty |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| codePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| intCodePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| codePhaseSearchWindow |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| azimuth |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| elevation |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| codePhase1023 |  | Time varying. Calculated from “codePhase”, see file: Sig GNSS Acquisition Assistance subtest2.csv (Note) |
| dopplerUncertaintyExt-r10 |  | Not present |
| Note: This field is “Time varying” and its value depends on the “current gnss-TimeOfDay”. The value of this field to be used shall be determined by taking the “current gnss-TimeOfDay” value and selecting the field value in the Sig GNSS Acquisition Assistance subtest2.csv file corresponding to the value of “current gnss-TimeOfDay”. |  |  |

In the case that the UE only supports Galileo E1 for Galileo then the GNSS-AcquisitionAssistance (Galileo E1) shall be used for Galileo.

In the case that the UE supports more than one Galileo signal then the GNSS-AcquisitionAssistance to be used for Galileo depends on the GNSS-AcquisitionAssistance(s) supported by the UE for Galileo. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (Galileo E1)

GNSS-AcquisitionAssistance (Galileo E5A)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_Galileo_E1 and pc_GNSS_AcquAssist_Galileo_E5A, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (Galileo E1) shall be used.

# GNSS-AcquisitionAssistance (Galileo E1)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (Galileo E1) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Galileo E1)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | SV IDs: 5, 10, 11, 18, 19, 20 |
| doppler0 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| doppler1 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| dopplerUncertainty |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| codePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| intCodePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| codePhaseSearchWindow |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| azimuth |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| elevation |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| codePhase1023 |  | Time varying. Calculated from “codePhase”, see file: Sig GNSS Acquisition Assistance subtest3.csv (Note) |
| dopplerUncertaintyExt-r10 |  | Not present |
| Note: This field is “Time varying” and its value depends on the “current gnss-TimeOfDay”. The value of this field to be used shall be determined by taking the “current gnss-TimeOfDay” value and selecting the field value in the Sig GNSS Acquisition Assistance subtest3.csv file corresponding to the value of “current gnss-TimeOfDay”. |  |  |

# GNSS-AcquisitionAssistance (Galileo E5A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 1 (Galileo E5A) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (Galileo E5A)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | SV IDs: 5, 10, 11, 18, 19, 20 |
| doppler0 |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| doppler1 |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| dopplerUncertainty |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| codePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| intCodePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| codePhaseSearchWindow |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| azimuth |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| elevation |  | Time varying. See file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| codePhase1023 |  | Time varying. Calculated from “codePhase”, see file: Sig GNSS Acquisition Assistance Gal E5A.csv (Note) |
| dopplerUncertaintyExt-r10 |  | Not present |
| Note: This field is “Time varying” and its value depends on the “current gnss-TimeOfDay”. The value of this field to be used shall be determined by taking the “current gnss-TimeOfDay” value and selecting the field value in the Sig GNSS Acquisition Assistance Gal E5A.csv file corresponding to the value of “current gnss-TimeOfDay”. |  |  |

In the case that the UE only supports BDS B1I for BDS then the GNSS-AcquisitionAssistance (BDS B1I) shall be used for BDS.

In the case that the UE supports more than one BDS signal then the GNSS-AcquisitionAssistance to be used for BDS depends on the GNSS-AcquisitionAssistance(s) supported by the UE for BDS. The possible GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (BDS B1I)

GNSS-AcquisitionAssistance (BDS B1C)

The GNSS-AcquisitionAssistance to be used shall be determined by the PICs pc_GNSS_AcquAssist_BDS_B1I and pc_GNSS_AcquAssist_BDS_B1C, in the case that both GNSS-AcquisitionAssistances are supported by the UE then the GNSS-AcquisitionAssistance (BDS B1I) shall be used.

# GNSS-AcquisitionAssistance (BDS B1I)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | 0 (BDS B1I) |
| gnss-AcquisitionAssistList |  | (SIZE) 6 |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B1I)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | SV IDs: 1, 2, 7, 18, 21, 27 |
| doppler0 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| doppler1 |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| dopplerUncertainty |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| codePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| intCodePhase |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| codePhaseSearchWindow |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| azimuth |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| elevation |  | Time varying. See file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| codePhase1023 |  | Time varying. Calculated from “codePhase”, see file: Sig GNSS Acquisition Assistance subtest9.csv (Note) |
| dopplerUncertaintyExt-r10 |  | Not present |
| Note: This field is “Time varying” and its value depends on the “current gnss-TimeOfDay”. The value of this field to be used shall be determined by taking the “current gnss-TimeOfDay” value and selecting the field value in the Sig GNSS Acquisition Assistance subtest9.csv file corresponding to the value of “current gnss-TimeOfDay”. |  |  |

# GNSS-AcquisitionAssistance (BDS B1C)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |
| gnss-SignalID |  | FFS |
| gnss-AcquisitionAssistList |  | FFS |
| confidence-r10 | % | 98 |

# GNSS-AcquisitionAssistElement (BDS B1C)

| Information Element | Units | Value/remark |
| --- | --- | --- |
| svID |  | FFS |
| doppler0 |  | FFS |
| doppler1 |  | FFS |
| dopplerUncertainty |  | FFS |
| codePhase |  | FFS |
| intCodePhase |  | FFS |
| codePhaseSearchWindow |  | FFS |
| azimuth |  | FFS |
| elevation |  | FFS |
| codePhase1023 |  | FFS |
| dopplerUncertaintyExt-r10 |  | FFS |

# GNSS ALMANAC:

# GNSS-Almanac (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | 1669 |
| toa |  | 249856 |
| ioda |  | Not present |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 31 |

# GNSS-AlmanacElement (Model-2): If GPS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| keplerianNAV-Almanac |  | Model-2 |
| svID |  | PRNs: 1, 2, 3 …21, 22, 23, 25, 26, 27... 30, 31, 32 |
| navAlmE |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmDeltaI |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmOMEGADOT |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmSVHealth |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmSqrtA |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmOMEGAo |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmOmega |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmMo |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmaf0 |  | See file: Sig GNSS Almanac subtest1.csv |
| navAlmaf1 |  | See file: Sig GNSS Almanac subtest1.csv |

Note: in the case that the UE supports Modernized GPS then the UE may also support the GNSS-Almanac (Model-3) and/or GNSS-Almanac (Model-4), however in this case the GNSS-Almanac (Model-2) for GPS shall still be used.

# GNSS-Almanac (Model-5): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 24 |

# GNSS-AlmanacElement (Model-5): If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| keplerianGLONASS |  | Model-5 |
| gloAlm-NA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmnA |  | Slot Numbers: 1, 2, 3 …….22, 23, 24 |
| gloAlmHA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmLambdaA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmtlambdaA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmDeltaIa |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmDeltaTA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmDeltaTdotA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmEpsilonA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmOmegaA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmTauA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmCA |  | See file: Sig GNSS Almanac subtest2.csv |
| gloAlmMA |  | See file: Sig GNSS Almanac subtest2.csv |

# GNSS-Almanac (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | 645 |
| toa |  | 147000 |
| ioda |  | 0 |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 29 |

# GNSS-AlmanacElement (Model-1): If Galileo supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| keplerianAlmanacSet |  | Model-1 |
| svID |  | SV IDs: 1, 2, 3 …21, 22, 23, 25, 26, 27... 30 |
| kepAlmanacE |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacDeltaI |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacOmegaDot |  | See file: Sig GNSS Almanac subtest3.csv |
| kepSV-StatusINAV |  | See file: Sig GNSS Almanac subtest3.csv |
| kepSV-StatusFNAV |  | 0. Present only if the UE supports multiple Galileo signals. |
| kepAlmanacAPowerHalf |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacOmega0 |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacW |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacM0 |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacAF0 |  | See file: Sig GNSS Almanac subtest3.csv |
| kepAlmanacAF1 |  | See file: Sig GNSS Almanac subtest3.csv |

# GNSS-Almanac (Model-7): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-Almanac |  |  |
| weekNumber |  | 313 |
| toa |  | 147456 |
| ioda |  | 0 |
| completeAlmanacProvided |  | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 30 |

# GNSS-AlmanacElement (Model-7): If BDS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| BDS-AlmanacSet-r12 |  | Model-7 |
| svID |  | SV IDs: 1, 2, 3, …, 29, 30 |
| bdsAlmToa-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmSqrtA-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmE-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmW-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmM0-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmOmega0-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmOmegaDot-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmDeltaI-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmA0-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsAlmA1-r12 |  | See file: Sig GNSS Almanac subtest9.csv |
| bdsSvHealth-r12 |  | See file: Sig GNSS Almanac subtest9.csv |

Note: in the case that the UE supports BDS B1C then the UE may also support the GNSS-Almanac (Model-3) and/or GNSS-Almanac (Model-4), however in this case the GNSS-Almanac (Model-7) for BDS shall still be used.

# GNSS UTC MODEL:

# GNSS-UTC-Model: If both GPS and GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-UTC-Model |  |  |
| utcModel1 |  |  |

# UTC-ModelSet1: If both GPS and GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| gnss-Utc-A1 |  | 0 |
| gnss-Utc-A0 |  | 0 |
| gnss-Utc-Tot |  | 249856 |
| gnss-Utc-WNt |  | 133 |
| gnss-Utc-DeltaTls |  | 15 |
| gnss-Utc-WNlsf |  | 158 |
| gnss-Utc-DN |  | 7 |
| gnss-Utc-DeltaTlsf |  | 16 |

# GNSS AUXILIARY INFORMATION:

# GNSS-AuxiliaryInformation: If multiple GPS signals supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-GPS |  | (SIZE) 6 |
| svID |  | PRNs: 1, 11, 17, 20, 23, 28. |
| signalsAvailable |  | As supported by the UE |

# GNSS-AuxiliaryInformation: If GLONASS supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-GLONASS |  | (SIZE) 6 |
| svID |  | Slot Numbers: 3, 4, 9, 10, 18, 20 |
| signalsAvailable |  | G1 |
| channelNumber |  | 5, 6, -2, -7, -3, 2 |

# GNSS-AuxiliaryInformation: If BDS B1C supported by the UE

| Information Element | Units | Value/remark |
| --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |
| gnss-ID-BDS-r16 |  | FFS |
| svID-r16 |  | FFS |
| satType-r16 |  | FFS |