3GPP TSG-RAN5 Meeting #90-e	R5-211838

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1168 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating AMPR testing for SUL band n83 and n84 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The AMPR requirements for SUL band n83 and n84 are updated in R17 core spec. So the AMPR testing for band n83 and n84 need to be updated correspondingly in TS 38.521-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating AMPR testing for band n83 and n84 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AMPR testing for n83 and n84 are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2C.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Table 6.2.3.4.1-29 for NS_49 in 6.2.3 is introduced in R5-210741 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-210931r2. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_10, NS_07, NS_40, NS_09 and NS_49 not complete.

- Seeking some clarification from RAN4 on how ΔTIB,c in TS 38.101-1 [2] subclause 6.2A.4.2/6.2C.2 and TS 38.101-3 [4] subclause 6.2A.4.2/6.2B.4.2 shall be applied for applicable bands.

<Unchanged Text Skipped>

6.2.3.3	Minimum conformance requirements

6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_24 | 6.5.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_50 | 6.5.3.3.3.16 | n39 | 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_47 | 6.5.3.3.3.17 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.18 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-1 |
| NS_49 | 6.5.3.3.3.19 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-1 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 |  |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 |  |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 |  |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

6.2.3.3.13	A-MPR for NS_18

Table 6.2.3.3.13-0: Band n28 and n83 30MHz A-MPR regions for NS_18

| Channel Bandwidth, MHz | Frequency range of UL transmission bandwidth configuration, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |  |
| 30 | 703~733 | >(LCRB*12*SCS)/2+5.22 | ≥Max(0, 12*SCS*NRB – 1.8 – RBstart*12*SCS) | A3 |
|  |  | ≤(LCRB*12*SCS)/2+5.22 | ≥5.4 | A4 |  |  |

<End of modified section 1>

<Start of modified section 2>

6.2C.5	UE additional maximum output power reduction for SUL

Editor’s notes:

- Tests for network signalling values NS_05, NS_17, NS_43 and NS_100 not complete.

6.2C.5.1	Test purpose

Same test purpose as in clause 6.2.3.1

6.2C.5.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.2C.5.3	Minimum conformance requirements

For a terminal that supports SUL for the band combination specified in Table 5.2C-1, the current version of the specification assumes the terminal is configured with active transmission either on UL carrier or SUL carrier at any time in one serving cell and the UE requirements for single carrier shall apply for the active UL or SUL carrier accordingly.

The normative reference for this requirement is TS 38.101-1 [2] clauses 4.3and 6.2.3.

6.2C.5.4	Test description

Same test description as specified in clause 6.2.3.4 with following exceptions:

Instead of table 5.3.5-1  use Table 5.5C-1.

For NS_03: Instead of table 6.2.3.4.1-3  use Table 6.2C.5.4-1

For NS_18: Instead of table 6.2.3.4.1-11  use Table 6.2C.5.4-2

For NS_48: Instead of table 6.2.3.4.1-19  use Table 6.2C.5.4-3

For NS_49: Instead of table 6.2.3.4.1-29  use Table 6.2C.5.4-4

Table 6.2C.5.4-1: Test Configuration Table for NS_03

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range , High range for SUL carrier Mid range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest for SUL carrier Lowest for Non-SUL carrierr |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for both SUL carrier and Non-SUL carrier |  |
| A-MPR test parameters for NS_03 |  |  |  |  |  |  |  |
| Test ID | Freq | Downlink Configuration | UL Configuration | SUL Configuration |  |  |  |
|  |  |  |  | Modulation |  |  | RB allocation (Note 2) |
| 1 | Low | N/A | N/A | DFT-s OFDM | PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | PI/2 BPSK |  | Outer_Full |
| 4 | Low |  |  |  | QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | QPSK |  | Outer_Full |
| 7 | Low |  |  |  | 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | 16 QAM |  | Outer_Full |
| 10 | Low |  |  |  | 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | 64 QAM |  | Outer_Full |
| 13 | Low |  |  |  | 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | 256 QAM |  | Outer_Full |
| 16 | Low |  |  | CP-s OFDM | QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | QPSK |  | Outer_Full |
| 19 | Low |  |  |  | 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | 16 QAM |  | Outer_Full |
| 22 | Low |  |  |  | 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | 64 QAM |  | Outer_Full |
| 25 | Low |  |  |  | 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | 256 QAM |  | Outer_Full |
| NOTE 1: Test Channel Bandwidths are checked separately for each SUL band combination, the applicable channel bandwidths are specified in Table 5.5C-1. NOTE 2: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 3: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |  |  |  |

Table 6.2C.5.4-2: Test Configuration Table for NS_18

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz 10MHz 20MHz 30MHz for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15 kHz for all carriers |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |
| Test ID | ChBw SCS | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | 5MHz, 10MHz, 20MHz | N/A | DFT-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  | CP-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 | 30MHz |  | DFT-s OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  |  | 256 QAM | Outer_Full | A4 |
| 19 |  |  | CP-s OFDM | 256 QAM | 87@73 | A3 |
| 20 |  |  |  | 256 QAM | Outer_Full | A4 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |

Table 6.2C.5.4-3: Test Configuration Table for NS_48

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for all carriers |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A for A-MPR testing. | N/A | DFT-s-OFDM |  | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 4 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 5 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 6 | Default | 40 |  |  |  |  | QPSK | 90@0 (A3) |
| 7 | Default | 40 |  |  |  |  | QPSK | 150@0 (A4) |
| 8 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 9 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 10 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 11 | Default | 50 |  |  |  |  | QPSK | 32@0 (A2) |
| 12 | Default | 50 |  |  |  |  | QPSK | 108@0 (A4) |
| 13 | Default | 50 |  |  |  |  | QPSK | 225@0 (A2) |
| 14 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 15 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 16 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 17 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 18 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 19 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 20 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 21 | Default | 40 |  |  |  |  | 256 QAM | 90@0 (A3) |
| 22 | Default | 40 |  |  |  |  | 256 QAM | 150@0 (A4) |
| 23 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 24 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 25 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 26 | Default | 50 |  |  |  |  | 256 QAM | 32@0 (A2) |
| 27 | Default | 50 |  |  |  |  | 256 QAM | 108@0 (A4) |
| 28 | Default | 50 |  |  |  |  | 256 QAM | 225@0 (A2) |
| 29 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 30 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 31 | Default | 25 |  |  | CP-OFDM |  | QPSK | Outer_Full (A3) |
| 32 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 33 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 34 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 35 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 36 | Default | 40 |  |  |  |  | QPSK | 95@0 (A3) |
| 37 | Default | 40 |  |  |  |  | QPSK | 152@0 (A4) |
| 38 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 39 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 40 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 41 | Default | 50 |  |  |  |  | QPSK | 34@0 (A2) |
| 42 | Default | 50 |  |  |  |  | QPSK | 115@0 (A4) |
| 43 | Default | 50 |  |  |  |  | QPSK | 228@0 (A2) |
| 44 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 45 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 46 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 47 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 48 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 49 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 50 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 51 | Default | 40 |  |  |  |  | 256 QAM | 95@0 (A3) |
| 52 | Default | 40 |  |  |  |  | 256 QAM | 152@0 (A4) |
| 53 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 54 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 55 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 56 | Default | 50 |  |  |  |  | 256 QAM | 34@0 (A2) |
| 57 | Default | 50 |  |  |  |  | 256 QAM | 115@0 (A4) |
| 58 | Default | 50 |  |  |  |  | 256 QAM | 228@0 (A2) |
| 59 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 60 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.2C.5.4-4: Test Configuration Table for NS_49

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15 kHz for all carriers |  |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A for A-MPR testing. | N/A | DFT-s-OFDM | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 4 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 5 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 6 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 7 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 8 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 9 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 10 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 11 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 12 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 13 | Default | 40 |  |  |  | QPSK | 150@33 (A2) |
| 14 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 15 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 16 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 17 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 18 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 19 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 20 | Default | 50 |  |  |  | QPSK | 216@0 (A1) |
| 21 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 22 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 23 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 24 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 25 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 26 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 27 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 28 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 29 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 30 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 31 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 32 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 33 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 34 | Default | 40 |  |  |  | 256 QAM | 150@33 (A2) |
| 35 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 36 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 37 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 38 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 39 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 40 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 41 | Default | 50 |  |  |  | 256 QAM | 216@0 (A1) |
| 42 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| 43 | Default | 25 |  |  | CP-OFDM | QPSK | Outer_Full (A3) |
| 44 | Default | 25 |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 45 | Default | 25 |  |  |  | QPSK | Edge_1RB_Left (A3) |
| 46 | Default | 30 |  |  |  | QPSK | 20@0 (A1) |
| 47 | Default | 30 |  |  |  | QPSK | 36@0 (A5) |
| 48 | Default | 30 |  |  |  | QPSK | 80@0 (A3) |
| 49 | Default | 30 |  |  |  | QPSK | 120@0 (A4) |
| 50 | Default | 30 |  |  |  | QPSK | Outer_Full (A2) |
| 51 | Default | 30 |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 52 | Default | 40 |  |  |  | QPSK | 40@0 (A1) |
| 53 | Default | 40 |  |  |  | QPSK | 5@53 (A5) |
| 54 | Default | 40 |  |  |  | QPSK | 100@0 (A4) |
| 55 | Default | 40 |  |  |  | QPSK | 159@33 (A2) |
| 56 | Default | 40 |  |  |  | QPSK | 5@187 (A5) |
| 57 | Default | 40 |  |  |  | QPSK | 192@0 (A1) |
| 58 | Default | 40 |  |  |  | QPSK | Outer_Full (A1) |
| 59 | Default | 50 |  |  |  | QPSK | 5@75 (A5) |
| 60 | Default | 50 |  |  |  | QPSK | 5@215 (A5) |
| 61 | Default | 50 |  |  |  | QPSK | 175@45 (A2) |
| 62 | Default | 50 |  |  |  | QPSK | 220@0 (A1) |
| 63 | Default | 50 |  |  |  | QPSK | Outer_Full (A1) |
| 64 | Default | 25 |  |  |  | 256 QAM | Outer_Full (A3) |
| 65 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 66 | Default | 25 |  |  |  | 256 QAM | Edge_1RB_Left (A3) |
| 67 | Default | 30 |  |  |  | 256 QAM | 20@0 (A1) |
| 68 | Default | 30 |  |  |  | 256 QAM | 36@0 (A5) |
| 69 | Default | 30 |  |  |  | 256 QAM | 80@0 (A3) |
| 70 | Default | 30 |  |  |  | 256 QAM | 120@0 (A4) |
| 71 | Default | 30 |  |  |  | 256 QAM | Outer_Full (A2) |
| 72 | Default | 30 |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 73 | Default | 40 |  |  |  | 256 QAM | 40@0 (A1) |
| 74 | Default | 40 |  |  |  | 256 QAM | 5@53 (A5) |
| 75 | Default | 40 |  |  |  | 256 QAM | 100@0 (A4) |
| 76 | Default | 40 |  |  |  | 256 QAM | 159@33 (A2) |
| 77 | Default | 40 |  |  |  | 256 QAM | 5@187 (A5) |
| 78 | Default | 40 |  |  |  | 256 QAM | 192@0 (A1) |
| 79 | Default | 40 |  |  |  | 256 QAM | Outer_Full (A1) |
| 80 | Default | 50 |  |  |  | 256 QAM | 5@75 (A5) |
| 81 | Default | 50 |  |  |  | 256 QAM | 5@215 (A5) |
| 82 | Default | 50 |  |  |  | 256 QAM | 175@45 (A2) |
| 83 | Default | 50 |  |  |  | 256 QAM | 220@0 (A1) |
| 84 | Default | 50 |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

Editor’s note: The following lines belong at the end of subclause 6.2.3.4.1. As new tables are added to this section, these lines should always follow the tables

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3.

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

Message contents in initial conditions are according to TS 38.508-1 [5] subclause 4.3.6.1.1.2 ensuring UL/SUL indicator in Table 4.3.6.1.1.2-1 with condition SUL, subclause 4.6 ensuring Table 4.6.1-28 with condition SUL AND RF, Tables 4.6.3-14, 4.6.3-15 and 4.6.3-19 with conditions SUL, and Table 4.6.3-167 with condition PUSCH_PUCCH_ON_SUL. All the AdditionalSpectrumEmission in 6.2.3.4.3 are sent in SIB1 as part of supplementaryUplink instead of uplinkConfigCommon.

Table 6.2C.5.4-1: Void

6.2C.5.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2C.5.5-1 to table 6.2C.5.5-4. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2C.4. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2C.5.5-1: UE Power Class 3 test requirements (NS_03) for band n86

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
| 4, 5 | NS_03 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7 | NS_03 | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03 | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03 | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03 | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 12 | NS_03 | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |
| 13, 14 | NS_03 | 23 | 4.5 | 4.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 15 | NS_03 | 23 | 4.5 | 4.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 16, 17 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 18 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 19, 20 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 21 | NS_03 | 23 | 3 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 22, 23 | NS_03 | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | NS_03 | 23 | 3.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25, 26 | NS_03 | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| 27 | NS_03 | 23 | 6.5 | 6.5 | 0 | 15.5 | 5 | 2 | 25+TT | 10.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-2: UE Power Class 3 test requirements (NS_18) for band n83

| Test ID | ChBw (MHz) | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | 5 | 23 | 1 | 2 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
|  | 10, 15, 20 | 23 | 1 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 3, 4 | 5 | 23 | 2 | 3 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
|  | 10, 15, 20 | 23 | 2 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
| 5, 6 | 5 | 23 | 2.5 | 4 | 0 | 19 | 3.5 | 2.5 | 25+TT | 15.5-TT |
|  | 10, 15, 20 | 23 | 2.5 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 7, 8 | 5 | 23 | 4.5 | 6 | 0 | 17 | 5 | 2.5 | 25+TT | 12-TT |
|  | 10, 15, 20 | 23 | 4.5 | 9 | 0 | 14 | 5 | 2.5 | 25+TT | 9-TT |
| 9, 10 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 6.5 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 11, 12 | 5 | 23 | 3 | 5 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
|  | 10, 15, 20 | 23 | 3 | 7 | 0 | 16 | 5 | 2.5 | 25+TT | 11-TT |
| 13, 14 | 5 | 23 | 3.5 | 5.5 | 0 | 17.5 | 5 | 2.5 | 25+TT | 12.5-TT |
|  | 10, 15, 20 | 23 | 3.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
| 15, 16 | 5 | 23 | 6.5 | 8.5 | 0 | 14.5 | 5 | 2.5 | 25+TT | 9.5-TT |
|  | 10, 15, 20 | 23 | 6.5 | 11.5 | 0 | 11.5 | 6 | 2.5 | 25+TT | 5.5-TT |
| 17 | 30 | 23 | 5 | 3 | 0 | 18 | 4 | 2.5 | 25+TT | 14-TT |
| 18 | 30 | 23 | 5 | 8 | 0 | 15 | 5 | 2.5 | 25+TT | 10–TT |
| 19 | 30 | 23 | 7 | 4.5 | 0 | 16 | 5 | 2.5 | 25+TT | 11–TT |
| 20 | 30 | 23 | 7 | 9.5 | 0 | 13.5 | 5 | 2.5 | 25+TT | 8.5–TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-3: UE Power Class 3 test requirements (NS_48) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 5 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 0 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 17 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 18 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 19 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 20 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 21 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 22 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 27 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 29 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 30 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 31 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 32 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 33 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 35 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 36 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 37 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 38 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 39 | 23 | 0 | 1.5 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 40 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 41 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 42 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 43 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 44 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 45 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 46 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 47 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 48 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 49 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 50 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 52 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 53 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 54 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 55 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 56 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 57 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 58 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 59 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 60 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

Table 6.2C.5.5-4: UE Power Class 3 test requirements (NS_49) for band n84

| Test ID | PPowerClass (dBm) | PPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 2 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 3 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 4 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 5 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 6 | 23 | 0 | 1 | 3 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 7 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 8 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 9 | 23 | 0 | 1 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 10 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 11 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 12 | 23 | 0 | 1 | 4 | 0 | 19 | 3.5 | 2 | 25+TT | 15.5-TT |
| 13 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 14 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 15 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 16 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 17 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 18 | 23 | 0 | 0 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 19 | 23 | 0 | 1 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 20 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 21 | 23 | 0 | 1 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 22 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 23 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 24 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 25 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 26 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 27 | 23 | 0 | 4.5 | 3 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 28 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 29 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 30 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 31 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 32 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 33 | 23 | 0 | 4.5 | 4 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 34 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 35 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 36 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 37 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 38 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 39 | 23 | 0 | 4.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 40 | 23 | 0 | 4.5 | 6 | 0 | 17 | 5 | 2 | 25+TT | 12-TT |
| 41 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 42 | 23 | 0 | 4.5 | 10 | 0 | 13 | 5 | 2 | 25+TT | 8-TT |
| 43 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 44 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 45 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 46 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 47 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 48 | 23 | 0 | 3 | 4.5 | 0 | 18.5 | 4 | 2 | 25+TT | 14.5-TT |
| 49 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 50 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 51 | 23 | 0 | 3 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 52 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 53 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 54 | 23 | 0 | 3 | 5.5 | 0 | 17.5 | 5 | 2 | 25+TT | 12.5-TT |
| 55 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 56 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 57 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 58 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 59 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 60 | 23 | 0 | 1.5 | 5 | 0 | 18 | 4 | 2 | 25+TT | 14-TT |
| 61 | 23 | 0 | 3 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 62 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 63 | 23 | 0 | 3 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 64 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 65 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 66 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 67 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 68 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 69 | 23 | 0 | 6.5 | 4.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 70 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 71 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 72 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 73 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 74 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 75 | 23 | 0 | 6.5 | 5.5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 76 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 77 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 78 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 79 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 80 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 81 | 23 | 0 | 6.5 | 5 | 0 | 16.5 | 5 | 2 | 25+TT | 11.5-TT |
| 82 | 23 | 0 | 6.5 | 7 | 0 | 16 | 5 | 2 | 25+TT | 11-TT |
| 83 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| 84 | 23 | 0 | 6.5 | 11 | 0 | 12 | 6 | 2 | 25+TT | 6-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |  |

<End of modified section 2>